3GPP TSG-RAN WG2 Meeting #129bis					           R2-2502464

Wuhan, China, 7 – 11 April, 2025

Agenda Item:		8.3.3

Source:			Huawei, HiSilicon

Title:			Further simulation results for measurement event prediction in FR2

Document for:		Discussion and Decision

# 1   Introduction

In the previous meetings RAN2 studied the prediction accuracy of L3 RSRP measurements using AIML models in different scenarios. The highest priority scenarios are mentioned below.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

As a next step, RAN2 decided to verify how AIML models can be utilized to predict mobility events, taking A3 event as an example. RAN2 also decided to prioritize so called “indirect method” where the AIML model is used to predict signal level (e.g. RSRP/SINR) while the occurrence of the measurement event is determined based on such predicted signal level, without further involvement of the AIML model. This will allow to verify how the inaccuracies related with the measurement predictions translate into accuracy of mobility event predictions and mobility performance metrics such as handover failure rate and number of handovers.

We have provided a first set of measurement results for measurement event prediction and its impact on system level mobility KPIs in [1]. During RAN2#129 meeting, RAN2 discussed the results brought by various companies and following conclusions were drawn:

For measurement event prediction case A in FR2: - Most of the results show that the F1 score for prediction of measurement event is very good. - For handover decision option 3 and 2 AI/ML performs better than baseline (legacy) in terms of HO failure rate. For measurement event prediction case B in FR1: - Measurement event case B can have very good F1 score depending on filtering approach/PW - F1 score is good for the PW window for case B (i.e. low PW) - F1 score decreases with an increasing MRRT value - We will focus on finding cases that bring benefits to the system rather than trying to compare - With indirect measurement event prediction based on temporal domain Case B (MRRT=50%), the AI-based HO has a minor/no system-level performance (i.e., HOF rate and HO number) decrease compared with the legacy HO mechanism.

However, it was noticed that companies may have applied different methods for F1 score calculation leading to different companies obtaining distinct results. Hence, further discussion took place on the alignment of this method which resulted in the following agreements being made, based on the summary in [2]:

Agreements - For evaluation purpose, it is assumed PW length = TTT length =320ms - For evaluation purpose, Option 2 is agreed i.e. each counter can be updated only one time for related predicted and/or ground truth measurement event. - To introduce “Total number of HOF per UE per second” as SLS metric

In addition, in the informal offline discussion between companies involved in this work, it was proposed by the SI rapporteur to also report one more metric for measurement event prediction for temporal domain case A i.e. the timing distance between the 1st time instance of positive true prediction and corresponding ground truth label. This is to understand how far in advance the model was able to truly predict the event to happen as it seems in some methods also the events predicted with a window<PW length were counted as true event prediction which enhances the F1 score, but does not truly reflect the purpose of agreeing to use an aligned PW length.

In this paper, we provide updated results related to Case 4 from the table above, considering the latest agreements as mentioned above and providing an information about the additional HO metrics as agreed by RAN2.

# 2   Measurement event prediction accuracy

## 2.1 Simulation assumptions

Simulation assumptions and evaluation methodology agreed previously for RRM measurement prediction were reused, as captured in [3]. Furthermore, we used the following settings specifically for measurement event prediction simulations purpose (also aligned with RAN2 agreements).

Table 2. Simulation assumptions for FR2 to FR2 intra-frequency temporal domain case A (Case 4)

| Parameter | Baseline value | Additional values |
| --- | --- | --- |
| A3 event offset (db) | 2 | N/A |
| TTT (ms) | 320 | N/A |
| UE speed (km/h) | 90 | 60, 120 |
| OW length (ms) | 400 | N/A |
| PW length (ms) | 400 | 80 |
| Max ETD (ms) | 80 | N/A |
| L1 filtering | Sliding window |  |

Additional information about the simulation setup, e.g. AIML model details, can be found in the Annex A and in the excel sheet attached to this Tdoc.

## 2.2 Simulation results

To evaluate the event prediction accuracy, RAN2 decided to use F1score which is defined as follows:

F1 score = 2*Precision*Recall/(Precision + Recall), where:

- Precision = n3/(n1+n3)

- Recall = n3/(n2+n3)

Counters n1, n2, n3 are defined as follows:

- Counter n1 (false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD]

- Counter n2 (missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD]

- Counter n3 (true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD]

Except for the baseline case as indicated in Table 2, we decided to check how TTT and ETD impact the results when the UE speed, OW/PW and consequently also RMSE remain constant. The results are presented in Table 3. In our methodology, only the events that are predicted 320ms in advance, i.e. according to the agreed PW length, are counted as positively predicted events.

Table 3. Measurement event prediction accuracy for FR2 to FR2 intra-frequency temporal domain case A (Case4)

|  | UE speed: 90km/h |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Precision | Recall | F1 score | RSRP diff [dB] | Average time distance between the 1st positive true prediction and corresponding ground truth label |
| OW: 320ms, PW: 320ms TTT=320ms, ETD=80ms | 0.38 | 0.28 | 0.32 | 1.36 | 320ms |
| OW: 320ms, PW: 320ms TTT=240ms, ETD=80ms | 0.58 | 0.55 | 0.56 | 1.36 | 320ms |
| OW: 320ms, PW: 320ms TTT=320ms, ETD=160ms | 0.62 | 0.44 | 0.52 | 1.36 | 320ms |
| OW: 320ms, PW: 320ms TTT=240ms, ETD=160ms | 0.89 | 0.85 | 0.87 | 1.36 | 320ms |

As we can see in the presented results, the accuracy of measurement event prediction is not always good and depends on the utilized TTT and ETD. For the baseline case the accuracy is not good and F1 score equals only 0.32. However, when TTT is shortened, F1 score rises to 0.56 already. Similarly, in case we keep TTT-320ms but extend the ETD, F1 score also increases to 0.52. When we combine an effect of decreased TTT and increased ETD (TTT=240ms, ETD=160ms) F1 score is already very good, i.e. F1 score=0.87.

It seems these phenomena be easily justified, i.e.:

- With ETD=80ms and the simulation sampling rate of 80ms, there is almost no room for mistakes from the prediction while increasing allowed error margin (ETD) to 160ms improves the F1 score quite significantly. It means that the model is able to predict that the event is going to happen, but its exact timing cannot be always predicted that accurately. System level KPIs are essential to understand how this timing inaccuracy would truly impact the HO performance, which is analyzed in the next section.

- With shorter TTT, the model needs to properly predict whether the event is met or not for a shorter duration which is an easier task and hence improves F1 score.

Based on the above results and considerations, we draw the following conclusions:

Observation 1: For FR2 to FR2 intra-frequency temporal domain case A (Case 4), F1 score is higher for shorter TTT values of the predicted event.

Observation 2: For FR2 to FR2 intra-frequency temporal domain case A (Case 4), it is challenging to achieve high F1 score for ETD=80ms in case sampling rate in the simulations is also equal to 80ms as it leaves very small margin for prediction mistakes. Extending ETD to 160ms improves the prediction accuracy significantly, e.g. with TTT=240ms and ETD=160ms, F1 score=0.87.

# 3   Mobility performance

## 3.1 Simulation assumptions

As per RAN2 agreements, the methodology of handover failure determination in our simulations followed the methodology from TR 36.839 [4], in particular section 5.2.1.3:

#### 5.2.1.3 Handover/PDCCH failure modelling Definition 3: A handover failure is counted if a RLF occurs in state 2, or a PDCCH failure is detected in state 2 or state 3. For calculating the handover failures for the two states: - In state 2: when the UE is attached to the source cell, a handover failure is counted if one of the following criteria is met: 1) Timer T310 has been triggered or is running when the HO_CMD is received by the UE (indicating PDCCH failure) or 2) RLF is declared in the state 2 - In state 3: after the UE is attached to the target cell a handover failure is counted if the following criterion is met: - target cell downlink filtered average (the filtering/averaging here is same as that used for starting T310) wideband CQI is less than the threshold Qout (-8 dB) at the end of the handover execution time (Table 5.1.4.1) in state 3. For the purpose of PDCCH failure condition monitoring in state 2: The L1 sample rate is once every 10ms and the L1 samples are filtered by a linear filter with a sliding window of 200ms (i.e. 20 samples). For the purpose of PDCCH failure condition monitoring in state 3: The L1 sample rate should be at least two samples during the 40ms (i.e. the handover execution time) and averaged over the number of samples. NOTE 1: The handover failure definition 3 above is different from the handover failure definition in TS 36.331 [5]. It serves the purpose of evaluating the handover performance at both serving and target cell while the definition in TS 36.331 [5] is from the UE point of view which only captures the failures in target cell. NOTE 2: PDCCH failure condition detection is based on power measurement with the simplified model for saving simulation time to allow the calibration with various sets of configurations and more efficient simulations. Definition 4: The handover failure rate is defined as: Handover failure rate = (number of handover failures) / (Total number of handover attempts). The total number of handover attempts is defined as: Total number of handover attempts = number of handover failures + number of successful handovers. The number of handover failures is in Definition 3.

The methodology is further explained in the figures below, also copied from [2]:

Figure 5.2.1.3.1: A handover failure is declared when the criterion 1) is met in state 2. Figure 5.2.1.3.2: A handover failure is declared when the criterion 2) is met in state 2.

To verify the impact of using predicted measurements/measurement event on handover KPIs, in addition to the parameters mentioned in section 2.1 of this document, following settings were used in order to be able to determine handover failure as per the methodology from TR 36.839 described above:

| Parameter | Value |
| --- | --- |
| Qin threshold | -6db |
| Qout threshold | -8db |
| Sample rate (TIndication_interval) | 40ms(FR1) |
| Qin evaluation period | 100ms |
| Qout evaluation period | 200ms |
| T310 | 1000ms |
| N310 | 1 |
| N311 | 1 |
| Handover preparation time | 40ms |
| Handover execution time | 40ms |

## 3.2 Simulation results

RAN2 agreed to utilize three KPIs for evaluation of impact that using predicted measurement event has on mobility performance:

- Handover failure rate, defined as: HOF rate = (number of handover failures) / (Total number of handover attempts).

- Total number of HO attempts per UE per second which includes both successful and failed handover attempts.

- Total number of HOF per UE per second, where HOF is defined as above.

Also, two distinct HO modelling options were agreed and captured in [3] (HO option 1 and HO option 2 for short):

Figure 5.5.1-1: Handover model option 1 Option 1 is illustrated in Figure 5.5.1-1. At current time i.e. t0 measurement event e.g. A3 event is predicted at some point of time in future. Network will not transmit handover command until a real measurement event is reported for the same neighbouring cell. In this way, the main benefit of this option is to save handover preparation time. Figure 5.5.1-2: Handover model option 2 Option 2 is illustrated in Figure 5.5.1-2. At current time i.e. t0 measurement event e.g. A3 event is predicted @ future time t1. Network transmits handover command when entry condition of the predicted measurement event based on actual measurement result @ t2 unless t2 is earlier than handover preparation phase. In later case, network transmits handover command immediately after handover preparation phase. In this way, not only handover preparation could be saved but also handover can be executed earlier.

The main difference between Option 1 and Option 2 is that in option 1 event prediction is used solely for the purpose of starting a HO preparation in advance, while the decision about sending a HO command is based on the usual measurement report (i.e. not based on predictions). In option 2, in addition to commencing the HO preparation in advance, the network sends HO command as soon as the event entering condition is met based on actual measurements and the AIML model predicts that an event will remain met for the duration of TTT. Hence, it allows not to delay sending the HO command unnecessarily (assuming that the prediction is correct).

Table below presents simulation results for the setup described above for FR2 to FR2 intra-frequency temporal domain case A (Case 4) with:

- UE speed=90km/h

- PW=320ms, OW=320ms

- Two different values of TTT, i.e. 320ms, 240ms

- Two HO modelling options, as described above

Table 4. Comparison of HO failure rate and total number of HO attempts between FR1 to FR1 intra-frequency temporal domain case B prediction and non-AIML based measurements, RSRP from 20 cells used as a noise

|  | TTT=320ms |  |  |  | TTT=240ms |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HO failure rate | Total number of HO attempts per UE per second | Total number of HOF per UE per second | F1 score of measurement event prediction | HO failure rate | Total number of HO attempts per UE per second | Total number of HOF per UE per second | F1 score of measurement event prediction |
| Baseline, i.e. KPIs when actual measurement results are used | 70.58% | 0.594 | 0.419 | N/A | 73.68% | 0.663 | 0.489 | N/A |
| AIML predicted measurement results with HO option 1 | 70.58% | 0.594 | 0.419 | 0.32 (ETD=80ms) 0.52 (ETD=160ms) | 73.68% | 0.663 | 0.489 | 0.56 (ETD=80ms) 0.87 (ETD=160ms) |
| AIML predicted measurement results with HO option 2 | 25% | 0.419 | 0.105 | 0.32 (ETD=80ms) 0.52 (ETD=160ms) | 22.22% | 0.628 | 0.14 | 0.56 (ETD=80ms) 0.87 (ETD=160ms) |

Very high HOF rates result from the fact that RAN2 agreed to use the RSRP from all 20 neighbouring cells as a noise. These KPIs are decreased significantly in case only 3 cells are used as a noise during metric calculations, as depicted in Table 5.

Table 5. Comparison of HO failure rate and total number of HO attempts between FR1 to FR1 intra-frequency temporal domain case B prediction and non-AIML based measurements, RSRP from 3 cells used as a noise

|  | TTT=320ms |  |  |  | TTT=240ms |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HO failure rate | Total number of HO attempts per UE per second | Total number of HOF per UE per second | F1 score of measurement event prediction | HO failure rate | Total number of HO attempts per UE per second | Total number of HOF per UE per second | F1 score of measurement event prediction |
| Baseline, i.e. KPIs when actual measurement results are used | 11.76% | 0.594 | 0.0983 | N/A | 10.53% | 0.663 | 0.07 | N/A |
| AIML predicted measurement results with HO option 1 | 11.76% | 0.594 | 0.0698 | 0.32 (ETD=80ms) 0.52 (ETD=160ms) | 10.53% | 0.663 | 0.07 | 0.56 (ETD=80ms) 0.87 (ETD=160ms) |
| AIML predicted measurement results with HO option 2 | 0% | 0.419 | 0 | 0.32 (ETD=80ms) 0.52 (ETD=160ms) | 0% | 0.628 | 0 | 0.56 (ETD=80ms) 0.87 (ETD=160ms) |

However, in both cases the trends are the same and the same observations and conclusions can be drawn.

As can be seen in the results, using AIML predicted measurement results using HO option 2 allows to improve mobility performance KPIs significantly, while utilizing HO option 1 brings no benefits. It seems that in the simulated scenarios, bad mobility performance is caused by too late handovers and saving only the HO preparation time is not sufficient to avoid these HO failures. HO option 2 on the other hand, allows to save much more time which in the simulated scenarios equals to TTT-(HO preparation time), as the PW length=TTT=320ms. For shorter TTT, the HO performance is also slightly improved for HO option 2 as in this case also HO preparation time can be saved, as PW length (320ms) is longer than a sum of TTT (240ms) and HO preparation time (40ms).

Observation 3: For FR2 to FR2 intra-frequency temporal domain case A (Case 4), HO performance can be improved significantly using HO option 2 while utilizing HO option 1 brings no mobility performance gains.

Observation 4: The performance gain is observed in terms of both HOF rate and total number of HOF per UE per second, e.g. in the RAN2 agreed scenario HOF rate drops from 70% to 25% while the total number of HOF per UE per second drops from 0.419 to 0.105.

Furthermore, it can be observed that there is no direct correlation between F1 score metric and the HO performance. This is best visible for in Table 5 where HOF rate equals to 0% for HO option 2 while the F1 score was equal to 0.32 for ETD=80ms and to 0.52 for ETD=160ms.

Observation 5: AIML models can help to improve the mobility performance significantly, even when the achieved F1 score is low.

Observation 6: F1 score alone does not provide sufficient information about the usefulness of measurement event prediction for mobility improvement in the system.

# 3   Summary

Based on the simulations presented in this paper, the following observations are made for measurement event prediction and its impact on system level KPIs for temporal domain prediction Case A in FR2:

Observation 1: For FR2 to FR2 intra-frequency temporal domain case A (Case 4), F1 score is higher for shorter TTT values of the predicted event.

Observation 2: For FR2 to FR2 intra-frequency temporal domain case A (Case 4), it is challenging to achieve high F1 score for ETD=80ms in case sampling rate in the simulations is also equal to 80ms as it leaves very small margin for prediction mistakes. Extending ETD to 160ms improves the prediction accuracy significantly, e.g. with TTT=240ms and ETD=160ms, F1 score=0.87.

Observation 3: For FR2 to FR2 intra-frequency temporal domain case A (Case 4), HO performance can be improved significantly using HO option 2 while utilizing HO option 1 brings no mobility performance gains.

Observation 4: The performance gain is observed in terms of both HOF rate and total number of HOF per UE per second, e.g. in the RAN2 agreed scenario HOF rate drops from 70% to 25% while the total number of HOF per UE per second drops from 0.419 to 0.105.

Observation 5: AIML models can help to improve the mobility performance significantly, even when the achieved F1 score is low.

Observation 6: F1 score alone does not provide sufficient information about the usefulness of measurement event prediction for mobility improvement in the system.

# References

- R2-2500892 Simulation results for measurement event prediction Huawei, HiSilicon

- R2- 2501555 Draft summary of [AT129][027][AI Mob] Simulation methodology (OPPO) OPPO

- R2-2500287 Text proposal of 38.744	OPPO	 pCR	Rel-19	38.744	0.0.6

- 3GPP TR 36.839	Evolved Universal Terrestrial Radio Access (E-UTRA); Mobility enhancements in heterogeneous networks

# Annex A – simulation assumptions

Below are the company selectable assumptions chosen by us.

| Configurable/selectable simulation assumption |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Spatial consistency | Whether LOSsoft is modeled or not | UE trajectory model | UE trajectory boundary processing | L1/L3 filtering (sliding/non-sliding) | Number of Tx beams | Number of Rx beams |
| Procedure B in TR38.901 | Not modelled | Option 1 in 38.843 section 6.3.1 | Wrap-around model | Both non-filtered and non-sliding L1 filtering cases were simulated | FR1: 4 FR2: 64 | FR1: 1 FR2: 8 |

All the simulation assumptions are listed in the table below with the selectable ones highlighted in yellow.

Table 1: Summarized simulation assumptions

| Parameter | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz, 15/30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading without LOSsoft and inter-frequency correlation; without Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading without LOSsoft; without Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage |
| System BW | 20MHz | 80MHz |
| UE speed | 30,60,90 km/h for study targeting measurement reduction | 60,90,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) |
| UE Antenna radiation pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 44dBm | 40 dBm |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | Procedure B in TR38.901 | Procedure B in TR38.901 |
| UE trajectory model | Option 1 in 38.843 section 6.3.1 | Option 1 in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | wrap round model | wrap round model |