3GPP TSG-RAN WG2 #129bis	R2-2502468

# Wuhan, China, 7th – 11th April 2025

Agenda Item:	8.13.2

Source:	Ericsson

Title:	Discussion on relay discovery and relay (re)selection

Document for:	Discussion, Decision

# 1	Introduction

For the WI on NR sidelink multiple-hop relay, the below objectives on relay discovery and relay (re)selection have been defined in the WID [1].

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3]

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

.

We discuss aspects on relay discovery and relay (re)selection in this paper. All discussions refer to L2 U2N Remote UE or L2 U2N Relay UE unless otherwise stated.

# 2	Discussion

RAN2 has made the below agreements in RAN2#128:

A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE.

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible).  FFS if operation as an intermediate relay UE towards a different cell would be supported.

The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.  If the upper bound is not configured, there is no threshold, but this does not override the previous agreement.  FFS if there would be additional restrictions in the case of no upper bound.

The following cases are supported for L2 multihop relay:

- One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE).

- Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE.

FFS if the last relay UE can use the same L2ID for both of the connections in either case.

Cases with two indirect paths to the gNB for the same remote UE are not supported.

An Intermediate Relay UE can serve multiple multi-hop indirect paths of different Remote UEs.

If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links.

RAN2 sends LS to ask SA2 to clarify whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message.

Wherein several FFSs need to be discussed in upcoming meetings.

## 2.1 Discovery forwarding

RAN2 has made below agreements on AS criteria for discovery forwarding

Agreements:

For model A discovery, there are no additional AS criteria on discovery message forwarding (the existence of the PC5 link is sufficient).

For model B discovery, if the PC5 link on which the discovery message is received has already been established, no additional AS criteria on discovery message forwarding are applied.  FFS if the PC5 link is not already established.

The above agreements are only concerning the case where the PC5 link between the relay UE and the UE from which the discovery messages are received. For the other case where the PC5 link is not already established, we think the AS criteria is still needed, due to below reasons

- The remote UE only checks the PC5 radio quality towards the first relay UE (candidate), without checking other links which have more than one hop away from the remote UE

- Checking additional AS criteria while forwarding discovery message or response can minimize the risk that a relay UE candidate with instable link is selected and trigger relay UE reselection after a short while.

Therefore, we would like to make the below proposals

For Model B discovery, the intermediate relay UE forwards the discovery solicitation/response message when the SD-RSRP between the relay UE and the UE from which the discovery solicitation/response message is received is above a configured threshold.

For Model B discovery, after receiving a discovery solicitation message from an intermediate relay, the last relay UE transmits the discovery response message when the SD-RSRP between the last relay UE and the intermediate relay is above a configured threshold.

## 2.2 Relay (re)selection

Regarding relay (re)selection, RAN2 has made the below agreements in RAN2#129

Agreements:

For multi-hop U2N relay selection at the remote UE, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE.

For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition:

-	PC5 signal strength of current first relay UE is below a (pre)configured signal strength threshold.

-	When U2N Remote UE receives a PC5-S link release message from current first relay UE.

-	When U2N Remote UE detects PC5 RLF with the current first relay UE.

-	Indicated by upper layer.

L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design):

-	cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network.

-	release of RRC connection between an intermediate relay UE and the network.

-	PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s).

-	FFS: link quality degradation of the upstream links resulting in a notification.

-	FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE).

-	FFS on the signalling contents of the PC5-RRC indication from the first relay UE.

-	FFS applicability to L3.

For the FFS on whether link quality degradation of the upstream links can result in a notification, we don’t think it is needed due to reasons

- The degradation of the link quality may be just a temporary/one-shot occurrence.

- There is already RLF trigger condition, which can cover the case where the link quality has a long term degradation

- Adopting additional criteria would cause additional design complexity for RAN2.

Therefore, we would like to make the below proposals

Not pursue link quality degradation of the upstream links as a trigger condition for notification message.

In Rel-17 U2N framework, for both L2 and L3 U2N Relay UEs in RRC_IDLE or RRC_INACTIVE, the PC5-RRC message(s) are used to inform their connected U2N Remote UE(s) when U2N Relay UEs select a new cell. The PC5-RRC message(s) are also used to inform their connected L2 or L3 U2N Remote UE(s) when L2 or L3 U2N Relay UE performs handover, detects Uu RLF, or its Uu RRC connection establishment/resume fails.

Based on which, U2N Remote UE may trigger relay reselection.

Therefore, we would like to make the below proposal.

Same as Rel-17 U2N, L3 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signalling from the first relay UE indicating:

  - cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network.

  - release of RRC connection between an intermediate relay UE and the network.

  - PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s).

In addition, regarding whether an intermediate relay UE can trigger relay (re)selection on behalf of remote UE, our understanding is that

- The SA2 conclusion/agreement on allowing an intermediate relay UE to trigger relay (re)selection as a relay UE, is only applicable/valid for L3 relay

- Allowing an L2 intermediate relay UE to trigger relay (re)selection as a relay UE, may result in that the relay UE selects a different cell from the serving cell of the last relay UE, which is not allowed according to RAN2 agreements.

- In Rel-17 U2N, it is the remote UE to make decision on relay (re)selection, it is reasonable to reuse it for multi-hop L2 remote UE.

Therefore, we would like to make the below proposal.

L2 multi-hop intermediate relay UE is not allowed to trigger relay (re)selection as a relay UE.

In Rel-17 single hop U2N relay, the PLMN ID and cell ID can be used as additional AS criteria. Similarly, Remote UE may need to consider both PLMN ID and cell ID in the relay (re)selection procedure. In the procedure, Remote UE may first select a U2N Relay UE considering its PLMN ID and cell ID. After that, Remote UE selects one or multiple intermediate Relay UE by checking if they are authorized to access the cell of U2N Relay UE. Therefore, each Relay UE candidate can include the PLMN info and cell ID in discovery messages as a container (which is transmitted or forwarded). In legacy, the discovery message can already contain PLMN ID and cell ID for single Relay UE. For multiple U2N relay, the discovery message can be extended to include PLMN ID and cell ID for each Relay UE in the path. Upon reception of discovery messages/responses, Remote UE considers received PLMN info and cell ID info in relay (re)selection procedure.

Same as in legacy, PLMN info and cell ID info of each Relay UE may be considered as additional AS criteria in relay (re)selection.

Each Relay UE candidate can include PLMN info and cell ID in discovery messages (which is transmitted or forwarded).

In addition, in Solution #1(TR 23.700-03), SA2 has discussed how to use Hop-Count value in relay (re)selection procedure. In an example, Remote UE selects the Relay UE announcing the least Hop-Count value to build the path with shortest hop numbers. We think it is not necessary to disclose Hop-Count value into AS, due to below reasons

1) Hop-count is included in Discovery or DCR/DCA messages, which are not seen by AS by nature.

2) It is sufficient to categorize such criteria as Upper layer criteria, and leave them to be handled by Upper layers. Disclose them to AS may introduce double control/usage of these criteria at both Upper layer and AS layer, which increases additional complexity.

Therefore, we propose that

Hop-Count can be categorized as an upper layer criterion and left to be handled by upper layer, and therefore, no need to be disclosed to AS layer.

# 3	Conclusion

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For Model B discovery, the intermediate relay UE forwards the discovery solicitation/response message when the SD-RSRP between the relay UE and the UE from which the discovery solicitation/response message is received is above a configured threshold.

Proposal 2	For Model B discovery, after receiving a discovery solicitation message from an intermediate relay, the last relay UE transmits the discovery response message when the SD-RSRP between the last relay UE and the intermediate relay is above a configured threshold.

Proposal 3	Not pursue link quality degradation of the upstream links as a trigger condition for notification message.

Proposal 4	Same as Rel-17 U2N, L3 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signalling from the first relay UE indicating:

1)	cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network.

2)	release of RRC connection between an intermediate relay UE and the network.

3)	PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s).

Proposal 5	L2 multi-hop intermediate relay UE is not allowed to trigger relay (re)selection as a relay UE.

Proposal 6	Same as in legacy, PLMN info and cell ID info of each Relay UE may be considered as additional AS criteria in relay (re)selection.

Proposal 7	Each Relay UE candidate can include PLMN info and cell ID in discovery messages (which is transmitted or forwarded).

Proposal 8	Hop-Count can be categorized as an upper layer criterion and left to be handled by upper layer, and therefore, no need to be disclosed to AS layer.

# 4	References

[1] RP-241609, “New WID on NR sidelink multi-hop relay”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17 – 20 June 2024.