3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502470

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.2.3

Source:	Huawei, HiSilicon

Title:	A-IoT random access procedure

Document for: Discussion and Decision

1	Introduction

In this contribution, the details of A-IoT random access procedure are further discussed.

2	Discussion

2.1	Msg1 resource selection

For Msg1 resource selection, we had the following agreements for time and frequency domain resource selection.

6 From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader, as the baseline for CBRA. FFS if this is applicable to FDMA. Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA. 2. RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

Thus, reader indicates the total number of access occasions in time-frequency domain by paging message, the device directly randomly selects one access occasion from the total number of access occasions.

In RFID [1], a slot counter decrement mechanism is supported to determine the slot for access.

Upon receiving a Query, Tags with matching Sel and Target shall pick a random value in the range (0, 2Q–1), inclusive, and shall load this value into their slot counter. If a Tag, in response to the Query, loads its slot counter with zero, then its reply to a Query shall be as shown in Table 6-33 using the immediate reply type specified in 6.3.1.6.1; otherwise the Tag shall remain silent. If a Tag, in response to the QueryRep, decrements its slot counter and the decremented slot value is zero, then its reply to a QueryRep shall be as shown in Table 6-37 using the immediate reply type specified in 6.3.1.6.1; otherwise the Tag shall remain silent. A Tag shall respond to a QueryRep only if it received a prior Query.

A similar device behaviour can be specified for A-IoT, based on the random number generated for access occasion. The example of text proposal for this kind of Msg1 resource selection procedure is as below:

=================================Text Proposal============================= 5.3.3.1 Selection of the access occasion for the D2R transmission of Random ID message The A-IoT MAC entity shall: 1> draw a random integer number 'i' with equal probability in the range: 0 ≤ i ≤ n-1, where n is the number of access occasions configured in A-IoT Paging message; 1> set the ACCESS_OCCASION_COUNTER to 'i'; Upon reception of the [Access Occasion Trigger] message, the A-IoT MAC entity shall: 1> if the [Access Occasion Trigger] message triggering 'm' access occasion(s) is received: 2> if ACCESS_OCCASION_COUNTER < m: 3> select the 'ACCESS_OCCASION_COUNTER+1' th access occasion from the m access occasion(s) triggered by this [Access Occasion Trigger] message for transmission of the Random ID message; 3> perform the transmission of the Random ID message procedure as defined in X.5.3.3.2; 2> else: 3> decrement ACCESS_OCCASION_COUNTER by m. =================================Text Proposal=============================

Proposal 1:	It is suggested to consider above TP for Msg1 resource selection procedure.

As to the last meeting way forward, we further discuss what will be the QueryRep-like message in Way-1.

- From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field

The detailed resource parameters and/or control information can be carried in the paging message, which means the “QueryRep-like” message is just one trigger. There is no other information needed in this MAC message, except for its message type. Considering that this message is the most frequently R2D message, it is beneficial for the efficiency to minimize the overhead. Thus, it can be one MAC message including only the message type field, i.e., 3 or 4bits MAC PDU. Noted that this MAC message does not need to be byte aligned. For example, in order to trigger 1000 devices for inventory, almost 1000 “QueryRep-like” messages are needed when one time-domain resource is triggered by a “QueryRep-like” message. It requires an additional 4000 bits, if this MAC message is 8bits which is byte aligned.

Proposal 2:	If “the R2D transmission which determines the Msg1 resource(s)” is concluded as the MAC signalling, it can be one MAC message including only the message type field, i.e., just 3 or 4-bits MAC PDU (Terminology can be “Access Occasion Trigger message”).

2.2	Msg2 related discussion

2.2.1	Msg2 monitoring

One R2D transmission can trigger one set of X*Y access occasions, where X is the number of time-domain resources and Y is the number of frequency-domain resources. After this bunch of Msg1 transmissions, the reader needs to send Msg2 to resolve the contention within these X*Y access occasions and to schedule the Msg3 transmissions. Figure 2.2.1-1 and Figure 2.2.1-2 give examples for A-IoT messages; there might be one or multiple Msg2s following multiple access occasions for Msg1.

Figure 2.2.1-1:	An example of one Msg2 following multiple access occasions for Msg1

Figure 2.2.1-2:	An example of multiple Msg2s following multiple access occasions for Msg1

In NR, RAR window is introduced for Msg2 monitoring. The motivations to introduce RAR window include: 1) the UE can determine contention resolution failure based on expiration of RAR window, in order to autonomously re-initiate the random access; 2) the UE can save power by limiting the time for monitoring Msg2.

Note that RAN1 is discussing the start of Msg2 monitoring and duration for Msg2 monitoring, while RAN2 can wait for more RAN1 progress for further discussion.

Proposal 3:	RAN2 waits for more RAN1 progress before discussing the RAN2 impacts related to Msg2 monitoring duration.

2.2.2	Msg2 content/format

In RAN1#120 meeting, RAN1 had the following agreement for Msg2.

Agreement Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices - FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2.

Observation 1:	The Msg2 needs to support multiplexing of random ID to multiple devices.

The probability that devices in different access occasions select the same random ID is estimated as follows. N represents the number of random ID successfully received by reader across different access occasions. To respond to Msg1s in access occasions assigned by one R2D message, either one (as shown in Figure 2.2.1-1) or multiple (as shown in Figure 2.2.1-2) Msg2(s) can be used, and these N random IDs are to be included in the Msg2(s). Note that no differentiation between different Msg2s is assumed. The probability under consideration is that at least two of these N random IDs are identical.

If the first random ID is random ID*, the probability that the second random ID is different from the first random ID is (216-1)/216. By analogy, the probability that the N-th random ID is different from the other N-1 random IDs is (216-(N-1))/216. Hence, the probability that N random IDs are totally different from each other is:

P0=(216)!216-N!216N=216×216-1…216-N-1216N.

The probability that at least two of these N random IDs are the same is:

P=1-P0=1-216×216-1…216-N-1216N.

Table 2.2.2-1	Calculation results of random ID collision probability P with different N

| N | P | N | P | N | P |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.00000% | 6 | 0.02289% | 11 | 0.08389% |
| 2 | 0.00153% | 7 | 0.03204% | 12 | 0.10066% |
| 3 | 0.00458% | 8 | 0.04272% | 13 | 0.11896% |
| 4 | 0.00916% | 9 | 0.05492% | 14 | 0.13877% |
| 5 | 0.01526% | 10 | 0.06864% | 15 | 0.16010% |

According to analysis in R1-2409419 [2], the total number of access occasions triggered by one R2D message is limited. For example, the access occasions assigned by one R2D message are 2 for time domain resources and 8 for frequency domain resources. Considering the optimal successful access probability in ALOHA [3] as 36.8%, the N is 6 and the probability P is just 0.02289%. Even for the large value, i.e., 5 and 8 time- and frequency-domain resources for Msg1 transmission, the N is 15 and the probability P is about 0.16%.

Observation 2:	The probability of random ID collision in Msg2 is below 0.16%, which can be considered as sufficiently low.

Even if there is a collision of random IDs among devices in different access occasions, the reader can identify it and avoid echoing the ambiguous random ID(s) in Msg2 by implementation. This will prevent further ambiguity for the devices when performing the subsequent RA procedure.

Observation 3:	In case there is a collision of random IDs among devices in different access occasions, the reader can identify it and avoid echoing the ambiguous random ID in Msg2 by implementation.

Proposal 4:	From RAN2 perspective, Msg2 does not require the information referring to the time-frequency resource of Msg1, i.e., the contained random ID alone is sufficient for the device to determine the response for its Msg1.

At the same time, the Msg2 schedules Msg3 transmission(s) for echoed device(s). In addition to echoing random ID, other information in Msg2, e.g. Msg3 scheduling information, is up to RAN1 further discussion.

Proposal 5:	RAN2 waits for more RAN1 progress about the Msg3 scheduling information in Msg2.

2.2.3	Msg3 re-transmission triggered by Msg2

There were the following agreements for Msg3 re-transmission triggered by Msg2.

4.	RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID

5.	If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported).

Based on the above discussion for Msg2 content, one Msg2 can include one or multiple echoed random ID for multiple devices and schedule associated Msg3 transmission. The reader may fail to receive one or multiple scheduled Msg3 and decide to trigger Msg2 re-transmission. Similarly, the re-transmitted Msg2 can echo one or multiple random ID for one or multiple failed device(s) in one-shot. Thus, the same format is considered for both initial Msg2 transmission and Msg2 re-transmission.

From the device side: Case 1: if the Msg2 reception failure results in Msg3 transmission failure, the device which misses the initial Msg2 will consider re-transmitted Msg2 as initial Msg2 and respond to this Msg2; Case 2: if the Msg2 is received successfully but Msg3 transmission fails, the device may receive another Msg2 targeting this device and needs to respond to it again. It is a unified behaviour that once the Msg2 is received targeting one device, this device considers contention resolution success and transmits the associated Msg3.

If we consider the Msg2 optionally assigning AS ID case, the Msg2 retransmission includes both Random ID and the optional AS ID, for the failed device. Device in case 1 will behave just like the reception of initial Msg2. Device in case 2 will use its stored AS ID (from initial Msg2), instead of random ID, to check the re-transmitted Msg2 content.

Proposal 6:	The same format is used for both initial transmission of Msg2 and re-transmission of Msg2, while, as to the exact content, it is up to the reader to include the only failed devices’ information in the re-transmitted Msg2. FFS any device behaviour differentiation.

2.3	Re-access related discussion

2.4.1	Feedback indication for re-access

During SI phase, the following agreements were made:

RAN2#127bis

2	Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.

RAN2#128

6.	The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose.

In RAN2#129 meeting, NACK based feedback indication was agreed for at least Msg3.

Agreements

- NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback.

The feedback will be introduced for Msg3 and following D2R data, to explicitly indicate whether the Msg3 or D2R data is successfully received and the service is successfully completed, so that device can know whether to skip the re-access for the subsequent paging. This will be one unified design for different use cases. Thus, NACK based feedback is also supported for the following D2R data to determine re-access. The reader can send “NACK feedback indication” either after Msg3 or after Msg5, whenever it intends to terminate the data transmission for device(s) in this paging round.

Proposal 7:	The reader can send “NACK feedback indication” either after Msg3 or Msg5 to device(s) that failed, whenever it intends to terminate the data transmission for device(s) in this “paging round”.

Within one paging procedure, some devices can finish the procedure successfully and some may not, thus the feedback indication should be per device feedback. ID information of the devices that failed can be included. The feedback can be sent to device in unicast way but it requires multiple R2D messages, each of which increases the signalling overhead, e.g., with more message type headers and CRCs in PHY layer. So, the feedbacks to multiple devices can be aggregated/carried into one broadcast R2D message.

Proposal 8:	The "NACK feedback indication" can be one broadcast R2D message, which can include multiple IDs of the devices that failed.

In device-side operation, if the "NACK feedback indication" is received, it needs to re-access in the subsequent paging. If no "NACK feedback indication” is received before subsequent paging, device assumes the access is successful by default and skips the re-access. Thus, there is no need for a timer for reception of “NACK feedback indication” from RAN2 perspective.

Proposal 9:	There is no need for a timer for reception of “NACK feedback indication”. The device shall consider itself successful by default, if no “NACK feedback indication” is received before subsequent paging.

2.4.2	Subsequent paging to trigger re-access

It was agreed that the subsequent paging message is reused to trigger re-access. However, the reader may not have an accurate count of triggered devices for a specific service, leading to an unsuitable initial allocation of access occasions. Thus, the reader may need to adjust the number of access occasions by using the subsequent paging message. For example, adding more access occasions if many collisions are observed, and reducing access occasions if there are many unoccupied opportunities. The paging identifier (e.g. few hundred bits) will be unnecessary overhead for this pure “reconfiguration” purpose. Thus, it can be considered that the paging identifier can be optional in subsequent paging message.

Observation 4:	RAN2 can discuss if the paging identifier can be optional in (subsequent) paging message.

# 3	Conclusion

This contribution makes the following proposals:

Observation 1:	The Msg2 needs to support multiplexing of random ID to multiple devices.

Observation 2:	The probability of random ID collision in Msg2 is below 0.16%, which can be considered as sufficiently low.

Observation 3:	In case there is a collision of random IDs among devices in different access occasions, the reader can identify it and avoid echoing the ambiguous random ID in Msg2 by implementation.

Observation 4:	RAN2 can discuss if the paging identifier can be optional in (subsequent) paging message.

Msg1 resource selection

Proposal 1:	It is suggested to consider above TP for Msg1 resource selection procedure.

Proposal 2:	If “the R2D transmission which determines the Msg1 resource(s)” is concluded as the MAC signalling, it can be one MAC message including only the message type field, i.e., just 3 or 4-bits MAC PDU (Terminology can be “Access Occasion Trigger message”).

Msg2 related discussion

Proposal 3:	RAN2 waits for more RAN1 progress before discussing the RAN2 impacts related to Msg2 monitoring duration.

Proposal 4:	From RAN2 perspective, Msg2 does not require the information referring to the time-frequency resource of Msg1, i.e., the contained random ID alone is sufficient for the device to determine the response for its Msg1.

Proposal 5:	RAN2 waits for more RAN1 progress about the Msg3 scheduling information in Msg2.

Proposal 6:	The same format is used for both initial transmission of Msg2 and re-transmission of Msg2, while, as to the exact content, it is up to the reader to include the only failed devices’ information in the re-transmitted Msg2. FFS any device behaviour differentiation.

Re-access related discussion

Proposal 7:	The reader can send “NACK feedback indication” either after Msg3 or Msg5 to device(s) that failed, whenever it intends to terminate the data transmission for device(s) in this “paging round”.

Proposal 8:	The "NACK feedback indication" can be one broadcast R2D message, which can include multiple IDs of the devices that failed.

Proposal 9:	There is no need for a timer for reception of “NACK feedback indication”. The device shall consider itself successful by default, if no “NACK feedback indication” is received before subsequent paging.

# 4	References

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard.

- R1-2409419, On frame structure and timing aspects of Ambient IoT, Huawei, HiSilicon, CBN, China Broadnet, Nov. 2024

- Y. Maguire and R. Pappu, “An Optimal Q-Algorithm for the ISO 18000-6C RFID Protocol,” in IEEE Transactions on Automation Science and Engineering, 2009.