3GPP TSG-RAN WG2 Meeting #129-Bis	R2-250xxxx

Wuhan, China, 7 – 11 April, 2025

Agenda item:	8.7.4.2

Source:	InterDigital

Title:	DSR enhancements for UL scheduling

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

RAN2#129 made the following agreements wrt. the DSR enhancement:

- One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE. - FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. - We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size. - Different LCGs may be configured with different number of reporting thresholds. - If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold. - Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold). - Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

# 2	Discussion

## 2.1	New DRX MAC CE format applicability

In the previous meeting, it was left FFS whether the new DSR MAC CE format will be always applied when at least one LCG is configured with multiple thresholds. Actually, since the new agreed DSR MAC CE format to report multiple reporting thresholds does not suffer from any higher overhead compared to legacy DSR MAC CE format when any LCG is configured with only one reporting threshold, there is no reason not to apply the new DSR MAC CE format always when at least one LCG is configured even with a single reporting threshold.

Proposal 1: New DSR MAC CE format will always be used when at least one LCG is configured with one or more reporting thresholds.

## 2.2	Naming of DSR MAC CEs

During the MAC running CR review, rapporteur proposes to modify the legacy DSR MAC CE naming to Single Entry DSR MAC CE. This is problematic in two ways: first, generally no renaming of legacy MAC CEs is done, and second, the new DSR MAC CE format can equally be “Single Entry” in case any LCG is configured with one reporting threshold only. Hence, while the naming of the new DSR MAC CE can follow the proposal (ie., “Multiple Entry DSR MAC CE”), no change in legacy naming should be done.

Proposal 2: Legacy DSR MAC CE is not renamed to “Single Entry DSR MAC CE”.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: New DSR MAC CE format will always be used when at least one LCG is configured with one or more reporting thresholds.

Proposal 2: Legacy DSR MAC CE is not renamed to “Single Entry DSR MAC CE”.