3GPP TSG-RAN WG2 Meeting #129bis  	                            R2-2502493

Wuhan, China, Apr. 7th – 11th, 2025							                   (Revision of R2-2500749)

Agenda Item:	8.8.2

Source: 	Sony

Title:	SMTC impacts due to NTN downlink coverage enhancements

Document for:	Discussion

# Introduction

In RAN2#129 meeting, RAN2 discussed SMTC impacts in NTN downlink coverage enhancement and the corresponding agreements are as follows.

Agreements:

- RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer

- RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame

- RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell)

- The number of SMTC/gaps a UE needs to consider at any time will not be increased further

In this contribution, we mainly discuss the SMTC impacts due to beam hopping/extended SSB periodicity and the way forward.

# Discussion

According to RAN1#118 meeting, RAN1 considers the extended SSB periodicity by NTN UE for initial access can be one or more values from the list {40ms, 80ms, 160ms, 320ms, 640ms} in the system level and link level evaluations. In NTN, the network can configure up to 4 SMTCs in parallel per carrier and for a given set of cells depending on UE capabilities. It was agreed in RAN2#129 meeting that the number of SMTC will not be extended for downlink coverage enhancement, but the behaviour to monitor the SMTC configuration can still be further improved in order to reduce the energy consumption on unnecessary SMTC monitoring e.g., if UE is too far away from certain cells, there is no need to monitor SMTC on those cells as neighbours.

In NTN, for RRC_CONNECTED UEs, network controlled adjustment of SMTCs can be based on the reported UE assistance information. The UE assistance information includes the service link propagation delay difference between the serving cell and the neighbour cells. For RRC_IDLE/RRC_INACTIVE UEs, the UE can adjust SMTCs based on its location and assistance information in SIB19 and the assistance information is based on the broadcasted reference location which is typically e.g., serving cell’s reference location.

With only the serving cell’s reference location and a threshold to that reference location, is not enough for UE to perform the proper the SMTC measurement selection. Furthermore, for the RRC_CONNECTED UEs, the propagation delay difference is not accurate enough either for network to configure the proper SMTCs for the UEs. Therefore, we propose to enhance the SMTC measurement adjustment, for both UEs in RRC_CONNECTED and RRC_IDLE/INACTIVE. The signalling details for additional assistance information e.g. additional reference location information from network, finer UE location information from UE if available and requested should be further discussed. The additional reference location information can be for example the neighbour cells’ location information, and together with the serving cell’s location information, UE can get a rough estimation of the distance to the neighbour cell and then can decide whether it needs to monitor the SMTC corresponding to that particular neighbour cell. On the other hand, if the UE would like to share its finer location information to the network, network can dynamically instruct the UEs the SMTCs to be monitored.

Proposal 1: UE autonomous SMTC measurement adjustment for UEs in RRC_IDLE/INACTIVE, and network controlled SMTC measurement adjustment for UEs in RRC_CONNECTED should be further enhanced, due to NTN downlink coverage enhancement.

# Conclusion

We propose RAN2 to discuss following proposals:

Proposal 1: UE autonomous SMTC measurement adjustment for UEs in RRC_IDLE/INACTIVE, and network controlled SMTC measurement adjustment for UEs in RRC_CONNECTED should be further enhanced, due to NTN downlink coverage enhancement.