3GPP TSG RAN WG2 Meeting #129                          	   R2-2502543

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.12.2

Source:	Qualcomm Incorporated

Title:	Discussion on UL only mTRP

WID/SID:	NR_MIMO_Ph5-Core – Release 19

Document for:	Discussion

# Introduction

For asymmetric DL sTRP/UL mTRP deployment scenarios, the updated subject (addition of two TA) was outlined in the WID of NR MIMO Phase 5” [1].

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources.

In this contribution, we discuss potential RAN2 work for supporting asymmetric DL sTRP/UL mTRP.

# Discussion

## PL offset

At RAN2 #129, the following agreements were made for Asymmetric DL sTRP/UL mTRP[4].

- Working assumption: - UE applies the latest PL offset value received in RRC or MAC CE. Can revisit if new issue is found. - Agreement - RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

For the possible race condition between RRC and MAC CE with different PL offset values, different UE capabilities may deploy different implementations. Thus, it is up to UE implementation for how to identify the latest PL offset. From our view, it is acceptable as long as UE applies the latest PL offset value based on its implementation. So, the working assumption should be confirmed.

Proposal 1. Confirm the following Working Assumption:

“UE applies the latest PL offset value received in RRC or MAC CE.”

As agreed, that the PHR calculation should include the PL offset value if configured or indicated. The equation of PHR is specified in 38.213, so it’s suitable for capturing this agreement.

Observation 1. TS 38.213 is suitable for capturing that PHR calculation includes the PL offset, if configured by RRC or indicated by MAC CE.

## Two TAs

As shown in Figure 1, a UE may have two TAs where one TA (e.g., TA1) is for the uplink transmission to DL sTRP and the other TA (e.g., TA2) is for the uplink transmission to UL mTRP.

Figure 1. Two TAs for asymmetric DL sTRP and UL mTRP

At RAN1 #118bis, the following agreements were made for 2TA [2].

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

Observation 2. RAN1’s agreements have outlined the basics of two-TA design considerations with one downlink reference.

Additionally, at RAN1 #119, the following agreement were made for ULmTRP [3].

Agreement The answer to the Question 1 in LS R1-2409353 is: From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

Based on the above agreement, an ULmTRP may support limited SSB transmissions on downlink if PL offset is not configured. In this case, each TA of the 2 TAs may have its respective downlink reference, similar to Release 18 multi-TRPs. However, there is only one sTRP for transmitting PDCCH order (i.e., one CORESET for monitoring the DCI).

Observation 3. If limited SSB transmission from ULmTRP is supported, each TA of the 2 TAs may have its respective downlink reference.

# Conclusion

In this contribution, we discussed potential RAN2 work for supporting asymmetric DL sTRP/UL mTRP and concluded with the following observations and proposals.

MAC CE for PL offset

Proposal 1. Confirm the following Working Assumption:

“UE applies the latest PL offset value received in RRC or MAC CE.”

Observation 1. TS 38.213 is suitable for capturing that PHR calculation includes the PL offset, if configured by RRC or indicated by MAC CE.

Two TAs

Observation 2. RAN1’s agreements have outlined the basics of two-TA design considerations with one downlink reference.

Observation 3. If limited SSB transmission from ULmTRP is supported, each TA of the 2 TAs may have its respective downlink reference.

# References

[1] RP-242394, “WID revision: NR MIMO Phase 5”, RAN Meeting #105, Melbourne, Australia, September 9-12, 2024.

[2] “Chair notes RAN1#118bis eom0” RAN1 #118bis, Hefei, China, October 14th – 18th, 2024

[3] “Chair notes RAN1#119 eom0”, RAN1 #119, Orlando, US, November 18th – 22nd, 2024

[4] “R2_129 R18 MIMO R19 MIMO LPWUS SBFD (Erlin) 2025-02-21 1025 eom” RAN2 #129, Athens, Greece, February 17th – 21st, 2025