3GPP TSG-RAN WG2 Meeting #129bis	R2-2502550

Wuhan, China, 7th – 11th of April 2025

Agenda item:	7.0.2.17

Source:	Nokia, Nokia Shanghai Bell

Title:	Further Considerations on SSBs from Source and Target Satellite in Soft Switching with Resynchronization

WID/SID:	NR_NTN_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

At RAN2#128 (November 2024) it was fiercely discussed how to avoid the SSB overlapping in time for soft-satellite switching with resynchronization. The discussion ended with the LS to RAN1 and RAN4 [3]. In February 2025 (RAN2#129), while awaiting RAN1 and RAN4 responses, the topic has been briefly discussed based on Nokia’s [6]. No conclusion has been reached [7], and the topic was postponed to RAN2#129bis when the response LSs from RAN1 and RAN4 are expected to arrive.

In this paper we present our view how this issue might be eventually addressed without major impact on the legacy specification. It considers also the response LSs from RAN1 and RAN4 [9][10].

# 2	Discussion

## 2.1	What happened at RAN2#128 and RAN2#129

The issue has been discussed via offline thread at RAN2#128 (summary in [2]) and three options have been listed there:

- Option 1: shifted cell timing

- Option 2: shifted SSB pattern

- Option 3: temporarily shifted SSB pattern

Companies in RAN2 were not in favour of Option 1 and preferred to further consider Option 2 or Option 3. However, there were also concerns regarding Option 3, as it may potentially result in the UE having to reacquire the DL synchronization at the t-Service. Thus, the LS to RAN1 and RAN4 has been agreed in [3] to confirm whether indeed Option 3 will result in the necessity to obtain DL synchronization once again.

Eventually the following was agreed at RAN2#128 [1]:

- Send a LS to RAN1/RAN4 on the feasibility of option 3. RAN2 will work on solution 2b or solution 3 depending on the LS response

Solution 2b is a subset of Option 2, wherein the SMTC is adjusted autonomously by the UE, as explained in [2].

At RAN2#129 the following was agreed [7] after a brief discussion based on Nokia’s [6].

R2-2500329 On SSBs from Source and Target Satellite in Soft Switching with Resynchronization Nokia, Nokia Shanghai Bell discussion Rel-18 NR_NTN_enh-Core Proposal 1: RAN2 to consider relying on NCD-SSBs to mitigate the SSB overlapping issue for soft-satellite switching with resynchronization. Proposal 2: RAN2 considers extending SatSwitchWithReSync in SIB19 by including NCD-SSB configuration which would be valid from t-ServiceStart to t-Service. Proposal 3: RAN2 may take the ultimate decision once the responses from RAN1 and RAN4 are received (i.e. at RAN2#129bis). - Sequans thinks the idea to shift SSB patterns was to do this only with a half-frame granularity - Session chair also thinks that the RAN2 envisaged solutions where SSB patterns are shifted (permanently or temporarily, i.e. option 2b and 3 previously discussed) referred to a SSB shift with a half-frame granularity Continue in the next meeting

As can be noticed, at RAN2#129 it was outlined only SSB shift with half-frame granularity was considered. However, this was nowhere clearly stated, neither when the agreements were reached at RAN2#128 [1] or in the offline discussion report [2].

Observation 1: Neither the conclusions reached at RAN2#128, nor the options analysed in R2-2410971 clearly limited the SSB shift to half-frame granularity.

Furthermore, as can be seen in [8], ssb-TimeOffset defined within SatSwitchWithReSync can have any value between 0 and 159 and is described as follows:

ssb-TimeOffset Indicates the time offset of the SSB from target satellite at its uplink time synchronization reference point with respect to the SSB from source satellite at its uplink time synchronization reference point. It is given in number of subframes.

Observation 2: ssb-TimeOffset defined in SatSwitchWithReSync is expressed in the number of subframes and can be configured to any value from 0 to 159.

## 2.2 	RAN4 Response LS

In February 2025 RAN4 and RAN1 have carefully discussed the content of the RAN2 LS [3] and provided the responses in [9] and [10], respectively.

In RAN4 response LS [9] provides assumptions under which the pre-acquired DL synchronization can be maintained when SSB pattern changes at t-Service. Those assumptions are as follows:

- Starting boundaries of SFN0 of the source satellite and the target satellite are aligned at the uplink time synchronization reference point; - The parameter ssb-TimeOffset is the relative offset between the SSB of the source satellite and the SSB of the target satellite during [t-ServiceStart, t-Service];

Observation 3: RAN4 in the response LS (R4-2502701) provides assumptions under which pre-acquired DL synchronization may be maintained after SSB pattern changes at t-Service.

However, at the same time RAN4 underlines certain concerns and informs RAN2 such scenario was not considered when RAN4 defined Release 18 requirements.

Observation 4: RAN4 has not considered the scenario with temporary SSB pattern shift when defining the Release 18 UE requirements. Such shift may require additional UE processing.

RAN4 also underlines that such temporary SSB pattern shift may impact other UEs, e.g. UE’s performing initial access during [t-ServiceStart, t-Service] window.

Observation 5: Temporary SSB pattern shift may impact other UEs, e.g. UE’s performing initial access during [t-ServiceStart, t-Service] window.

In a nutshell, RAN4 seems to say this scheme could be possible, but:

- only under certain assumptions

- it requires RAN4 specification work (UE requirements for shifting the SSB pattern)

- it may impact other UEs (e.g. those performing initial access during [t-ServiceStart, t-Service] window)

In addition, RAN4 has further doubts which RAN2 shall attempt to clarify [9]:

In addition, RAN4 would like RAN2 to confirm which one of the following is the correct understanding of parameter ssb-TimeOffset, - Option 1: the relative offset between the SSB of the source satellite and the SSB of the target satellite, and the starting boundaries of SFN0 of the source satellite and the target satellites are aligned at the uplink time synchronization reference point - Option 2: the relative offset between the starting boundaries of SFN0 of the source satellite and the target satellite

As can be seen above (in section 2.1) and in [8], Option 1 is the correct interpretation of the ssb-TimeOffset field. Thus, RAN4 has two related questions:

1) Whether it is feasible to configure ssb-TimeOffset with values other than multiple of 5. 2) How the half-frame indication and SFN indication bits are configured during [t-ServiceStart, t-Service].

The answer to the first question is YES, ssb-TimeOffset provided within SatSwitchWithReSync IE, according to NR RRC [8], can be configured to any value from the (0…159) range.

Proposal 1: RAN2 responds to RAN4 and states ssb-TimeOffset provided within SatSwitchWithReSync can be configured to other value than multiples of 5.

Regarding the second question, there is no explicit half-frame indication or SFN indication specifically for the satellite switching with resynchronization (e.g. inside SatSwitchWithReSync IE). Thus, what the UE gets from MIB and PBCH regarding SFN still holds during [t-ServiceStart, t-Service].

Proposal 2: RAN2 responds to RAN4 and indicates there is no separate half-frame or SFN indication for satellite switching with resynchronization which would apply during [t-ServiceStart, t-Service].

More details on the response LS to RAN4 can be found in Annex A.

## 2.3 	RAN1 Response LS

RAN1 has also responded [10] to the original RAN2 LS [3]. However, as can be seen in [10], RAN1 provides a list of subsequent questions. The first RAN1 doubt is as follows:

Is RAN2 asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service, or whether RAN2 assumes that some RAN1 specification work may be needed?

In our understanding, the main intention of the original RAN2 LS [3] was to check if the existing RAN1 specification supports such behaviour. If that is not the case and RAN2 still believes such behaviour is the best way forward one year after freezing Release 18, RAN1 should be kindly asked to pursue corresponding changes.

Proposal 3: RAN2 responds to RAN1 and indicates the intention was to check if the existing RAN1 specification supports such behaviour.

The second assumption is formulated as follows:

RAN1 assumes that the meaning of “the SSB pattern is changed” is that only the time offset is changed (may be determined by ssb-TimeOffset).

Here RAN1 is right, the assumption made in RAN2 was that time location of the SSB is shifted by applying ssb-TimeOffset.

Proposal 4: RAN2 responds to RAN1 and indicates the “SSB pattern change” was supposed to refer to SSB shift in time by applying ssb-TimeOffset.

The third point made by RAN1 looks as follows:

RAN1 assumes that the SIB19 of the source satellite includes ephemeris information of the target satellite, and that such information will not get outdated by the time the satellite switch occurs.

Indeed, also this assumption made by RAN1 is pertinent. SIB19 comprises SatSwitchWithReSync, wherein ntn-Config associated with target satellite is conveyed. It is expected such information is also valid at the time of satellite switching.

Proposal 5: RAN2 responds to RAN1 and indicates SIB19 comprises SatSwitchWithReSync, wherein ntn-Config associated with target satellite is conveyed. It is expected such information is also valid at the time of satellite switching.

More details on the response LS to RAN1 and RAN4 can be found in Annex A.

Proposal 6: RAN2 considers a draft LS shown in Annex A when preparing a response LS to RAN1 and RAN4.

## 2.4	Use of NCD-SSBs during t-ServiceStart to t-Service period

As can be seen in the response LSs from RAN1 and RAN4 [9][10], there is no clear green light to pursue the changes considered in RAN2 in the end of 2024. Quite the contrary, instead there is a list of questions and potential concerns if such solution is to be introduced still in Release 18 (as described in sections 2.2 and 2.3).

If the issue identified in [2] for soft-satellite switching is to be resolved without significantly impacting the legacy specification, then none of the Options considered can be implemented, as indicated in our previous RAN2 paper [6]. A significant work may be needed also in RAN1 and RAN4.

We propose an alternative approach which takes advantage of the Non-Cell Defining (NCD) SSBs. As a brief reminder, NCD-SSBs are capable of providing DL synchronization but these are not associated with RMSI (Remaining Minimum System Information, including SIB1). The main purpose of NCD-SSBs was to serve in bandwidth parts (BWPs) which do not comprise regular SSBs and help e.g. the RedCap UEs to obtain/maintain connectivity.

Observation 6: NCD-SSBs are capable of providing DL synchronization but are not associated with RMSI.

It is worth noting that NCD-SSBs are configurable by the NW in the orthogonal resources to those resources that are used for the transmission of regular SSBs. The position of NCD-SSBs within the radio frame does not have to be fixed.

Observation 7: NCD-SSBs are configurable by the NW in the orthogonal resources to those resources that are used for the transmission of regular SSBs.

Observation 8: The position of NCD-SSBs within the radio frame does not have to be fixed.

As the result of the abovementioned benefits of NCD-SSBs, we suggest to resolve the SSB timing issue for soft-satellite switching with resynchronization by using NCD-SSBs during the time period (t-ServiceStart, t-Service). The high-level sequence of actions could be as follows:

- After t-ServiceStart the target satellite will start broadcasting NCD-SSBs. There is no transmission of the regular SSBs from the target satellite during that period. Source satellite continues transmitting regular SSBs between the time period (t-ServiceStart, t-Service).

- During the time window (t-ServiceStart, t-Service) the UE acquires synchronization with the target satellite using NCD-SSBs.

- At time t-Service the source satellite stops broadcasting SSBs and covering that area. Target satellite stops the transmission of NCD-SSBs and switches to regular (cell-defining) SSB transmission. UE does not have to reacquire the DL synchronization with the target satellite.

Proposal 7: RAN2 to consider relying on NCD-SSBs to mitigate the SSB overlapping issue for soft-satellite switching with resynchronization.

If the scheme described in Proposal 7 is to be operational then System Information Block (SIB) should convey the details regarding the temporary configuration of NCD-SSBs. Most logically this shall be included in SatSwitchWithReSync IE within SIB19.

Proposal 8: RAN2 considers extending SatSwitchWithReSync in SIB19 by including NCD-SSB configuration which would be valid from t-ServiceStart to t-Service.

A detailed NR RRC change proposal is shown in Annex B.

Proposal 9: RAN2 considers the NR RRC changes shown in Annex B.

# 3	Conclusion

This document has made the following observations:

Observation 1: Neither the conclusions reached at RAN2#128, nor the options analysed in R2-2410971 clearly limited the SSB shift to half-frame granularity.

Observation 2: ssb-TimeOffset defined in SatSwitchWithReSync is expressed in the number of subframes and can be configured to any value from 0 to 159.

Observation 3: RAN4 in the response LS (R4-2502701) provides assumptions under which pre-acquired DL synchronization may be maintained after SSB pattern changes at t-Service.

Observation 4: RAN4 has not considered the scenario with temporary SSB pattern shift when defining the Release 18 UE requirements. Such shift may require additional UE processing.

Observation 5: Temporary SSB pattern shift may impact other UEs, e.g. UE’s performing initial access during [t-ServiceStart, t-Service] window.

Observation 6: NCD-SSBs are capable of providing DL synchronization but are not associated with RMSI.

Observation 7: NCD-SSBs are configurable by the NW in the orthogonal resources to those resources that are used for the transmission of regular SSBs.

Observation 8: The position of NCD-SSBs within the radio frame does not have to be fixed.

and proposed the following:

Proposal 1: RAN2 responds to RAN4 and states ssb-TimeOffset provided within SatSwitchWithReSync can be configured to other value than multiples of 5.

Proposal 2: RAN2 responds to RAN4 and indicates there is no separate half-frame or SFN indication for satellite switching with resynchronization which would apply during [t-ServiceStart, t-Service].

Proposal 3: RAN2 responds to RAN1 and indicates the intention was to check if the existing RAN1 specification supports such behaviour.

Proposal 4: RAN2 responds to RAN1 and indicates the “SSB pattern change” was supposed to refer to SSB shift in time by applying ssb-TimeOffset.

Proposal 5: RAN2 responds to RAN1 and indicates SIB19 comprises SatSwitchWithReSync, wherein ntn-Config associated with target satellite is conveyed. It is expected such information is also valid at the time of satellite switching.

Proposal 6: RAN2 considers a draft LS shown in Annex A when preparing a response LS to RAN1 and RAN4.

Proposal 7: RAN2 to consider relying on NCD-SSBs to mitigate the SSB overlapping issue for soft-satellite switching with resynchronization.

Proposal 8: RAN2 considers extending SatSwitchWithReSync in SIB19 by including NCD-SSB configuration which would be valid from t-ServiceStart to t-Service.

Proposal 9: RAN2 considers the NR RRC changes shown in Annex B.

# References

- R2-2410913 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #128 Orlando, USA, Nov 18th – 22nd, 2024

- R2-2410971 Report of [AT128][301][NR NTN] SMTC issues 3GPP TSG-RAN WG2 Meeting #128 Orlando, USA, Nov 18th – 22nd, 2024

- R2-2410978 LS soft satellite switch with resync 3GPP TSG-RAN WG2 Meeting #128 Orlando, USA, Nov 18th – 22nd, 2024

- 3GPP TS 38.213 NR; Physical layer procedures for control, v18.5.0, January 2025

- R2-2409213 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127bis Changsha, China, Apr 15th – 19th, 2024

- R2-2500329 On SSBs from Source and Target Satellite in Soft Switching with Resynchronization 3GPP TSG-RAN WG2 Meeting #129 Athens, Greece, Feb 17th – 21st, 2025

- R2-2501333 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #129 Athens, Greece, Feb 17th – 21st, 2025

- 3GPP TS 38.331 NR Radio Resource Control (RRC)

- R4-2502701 Reply LS on soft satellite switch with re-sync 3GPP TSG RAN4#114, Athens, Greece, Feb 17th – 21st, 2025

- R1-2501606 Reply LS on soft satellite switch with re-sync 3GPP TSG RAN WG1#120, Athens, Greece, Feb 17th – 21st, 2025

# Annex A – Response LS to RAN1 and RAN4

3GPP TSG-RAN WG2 Meeting #129bis	R2-250xxxx

Wuhan, China, 7th – 11th of April 2025

Title:	Response LS on soft satellite switch with re-sync

Response to:	R4-2502701/R1-2501606

Release:	Release 18

Work Item:	NR_NTN_enh-Core

Source:	TSG RAN WG2

To:	TSG RAN WG4, TSG RAN WG1

Cc:	-

Contact Person:

#### Name:	Jedrzej Stanczak

E-mail Address:	jedrzej.stanczak[at]nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to thank RAN1 and RAN4 for their response LSs. RAN2 has re-discussed the aspects of SSB overlapping in soft satellite switching and would like to provide the following responses:

Questions asked by RAN4:

Regarding the following:

“In addition, RAN4 would like RAN2 to confirm which one of the following is the correct understanding of parameter ssb-TimeOffset, - Option 1: the relative offset between the SSB of the source satellite and the SSB of the target satellite, and the starting boundaries of SFN0 of the source satellite and the target satellites are aligned at the uplink time synchronization reference point - Option 2: the relative offset between the starting boundaries of SFN0 of the source satellite and the target satellite”

RAN2 would like to confirm Option 1 is the correct understanding.

Regarding subsequent questions, related to Option 1:

“1) Whether it is feasible to configure ssb-TimeOffset with values other than multiple of 5. 2) How the half-frame indication and SFN indication bits are configured during [t-ServiceStart, t-Service].”

1) ssb-TimeOffset provided within SatSwitchWithReSync IE, according to NR RRC, can be configured to any value from the (0…159) range.

2) there is no explicit half-frame indication or SFN indication specifically for the satellite switching with resynchronization (e.g. inside SatSwitchWithReSync IE). Thus, what the UE gets from MIB and PBCH regarding SFN still holds during [t-ServiceStart, t-Service].

Questions asked by RAN1:

Regarding the following:

Is RAN2 asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service, or whether RAN2 assumes that some RAN1 specification work may be needed?

The main intention of the original RAN2 LS was to check if the existing RAN1 specification supports such behaviour. Please note RAN2 has not decided yet such solution is to be adopted.

Regarding the following:

RAN1 assumes that the meaning of “the SSB pattern is changed” is that only the time offset is changed (may be determined by ssb-TimeOffset).

Yes, the assumption made in RAN2 was that time location of the SSB is shifted by applying ssb-TimeOffset.

Finally, regarding the following:

RAN1 assumes that the SIB19 of the source satellite includes ephemeris information of the target satellite, and that such information will not get outdated by the time the satellite switch occurs.

Yes, this assumption is pertinent. SIB19 comprises SatSwitchWithReSync, wherein ntn-Config associated with target satellite is conveyed. It is expected such information is also valid at the time of satellite switching.

2. Actions:

To RAN1 and RAN4 :

ACTION: 	RAN2 respectfully asks RAN1 and RAN4 to take into account RAN2 responses and provide further feedback, if necessary.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#130	from 2025-05-19	to 2025-05-23		Malta, MT

RAN2#131	from 2025-08-25	to 2025-08-29		India, IN

# Annex B – Text Proposal for NR RRC

#### –	SIB19

SIB19 contains satellite assistance information for NTN access.

SIB19 information element

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...,

[[

ntn-NeighCellConfigListExt-v1720         NTN-NeighCellConfigList-r17                     OPTIONAL        -- Need R

]],

[[

movingReferenceLocation-r18              ReferenceLocation-r17                           OPTIONAL,       -- Need R

ntn-CovEnh-r18                           NTN-CovEnh-r18                                  OPTIONAL,       -- Need R

satSwitchWithReSync-r18                  SatSwitchWithReSync-r18                         OPTIONAL        -- Need R

]],

[[

satSwitchWithReSync-v18xy                SatSwitchWithReSync-v18xy                       OPTIONAL        -- Need R

]]

}

NTN-NeighCellConfigList-r17 ::=          SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-r17

NTN-NeighCellConfig-r17 ::=              SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

carrierFreq-r17                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

physCellId-r17                           PhysCellId                                      OPTIONAL        -- Need R

}

NTN-CovEnh-r18 ::=                       SEQUENCE {

numberOfMsg4HARQ-ACK-Repetitions-r18     BIT STRING (SIZE(4)),

rsrp-ThresholdMsg4HARQ-ACK-r18           RSRP-Range                                      OPTIONAL        -- Need R

}

SatSwitchWithReSync-r18 ::=              SEQUENCE {

ntn-Config-r18                           NTN-Config-r17,

t-ServiceStart-r18                       INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

ssb-TimeOffset-r18                       INTEGER (0..159)                                OPTIONAL        -- Need R

}

SatSwitchWithReSync-v18xy ::=              SEQUENCE {

ntn-Config-r18                           NTN-Config-r17,

t-ServiceStart-r18                       INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

ssb-TimeOffset-r18                       INTEGER (0..159)                                OPTIONAL,       -- Need R

nonCellDefiningSSB-r18                   NonCellDefiningSSB-r17							 OPTIONAL        -- Need R

}

-- TAG-SIB19-STOP

-- ASN1STOP

SatSwitchWithReSync field descriptions

nonCellDefiningSSB If configured, the UE supporting satellite switching with resynchronization uses this SSB for the purposes for which it would otherwise have used the CD-SSB of the target satellite (e.g. obtaining sync, measurements, RLM, BFD, beam management). It remains valid between t-ServiceStart and t-Service.

ssb-TimeOffset Indicates the time offset of the SSB from target satellite at its uplink time synchronization reference point with respect to the SSB from source satellite at its uplink time synchronization reference point. It is given in number of subframes.

t-ServiceStart Indicates the time information on when the target satellite is going to start serving the area currently covered by the serving satellite. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1st January 1900 (midnight between Sunday, December 31, 1899, and Monday, January 1, 1900). The exact start time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field. The reference point for t-ServiceStart is the uplink time synchronization reference point of the serving satellite.