3GPP TSG-RAN WG2 Meeting #129bis 	R2-2502552

Wuhan, China, 7th – 11th of April 2025	revision of R2-2500332

Agenda item:	8.6.3

Source:	Nokia

Title:	Further Details on L1 Measurement Reporting Enhancements for Rel-19 LTM

WID/SID:	NR_Mob_Ph4 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The phase 4 mobility enhancements work item description captures the measurement enhancements in the [1] as follows:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

Following agreements have been made in the previous RAN2 meetings [2-4]:

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12.	NW can configure a dedicated SR configuration for MR MAC CE transmission.

Agreements made at the most recent RAN2 meeting (RAN2#128 [5]):

Agreements on L1 event-driven MR:

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

- TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

- TTT is applied to the leaving condition.

- Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

- Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

- A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

- Support the truncated measurement report MAC CE.

And the following decisions have been made at RAN2#129 (February 2025) [6]:

Agreements on L1 event-triggered MR:

- Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused".

- Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide).

- Do not create a new resource configuration just for the event triggered measurements.

- Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused".

- Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report.

- Reuse the existing RSRP values and RSRP offset values and their interpretations in TS 38.133 for event-triggered measurement report. Can revisit if RAN4 identifies any issue.

- TimeToTrigger and reportInterval of L3 measurement report can be reused. FFS on additional value(s) of reportInterval.

- Cell specific offset is not supported for LTM L1 event evaluation.

- The trigger quantity for LTM L1 event triggered measurement is the same as the report quantity.

- Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

- Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

- R18 differential RSRP encoding is used as baseline.

- LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

- Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s))

- Truncated MR MAC CE includes the triggered beam information:

- One report config ID (event config ID)

- At least one triggered beam id (SSBRI or CRI) + L1 RSRP

- Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur.

- For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used.

- S-measureConfig is applicable to in the L1 event-triggered MR. No additional change is expected on RRC and MAC.

In this paper we present our views on the remaining open issues for event-triggered L1 measurement reporting framework for LTM.

# 2	Event-triggered reporting for LTM

## On the explicit indication of event condition in MR MAC CE

10.  Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

In the previous RAN2 meeting the need for explicit indication which beams meet the event condition was discussed and agreed. The details of this indication remain FFS. As one solution it was proposed to use individual indication per beam whether the beam triggered the report. However, since the differential RSRP reporting is a Rel-19 baseline, the highest reported RSRP value may not be for the beam that triggered the report. This creates an issue where the additional beams or beam not fulfilling the report criteria are reported with higher RSRP value than the beam that triggered the report.

Observation 1: Using the differential RSRP reporting may lead to the situation where the highest reported RSRP is not for the beam that triggered the report. The beam that triggered the report may not be reported with highest resolution RSRP value.

This means that for the beam that triggered the report, the differential RSRP value may be used which leads to lower resolution of RSRP level due to differential encoding.

The MAC CE encoding and decoding is simplified since the meaningful information is encoded first in the MAC CE. The information that is additional is added last i.e. the RSRI that do not fulfil the reporting criteria.

Observation 2: Reporting the RSRIs that fulfil criteria first can simplify the encoding for the MR MAC CE since the truncated format and non-truncated format are encoded in similar manner.

Due to the issues described in the preceding observations, we think RAN2 shall consider another approach, wherein the order of the information in the MAC CE has a significant meaning. I.e. initial RSRIs correspond to the RS that fulfilled the criteria, subsequent are for those that do not fulfil the criteria.

Observation 3: Reporting the RSRI that fulfilled the even criteria first with absolute RSRP gives the network more exact information on the candidate cell RSRI that triggered the report.

Proposal 1: The RSRIs that have fulfilled the event criteria are reported first in the MR MAC CE. The RSRI that fulfilled the criteria and has the highest RSRP value is first in the list.

Proposal 2: The MR MAC CE contains indication whether the RSRI not fulfilling report criteria will be listed next to the RSRI that have fulfilled the event criteria.

The indication in the MR MAC CE can be as illustrated in the Figure 1. MR MAC CE format where the RSRI fulfilling criteria are reported first. The S field can indicate whether the succeeding RSRI has fulfilled the reporting criteria or whether it would be just additionally reported RSRI. If the S field indicates that following RSRI field will not fulfil the reporting criteria the first reported RSRI is reported with absolute RSRI and the following RSRP would be encoded using differential encoding.

Figure 1. MR MAC CE format where the RSRI fulfilling criteria are reported first.

The proposed approach makes the MAC CE encoding similar for each of the case i.e. in case of truncated MR or non-truncated MR since the beams that fulfilled the reporting criteria are always reported first.

## 2.2 	Time to Trigger and Event Evaluation at MAC Layer

The RAN plenary has revised the Mobility enhancement WI objectives [1] and it is not required anymore to manage potential overlaps between the MIMO WI and Mobility WI.

To prevent too frequent event triggered reporting, a mechanism to perform at least time domain filtering would be needed. It has been agreed that for LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific offset can be applied [4].

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

For the LTM event evaluation, the TTT concept, similar to L3 reporting, was adopted. In the TTT approach, the UE is configured to observe whether the evaluated criteria (e.g. LTM3 event) is fulfilled for the duration of the TTT. In a certain case, the TTT duration could be configured to be ‘0’ which means that if the event criteria is fulfilled, the event and the associated reporting is triggered immediately. As the MAC layer is used for event evaluation, the TTT related procedure (start /stop/expiry) is performed at MAC layer where the event evaluation procedure is defined.

In the agreed operation, depicted in Figure 2, the L1 is providing the measurement results to MAC layer (e.g. on RS1… RSk) that includes the candidate RS measurements and serving cell RS measurements, depending on the LTM event type. MAC layer determines based on the configured event criteria when the event criteria is fulfilled and starts the TTT timer. MAC layer also observes that whether the criteria is fulfilled for the duration of the TTT. Once the TTT expires, the event triggered report is generated, reporting the RS or RSs fulfilling the criteria.

Figure 2. Event evaluation by MAC layer processing the L1 measurements.

Modelling of the TTT timer handling may have the following options:

- either the TTT timer start/stop/expiry is modelled explicitly or

- only the reporting procedure is modelled as result of the evaluation.

Observation 4: Modelling of the TTT timer handling may have the following options: either the TTT timer start/stop/expiry is modelled explicitly or only the reporting procedure is modelled as result of the evaluation.

Proposal 3: Model the TTT timer handling explicitly, i.e. TTT timer start/stop/expiry is modelled at MAC.

Proposal 4: The value of TTT is configurable by NW and one TTT value is configured per LTM reporting configuration.

Proposal 5: The TTT range is to be decided in the later phase of Stage-3 discussions.

## 2.3 	Report on Leave

The measurement framework for NR has numerous components which were defined over several releases, starting from NR Rel-15. One such configurable aspect is reportOnLeave. It is a flag indicating if the UE should report also when one of the cells that previously met a reporting criteria has now left the cellsTriggeredList. A similar mechanism was agreed to be defined for LTM L1 measurements events where the report on leave could report on beam level that the reporting criteria is not fulfilled anymore:

MR can be sent when the leaving condition is met, based on NW configuration.

Nevertheless, RAN2 needs to discuss how report on leave in MAC layer should look like. Among the questions to be answered one can list at least the following:

- Is the report on leave type of event indicated explicitly or implicitly?

- Is the leaving condition an equivalent of the entry condition or can it be configured separately and in a different manner?

Proposal 6: RAN2 considers at least the following list of open issues when designing the report on leave for L1 measurement reporting:

- Is the report on leave type of event indicated explicitly or implicitly?

- Is the leaving condition an equivalent of the entry condition or can it be configured separately and in a different manner?

- Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

We think the same MAC CE should be used for entry and leave report.

Proposal 7: RAN2 agrees that same MR MAC CE format is used for report on leave and for the entry report.

To support reporting beams with on leave criteria it is enough to have one indication per report that the report was triggered due to leaving condition being met. The MR MAC CE that reports on leave information, should only include RSRIs for which the leave condition has been met. The MR MAC CE that reports on leave information should not include any additional beams/RSRIs.

Observation 5: It is enough to have one indication in the MR MAC CE to indicate this report has been triggered on leave.

Proposal 8: RAN2 agrees that MR MAC CE has a single field indicating that the report is for report on leave.

Observation 6: The MR MAC CE that reports on leave information should include only RSRIs for which the report on leave criteria is fulfilled. Reporting of additional beams is not supported.

Proposal 9: If the MR MAC CE is for report on leave it contains only RSRIs that are reported as they have met the event leaving condition.

## 2.4. Truncating MR MAC CE

At RAN2#129 the following has been agreed:

- Truncated MR MAC CE includes the triggered beam information:

- One report config ID (event config ID)

- At least one triggered beam id (SSBRI or CRI) + L1 RSRP

It was also discussed via offline thread when the UE uses truncated MR MAC CE. The following was concluded [6]:

13. Truncated event-triggered L1 measurement report MAC CE can be used when the available grant is not sufficient to accommodate the normal measurement report MAC CE plus subheader

In our opinion this should be clarified further – the UE may use truncated format only when the normal MAC CE is not able to accommodate the RSRIs that fulfil the criteria for the measurement report event (i.e. not just when the total number of N beams cannot be accommodated in the normal MAC CE).

Proposal 10: The UE uses truncated MR MAC CE when it is not able to report all the beams that triggered the report, up to the configured number of beams.

Proposal 11: The truncated MR MAC CE can only include RSRIs that fulfil the triggering conditions.

## 2.5 Periodic Transmission of the MAC CE Report

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

One further discussion point for periodic reporting is the resource allocation. RAN2 should discuss how to support periodic resource allocation for the report. One option is to provide periodic resources when the UE sends an SR. In such cases, the details of UL grant resource allocation via PDCCH can be left to RAN1. Also, if the network does not configure periodic resource allocation, the UE needs to request the resources separately for each MAC CE based measurement reporting. The measurement report should be able to be provisioned on any available UL grant.

Proposal 12: RAN2 agree to support the configuration of periodic resource allocation for event triggered periodic MR MAC CE reporting.

## 2.6 Prohibit Timer for triggering the MAC CE Report

To avoid too frequent measurement reporting by the UE, a prohibit timer could be applied for the event triggered reporting. The prohibit timer could be applied per reporting configuration i.e. the UE would not be allowed to trigger another MR MAC CE for the same reporting configuration while the associated prohibit timer is running.

Observation 7: A prohibit timer could be applied for the LTM event triggered reporting to avoid too frequent measurement reporting.

Proposal 13: RAN2 to support a prohibit timer for event triggered reporting. The prohibit timer is applied per reporting configuration.

## 2.7 Number of reported beams in MR MAC CE

The following has been agreed so far regarding the number of beams and what kind of beams could be reported:

7. For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

The definition of Rel-18 LTM report content in TS 38.331 is as follows:

LTM-ReportContent-r18 ::=     SEQUENCE {

nrOfReportedCells-r18                          ENUMERATED {n1,n2,n3,n4},

nrOfReportedRS-PerCell-r18                     ENUMERATED {n1,n2,n3,n4},

spCellInclusion-r18                            ENUMERATED {true}                                          OPTIONAL -- Need R

}

The maximum number of reported beams in R18 LTM was specified to be 16. We see no strong reason to deviate from this and thus the number of reported beams can be up to N=16 (4 cells x 4 beams per cell) and the value is network configurable.

Observation 8: There is no reason to deviate from the R18 reporting parameters, where reporting of up to 16 beams can configured (4 cells x 4 beams per cell).

However, RAN2 could not resolve this doubt by itself and decided to ask RAN1 what should be the maximum number of beam-level measurement results that can be included in MR MAC CE [8].

Proposal 14: RAN2 postponed the decision on the maximum number for N until the response to [8] is received.

In some case, the number of reported beams not fulfilling the criteria should be limited to prevent the UE from including information that may not be useful for the network. For example, when the maximum number of beams which can be reported is relatively large, but only a few beams fulfil the reporting criteria, it is not beneficial to always include up to N beams in the MR MAC CE. This could unnecessarily consume UL resources that could otherwise be available for reporting other information or sending UL user data.

Also, when truncating the MR MAC CE, any beam not fulfilling the reporting criteria should be omitted, like we have suggested already above.

Observation 9: The number of reported candidate beams that do not fulfil the reporting criteria should be limited.

Proposal 15: A maximum limit should be set for the number of candidate beams in the MR MAC CE that do not fulfil the reporting criteria.

## 2.8 Applying Beam Specific offset

Regarding the beam-specific offset to be used in LTM measurement event evaluation the only relevant RAN2 agreement made as of now was formulated as follows:

6. For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

This aspect was also briefly discussed in post-RAN2#128 e-mail discussion [109], devoted to MAC CR drafting. In the parallel thread “[POST128][108][MOB] RRC running CR” it was also proposed to capture the offset in NR RRC in the following way:

LTM-CSI-ReportConfig information element

-- ASN1START

-- TAG-LTM-CSI-REPORTCONFIG-START

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

...,

eventTriggered-r19                        SEQUENCE {

ltm-EventTriggeredMeasurement-r19             LTM-EventTriggeredResourceConfigId-r19,

eventId-r19                                     CHOICE {

eventLTM2-r19                                     SEQUENCE {

ltm2-Threshold-r19                                MeasTriggerQuantity,

hysteresis-r19                                    Hysteresis,

timeToTrigger-r19                                 TimeToTrigger

},

eventLTM3-r19                                     SEQUENCE {

ltm3-Offset-r19                                   MeasTriggerQuantityOffset,

ltm3-Threshold-r19                                MeasTriggerQuantity,

hysteresis-r19                                    Hysteresis,

timeToTrigger-r19                                 TimeToTrigger

},

eventLTM4-r19                                     SEQUENCE {

ltm4-Threshold-r19                                MeasTriggerQuantity,

hysteresis-r19                                    Hysteresis,

timeToTrigger-r19                                 TimeToTrigger

},

eventLTM5-r19                                     SEQUENCE {

ltm5-Threshold1-r19                               MeasTriggerQuantity,

ltm5-Threshold2-r19                               MeasTriggerQuantity,

hysteresis-r19                                   Hysteresis,

timeToTrigger-r19                                TimeToTrigger

},

...

},

…

}

#### - MeasTriggerQuantityOffset Offsets defined in measurement report configurations (e.g., events defined in ReportConfig-NR, or LTM-CSI-ReportConfig) for triggering measurement reports (e.g., by RRC message MeasurementReport or by MAC CE in TS 38.321 [3]). Values in the unit of 'dB'. -- ASN1START -- TAG-MEASTRIGGERQUANTITYOFFSET-START MeasTriggerQuantityOffset ::= CHOICE { rsrp INTEGER (-30..30), rsrq INTEGER (-30..30), sinr INTEGER (-30..30) } -- TAG-MEASTRIGGERQUANTITYOFFSET-STOP -- ASN1STOP

Based on that, for a report configuration, the offset is configured for each event and should be applied to all RSs/beams in the resource set associated with the report configuration. This could be a simple way to implement beam level offset. The other possible options include inserting the offset to CSI resource configuration or even linking it to a TCI state of the serving beam.

However, for simplicity we propose the following:

Proposal 16: Apply the event evaluation offset for LTM3 on LTM CSI resource configuration level. Each beam in the CSI resource configuration or in reporting configuration is applied with the same offset value.

## 2.9 Event/Reporting Identifier

Basic information included in MR MAC CE:

- Triggered event information (e.g., ReportConfigID)

Since the L1 event triggered reporting supports multiple event types, at least one field in MAC CE should indicate:

- the event for which the reporting is triggered and

- the reporting configuration

For example, a field in the MAC CE should point to the reporting configuration associated with the event and resource configuration for which the reporting is performed.

Proposal 17: The LTM Report MAC CE includes an identifier that identifies the LTM reporting configuration. The LTM reporting configuration is associated with the reporting event type (e.g. LTM3) and the measurement resource configuration for which the reporting is performed.

## 2.10 Reporting Serving Cell Information

10. Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

It makes sense, especially when the LTMx event compares the candidate against the serving beam or serving beam is verified against the threshold.

Proposal 18: Include serving cell RSRP measurement for events for which the current serving beam RS is used for event evaluation i.e. LTM 2/3/5.

In order to consider to inter-working with the beam management procedure, it might be beneficial if the UE includes RS associated with indicated (and activated TCI states) in the event based measurement report. Network can use this to delay LTM cell switch if another serving cell beam is strong, e.g. based on beam management procedure.

Proposal 19: UE can be configured to include RS associated with serving cell’s activated TCI states in the event based measurement reporting.

The MAC CE format could include indication whether the serving cell information is included (e.g. when the MAC CE is truncated).

Proposal 20: MAC CE can comprise an explicit indication whether serving cell/beam information is also included.

## 2.11 Support for mTRP Operation

In RAN1#120 following conclusion was reached:

Conclusion

The following is up to RAN2: Coexistence of LTM event triggered reporting and mTRP operation at serving cells

In RAN2#127bis the following was agreed:

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

Supporting multiple TRP operation at serving cell implies that the UE may have two indicated TCI states. As the UE would be able to derive two measurement RS for the serving cell this would result in the following two issues to be discussed in RAN2:

- which of the indicated TCI states (or perhaps both?) are used for event evaluation?

- inclusion of the serving cell information based on the measurements on RS of indicated TCI states.

Regarding 1), we believe that it might be left to the UE implementation which of the TCI states to use.

Proposal 21: If the UE is configured with mTRP operation in the cell, the UE may select which of the TCI states it uses for event evaluation.

Regardless of which TCI state was used for event evaluation, the UE should include the RSRP measurement of both TCI states as “current beam” information in the MR MAC CE.

Proposal 22: If the network has configured the UE to report current beam information when UE is configured with multi TRP operation in the source cell, the MR MAC CE includes 2 RSRP fields for the current beam information (for the first and second indicated TCI State).

## 2.12 Scheduling Request to indicate a reporting event

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12. NW can configure a dedicated SR configuration for MR MAC CE transmission.

If the SR ID is associated with LTM event triggered reporting, it should be possible to share the same SR ID with SR for other purposes. If the SR is not configured, the MAC CE reporting the event is provisioned by triggering CBRA. The dedicated SR should not be associated with specific event but shared between configured events. Further information on the reported event is provided by the MR MAC CE.

Proposal 23: If dedicated SR is not configured, CBRA procedure (for BSR) is triggered for providing the MAC CE reporting the measurements.

# 3 Activation/Deactivation of Semi-Persistent CSI-RS Resources

RAN1 has agreed to support measurements using semi-persistent (SP) CSI-RSs from candidate cells for gNB-scheduled reporting (non-event triggered reporting) and has sent an LS to RAN2 to inform accordingly [7].

Agreement (RAN1 # 120 Feb 2025) - Confirm the following working assumption made in RAN1#118bis Working Assumption(RAN1#118bis) In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2

For serving cell measurements, SP CSI-RSs associated with the serving cell are activated or deactivated using a MAC-CE that indicates the CSI-RS resource set ID(s) to be activated or deactivated, as specified in Section 6.1.3.12 of TS 38.321.

To extend this functionality for SP CSI-RSs associated with candidate cell(s), the following approaches may be considered:

- When each candidate cell is configured with its own SP CSI-RS resource sets:

  - Approach 1:

    - If activation/deactivation is performed per candidate cell, the existing SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE can be enhanced to indicate whether the activation/deactivation of one or more SP CSI-RS resource sets applies to an LTM candidate cell or a serving cell.

  - Approach 2:

    - To allow the UE to report measurements from multiple candidate cells (as allowed with periodic RSs), activation/deactivation can be performed for multiple candidate cells using the same MAC CE, a new MAC CE could be introduced to allow the activation/deactivation of one or more SP CSI-RS resource sets from one or more candidate cells.

- When an SP CSI-RS resource set contains SP CSI-RSs from multiple candidate cells (e.g., similar to ltm-CSI-RS-ResourceSet):

  - A MAC CE could be introduced that indicates for which CSI-RSs or for which cells the CSI-RSs from the SP CSI-RS resource set are activated/deactivated.

Observation 10: For serving cell measurements, SP CSI-RSs associated with the serving cells are activated or deactivated using a MAC CE that indicates the CSI-RS resource set ID(s) to be activated or deactivated, as specified in Section 6.1.3.12 of TS 38.321.

Proposal 24: For the activation/deactivation of candidate cell SP CSI-RSs, one of the following options can be considered for the provisioning of the MAC CE:

- Enhance the existing SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE.

- Introduce a new MAC CE for activating/deactivating multiple SP CSI-RS resource sets across candidate cells.

- Introduce a MAC CE that specifies which CSI-RSs or cells from the common SP CSI-RS resource set should be activated/deactivated.

# 4	Conclusion

This paper discussed RAN2 aspects of LTM L1 measurement framework which shall be addressed in Release 19. The following observations have been made:

Observation 1: Using the differential RSRP reporting may lead to the situation where the highest reported RSRP is not for the beam that triggered the report. The beam that triggered the report may not be reported with highest resolution RSRP value.

Observation 2: Reporting the RSRIs that fulfil criteria first can simplify the encoding for the MR MAC CE since the truncated format and non-truncated format are encoded in similar manner.

Observation 3: Reporting the RSRI that fulfilled the even criteria first with absolute RSRP gives the network more exact information on the candidate cell RSRI that triggered the report.

Observation 4: Modelling of the TTT timer handling may have the following options: either the TTT timer start/stop/expiry is modelled explicitly or only the reporting procedure is modelled as result of the evaluation.

Observation 5: It is enough to have one indication in the MR MAC CE to indicate this report has been triggered on leave.

Observation 6: The MR MAC CE that reports on leave information should include only RSRIs for which the report on leave criteria is fulfilled. Reporting of additional beams is not supported.

Observation 7: A prohibit timer could be applied for the LTM event triggered reporting to avoid too frequent measurement reporting.

Observation 8: There is no reason to deviate from the R18 reporting parameters, where reporting of up to 16 beams can configured (4 cells x 4 beams per cell).

Observation 9: The number of reported candidate beams that do not fulfil the reporting criteria should be limited.

Observation 10: For serving cell measurements, SP CSI-RSs associated with the serving cells are activated or deactivated using a MAC CE that indicates the CSI-RS resource set ID(s) to be activated or deactivated, as specified in Section 6.1.3.12 of TS 38.321.

And the following has been proposed:

Proposal 1: The RSRIs that have fulfilled the event criteria are reported first in the MR MAC CE. The RSRI that fulfilled the criteria and has the highest RSRP value is first in the list.

Proposal 2: The MR MAC CE contains indication whether the RSRI not fulfilling report criteria will be listed next to the RSRI that have fulfilled the event criteria.

Proposal 3: Model the TTT timer handling explicitly, i.e. TTT timer start/stop/expiry is modelled at MAC.

Proposal 4: The value of TTT is configurable by NW and one TTT value is configured per LTM reporting configuration.

Proposal 5: The TTT range is to be decided in the later phase of Stage-3 discussions.

Proposal 6: RAN2 considers at least the following list of open issues when designing the report on leave for L1 measurement reporting:

- Is the report on leave type of event indicated explicitly or implicitly?

- Is the leaving condition an equivalent of the entry condition or can it be configured separately and in a different manner?

Proposal 7: RAN2 agrees that same MR MAC CE format is used for report on leave and for the entry report.

Proposal 8: RAN2 agrees that MR MAC CE has a single field indicating that the report is for report on leave.

Proposal 9: If the MR MAC CE is for report on leave it contains only RSRIs that are reported as they have met the event leaving condition.

Proposal 10: The UE uses truncated MR MAC CE when it is not able to report all the beams that triggered the report, up to the configured number of beams.

Proposal 11: The truncated MR MAC CE can only include RSRIs that fulfil the triggering conditions.

Proposal 12: RAN2 agree to support the configuration of periodic resource allocation for event triggered periodic MR MAC CE reporting.

Proposal 13: RAN2 to support a prohibit timer for event triggered reporting. The prohibit timer is applied per reporting configuration.

Proposal 14: RAN2 postponed the decision on the maximum number for N until the response to [8] is received.

Proposal 15: A maximum limit should be set for the number of candidate beams in the MR MAC CE that do not fulfil the reporting criteria.

Proposal 16: Apply the event evaluation offset for LTM3 on LTM CSI resource configuration level. Each beam in the CSI resource configuration or in reporting configuration is applied with the same offset value.

Proposal 17: The LTM Report MAC CE includes an identifier that identifies the LTM reporting configuration. The LTM reporting configuration is associated with the reporting event type (e.g. LTM3) and the measurement resource configuration for which the reporting is performed.

Proposal 18: Include serving cell RSRP measurement for events for which the current serving beam RS is used for event evaluation i.e. LTM 2/3/5.

Proposal 19: UE can be configured to include RS associated with serving cell’s activated TCI states in the event based measurement reporting.

Proposal 20: MAC CE can comprise an explicit indication whether serving cell/beam information is also included.

Proposal 21: If the UE is configured with mTRP operation in the cell, the UE may select which of the TCI states it uses for event evaluation.

Proposal 22: If the network has configured the UE to report current beam information when UE is configured with multi TRP operation in the source cell, the MR MAC CE includes 2 RSRP fields for the current beam information (for the first and second indicated TCI State).

Proposal 23: If dedicated SR is not configured, CBRA procedure (for BSR) is triggered for providing the MAC CE reporting the measurements.

Proposal 24: For the activation/deactivation of candidate cell SP CSI-RSs, one of the following options can be considered for the provisioning of the MAC CE:

- Enhance the existing SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE.

- Introduce a new MAC CE for activating/deactivating multiple SP CSI-RS resource sets across candidate cells.

- Introduce a MAC CE that specifies which CSI-RSs or cells from the common SP CSI-RS resource set should be activated/deactivated.

# References

[1]	RP-242356 Revised Work Item: NR mobility enhancements Phase 4 3GPP TSG RAN Meeting #105, Sep 9-12, 2024

[2] R2-2403731 	Report from session on V2X/SL, R19 NES and MOB 3GPP TSG-RAN WG2 Meeting #125bis Changsha, China, April 15 – 19, 2024

[3] R2-2405701	Report from session on V2X/SL, R19 NES and MOB 3GPP TSG-RAN WG2 Meeting #126	 Fukuoka, Japan, May 20 – 24, 2024

[4] R2-2409211 Report from session on V2X/SL, R18/19 MOB, and R19 NES 3GPP TSG-RAN WG2 Meeting #127bis Hefei, China, October 14th – 18th, 2024

[5] R2-2410911 Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES 3GPP TSG-RAN WG2 Meeting #128	, Orlando, USA, November 18th – 22nd, 2024

[6] R2-2501331 Report from session on R17/18 V2X/SL, R18/19 MOB and R19 NES 3GPP TSG-RAN WG2 Meeting #129, Athens, Greece, February 17 – 21, 2025

[7] R1-2501500 LS on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells 3GPP TSG-RAN WG1 Meeting #120, Athens, Greece, February 17 – 21, 2025

[8] R2-2501485 LS on number of beam measurements in the measurement report MAC CE TSG-RAN WG2 Meeting #129, Athens, Greece, February 17 – 21, 2025