3GPP TSG-RAN WG2 Meeting #129bis	R2-2502553

Wuhan, China, 7th – 11th of April 2025

Agenda item:	7.2

Source:	Nokia

Title:	Remaining Rel-18 LTM Corrections

WID/SID:	NR_Mob_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this paper we are addressing several remaining issues for Release 18 LTM. In particular our focus is on the coordination between MCG and SCG when LTM is configured by each of those nodes. We also depict the changes required to 38.306 resulting from the RAN4 LS [8].

# 2	Discussion

## 2.1 	On UE capabilities for NW-triggered early TA acquisition

For Rel. 18 LTM, the following capability is described in 38.306 [1].

| rach-EarlyTA-Measurement-r18 Indicates the maximum number of candidate cells for TA acquisition based on PDCCH ordered CFRA procedure before receiving cell switch command MAC-CE. Power ramping for PRACH retransmission based on PDCCH order indication. UE also supports dropping the serving cell UL to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s). A UE supporting this feature shall also indicate support of ta-IndicationCellSwitch-r18 and at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. For cross-band operation, the capability refers to the source band. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Observation 1: Considering the definition of rach-EarlyTA-Measurement-r18, the number of cells towards which NW can trigger early TA acquisition is limited.

Considering the way the NW-triggered early TA acquisition procedure is specified, the UE needs to perform only the following actions:

- Listen to the PDCCH order of the serving cell.

- Send CFRA preamble to the indicated target cell.

The TA value will then be sent from the NW, which is also responsible for maintaining the TA value. Thus, restricting the number of cells for which NW-triggered early TA acquisition procedure can be triggered does not seem to be needed.

Proposal 1: Update the first sentence to "Indicates whether the UE supports early TA acquisition based on PDCCH ordered CFRA procedure before the cell switch command MAC CE”. Adopt the TP in Annex A for TS 38.306.

## 2.2 	On UE-based TA estimation capability

The capability for performing UE-based TA estimation is defined as follows [1].

| ue-TA-Measurement-r18 Indicates whether the UE supports UE-based TA measurement by indicating the maximum number of candidate cells that the UE maintains the TA for. A UE supporting this feature shall also indicate the support of at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. For cross-band operation, this capability refers to the source band. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Observation 2: Considering the definition of ue-TA-Measurement-r18, the number of cells for which the UE maintains the TA for is limited.

The timeframe associated with UE “maintaining” the UE-measured TA values is not clear. At least the following options are possible:

- UE measures and maintains the TA value starting from the moment when it is configured to perform UE-based TA measurement.

- UE measures and maintains the TA value starting from the moment when it receives the CSC.

Observation 3: The timeframe associated with UE “maintaining” the UE-measured TA values requires further clarification.

Depending on the timeframe associated with UE “maintaining” the UE-measured TA, we believe the capability description might need to updated. For instance, if UE measures and maintains the TA value from the moment when it knows it is configured to perform UE-based TA measurement then the current definition of restricting the number of cells for which the UE can perform UE-based TA measurements is needed. On the other hand, if UE measures and estimates the TA value after reception of LTM CSC, the above restriction may not be needed since UE is anyway instructed to switch to only 1 cell.

Proposal 2: RAN2 to clarify the understanding on the timeframe associated with UE “maintaining” the UE-measured TA and to reflect this in 38.306. The following options can be considered as a starting point (other alternatives are not precluded):

- UE measures and maintains the TA value starting from the moment when it is configured to perform UE-based TA measurement.

- UE measures and maintains the TA value starting from the moment when it receives the CSC.

## 2.3 	On DC LTM configured simultaneously in MCG and SCG

For Rel. 18 LTM, we have agreed that MCG and SCG LTM can be configured but not triggered simultaneously.

RAN2#124 LTM for simultaneous PCell and PSCell change is not supported in Rel 18 RAN2#127bis RAN2 understands that if UE supports all ltm-MCG-NRDC-r18, ltm-MCG-IntraFreq-r18 and ltm-SCG-IntraFreq-r18, the UE is also able to support the configuration of LTM for MCG and LTM for SCG simultaneously.

In RAN2#129 (February) the following was discussed and concluded [2], based on Nokia’s [3]:

Proposal 5: In addition to the number of LTM cells and L1 measurement configuration, MN informs SN about the number of cells for which early synchronization procedures can be performed. Adopt the TP in Annex D in TS 38.331. Will be revisited next meeting. [Ericsson]: We have “the MN indicates the maximum number of LTM candidate configurations the SN is allowed to configure for SCG LTM, to ensure that UE capabilities are not exceeded”. This already implies that MN is responsible that capabilities are not exceeded. We do not need additional signalling for the early TA acquisition procedure. [Nokia]: Consider the number of LTM candidate configurations to the SN doesn’t mean number of cells for which early synchronization procedures. [Huawei]: Understand the rational and need more time to think of. Comeback session: [Nokia]: Companies understand there is no issue on the number of cells for which early synchronization procedures, however there would be some issue regarding other UE capabilities on TCI states, therefore suggest to revisit it next meeting all together.

In Section 5.3.5.18.1 in 38.331 [4] we have captured the following:

In NR-DC, the UE may receive two independent ltm-Config: - an ltm-Config associated with the MCG that is included within an RRCReconfiguration message received via SRB1; and - an ltm-Config associated with the SCG that is included within an RRCReconfiguration message either received via SRB3, or, alternatively, embedded in an RRCReconfiguration message received via SRB1. In case the UE receives two independent ltm-Config: - the UE maintains two independent ltm-Config; - the UE maintains two independent VarLTM-ServingCellNoResetID, one associated with each ltm-Config; - the UE maintains two independent VarLTM-ServingCellUE-MeasuredTA-ID, one associated with each ltm-Config; - the UE independently performs all the procedures in clause 5.3.5.18 for each ltm-Config and the associated VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID unless explicitly stated otherwise.

Moreover, in Section 7.3 in 37.340 [5] we have captured the following:

For LTM operation, the MN indicates the maximum number of LTM candidate configurations the SN is allowed to configure for SCG LTM, to ensure that UE capabilities are not exceeded. The SN can also request the MN for a new value of the maximum number of allowed LTM candidate configurations to configure for SCG LTM, and it is up to the MN whether to accommodate the SN request. If the SN receives from the MN a new value for the maximum number of LTM candidate configurations, it is SN responsibility to ensure that its configured LTM candidate configurations to comply with the new limit. For the LTM related UE capabilities about L1 measurements requiring coordination between the MN and the SN, it is up to the MN to decide on how to resolve the dependency between MN and SN configurations and ensure that the UE capabilities are not exceeded. The MN then provides the resulting UE capabilities about L1 measurements usable for SCG configuration to the SN. The SN can also request a new value for such UE capabilities for L1 measurements, and it is up to the MN whether to accommodate the SN request. If the SN receives from the MN a new value for such UE capabilities for L1 measurements, it is SN responsibility to ensure that its configured L1 measurement configurations to comply with the new limit.

In 38.331 [5], the above text is reflected in the following IE:

CG-ConfigInfo message

-- ASN1START

-- TAG-CG-CONFIG-INFO-START

CG-ConfigInfo ::=               SEQUENCE {

-- Text Omitted

[[

maxNumberLTM-CandidatesSCG-r18   INTEGER(0..maxNrofLTM-Configs-r18)                               OPTIONAL

]],

[[

allowedL1-MeasConfigNRDC-r18     L1-MeasConfigNRDC-r18                                            OPTIONAL

]]

-- Text Omitted

-- TAG-CG-CONFIG-INFO-STOP

-- ASN1STOP

Looking at the above agreements and specifications text, it is clear that simultaneous configuration for MCG and SCG LTM is possible. As long as both MCG and SCG LTM are configured at the same time for a UE, either MN or SN can trigger RACH-based LTM.

Observation 4: Rel. 18 supports simultaneous configuration, but not simultaneous triggering of RACH-based MCG and SCG LTM.

For Rel. 18 LTM, we have defined several enhancements, which are signaled from the UE to the NW through the UE capabilities, such as:

- Early DL synchronization (e.g., number of DL TCI states that can be activated for a UE)

- Early UL synchronization (e.g., number of cells for which early TA acquisition or UE-based TA acquisition can be performed)

- Early configuration decoding

Examples of relevant capabilities regarding early DL synchronization as described in 38.306 [1] are given below:

| ltm-MAC-CE-JointTCI-r18 Indicates whether the UE supports MAC-CE activated joint LTM TCI states. This capability comprises the following parameters: - qcl-Resource-r18 indicates the supported QCL source RS for MAC-CE activated DL/UL LTM TCI states configuration. - maxNumberJointTCI-PerCell-r18 indicates the maximum number of MAC-CE activated joint LTM TCI states per candidate cell. - maxNumberJointTCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated joint LTM TCI states across candidate cells and serving cell TCI states across serving cells in the band. A UE supporting this feature shall also indicate support of ltm-BeamIndicationJointTCI-r18. NOTE: The maximum number of MAC-CE activated joint TCI states across all servings cells is limited by of unifiedJointTCI-r17. For cross-band operation, this capability refers to the source band. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| ltm-MAC-CE-SeparateTCI-r18 Indicates whether the UE supports MAC-CE activated DL/UL LTM TCI states. This capability comprises the following parameters: - qcl-Resource-r18 indicates the supported QCL source RS for MAC-CE activated DL/UL LTM TCI states configuration. - maxNumberDL-TCI-PerCell-r18 indicates the maximum number of MAC-CE activated DL TCI states per candidate cell. - maxNumberUL-TCI-PerCell-r18 indicates the maximum number of MAC-CE activated UL TCI states per candidate cell. - maxNumberDL-TCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated LTM DL TCI states across all candidate cells and serving cell DL TCI states across all serving cells. - maxNumberUL-TCI-AcrossCells-r18 indicates the maximum number of MAC-CE activated UL TCI states across all candidate cells and serving cell UL TCI states across all serving cells in the band. A UE supporting this feature shall also indicate support of ltm-BeamIndicationSeparateTCI-r18. The maximum number of MAC-CE activated DL/UL TCI states across all servings cells is limited by unifiedSeparateTCI-r17. For cross-band operation, this capability refers to the source band. | Band | No | N/A | N/A |

Considering the excerpts from the specification text, only LTM related capabilities about L1 measurements are exchanged between MN and SN, i.e., allowedL1-MeasConfigNRDC-r18. Thus, if both MCG and SCG LTM are simultaneously configured, and UE supports capabilities such as early DL synchronization, early decoding, etc., neither MN nor SN can trigger additional procedures (e.g., early synchronization). The reason for this is because MN does not know which UE capabilities are currently being configured from SN. For instance, consider the early TCI state activation. Based on the capabilities highlighted in the above table, UE reports the maximum number of (joint or separate) TCI states that can be activated per candidate cell and across all candidate cells and serving cells. In this case, MN does not know the number of TCI states for cells configured as SCG LTM candidates for which SN has triggered early TCI state activation, so it does not know if it can trigger procedure for additional TCI states for cells configured as MCG LTM candidates without violating the UE capabilities.

Observation 5: Based on the current specification, if LTM is configured in both MCG and SCG, the MN or SN cannot independently trigger e.g., early DL synchronization while ensuring that the UE capability on the maximum number of activated TCI states across all candidate cells and the serving cell is not exceeded since they are not aware of what the other node has triggered.

Considering that the true gains of LTM come from UEs supporting such capabilities (even if these are not mandatory features), we think that the specification should provide support for the coordination in case both MCG and SCG LTM are configured towards the UE.

Proposal 3: In addition to the number of LTM cells and L1 measurement configuration, MN informs SN about the number of cells or/and TCI states for which early DL synchronization procedures can be performed. Adopt the TP in Annex B in TS 38.331.

Depending on the conclusions for the discussion in Section 2.1 and 2.2 above, the number of cells for which early TA acquisition can be configured towards the UE when both MCG and SCG LTM are configured may also need to be exchanged between MN and SN. Therefore, we propose the following:

Proposal 4: MN informs SN about the number of cells which early UL synchronization procedures can be performed. Adopt the TP in Annex B in TS 38.331.

## 2.4 	On UE capability for L1-RSRP measurements (RAN4 LS)

RAN4 in their LS [8] indicated they have changed the description of FG 39-3-1, FG 39-3-3, FG 39-3-5 and FG 39-3-6. As a result, RAN4 expect us to pursue corresponding changes in TS 38.306 and update the following capabilities:

- Number of frequency layers for L1-RSRP measurement (maxFreqLayersL1-Meas-r18)

- Number of total cells to be measured (supportedMaxCellsWithoutGapsL1-Meas-r18)

- Number of SSB resources for L1-RSRP measurement per frequency layer (maxSSB-PerFreqLayerL1-Meas-r18)

- Number of total SSB resources to be measured (supportedMaxSSB-L1-Meas-r18)

The requested updates are related to counting the serving and candidate LTM cells for SSB-based L1-RSRP measurements. We show how to adopt these changes in Annex C and Annex D.

Proposal 5: Adopt the changes to TS 38.306 and TS 38.331 shown in Annex C and Annex D, respectively.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: Considering the definition of rach-EarlyTA-Measurement-r18, the number of cells towards which NW can trigger early TA acquisition is limited.

Observation 2: Considering the definition of ue-TA-Measurement-r18, the number of cells for which the UE maintains the TA for is limited.

Observation 3: The timeframe associated with UE “maintaining” the UE-measured TA values requires further clarification.

Observation 4: Rel. 18 supports simultaneous configuration, but not simultaneous triggering of RACH-based MCG and SCG LTM.

Observation 5: Based on the current specification, if LTM is configured in both MCG and SCG, the MN or SN cannot independently trigger e.g., early DL synchronization while ensuring that the UE capability on the maximum number of activated TCI states across all candidate cells and the serving cell is not exceeded since they are not aware of what the other node has triggered.

Proposal 1: Update the first sentence to "Indicates whether the UE supports early TA acquisition based on PDCCH ordered CFRA procedure before the cell switch command MAC CE”. Adopt the TP in Annex A for TS 38.306.

Proposal 2: RAN2 to clarify the understanding on the timeframe associated with UE “maintaining” the UE-measured TA and to reflect this in 38.306. The following options can be considered as a starting point (other alternatives are not precluded):

- UE measures and maintains the TA value starting from the moment when it is configured to perform UE-based TA measurement.

- UE measures and maintains the TA value starting from the moment when it receives the CSC

Proposal 3: In addition to the number of LTM cells and L1 measurement configuration, MN informs SN about the number of cells or/and TCI states for which early DL synchronization procedures can be performed. Adopt the TP in Annex B in TS 38.331.

Proposal 4: MN informs SN about the number of cells which early UL synchronization procedures can be performed. Adopt the TP in Annex B in TS 38.331.

Proposal 5: Adopt the changes to TS 38.306 and TS 38.331 shown in Annex C and Annex D, respectively.

# References

- 3GPP TS 38.306 NR UE capabilities, V18.5.0 (2025-03)

- R2-2501331 Report from session on R17/18 V2X/SL, R18/19 MOB and R19 NES 3GPP TSG-RAN WG2 Meeting #129, Athens, Greece, February 17th  – 21st 2025

- R2-2500333 On the Remaining Rel-18 LTM Aspects and Corrections 3GPP TSG-RAN WG2 Meeting #129, Athens, Greece, February 17th  – 21st 2025

- 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification, V18.5.0 (2024-03)

- 3GPP TS 37.340 Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Overall Description; Stage-2, V18.5.0 (2024-03)

- R2-2410442 Miscellaneous Rel-18 LTM Aspects and Corrections 3GPP TSG-RAN WG2 Meeting #128	Orlando, USA, November 18th – 22nd, 2024

- R2-2410911 Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES 3GPP TSG-RAN WG2 Meeting #128	Orlando, USA, November 18th – 22nd, 2024

- R4-2502619 LS on UE capability for L1-RSRP measurement in LTM 3GPP TSG-RAN WG4 Meeting #114 Athens, Greece, 17 – 21 February, 2025

# Annex A – TP for TS 38.306 – UE capability for NW-triggered early TA acquisition

Update the description of the capability as follows:

| rach-EarlyTA-Measurement-r18 Indicates whether the UE supports early TA acquisition based on PDCCH ordered CFRA procedure before receiving cell switch command MAC-CE. Power ramping for PRACH retransmission based on PDCCH order indication. UE also supports dropping the serving cell UL to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s). A UE supporting this feature shall also indicate support of ta-IndicationCellSwitch-r18 and at least one of ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18. For cross-band operation, the capability refers to the source band | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

# Annex B – TP for TS 38.331 – Early Synchronization for MCG and SCG LTM simultaneously configured

It is sufficient to update CG-ConfigInfo. For simplicity, in the sequel we show an example of adding the IEs related to early TCI state activation. Depending on the group discussion, additional capabilities can also be considered.

-- ASN1START

-- TAG-CG-CONFIG-INFO-START

CG-ConfigInfo ::=               SEQUENCE {

-- Text omitted

[[

maxNumberLTM-CandidatesSCG-r18   INTEGER(0..maxNrofLTM-Configs-r18)                               OPTIONAL

]],

[[

allowedL1-MeasConfigNRDC-r18     L1-MeasConfigNRDC-r18                                            OPTIONAL

]],

[[

-- Proposal 3

maxNumberJointTCI-AcrossCells-r18   INTEGER(1..32)                               OPTIONAL,

maxNumberDL-TCI-AcrossCells-r18   INTEGER (1..32)                               OPTIONAL,

maxNumberUL-TCI-AcrossCells-r18   INTEGER (1..32)                               OPTIONAL,

-- Proposal 4

rach-EarlyTA-Measurement-r18   INTEGER(0..maxNrofLTM-Configs-r18)                               OPTIONAL,

ue-TA-Measurement-r18   INTEGER (0..maxNrofLTM-Configs-r18)                               OPTIONAL

]]

}

-- Text omitted

-- TAG-CG-CONFIG-INFO-STOP

-- ASN1STOP

# Annex C – 38.306 TP (related to RAN4 LS in R4-2502619)

First Modified Subclause

| maxFreqLayersL1-Meas-r18 Indicates the number of frequency layers for L1-RSRP measurement. This capability signalling comprises of the following parameters: - supportedMaxIntraInterFreqLayersWithoutGaps-r18 indicates the maximum number of frequency layers UE can measure for intra- and inter-frequency without measurement gaps L1-RSRP measurement. A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 and/or interFreqSSB-L1-MeasWithoutGaps-r18. - supportedMaxInterFreqLayersWithGaps-r18 indicates the maximum number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps. A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18. For LTM, only the frequency layers which are configured with SSB-based L1-RSRP measurements on candidate LTM cells in LTM-CSI-ResourceConfig-r18 are considered. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Next Modified Subclause

| supportedMaxCellsWithoutGapsL1-Meas-r18 Indicates the max number of total cells of serving cells and neighbouring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. For LTM, candidate cell(s) and serving cells for SSB-based L1-RSRP measurement are considered, regardless of whether SSB-based L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Next Modified Subclause

| maxSSB-PerFreqLayerL1-Meas-r18 Indicates the maximum number of SSB resources for L1-RSRP measurement per frequency layer UE can measure. This capability signalling comprises of the following parameters: - supportedMaxSSB-PerFreqLayersWithoutGaps-r18 indicates the max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps. A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. - supportedMaxSSB-PerFreqLayersWithGaps-r18 indicates the max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps. A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18. For LTM, candidate cell(s) and serving cells for SSB-based L1-RSRP measurement are considered, regardless of whether SSB-based L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

First Modified Subclause

supportedMaxSSB-L1-Meas-r18 Indicates the max number of total SSB resources of serving cells and neighbouring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. For LTM, candidate cell(s) and serving cells for SSB-based L1-RSRP measurement are considered, regardless of whether SSB-based L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18.

End of Modified Subclauses

# Annex D – 38.331 TP (related to RAN4 LS in R4-2502619)

-- R4 39-3-5: Number of SSB resources for L1-RSRP measurement per frequency layer

maxSSB-PerFreqLayerL1-Meas-r18                        SEQUENCE {

supportedMaxSSB-PerFreqLayerWithoutGaps-r18            INTEGER (1..8)                            OPTIONAL,

supportedMaxSSB-PerFreqLayerWithGaps-r18               INTEGER (1..8)                            OPTIONAL

}

maxSSB-PerFreqLayerL1-Meas-v18xy                          SEQUENCE {

supportedMaxSSB-PerFreqLayerWithoutGaps-v18xy          ENUMERATED {n1, n2, n3, n4, n5, n6, n7, n8, n12, n16, n20, n24}                            OPTIONAL,

supportedMaxSSB-PerFreqLayerWithGaps-r18               INTEGER (1..8)                            OPTIONAL

}