3GPP TSG-RAN2 Meeting #129bis	R2-2502561

# Wuhan, China, Apr 7th – 12th, 2025

Agenda item:		4.1

Source:	Nordic Semiconductor ASA, Gatehouse

Title:					Correction to explicit indication of epoch in SIB31

Document for:		Discussion and Decision

# Introduction

In R17 upon introduction of SIB31, two possible methods of epoch time were agreed: 1) default epoch time is set to the end of SI window when explicit IE is not configured 2) explicit IE of SF and SFN is included in the SIB31 to indicated epoch time.

ServingSatelliteInfo-r17 ::=	SEQUENCE {

ephemerisInfo-r17			CHOICE {

stateVectors				EphemerisStateVectors-r17,

orbitalParameters			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r17			SEQUENCE {

nta-Common-r17					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r17				INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r17	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

ul-SyncValidityDuration-r17		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900},

epochTime-r17					SEQUENCE {

startSFN-r17					INTEGER (0..1023),

startSubFrame-r17				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Offset-r17					INTEGER (0..1023),

k-Mac-r17						INTEGER (1..512)				OPTIONAL,	-- Need OP

...

}

When default epoch time is used, there is no ambiguity between network and UE, and all is clear. However, when explicit epoch is used the specification states that it indicates current or next upcoming SFN of frame where the SIB31 is received.

epochTime Epoch time of the satellite ephemeris data and common TA parameters, see TS 36.213 [23]. This field also indicates the epoch time for the reference location of earth moving cells if present. The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point of the serving cell. epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. For serving cell, the startSFN indicates the current SFN or the next upcoming SFN after the frame where the message indicating the epochTime is received. If the field is absent, the epoch time is the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIB31(-NB) is transmitted. E-UTRAN always includes epochTime when SIB31(-NB) is provided through dedicated signalling. In case of handover or conditional handover, this field is based on the timing of the target cell, i.e. the startSFN and startSubFrame number indicated in this field refers to the SFN and sub-frame of the target cell, and UE considers the target cell epoch time (indicated by the startSFN and startSubFrame in this field) to be the frame nearest to the frame where RRCConnectionReconfiguration message is received. In case of handover or conditional handover, the reference point for epoch time of the target NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point of the target cell.

However, this is very ambiguous. For example, if SI window is 160ms, and SIB31 TBS is configured to use 8 subframes, and there are 4 repetitions configured within SI window as shown in Figure 1 below. Each repetition can be in current SFN or due to postponing leak into next SFN.

Figure 1 Illustration of an issue of ambiguity of "where the message is received" in NB-IoT

Firstly, eNB does not know when message indicating EpochTime is received by UE. In which of the repetitions, this depends on UE SNR.  Secondly, the exact reference time of “when message indicating EpochTime is received by UE” is extremely important for UE to determine correct H-SFN.

Specification can be read with different interpretations:

Interpretation#1: Next upcoming SFN is one-and-only SFN after SFN where message was received. However, this interpretation is infeasible, because same message is transmitted within SI window, and different UEs may receive message in 4 different SFNs.

Interpretation#2: Next upcoming SFN is any SFN after SFN where message was received. This interpretation may work, if eNB indicates Epoch SFN to be SFN of 4th Rep or any SFN after that. If UE knows that 4th Rep SFN is the reference point, it may recover H-SFN unambiguously and still allow 10.24s flexibility for eNB to indicate SFN.

Proposal: Clarify in 36.331 specification or conclude in RAN2 that for

epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. For serving cell, the startSFN indicates the current SFN or the next upcoming SFN after the frame where the message indicating the epochTime is received.

- Alt1: the “next upcoming SFN” means any SFN after SFN n that is carrying last subframe of last repetition of message within SI window until SFN n+1023.

- Alt2: the “next upcoming SFN” means any SFN after SFN n being the last subframe of SI window until SFN n+1023.

# Conclusions

To ensure that Explicit epoch time can be used in the field, we propose adopting one of the following proposals:

Alt1: the “next upcoming SFN” means any SFN after SFN n that is carrying last subframe of last repetition of message within SI window until SFN n+1023.

TS36.331 Section 6.3.1

SystemInformationBlockType31 field descriptions

epochTime Epoch time of the satellite ephemeris data and common TA parameters, see TS 36.213 [23]. The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point. epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. For serving cell, the startSFN indicates any SFN after the SFN n that is carrying the last subframe of the last repetition of the message within SI-window ,where the message indicating the epochTime is received, until SFN n+1023. If the field is absent, the UE uses the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIB31(-NB) is transmitted. E-UTRAN always includes epochTime when SIB31(-NB) is provided through dedicated signalling. In case of handover or conditional handover, this field is based on the timing of the target cell, i.e. the startSFN and startSubFrame number indicated in this field refers to the SFN and sub-frame of the target cell, and UE considers the target cell epoch time (indicated by the startSFN and startSubFrame in this field) to be the frame nearest to the frame where RRCConnectionReconfiguration message is received.

Alt2: the “next upcoming SFN” means any SFN after SFN n being the last subframe of SI window until SFN n+1023.

TS36.331 Section 6.3.1

SystemInformationBlockType31 field descriptions

epochTime Epoch time of the satellite ephemeris data and common TA parameters, see TS 36.213 [23]. The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point. epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. For serving cell, the startSFN indicates any SFN after the SFN n that is the last subframe of the SI-window .where the message indicating the epochTime is received, until SFN n+1023. If the field is absent, the UE uses the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIB31(-NB) is transmitted. E-UTRAN always includes epochTime when SIB31(-NB) is provided through dedicated signalling. In case of handover or conditional handover, this field is based on the timing of the target cell, i.e. the startSFN and startSubFrame number indicated in this field refers to the SFN and sub-frame of the target cell, and UE considers the target cell epoch time (indicated by the startSFN and startSubFrame in this field) to be the frame nearest to the frame where RRCConnectionReconfiguration message is received.