3GPP TSG-RAN WG2 Meeting #129bis	R2-2502563

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.7.4.2

Source:	China Telecom

Title:	Remaining Issues on DSR Enhancement

Document for:	Discussion and Decision

- Introduction

In the previous meetings, the agreements one DSR enhancements were made as follow.

- RAN2#127bis: - DSR MAC CE details For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - RAN2#129: One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE. FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size. Different LCGs may be configured with different number of reporting thresholds. If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold. Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold). Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

The paper aims to discuss the remaining issues on DSR enhancements and provide some considerations.

2 Discussion

## 2.1	DSR MAC CE

As we all know, the legacy DSR MAC CE can indicate at most one reporting threshold for each LCG. To indicate multiple reporting thresholds for each LCG, the enhanced DSR MAC CE is introduced in the previous meetings. Regarding the coexistence of the new DSR MAC CE and legacy DSR MAC CE, if all LCGs are configured with at most one pair of remaining time and buffer size information to be reported, the legacy DSR MAC CE could be used. Otherwise, new DSR MAC CE will be employed. In other words, new DSR MAC CE will be always used when at least one LCG is configured with multiple reporting thresholds.

Proposal 1: New DSR MAC CE is always used when at least one LCG is configured with multiple reporting thresholds.

Furthermore, if the proposed new MAC CE format is adopted, the legacy DSR MAC CE can be considered a special case of the new DSR MAC CE, e.g., all extension bits E are set to 0. Therefore, the new DSR MAC CE can be used to indicate one or multiple reporting thresholds for LCG. In other words, there is no need to configure both the legacy and new DSR MAC CE simultaneously.

Proposal 2: Legacy and new DSR MAC CE are not allowed to be sent to one single UE.

## 2.2	Data volume calculation

In the last meeting, the following options were proposed regarding to the data volume calculation.

- Option 1. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about the issue of LCH only having the control PDU and retransmitted data at the next meeting.

- Option 2. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest configured reporting threshold.

- Option 3. PDCP considers Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about how RLC considers Control PDU and retransmitted data at the next meeting.

Per TS38.321, the buffer size field in DSR MAC CE specifies the total volume of delay-critical UL data for an LCG, and the buffer size is calculated according to the data volume calculation procedure for the associated RLC in TS38.322 and the data volume calculation procedure for the associated PDCP in TS 38.323.

In TS38.322, it specifies that the RLC layer should consider the following fields as part of the delay-critical data volume calculation.

- RLC data PDUs that are pending for retransmission (RLC AM)

- STATUS PDU

And in TS38.323, it specifies that the PDCP layer should consider the following fields as part of the delay-critical data volume calculation.

- the PDCP Control PDUs;

- for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13;

- for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

It can be observed that the RLC entity prioritizes the control PDU and retransmitted RLC data PDUs. Similarly, the PDCP entity also priorities the control PDUs and retransmitted PDCP data PDUs/ PDCP SDUs over other delay-critical data transmission. Therefore, it is reasonable to consider the control PDUs and the PDCP SDUs/PDCP PDUs to be retransmitted in the buffer size calculation associated with the smallest remaining time threshold. In other words, with regard to data volume calculation, it is proposed to support Option 2, i.e., both PDCP and RLC consider Control PDU and retransmitted data into the shortest configured reporting threshold.

Proposal 3: For data volume calculation, it is proposed to support Option 2, i.e., both PDCP and RLC consider Control PDU and retransmitted data into the shortest configured reporting threshold.

- Conclusions

Based on the above analysis, we have the following proposals:

Proposal 1: New DSR MAC CE is always used when at least one LCG is configured with multiple reporting thresholds.

Proposal 2: Legacy and new DSR MAC CE are not allowed to be sent to one single UE.

Proposal 3: For data volume calculation, it is proposed to support Option 2, i.e., both PDCP and RLC consider Control PDU and retransmitted data into the shortest configured reporting threshold.

- References

- R2-25xxxxx, Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece