3GPP TSG-RAN2 Meeting #129bis	R2-2502589

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.11.3

Source:	Qualcomm Incorporated

Title:	Other aspects of SBFD

WID/SID:	NR_duplex_evo-Core – Release 19

Document for:	Discussion and Decision

# Introduction

RAN plenary #102 approved the work item on NR duplex operation, Sub-Band Full Duplex (SBFD) and the latest WID is in [1]. The objective of the work item is to specify SBFD operation at the gNB side within a TDD carrier. The objectives include to specify the semi-static indication of time and frequency domain locations of the SBFD subbands to the UE and specify UE transmission, reception and measurement procedures in SBFD and non-SBFD symbols. The other objectives of the work item are to study the enhancements for CLI handling, and RAN2 will focus on the signaling design for UL resource muting for PUSCH, and L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework.

In RAN2 #127, the following agreements were made about other aspects of SBFD.

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. FFS whether/how early indication is used during a SBFD RA procedure. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed. Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

In RAN2 #127bis, the following agreements were made about other aspects of SBFD.

For UL resource muting for PUSCH, the configuration of time and frequency location for UL resource muting should be introduced based on R1 agreement. For L1 based UE-to-UE CLI measurement mechanism, the configuration of periodic, semi-persistent or aperiodic UE-to-UE CLI measurement resource (set) should be introduced based on R1 agreement. For L1 based UE-to-UE CLI reporting mechanism, the configuration of report quantities should be introduced based on R1 agreement.

RAN2 #128 made the following agreements about the other aspects of SBFD.

Prioritization of SBFD cells / frequencies during cell reselection is not considered. RAN2 wait for input from the other WGs regarding whether for inter-cell CSI-RS measurements, UE needs to be provided with information of the SBFD configuration of neighbouring cells.

RAN2 #129 has agreements about the other aspects of SBFD.

A new SP CLI measurement resource set activation/deactivation MAC CE is introduced to activate/deactivate the SP CLI measurement resource.

In this contribution, we would like to discuss the configuration of SBFD time and frequency locations, the configuration signaling of the L1 based inter-UE CLI measurement and reporting, and RAN2 impacts on transmission, reception in SBFD symbols.

# Discussion

RAN1 has agreed to support at least cell-specific configuration of SBFD time and frequency and will further discuss the need for additional UE-specific SBFD time and frequency configuration.

Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

In our understanding, to accommodate SBFD operation for RRC_IDLE/INACTIVE, UE-dedicated frequency configurations are not applicable and cell-specific configuration frequency configuration are needed. For semi-static indication of SBFD subbands time location, it is desirable to configure together with the existing parameter with modification, i.e., TDD-UL-DL-ConfigCommon.

RAN2 #127 meeting agreed that cell-specific SBFD time/frequency configuration is provided by SIB1 (or via RRC dedicated signaling to covey cell specific configuration). Regarding the UE-specific configuration, RAN2 can wait for RAN1 further discussion and decision.

Proposal 1: For UE-specific dedicated RRC configuration on SBFD time/frequency configuration, RAN2 wait RAN1 further decision on whether support.

RAN1 #119 made following agreements regarding the CLI measurement configuration.

Agreement Define a new IE SRS-RSRP-MeasurementResourceSet containing a set of SRS-RSRP measurement resource(s) SRS-RSRP-MeasurementResource for L1 SRS-RSRP measurement - Configuration of slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured (already agreed in RAN1#118bis) - FFS: Other configurations Define SRS-RSRP-MeasurementResource with the following parameters: - Legacy SRS Resource IE - FFS: Other parameters Agreement Define a new IE CLI-RSSI-MeasurementResourceSet containing a set of CLI-RSSI measurement resource CLI-RSSI-MeasurementResource for L1 CLI-RSSI measurement. - Configuration of the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured (already agreed in RAN1#118bis) Define CLI-RSSI-MeasurementResource with the following parameters: - CLI-RSSI measurement resource ID - Starting PRB index - Number of PRBs - Starting symbol of the CLI-RSSI resource within a slot - Number of symbols of the CLI-RSSI resource within a slot - Periodicity and slot offset for the CLI-RSSI resource - FFS: Other parameters

In the existing CSI framework, the CSI-measConfig is used to manages the CSI-RS/CSI-IM resources pool along with the CSI-ReportConfig, and CSI-ResourceConfig defines a group of CSI-RS/CSR-IM resource. According to RAN1 agreements, two new separate CLI resource configurations should be introduced, i.e., SRS-RSRP resource config and CLI-RSSI resource config. The SRS-RSRP resource configure contains a set of SRS-RSRP measurement resource(s) for L1 SRS-RSRP measurement. The CLI-RSSI resource configuration contains a set of CLI-RSSI measurement(s) resource for L1 CLI-RSSI measurements

Proposal 2: For L1 based UE-to-UE CLI measurement and reporting configuration based on the existing CSI framework, two new separate CLI measurement resource configurations, i.e., SRS-RSRP resource set and CLI-RSSI resource set are introduced under CSI-ResourceConfig. Each resource set contains a set of measurement resources, i.e., SRS-RSRP resource and CLI-RSSI resource separately.

RAN1 #119 further made following agreements on activation/deactivation SP CLI measurement resource set.

Agreement For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of periodic CLI measurement resources, the TCI state(s) with QCL typeD for the CLI measurement resources in the periodic CLI measurement resource set are configured per CLI measurement resource (either for all resources in the set or none) by higher-layer parameter - For each periodic CLI measurement resource, this higher-layer parameter provides a reference to one TCI-State in TCI-States for providing the QCL source and QCL typeD. - If the TCI state is not configured, the UE QCL assumptions are the default rules agreed in RAN1#118bis For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE - TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none. - If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis

For aperiodic L1 based UE-to-UE CLI reporting on SP CLI measurement resources, a new SP CLI measurement resource set activation/deactivation MAC CE should be introduced to activate and deactivate the SP CLI measurement resource. Similar to SP CSI-RS/CSI-IM resource set activation/deactivation MAC CE, the following fields could be included, i.e., A/D field, serving cell/BWP ID field, CLI-RSSI/SRS-RSRP resource set ID field and the corresponding TCI state ID fields.

Proposal 3: In the new SP CLI measurement resource set activation/deactivation MAC CE, the following fields are included, i.e., A/D field, serving cell/BWP ID field, CLI-RSSI or SRS-RSRP resource set ID field and the corresponding TCI state ID fields.

For CLI reporting, several existing CSI reporting configure IEs could be reused for the CLI reporting configuration. RAN1 #118bis made the following agreements.

Agreement In CSI-ReportConfig, reuse resourcesForChannelMeasurement providing the ID of a CSI-ResourceConfig that contains configuration of measurement resources for L1-SRS-RSRP and/or L1-CLI-RSSI.

Proposal 4: For L1 based UE-to-UE CLI reporting configuration, the resourcesForChannelMeasurement is reused for CLI measurement with the associated ID of CLI resource, i.e., SRS-RSRP resource and CLI-RSSI resource.

For L1 based UE-to-UE CLI reporting triggering, RAN1 made following agreements.

Agreement For aperiodic CLI reporting on PUSCH - Reuse CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig - Reuse the existing DCI field 'CSI request' to trigger an aperiodic CLI report - Introduce an indication in CSI-AssociatedReportConfigInfo that selects a set from the list of CLI-RSSI measurement resource sets or SRS-RSRP measurement resource sets configured in CSI-ResourceConfig - Note: As in current specifications, the parameter for indication (e.g., resourceSet) is always present, but only used in the case that the CSI-ResourceConfig is configured with multiple sets of aperiodic CLI measurement resources.

RAN1#120 further made the following agreements.

Agreement - Periodic L1 CLI-RSSI reporting for L1 based UE-to-UE CLI measurement and reporting is supported - Semi-persistent L1 CLI-RSSI reporting for L1 based UE-to-UE CLI measurement and reporting is NOT supported - Periodic/Semi-persistent L1 SRS-RSRP reporting for L1 based UE-to-UE CLI measurement and reporting is NOT supported

According to RAN1 agreements, we have following proposals.

Proposal 5:  For L1 based UE-to-UE CLI reporting configuration, the AP trigger based CLI-RSSI and SRS-RSRP reporting and periodic based CLI-RSSI reporting is supported.

Since it has been agreed that CSI-AperiodicTriggerStateList to configure the trigger state is reused, and the existing DCI field ‘CSI request’ to trigger an aperiodic CLI report is also reused, the legacy Aperiodic CSI Trigger State Subselection MAC CE can be reused to indicate the selection status of the aperiodic trigger states configured for CLI reporting.

Proposal 6: The legacy Aperiodic CSI Trigger State Subselection MAC CE is reused to indicate the selection status of the aperiodic trigger states configured for CLI reporting.

For CLI-RSSI resource configuration, Rel-16 has defined CLI-RSSI resource for L3 UE to UE CLI measurement, although the detailed CLI-RSSI resource pattern is discussed in RAN1. But we think reusing the existing Rel-16 contiguous CLI-RSSI pattern, i.e., contiguous frequency resources via start PRB and number of PRBs and time resources via start position and number of symbols can minimize the spec impact and reduce the UE implementation complexity.

Proposal 7: For L1 based UE-to-UE CLI measurement configuration, the configuration of CLI-RSSI resource based on the existing Rel-16 CLI-RSSI resource pattern is supported.

RAN1 #120 made following agreement regarding the RRC parameter carrier in CSI-ReportConfig and bwp-Id in the associated CSI-ResourceConfig.

Agreement For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’ - carrier in CSI-ReportConfig is used to indicate in which serving cell the CLI-RSSI measurement resources or SRS-RSRP measurement resources in CSI-ResourceConfig are to be found. If the field is absent, the resources are on the same serving cell as this report configuration. - bwp-Id in the associated CSI-ResourceConfig is used to indicate the DL BWP where the CLI-RSSI measurement resources or SRS-RSRP measurement resources are located in. Note: No RAN1 specification impact

According to RAN1 agreements, the description of carrier in CSI-ReportConfig and bwp-Id in the associated CSI-ResourceConfig should be associated with the corresponding CLI measurement resource configuration. The description of these two parameters in current RRC spec is related to CSI resource.

Proposal 8: The RRC parameters of carrier in CSI-ReportConfig and bwp-Id in the associated CSI-ResourceConfig are reused for CLI measurement resource configurations. The description of these two RRC parameters is updated accordingly to associate with the CLI resource.

In RAN1 reply LS [2] to RAN4, the Configuration 1 and Configuration 2 of the UL transmission and DL reception across SBFD symbols and non-SBFD symbols are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement. It means there are no separate configurations for CSI-RS on the SBFD symbols. Further, for the collision case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission), RAN1 indicates that the existing prioritization rules can be applied to this case.

Thus, for the CSI measurements for RLM/BFD/CBD, there are no different radio or interference conditions across SBFD symbols or non-SBFD symbols. In that sense, UE does not need to separately report the measurement results of L1-RSRP or from L3. There is neither technical requirements nor benefits to have separate RLM/BFD/CBD measurements procedure.

Proposal 9: The separate BFD/BFR procedure on SBFD symbols is not supported.

Proposal 10: The separate Layer3 measurement report for CSI-RS resources in SBFD symbol is not supported.

For Rel-16 L3 based CLI measurement, and Rel-19 L1 based CLI measurement, it is not clear whether UE can be simultaneously configured with both L3 and L1 UE-to-UE CLI measurement resources or not. RAN1 is still discussing the UE capability on whether it is possible to configure both or limit to either L1 or L3 based configuration for UEs to simplify complexity. Further, DU configures L1 measurement resources and CU configures L3 measurement resources. There are also RAN3 impacts that need involve RAN3 discussions. In our view, for simplicity, UE does not expect to be simultaneously configured with both L3 and L1 UE-to-UE CLI measurement resources.

Proposal 11: UE does not expect to be simultaneously configured with both L3 and L1 UE-to-UE CLI measurement resources.

For the UL transmission and DL reception across SBFD symbols and non-SBFD symbols, RAN1 has made several agreements about configuration 1 and configuration 2 to configure the UL and UL resources.

Agreement (RAN1#117) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only. - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols. - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. FFS whether support of configuration 2 is subject to UE capability. Agreement (RAN1#118bis) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. For SRS, only Configuration 1 is applicable. Agreement (RAN1#120) For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, - The valid symbol type for Type 1 CG PUSCH is configured in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig. - The valid symbol type for PUCCH configured for SR is configured in SchedulingRequestResourceConfig. - The valid symbol type for PUCCH carrying P-CSI is configured in periodic in CSI-ReportConfig.

For the configuration1, the PUCCH and CG-PUSCH resource are restricted to configure on one type of symbol, i.e., SBFD symbols only or non-SBFD symbols only. Then for CG resource and SR resource, network should indicate which type of symbol is applied. The valid symbol type should be introduced for CG resource configuration and SR resource configuration. RAN1 also agreed the valid symbol type of PUCCH resource for periodic CSI reporting should be supported.

Proposal 12: When UE is provided with the resource Configuration 1, the indication of the valid symbol type for type-1 CG resource, PUCCH resource for SR and PUCCH resource carrying periodic CSI reporting should be introduced separately.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: For UE-specific dedicated RRC configuration on SBFD time/frequency configuration, RAN2 wait RAN1 further decision on whether support.

Proposal 2: For L1 based UE-to-UE CLI measurement and reporting configuration based on the existing CSI framework, two new separate CLI measurement resource configurations, i.e., SRS-RSRP resource set and CLI-RSSI resource set are introduced under CSI-ResourceConfig. Each resource set contains a set of measurement resources, i.e., SRS-RSRP resource and CLI-RSSI resource separately.

Proposal 3: In the new SP CLI measurement resource set activation/deactivation MAC CE, the following fields are included, i.e., A/D field, serving cell/BWP ID field, CLI-RSSI or SRS-RSRP resource set ID field and the corresponding TCI state ID fields.

Proposal 4: For L1 based UE-to-UE CLI reporting configuration, the resourcesForChannelMeasurement is reused for CLI measurement with the associated ID of CLI resource, i.e., SRS-RSRP resource and CLI-RSSI resource.

Proposal 5:  For L1 based UE-to-UE CLI reporting configuration, the AP trigger based CLI-RSSI and SRS-RSRP reporting and periodic based CLI-RSSI reporting is supported.

Proposal 6: The legacy Aperiodic CSI Trigger State Subselection MAC CE is reused to indicate the selection status of the aperiodic trigger states configured for CLI reporting.

Proposal 7: For L1 based UE-to-UE CLI measurement configuration, the configuration of CLI-RSSI resource based on the existing Rel-16 CLI-RSSI resource pattern is supported.

Proposal 8: The RRC parameters of carrier in CSI-ReportConfig and bwp-Id in the associated CSI-ResourceConfig are reused for CLI measurement resource configurations. The description of these two RRC parameters is updated accordingly to associate with the CLI resource.

Proposal 9: The separate BFD/BFR procedure on SBFD symbols is not supported.

Proposal 10: The separate Layer3 measurement report for CSI-RS resources in SBFD symbol is not supported.

Proposal 11: UE does not expect to be simultaneously configured with both L3 and L1 UE-to-UE CLI measurement resources.

Proposal 12: When UE is provided with the resource Configuration 1, the indication of the valid symbol type for type-1 CG resource, PUCCH resource for SR and PUCCH resource carrying periodic CSI reporting should be introduced separately.

# References

- RP‑241614, Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

- R1-2501560, Reply LS on CSI-RS measurement with SBFD operation, MediaTek.