3GPP TSG-RAN WG2 Meeting #129bis	R2-2502594

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.1.3

Source:	Nokia

Title:	Data Collection for Training of network-side ML models

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This contribution elaborates on the issues related to data collection for network-side models for beam management. The issues addressed are logged measurement configuration, triggering of logging and reporting, AS buffer for logging, data availability indicator, logging data handling during handover and RRC state changes, and SRB for reporting.

# 2	Discussion

## 2.1	Logged Measurement Configuration

### 2.1.1	Logged Measurement Configuration Activation

From RAN1#116bis, it is clarified that existing CSI framework can provide necessary support for configuration of network-side ML model for BM-Case1 and BM-Case2. The configuration for data collection for offline training will rely on UEs which receive CSI measurement configurations for RS resources and resource sets (e.g. NZP-CSI-RS resources or SSB-RS resources) specified in CSI-MeasConfig. UE can perform measurements on the configured CSI-RS resources and log the measurement results in AS layer buffer. Therefore, CSI-MeasConfig framework can be a pre-requisite for configuration of measurements and logging of data collection for network-side models. The CSI-ReportConfig Information Element (IE) within CSI-MeasConfig specifies which of CSI ResourceConfig to be used for the measurement. The reporting conditions are determined based on reportConfigType of CSI-ReportConfig, while reportQuantity defines what quantities to be measured and reported. Even if some the fields may not be relevant for data collection for network-side models, the reuse of CSI-ReportConfig would be beneficial to avoid the definition of the same fields in other IEs and to harmonize the data collection configurations with the eventual inference configurations which would configure measurements on the same CSI-RS resources.

Proposal 1: Legacy CSI-ReportConfig is reused to provide configuration for measurement and logging of CSI-RS for network-side models, i.e., added or modified with csi-ReportConfigToAddModList, and released with csi-ReportConfigToRemoveList.

For data collection, RAN2 agreed that UE is to be configured to log the CSI-RS measurements instead of reporting the measurements via PUCCH/PUSCH. To enable such feature, the key principle is to reuse this CSI-ReportConfig to convey the CSI-RS resource set for measurement without enabling the legacy reporting. The UE can log the measurements based on the configured CSI-RS resource sets associated with CSI-ReportConfig and network can retrieve the logged data via on-demand reporting. This means we need to differentiate the data collection and logging configuration from legacy report configurations for L1 measurement report. This can be achieved by for instance, adding a new logging indicator in the relevant report configuration.

Proposal 2: The UE shall be able to identify from the CSI-ReportConfig which report configurations are for logging and data collection.

### 2.1.2	Triggering Event Based Logging of Measurements

In RAN2#129 it was agreed to “Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported. FFS what to log.”

Based on the agreement, the existing event A1 (serving cell becomes better than absolute threshold) and event A2 (serving cell becomes worse than absolute threshold) could be reused with related configurations such as the amount of averaging or filtering (nrofCSI-RS-ResourcesToAverage) and limiting of measurements to specific SSBs (ssb-ToMeausure) specified in TS 38.331. The measurements which are collected are on fine beams used for data logging, and these beams are usually located within the footprint of a single coarse SSB beam. The RRM measurement reporting framework already supports limiting measurements to specific SSBs. Note as well that the specification supports this type of configuration today, meaning that as long as the configuration doesn’t require a configurable link to a specific L1 measurement configuration, there would be no specification impact to use event A1 and event A2 as triggers.

The call flow of Figure 2.1.2-1 illustrates the procedure for event-triggered logging.

Figure 2.1.2-1: L3 Event-Triggered L1 CSI-RS Logging Activation with Event A1.

As shown in the Figure 2.1.2-1, we can configure RRM MeasurementReport with L3 event (e.g., A1/A2). UE will send the L3 report when the event is triggered. When network receives this report, network can configure for instance, CSI-ReportConfig with periodic CSI-RS resources for logging (Step 5).  When UE receives the logging configuration, then it starts the logging of L1 measurements. Thus, the L1 measurements can be logged without any additional specification impact. Regarding the ‘FFS what to log’ can be left to RAN1.

Observation 1: What to log can be left to RAN1.

If the UE is configured with L1 measurements in Step 1 (not shown in the Figure), then in Step 5, a logging activation trigger could activate the logging of L1 measurements. This allows gNB to rapidly response to the event triggered logging to enable or disable logging in UE.

Observation 2: To support L3 event-triggered measurement logging for network-side AI/ML beam management data collection, MeasurementReports resulting from the legacy ReportConfigNR, configured with events A1 and A2, can be used by the gNB to trigger the activation or deactivation of CSI-RS measurement logging.

Observation 3: The network can activate or deactivate logging by initiating a subsequent (to L3-trigger) procedure or command, where the exact mechanism to support the activation and deactivation of logging is FFS.

Proposal 3: To support a gNB’s rapid response to a measurement report configured to enable or disable logging, a MAC-CE is introduced to enable and disable logging dynamically.

### 2.1.4	AS buffer for logging

While it has been agreed that there will be an AS buffer for logging for network-side data collection, we have yet to discuss two key issues: how many buffers there are; how a single buffer would be shared for different configurations/use-cases if there is only one; and how the context of measurements in the buffer is kept when the gNB releases a configuration for measurement logging prior to retrieving the measurements logged, associated with the prior configuration(s).

#### 2.1.4.1	Number of Buffers

The discussion until this point has implied the use of a single buffer for all network-side data collection configurations, including in the running CR for 38.331, which shows an element called CSI-LogMeasInfo-r19, containing a reference, refCSI-LoggedMeasurementConfigId-r19, pointing to a specific CSI-LoggedMeasurementConfigId-r19. The only reason to include such a field is to associate the measurement result(s) with a configuration, implying that the measurement results associated with multiple logged measurement configurations would be stored in the same buffer.

Observation 4: Discussion in RAN2 thus far implies the use of a single AS buffer for logging measurements for network-side data collection.

#### 2.1.4.2	Identifying Measurements in the Log

To support data collection on component carriers in a carrier aggregation configuration, measurements must be tagged with the serving cell index since the measurement logging configuration IDs may overlap and the gNB needs to know how to identify on which cell measurements were taken. Since the primary cell, or PCell would be responsible for receiving the measurement reports on behalf of the SCells, we think that the FFS item on the cellId-r19 could be updated to indicate the SCellIndex, the maximum number of which is 31, and whose range is between 1 and 31. Because the first index is implicitly reserved for the PCell, the range of cellId-r19 could start from 0, as shown in the excerpt from the running 38.331 CR below.

Additionally, we see no reason to tie any of this logged measurement framework to logged MDT. Therefore, we propose creating a new IE for the maximum number of logged measurement reports specific to beam management use case.

Proposal 4: Create a new IE to define the maximum number of logged measurement reports for measurement logging for network-side data collection.

CSI-LogMeasInfoList-r19 ::=          SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF CSI-LogMeasInfo-r19

CSI-LogMeasInfo-r19 ::=              SEQUENCE {

cellId-r19                              INTEGER (0..maxNrOfSCells),

refCSI-LoggedMeasurementConfigId-r19    CSI-LoggedMeasurementConfigId-r19,

csi-RS-MeasResultList-r19               SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF CSI-MeasResults-r19    OPTIONAL,

csi-SSB-MeasResultList-r19              SEQUENCE (SIZE (1..maxNrofSSBs)) OF CSI-MeasResults-r19                    OPTIONAL,

FFS

...

}

Proposal 5: The cellId-r19 indicated in the CSI-LogMeasInfo-r19 IE is the serving cell index with a range from 0 to maxNrOfSCells, and the index 0 indicates that the measurements are for the PCell.

#### 2.1.4.3	Releasing Logged Measurement Configurations

While a UE is configured with a logged measurement configuration for network-side data collection, the gNB knows the context of the reports being generated by the ID through the reference to the logged measurement configuration ID. However, we must consider what happens when the gNB releases the configuration from the UE. On the gNB side, once the configuration is released, normally, the gNB would lose the context of the configuration, rendering the referenced to the logged measurement configuration ID in the measurement log uninterpretable.

Observation 5: The gNB could lose the context of measurements logged and reported by the UE if it releases the measurement logging configuration from the UE.

Releasing the configuration should not cause the discarding of measurements as we have agreed to support handover. However, if a new logged measurement configuration is configured in place of an old one and is identified by the same logged measurement configuration ID, the UE shall discard the old samples because it would be nearly impossible for the gNB to properly identify which samples were of an old or new logged measurement configuration with the same ID.

Proposal 6: If a new logged measurement configuration is configured in the UE with the same ID as an existing logged measurement configuration, the UE shall discard samples belonging to the old configuration.

# 2.2	Logged Measurements Transmission

### 2.2.1	Availability Indicator

During RAN2#129, it was agreed that “Data availability indication is supported.  FFS when this would be triggered”. However, it is still an open issue when the availability indication can be sent by the UE to the network. Previously, it was also discussed whether the UE should send an indication to inform the network when its data logging buffer is already full or is about to be full, as in certain cases such as the event-based data logging network may not have full knowledge of the amount of data collected in the UE buffer.

It needs to be considered what would be the implications in each of this case. We think that sending an availability indication, when UE buffer is already full, has the risk of losing some data due to overwrite. Since UE may still be logging data while the network initiates the collection of data already presented in the UE buffer. In case of an indication before when the buffer is about to be full, it needs to be clarified what it would mean for different UEs based on the size of their buffers as well as the logic of their buffer management. At the same time, it may also not be efficient to have frequent data availability indicator from the UE with a very small amount of data available in its buffer.

Observation 6: When the UE sends a data availability to the network, it has implications for safe retrieval of data as well as efficiency of data reporting.

Therefore, it may be efficient that the criterion for data availability indicator is configurable by the network. This can be achieved by the network by configurating a threshold for the amount of data collected in the UE buffer. For this purpose, network may take into account the size of the UE buffer and configure a threshold value such that the data availability indicator is triggered well before the UE buffer is full and at the same time not early enough to have frequent indications for very small amount of data.

Proposal 7: Network can configure a threshold for data availability indication. UE sends an availability indication when the data it has collected is more than this threshold via UEAssistanceInformation (UAI).

### 2.2.2	Low Power Indication

During RAN2#129, it was agreed that “Low power bit indication is supported.” However, we have not yet agreed on how many logged data collection configurations can be configured in the UE simultaneously, including for data collection for UE-side models. If more than one can be configured, then we think that the low power indication should be signalled generally enough to cover all data collection configurations. That is, it should not be possible that the UE’s battery is too low to collect data for training network-side models, but is sufficiently charged to collect data for UE-side models or that specific data collection configurations are possible, but others aren’t. The low battery indication should be treated as a binary – data collection for training models (network-side or UE-side) is possible or it is not based on the battery state.

Proposal 8: The low battery indication should be sent once and is applicable to both network-side and UE-side data collection configurations.

### 2.2.3	Logged Data Handling for RRC IDLE and RRC INACTIVE

According to the open issue related to “FFS how to handle idle/inactive and RLF cases and whether we have a unified”, we need to understand the UE behaviour during RRC idle and RRC inactive case.

The logged data is stored during RRC Connected state. The UE moves to IDLE/INACTIVE state when it receives an RRCRelease message from the network, i.e., it is a network decision when the UE moves to IDLE/INACTIVE. The network is also aware that the UE has been configured, the network can retrieve the logged data from the UE before moving the UE to IDLE/INACTIVE.

Observation 7: The network can retrieve the logged data before moving the UE to IDLE/INACTIVE state.

If the data is kept the issue of confidentiality of the logged data arises. This issue is discussed in detail in the paper [R2-2500391]. Another issue is how to handle the case when the new node would like to collect new data without retrieving the data collected in the previous node. We think that these issues can be solved, but this would require additional discussion and specification work including RAN3 work on data forwarding.

Additionally, a UE in IDLE mode could reselect to a different cell in an unpredictably long amount of time. The gNB need to have the context of the UE to be able to correctly interpret the reported logged data, therefore the gNB would have to maintain the context of every UE which was configured for data collection until the UE transmitted its data.

Observation 8: The gNB could have to store the context of previously released UEs indefinitely to be able to retrieve and provide context to the UE’s logged measurements.

As the network can retrieve the data before the UE is moved to IDLE/INACTIVE, and keeping the data could cause additional complexity, we think that the simple solution is to discard the logged when the UE moves to IDLE/INACTIVE.

Proposal 9: Unretrieved logged measurements shall be discarded when the UE transitions to RRC IDLE or RRC INACTIVE state.

### 2.2.4	Logged Data Handling During Handover

At RAN2#129 it was agreed that UE retains logged data during handover (HO). The issues of keeping the data (confidentiality, buffer handling), discussed above for RRC IDLE/INACTIVE cases, are also valid for this case. In this case, the network may have no time to retrieve the logged data from the UE, so there can be cases when there is logged data in the UE that network would like to retrieve. Because handover is negotiated between a source gNB and a target gNB and gNBs can be configured to be aware of one another, it can be determined in advanced whether the target gNB can support the retrieval of logged measurements on behalf of the source gNB. Therefore, we find it useful to check in advanced whether the target gNB can support the retrieval to avoid unnecessary data retention during handover.

Proposal 10: Whether the UE retains logged measurements during handover from the source cell to a target cell is configured by the gNB.

There are at least two decision points where the network can configure the UE to keep or discard the data. The first point is when the data collection is configured. If the networks know at the point of the data collection configuration that it is important to keep the logged data at handover for all possible target cells, then it can send an indicator to keep the logged data at handover along with the data collection configuration. The other option is the network makes the decision on keeping or discarding the data based on the target cell (e.g., if the target cell belongs to the same gNB then it is reasonable to keep the data). In this case the network decision for keeping the data can be sent along with the handover command (RRCReconfiguration message for handover).

Proposal 11: The indication whether the UE retains logged measurements during handover can be sent when the network configures the UE to collect the data or when it sends the handover command (RRCReconfiguration message for handover).

The concept of retaining data in the UE and sending the report to the target node is simple from UE perspective, but it also has network impacts. The first issue is that after the target node retrieves the logged data from the UE, it should forward this to the source gNB. The support of forwarding the logged data would require RAN3 specification work.

Observation 9: Transferring logged measurements from a target gNB to a source gNB has RAN3 specification impacts.

For correct interpretation of the logged data requires the knowledge of what exactly has been measured. This means that the gNB will need to retain the logged measurement related configuration of the UE for a duration long enough for the target gNB to have retrieved the logged measurements from the UE and transmit them to the source gNB.

Observation 10: The source gNB will need to retain the logged measurement configuration(s) for UEs which are handed over to a target gNB and which are expected to transmit logged measurements, configured by the source gNB, until the target gNB retrieves the logged measurements from the UE and forwards them to the source gNB.

If the target gNB is unable or unwilling to retrieve logged measurements in a timely manner after handover, the UE should remove the logged measurements belonging to the source gNB from the buffer. If the UE is subsequently handed over to a new target gNB prior to transmitting measurements for the source gNB, the UE should remove the logged measurements belonging to the source gNB from the buffer.

Proposal 12: A UE configured to transmit logged measurements for a source gNB to a target gNB shall discard logged measurements for the source gNB after handover if the UE is subsequently handed over to a next target gNB prior to transmitting logged measurements for the original source gNB.

Proposal 13: A UE configured to transmit logged measurements for a source gNB to a target gNB shall discard unretrieved logged measurements for the source gNB after handover upon expiration of a timer.

As the decision of keeping the data at HO has RAN3 impact due to the potentially necessary data forwarding, RAN2 should inform RAN3 about these decisions.

Proposal 14: Send an LS to RAN3 to ask the following questions:

- For a UE configured to log measurements in RRC CONNECTED mode, is it feasible to specify in Release 19 that after handover for a target gNB to retrieve logged measurements collected by the UE at the source gNB and forward the logged measurements retrieved from the UE to the source gNB?

And provide the following information:

- Clarify that the data collection configuration could originate in the source gNB or from OAM and that when the configuration originates in the gNB, the target of the collected data is the gNB and when the configuration originates in OAM, the target of the collected data is the Trace Collection Entity (TCE) to which the target gNB would forward the logged measurements as Trace Records.

- Both mechanisms are unrelated to legacy logged MDT.

- A UE does not retain logged measurements for a source cell when a subsequent handover occurs; e.g., the UE collects data for gNB1, hands over to gNB2, and then to gNB3. gNB3 is not expected to forward logged measurements to gNB1.

(Draft LS proposal can be found in the Annex.)

### 2.2.5	Which SRB to use?

One open issue stating “Low priority SRB will be used. FFS new SRB or use of SRB4” from RAN2#127-bis is related to which SRB is to be used for the data transfer from the UE to the network for data collection of network-side models. It has already been agreed that a low priority SRB will be used for this purpose. Even so, there is no proper motivation whether it will be an existing low priority SRB, or a new SRB is to be specified. A new SRB would require specification impact as well as additional configuration for PDCP, RLC and MAC to establish the SRB.

Observation 11: Introducing of a new SRB would require additional configurations of PDCP, RLC and MAC.

On using an existing SRB, previous discussions have centred on SRB4 as an option for low priority SRB. However, we notice that there is no clear priority defined between SRB2 and SRB4.

According to TS 38.331:

-     SRB2 is for NAS messages and for RRC messages which include logged measurement information, all using DCCH logical channel (except SRB2 of L2 U2N Remote UE). SRB2 has a lower priority than SRB1 and may be configured by the network after AS security activation;

-     SRB4 is for RRC messages which include application layer measurement report information, all using DCCH logical channel. SRB4 has a lower priority than SRB1 and can only be configured by the network after AS security activation.

As it appears from the specification that both SRB2 and SRB4 have lower priority than SRB1, but there is no clear priority difference between these two SRBs.

Observation 12: There is no clear priority difference defined for SRB2 and SRB4 in the specification.

Now, the question is which SRB would be the most suitable choice for the data reporting purpose. In our understanding there are at-least two reasons to choose SRB2 for the data reporting over SRB4:

- Data reporting is agreed to be done via UEInformationResponse message, and SRB2 already is used for RRC messages which include logged measurement reporting. Therefore, data reporting over SRB2 may be more straightforward.

- SRB4 is used for application layer reporting and is also not widely in use. Mandating data reporting over SRB4 may require both gNB and UEs to implement application layer reporting as well which may limit the UE’s participation.

Due to above reasons we think that SRB2 may be a better option for data reporting than SRB4.

Proposal 15: RAN2 to consider SRB2 for AIML logged measurement reporting.

# 3	Conclusion

This document has made the following observations:

Observation 1: What to log can be left to RAN1.

Observation 2: To support L3 event-triggered measurement logging for network-side AI/ML beam management data collection, MeasurementReports resulting from the legacy ReportConfigNR, configured with events A1 and A2, can be used by the gNB to trigger the activation or deactivation of CSI-RS measurement logging.

Observation 3: The network can activate or deactivate logging by initiating a subsequent (to L3-trigger) procedure or command, where the exact mechanism to support the activation and deactivation of logging is FFS.

Observation 4: Discussion in RAN2 thus far implies the use of a single AS buffer for logging measurements for network-side data collection.

Observation 5: The gNB could lose the context of measurements logged and reported by the UE if it releases the measurement logging configuration from the UE.

Observation 6: When the UE sends a data availability to the network, it has implications for safe retrieval of data as well as efficiency of data reporting.

Observation 7: The network can retrieve the logged data before moving the UE to IDLE/INACTIVE state.

Observation 8: The gNB could have to store the context of previously released UEs indefinitely to be able to retrieve and provide context to the UE’s logged measurements.

Observation 9: Transferring logged measurements from a target gNB to a source gNB has RAN3 specification impacts.

Observation 10: The source gNB will need to retain the logged measurement configuration(s) for UEs which are handed over to a target gNB and which are expected to transmit logged measurements, configured by the source gNB, until the target gNB retrieves the logged measurements from the UE and forwards them to the source gNB.

Observation 11: Introducing of a new SRB would require additional configurations of PDCP, RLC and MAC.

Observation 12: There is no clear priority difference defined for SRB2 and SRB4 in the specification.

And proposed the following:

Proposal 1: Legacy CSI-ReportConfig is reused to provide configuration for measurement and logging of CSI-RS for network-side models, i.e., added or modified with csi-ReportConfigToAddModList, and released with csi-ReportConfigToRemoveList.

Proposal 2: The UE shall be able to identify from the CSI-ReportConfig which report configurations are for logging and data collection.

Proposal 3: To support a gNB’s rapid response to a measurement report configured to enable or disable logging, a MAC-CE is introduced to enable and disable logging dynamically.

Proposal 4: Create a new IE to define the maximum number of logged measurement reports for measurement logging for network-side data collection.

Proposal 5: The cellId-r19 indicated in the CSI-LogMeasInfo-r19 IE is the serving cell index with a range from 0 to maxNrOfSCells, and the index 0 indicates that the measurements are for the PCell.

Proposal 6: If a new logged measurement configuration is configured in the UE with the same ID as an existing logged measurement configuration, the UE shall discard samples belonging to the old configuration.

Proposal 7: Network can configure a threshold for data availability indication. UE sends an availability indication when the data it has collected is more than this threshold via UEAssistanceInformation (UAI).

Proposal 8: The low battery indication should be sent once and is applicable to both network-side and UE-side data collection configurations.

Proposal 9: Unretrieved logged measurements shall be discarded when the UE transitions to RRC IDLE or RRC INACTIVE state.

Proposal 10: Whether the UE retains logged measurements during handover from the source cell to a target cell is configured by the gNB.

Proposal 11: The indication whether the UE retains logged measurements during handover can be sent when the network configures the UE to collect the data or when it sends the handover command (RRCReconfiguration message for handover).

Proposal 12: A UE configured to transmit logged measurements for a source gNB to a target gNB shall discard logged measurements for the source gNB after handover if the UE is subsequently handed over to a next target gNB prior to transmitting logged measurements for the original source gNB.

Proposal 13: A UE configured to transmit logged measurements for a source gNB to a target gNB shall discard unretrieved logged measurements for the source gNB after handover upon expiration of a timer.

Proposal 14: Send an LS to RAN3 to ask the following questions:

- For a UE configured to log measurements in RRC CONNECTED mode, is it feasible to specify in Release 19 that after handover for a target gNB to retrieve logged measurements collected by the UE at the source gNB and forward the logged measurements retrieved from the UE to the source gNB?

And provide the following information:

- Clarify that the data collection configuration could originate in the source gNB or from OAM and that when the configuration originates in the gNB, the target of the collected data is the gNB and when the configuration originates in OAM, the target of the collected data is the Trace Collection Entity (TCE) to which the target gNB would forward the logged measurements as Trace Records.

- Both mechanisms are unrelated to legacy logged MDT.

- A UE does not retain logged measurements for a source cell when a subsequent handover occurs; e.g., the UE collects data for gNB1, hands over to gNB2, and then to gNB3. gNB3 is not expected to forward logged measurements to gNB1.

(Draft LS proposal can be found in the Annex.)

Proposal 15: RAN2 to consider SRB2 for AIML logged measurement reporting.

# 4	Annex: Draft LS to RAN3

3GPP TSG-RAN WG2 Meeting #129bis	R2-250xxxx

Wuhan, China, 07 – 11 April 2025

Title:	[DRAFT] LS on Logged Data Forwarding from a Target to a Source Cell

Response to:	-

Release:	Release 19

Work Item:	NR_AIML_air-core

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG3

Cc:

Contact Person:

#### Name:	Gyorgy Wolfner

E-mail Address:	gyorgy.wolfner@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

During the discussion of network-side (NW-side) data collection (DC) RAN2 has concluded that the UE can retain logged data during handover (HO) and the UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE). It was also agreed that a UE does not retain logged measurements for a source cell when a subsequent handover occurs; e.g., the UE collects data for gNB1, hands over to gNB2, and then to gNB3. gNB3 is not expected to forward logged measurements to gNB1.

That is, a UE configured with measurement logging for NW-side DC shall retain logged data (measurements) during a handover procedure and report the availability of logged data to the target cell in RRCReconfigurationComplete. Next, the network would retrieve the logged data from the UE in the target cell and forward it to the gNB controlling source cell.

The data collection configuration could originate in the source gNB or from OAM and that when the configuration originates in the gNB, the target of the collected data is the gNB and when the configuration originates in OAM, the target of the collected data is the Trace Collection Entity (TCE) to which the target gNB would forward the logged measurements as Trace Records. Remark that these mechanisms are unrelated to legacy logged MDT.

RAN2 respectfully asks RAN3 whether it is feasible for a UE configured to log measurements in RRC CONNECTED mode to specify in Release 19 that after handover for a target gNB to retrieve logged measurements collected by the UE at the source gNB and forward the logged measurements retrieved from the UE to the source gNB. If yes, then RAN2 would like to ask RAN3 to work on the necessary specifications.

2. Actions:

To RAN3.

ACTION: 	RAN2 respectfully asks RAN3 whether it is feasible for a UE configured to log measurements in RRC CONNECTED mode to specify in Release 19 that after handover for a target gNB to retrieve logged measurements collected by the UE at the source gNB and forward the logged measurements retrieved from the UE to the source gNB. If yes, then RAN2 would like to ask RAN3 to work on the necessary specifications.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#130	from 2025-05-19	to 2025-05-23		Malta, MT

RAN2#131	from 2025-08-25	to 2025-08-29		India (TBC), IN