3GPP TSG-RAN2#129bis	R2-2502597

Wuhan, China, Apr 7th-11th, 2025

Agenda item:		8.4.2

Source:		Lenovo

Title:		 Procedure and Configuration of LP-WUS in RRC Idle/ Inactive

Document for:		Discussion and Decision

- Introduction

In RAN#103 meeting, WID of Rel-19 Low-power wake-up signal and receiver for NR (LP-WUS/ WUR) [1] was discussed and confirmed. Currently, the UE’s Main Radio (MR) needs to periodically monitor PDCCH once per DRX cycle, which dominates the power consumption in periods with no signaling or data traffic. If the UE’s MR can wake up only when they are triggered (e.g. paging), power consumption could be dramatically reduced. This can be achieved by using a wake-up signal to trigger the main radio and a separate receiver which has the ability to monitor wake-up signal with ultra-low power consumption. Main radio works for data transmission and reception, which can be turned off or set to deep sleep unless it needs to monitor PDCCH [2].

Furthermore, the following agreements were made during previous RAN2 meetings:

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-WUS configuration - - Entry/exit condition for LP-WUS monitoring (FFS if it always configured) Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. From RAN2 perspective, no new procedure is introduced for SI reception/updates. If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). The LPWUS monitoring exit condition does not include MR measurements. For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion. For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping. FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session. FFS on the following options - - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping. - - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed. - - Option 3: The formula for UE_ID based PEI subgrouping is reused. RAN2 understands that UE can report which band(s) is supported by LR to NW. RAN2 understands that any potential overload issues could be addressed by current mechanism in spec. Send LS to RAN1 and RAN4 to inform the agreements. For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - - UE_ID is related to 5G-S-TMSI, - - N is the number of total paging frames in one DRX cycle, - - Ns is the number of the PO for a PF, - - Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively. RAN2 sends reply LS to SA2/RAN3/CT1, and CC RAN1 and RAN4 to confirm the “LP-WUS” terminology. Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.

This contribution looks at the possible RRC Idle/ Inactive procedures for an LP-WUR enabled UE including entry and exit conditions for LR activation, subgrouping and possible paging enhancements.

- Discussion

  - Entry and Exit Conditions

It has been agreed in previous meetings that the baseline entry/exit conditions for LR to monitor for LP-WUS will be based on the serving cell measurements and/or serving cell quality (e.g., RSRP or RSRQ) from MR. Whether these conditions are always configured is still FFS. As per the WID objectives, it has been captured that the target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for msg3. Some companies have pointed out that this means it is expected for LR coverage to match that of MR and that an entry/exit condition need not be always configured. In our view, although the LR coverage shall match that of the MR’s PUSCH coverage for msg3, this is still lesser than the MR’s DL coverage, i.e., the LR shall have a smaller coverage with respect to the MR’s DL coverage. Furthermore, since the LP-WUR architecture may vary from one UE to another (OOK-based or OFDM-based LR), it is necessary to always provide an entry/ exit condition in the SIB for a cell supporting LP-WUS.

The entry/exit condition for LP-WUS monitoring is always configured in SIB when the cell supports LP-WUS.

  - Paging monitoring triggered by LP-WUS

According to RAN1’s conclusions: for the cases where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not. And 3 potential options were given from RAN1’s conclusions for the association between LO and PO, as below:

- Option 1: UEs monitoring the same PO monitor the same LO.

- Option 2: UEs corresponding to different POs monitor the same LO.

- Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.

RAN1 further agreed that at least Option 1 is supported in the RAN1#118bis meeting [3] and also agreed that Option 2 can be supported during the RAN1 #120 meeting. In some exceptional cases, UE may fail to receive the paging message after receiving the LP-WUS indicating wake-up. For example, there was no paging transmitted during the target PO(s), or UE misses the target PO/PEI(s) due to e.g., potential transition time, or if the UE has been falsely alarmed by the LP-WUS targeted to other UEs.  RAN2 is suggested to discuss the UE behaviors for the cases that UE fails to receive paging after being woken up by LP-WUS. From our understanding, if UE keeps monitoring paging, then the energy consumption can be aggravated. UE may re-enter LP-WUS monitoring mode in case no paging is received in target PO(s) after being woken up by LP-WUS provided that at least the entry condition for LP-WUS monitoring is satisfied.

UE may re-enter LP-WUS monitoring mode in case no paging is received in target PO(s) after being woken up by LP-WUS provided that at least the entry condition for LP-WUS monitoring is satisfied.

- Additionally, in the case that UE determines to monitor PEI (if configured) after being woken up by LP-WUS and fails to detect the PEI occasion due to some exceptional cases as mentioned above, to avoid UE missing potential paging during the subsequent PO, it is proposed that the UE may keep monitoring associated PO of this PEI.

In the case that UE fails to monitor PEI occasion (if configured) after being woken up by LP-WUS, UE continues to monitor for the associated PO.

  - Issue on Radio Capability for Paging Information

In the previous meeting, SA2 had sent a LS to RAN2 regarding the case where initial registration is performed on a pre-Rel-19 gNB [4]. RAN2 has briefly discussed the issue in the previous meeting and concluded that there is no issue for networks after Rel-17 since the LP-WUS UE-ID based subgrouping UE capability can be included in the UE-RadioPagingInfo container. In our view, the issue remains a bit unclear as it is our understanding that the UE can report its capability to the network during initial registration regardless of the release implemented by the network. When the gNB acquires the capability and creates the UE Radio Capability for Paging Information for transmission to AMF, the gNB is not prohibited from including any containers that it does not understand/ recognize. For example, a Rel-16 gNB is not prohibited from including a Rel-17 capability container. Furthermore, when AMF acquires the UE Radio Capability for Paging Information from the NG-RAN, it stores this information regardless of whether it understands the contents or not [5]. Hence, we are unsure of whether this issue really exists, and if it does, we think that this can be resolved by network implementation to include paging related capabilities in the UE Radio Capability for Paging Information message even when the gNB does not understand the capability informed by the UE.

RAN2 to discuss whether there is an issue when creating the UE Radio Capability for Paging Information and whether network implementation can resolve such issue (if exists).

# Conclusion

This document discusses some procedures for a LR capable UE in the RRC_IDLE/ RRC_INACTIVE Mode. Following are the proposals and observations made in this document:

- The entry/exit condition for LP-WUS monitoring is always configured in SIB when the cell supports LP-WUS.

- UE may re-enter LP-WUS monitoring mode in case no paging is received in target PO(s) after being woken up by LP-WUS provided that at least the entry condition for LP-WUS monitoring is satisfied.

- In the case that UE fails to monitor PEI occasion (if configured) after being woken up by LP-WUS, UE continues to monitor for the associated PO.

RAN2 to discuss whether there is an issue when creating the UE Radio Capability for Paging Information and whether network implementation can resolve such issue (if exists).

# References

[1] 3GPP RAN#103, RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] 3GPP TR 38.869 v18.0.0: Study on low-power wake up signal and receiver for NR (Release 18)

[3] 3GPP TSG RAN WG1#118bis, Chair’s Notes

[4] 3GPP RAN2 #129, R2-2500050

[5] 3GPP TS 23.501