3GPP TSG-RAN2 #129bis	R2-2502602

Wuhan, China, Apr.  7th – 11th, 2025

Agenda item:	8.6.3 (NR_Mob_Ph4-Core)

Source:	Ofinno

Title: 	Discussion on L1 event triggered measurement report

Document for:	Discussion and Decision

# Introduction

An objective of the WID [1] for R19 mobility enhancements is to specify necessary components to support event triggered L1 measurement reporting. RAN2 made the following agreements:

RAN2#129 agreed the following [2]: - Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling. - Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam. - LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant. - Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s)) - Truncated MR MAC CE includes the triggered beam information: - One report config ID (event config ID) - At least one triggered beam id (SSBRI or CRI) + L1 RSRP - The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR. - The max number of N (included in the MR MAC CE) is the total number of all candidate beams without counting current serving beam. - Truncated event-triggered L1 measurement report MAC CE can be used when the available grant is not sufficient to accommodate the normal measurement report MAC CE plus subheader. RAN2#128 agreed the following [3]: - For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE. RAN2#127b agreed the following [4]: - MR can be sent when the leaving condition is met, based on NW configuration. - Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) - The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. - NW can configure a dedicated SR configuration for MR MAC CE transmission. RAN2#126 agreed the following [5]: - Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure.

In this contribution, we discuss issues related to event triggered LTM measurement report.

# 2. Discussion

## 2.1 LTM event evaluation

RAN2#127 agreed the beams used for LTM event evaluation. Based on the agreement, current beam is used for event evaluation of a serving cell, and any beam in the candidate RS configuration can be used for LTM event evaluation. Further, RAN#127b confirmed the working assumption that the same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

A UE may evaluate an LTM event using multiple candidate beams. The UE may also be configured with multiple candidate cells. This implies that the UE may evaluate each LTM event using multiple candidate beams for each of the multiple candidate cells, according to the measurement resource configuration and the measurement reporting configuration. This leads to a case where there is frequent triggering of event triggered L1 measurement report (MR) by the UE. Some of these may be redundant or unnecessary. Several scenarios may occur where multiple candidate beams satisfy the entry condition of an LTM event for TTT (i.e., event condition).

A UE may evaluate the LTM event using multiple candidate beams of a candidate cell. In an example, a first candidate beam may satisfy the entry condition for the LTM Event for TTT. Before triggering the MR for the first candidate beam satisfying the entry condition for the LTM event for TTT, a second candidate beam may also satisfy the entry condition of the LTM event for TTT. RAN2 agree [2] that a L1 MR MAC CE is specific to a triggered measurement event. Therefore, the UE may trigger a single L1 MR MAC CE for the multiple candidate beams that satisfied the entry condition of the LTM event.

In another example, a candidate beam satisfies the event, soon after triggering an MR for the LTM event for a different candidate beam. The UE triggers an MR after a first candidate beam of the candidate cell satisfies the entry condition for the LTM Event. After triggering the MR, a second candidate beam of the same candidate cell may satisfy the entry condition for the same LTM event.

Observation 1: For a candidate cell, after triggering MR for a first candidate beam satisfying the LTM event, a second candidate beam may satisfy the LTM event.

In this case, there are two possible behaviors. In a first option, the UE may trigger a new MR for the second candidate beams that satisfied the entry condition of the LTM event for TTT, irrespective of how much time has elapsed since triggering the MR for the first candidate beam. This may lead to triggering many MRs. In a second option, the UE may trigger a new MR for the second candidate beam, if a significant amount of time has elapsed since triggering the MR for the first candidate beam. For example, to the UE doesn’t trigger a new MR if the time elapsed since triggering the MR for the first candidate beam is small or an insignificant amount. On the other hand, if the time elapsed is a significant one, the UE triggers a new MR. The network may configure the time value based on which the UE may trigger the new MR. This is similar to operation of a prohibit timer.

Proposal 1: Introduce prohibit timer for event triggered L1 MR, with the following behavior:

- The prohibit timer is started after triggering a new MR for a candidate cell satisfying an entry condition of an LTM event for TTT.

- While the prohibit timer is running, a new MR is not triggered for the same candidate cell for the same LTM event.

- While the prohibit timer is running, a new MR is triggered for the same candidate cell satisfying an entry condition of a different LTM event.

## 2.2 L1 MR validity

RAN2 agreed [2] that an L1 MR MAC CE only includes one triggered measurement event.

A UE may trigger an MR if a candidate beam satisfies the entry condition of an LTM event for TTT. The UE may transmit the MR if an uplink resource is available. If an uplink resource is not available, the UE may wait to receive an uplink resource for transmitting the MR. RAN#127b agreed [4] that the UE triggers SR for resource allocation to send the event triggered MR. If an uplink resource is received after sending SR, the UE may transmit the MR. If an uplink resource is not received after transmitting the SR, the UE may initiate a RACH procedure and transmit the MR after receiving the uplink resource. In another example, UE may transmit a truncated measurement report MAC CE when the available grant is not sufficient to accommodate the normal measurement report MAC CE plus subheader [2]. The UE may transmit a normal measurement report MAC CE following the transmission of the truncated measurement report MAC CE. This leads to a case where the information in the triggered MR may be outdated by the time uplink resource for transmitting the MR is available.

Observation 2: The measurement results in a triggered MR may be outdated if an uplink resource for transmitting the triggered MR is available only after a significant time has elapsed since triggering the MR.

Proposal 2: RAN2 to discuss handling of outdated information in event triggered L1 MR.

### 2.3 L1 MR MAC CE

In the case of layer 3 measurements, an RRC layer may trigger multiple layer 3 measurement reports. The RRC layer may submit to the MAC entity, the triggered layer 3 measurement reports for transmission. The MAC entity, based on the available uplink resource and a logical channel prioritization procedure, may multiplex the triggered layer 3 measurement reports into a single MAC PDU and transmit to the network.

As discussed above, there may be frequent triggering of event triggered L1 MR. Some of these MRs may be triggered concurrently, e.g., for different LTM events. There may be multiple MRs triggered and pending for transmission. RAN2 has already agreed that the MRs is sent via a MAC CE over any available UL grant [2]. However, according to the current MAC specification [6], at most one MAC CE for a procedure is multiplexed into one MAC PDU.

Observation 3: A MAC PDU can accommodate multiple layer 3 measurement reports while a MAC PDU can only accommodate one event triggered L1 MR MAC CE.

When multiple MRs are triggered and if the MAC entity can accommodate only one L1 MR MAC CE in a MAC PDU, and if an L1 MR MAC CE can accommodate only one triggered MR, this may cause frequent triggering of L1 MR MAC CEs. This leads to higher signalling due to different MRs being transmitted via different MAC PDUs and each L1 MR MAC CE having its subheader. This may even lead to frequent transmission of scheduling request if uplink resource for transmitting the MR is not available.

Proposal 3: Multiple event triggered L1 MR MAC CEs can be included in a MAC PDU.

A UE may be configured to transmit, in a normal L1 MR MAC CE, measurement results of up to N candidate beams. Additionally, the UE maybe configured to transmit measurement results for candidate beams that do not satisfy an LTM event condition. In this case, the N candidate beams may consist of X candidate beams that satisfy an LTM event condition (i.e., LTM entry condition for TTT) and Y candidate beams that do not satisfy the LTM event condition. In addition to the N candidate beams, the UE may be configured to transmit measurement result of current beam of the serving cell.

According to RAN2 agreements, a UE transmits Truncated event-triggered L1 measurement report MAC CE when the available grant is not sufficient to accommodate the normal measurement report MAC CE plus subheader. This means that even when the available uplink grant is sufficient to accommodate beam information (i.e., measurement results, and RS resource index of the corresponding candidate beam) for all of the X candidate beams satisfying the LTM event condition, the UE still transmits truncated L1 MR MAC CE if the available uplink grant is not sufficient to accommodate the Y beams not satisfying the LTM event condition. The truncated L1 MR MAC CE consists of all of the X candidate beams satisfying the LTM event condition. Similarly, when the available uplink grant is sufficient to accommodate beam information (i.e., measurement results, and RS resource index of the corresponding candidate beam) for all of the X candidate beams satisfying the LTM event condition, the UE still transmits truncated L1 MR MAC CE if the available uplink grant is not sufficient to accommodate the beam information for current beam of serving cell.

Observation 4: A UE configured to transmit candidate beams not satisfying an LTM event condition, transmits a truncated L1 MR MAC CE when an available UL grant can accommodate all the candidate beams that satisfies an LTM event condition but cannot accommodate candidate beams that does not satisfy the LTM event condition.

Observation 5: A UE configured to transmit current serving beam information, transmits a truncated L1 MR MAC CE when an available UL grant can accommodate all the candidate beams that satisfies an LTM event condition but cannot accommodate the current serving beam information.

The main objective of sending an L1 MR for an LTM event is to inform a network about the candidate beams that satisfy an LTM event condition of the LTM event. This is the essential information for the triggered measurement event. The network can take LTM cell switch decision or trigger an early sync, based on the beam information of the candidate beams that satisfy the LTM event condition. Every other information in the L1 MR MAC CE i.e., the candidate beams not satisfying the LTM event condition, the current serving beam etc. are useful information but not essential information. If an available uplink grant can accommodate beam information for all the candidate beams (i.e., X candidate beams) that satisfy the LTM event condition, the UE should transmit a normal L1 MR MAC CE and not a truncated L1 MR MAC CE. This is irrespective of whether the uplink grant can accommodate beam information for the Y candidate beams or the current serving beam. The triggered measurement report can be cancelled after transmitting beam information for all of the X candidate beams.

Proposal 4: A normal L1 MR MAC CE is transmitted if available uplink grant is sufficient to accommodate a normal L1 MR MAC CE, comprising beam information of all the candidate beams satisfying the LTM event condition, plus its subheader.

Proposal 5: A L1 measurement report is cancelled when beam information for all the candidate beams satisfying entry condition of an LTM event for TTT is transmitted.

Transmitting a truncated L1 MR MAC CE may inform a network that the beam information in the truncated L1 MR MAC CE is only for some of the candidate beams that satisfies the LTM event condition. The network cannot identify if all the candidate beams that satisfied the event condition is included in the truncated L1 MR MAC CE transmitted by UE. Similarly, the network cannot identify if the beam information pending for transmission consist of any candidate beams that satisfied the LTM event condition. Transmitting a truncated L1 MR MAC CE may inform the network that there are more beam information pending for transmission, for the triggered MR that resulted in UE transmitting the truncated L1 MR MAC CE.

Based on receiving a truncated L1 MR MAC CE, the network may schedule uplink grant to the UE for transmitting normal L1 MR MAC CE. There are 2 possible options on what to transmit in the normal L1 MR MAC CE. In a first option, UE transmits the normal L1 MR MAC CE containing the beam information for the N candidate beams - the candidate beams satisfying the LTM Event condition (i.e., all of the X candidate beams) and the candidate beams not satisfying the LTM Event condition (i.e., all of the Y candidate beams) - and the current serving beam. This includes the beam information for the candidate beams that have already been transmitted in the truncated L1 MR MAC CE. In a second option, the UE transmits the normal L1 MR MAC CE containing the beam information only for the candidate beams that are not transmitted in the truncated L1 MR MAC CE. The first option is not efficient as there is no advantage to the UE or the network in retransmitting the information that is already transmitted in the truncated L1 MR MAC CE. Instead, this may cause a problem if the available UL grant is sufficient to accommodate all the N candidate beams but is sufficient to accommodate all the N beams except the beams included in the truncated L1 MR MAC CE.

Proposal 6: A normal L1 MR MAC CE that transmitted after transmitting a truncated L1 MR MAC CE for a triggered measurement event does not contain the beam information for the candidate beams included in the truncated L1 MR MAC CE.

A UE triggers an SR, for the L1 MR, if there is no UL grant or if an available UL grant is not sufficient to accommodate either a normal L1 MR MAC CE plus its subheader, or a truncated L1 MR MAC CE plus its subheader. Transmitting a truncated L1 MR MAC CE may also serve as an indication to the network that there are pending beam information for the candidate beams or the current serving beam to be transmitted i.e., the truncated L1 MR MAC CE may serve the purpose of transmitting SR. However, unlike an SR, there is no procedure for the UE when the UE does not receive an uplink grant after transmitting the truncated L1 MR MAC CE. This may lead to a scenario where the UE is waiting for an uplink grant indefinitely, after transmitting the truncated L1 MR MAC CE. A UE behavior to handling this scenario is required. 1 possible way to handle this is by sending an SR or performing a RACH if the UE does not receive an uplink grant after transmitting the truncated L1 MR MAC CE.

Observation 6: There is no procedure to handle a scenario where a UE does not receive an uplink grant after transmitting a truncated L1 MR MAC CE.

Proposal 7: Introduce a timer for truncated L1 MR MAC CE, with the following behavior:

- The timer is started after transmitting a truncated L1 MR MAC CE for a triggered measurement event.

- The timer is stopped when an uplink grant is received.

- When the timer expires, transmit SR.

RAN2 agreed that normal L1 MR MAC CE and truncated L1 MR MAC CE are MAC CEs of variable size. Therefore, in order for the network to demultiplex the truncated/ normal L1 MR MAC CE in a MAC PDU, L1 MR MAC CE should include one or more fields to determine the size of the L1 MR MAC CE. As discussed above, a normal L1 MR MAC CE may contain beam information of up to N candidate beams. However, N is the maximum number of beams that can be included in the normal L1 MR MAC CE and not the actual number. If a UE has measurements for less than N candidate beams e.g., for M candidate beams, then the normal L1 MR MAC CE includes beam information for only the M candidate beams. A field in the normal L1 MR MAC CE should indicate that it contains beam information for M candidate beams. Additionally, the normal L1 MR MAC CE may contain beam information of the current serving beam. Another field in the normal L1 MR MAC CE should indicate whether L1 MR MAC CE contains beam information for the current serving beam. The network can determine the size of the MAC CE based on these fields.

Proposal 8: Introduce an explicit indication in MR MAC CE to inform the number of candidate beams included in the MR MAC CE.

Proposal 9: Introduce an explicit indication in MR MAC CE to indicate whether the MR MAC CE contains the current serving beam information.

RAN2 agreed [2] that explicit indications are introduced in MR MAC CE to inform which beams satisfy an event entry/ leaving condition for TTT. This will help the network to differentiate between a candidate beam that satisfies the event entry/ leaving condition from a candidate beam that does not satisfy the event entry/ leaving condition. In addition to this, it is required to also differentiate between the candidate beams and the current serving beam is some scenarios. For example, for event LTM2 and event LTM5, the current serving beam may also be a beam satisfying entry/ leaving condition for the event. There are 2 possible ways to achieve this. In an first option, introduce an explicit indication in the MR MAC CE to indicate the current serving beam. In a second option, a common field e.g., 2 bit length, to indicate all the type of beams.

Proposal 10: Introduce an explicit indication in MR MAC CE to indicate the current serving beam.

RAN2 agreed [2] that L1 MR MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR. According to [5], Network may configured a UE for reporting event triggered L1 MR for 2 LTM purposes – to select candidate beam/ cell to trigger early synchronization, or to select target beam/ cell to trigger LTM cell switch. Event LTM2 is only based on the current serving beam and does not depend on whether a candidate beam is strong or weak. Similarly, event LTM4 is based only on the candidate beam measurements and not the current serving beam. These LTM events are typically not used to trigger an LTM cell switch. For example, event LTM4 may inform the network that the UE is in an area where the candidate beam is available. Network may trigger early synchronization to the candidate beam. Event LTM2 may indicate that the current serving beam is becoming weak. Network may (re)configured event LTM 3 or event LTM 5. It can be seen that some of the above mentioned scenarios are more urgent in nature than the others. For example, a MR that can help the network to trigger an LTM cell switch is more urgent than an MR for event LTM 2 or event LTM4. We think the MRs triggered for some of these less urgent use cases can be transmitted with a lower LCH priority as compared to that of the more urgent use cases. We think a MAC CE for (Enhanced) BFR should be considered a higher LCH priority than the MAC CE for MR of less urgent use cases.

Proposal 11: RAN2 to discuss whether some MAC CE for L1 MR is considered as lower LCH priority than MAC CE for (Enhanced BFR).

# 3. Conclusion

The contribution discussed aspects related to event triggered L1 measurement report. The following observations and proposals are made:

LTM Event evaluation:

Observation 1: For a candidate cell, after triggering MR for a first candidate beam satisfying the LTM event, a second candidate beam may satisfy the LTM event.

Proposal 1: Introduce prohibit timer for event triggered L1 MR, with the following behavior:

- The prohibit timer is stared after triggering a new MR for a candidate cell satisfying an entry condition of an LTM event for TTT.

- While the prohibit timer is running, a new MR is not triggered for the same candidate cell for the same LTM event.

- While the prohibit timer is running, a new MR is triggered for the same candidate cell satisfying an entry condition of a different LTM event.

L1 MR validity:

Observation 2: The measurement results in a triggered MR may be outdated if an uplink resource for transmitting the triggered MR is available only after a significant time has elapsed since triggering the MR.

Proposal 2: RAN2 to discuss handling of outdated information in event triggered L1 MR.

L1 MR MAC CE:

Observation 3: A MAC PDU can accommodate multiple layer 3 measurement reports while a MAC PDU can only accommodate one event triggered L1 MR MAC CE.

Proposal 3: Multiple event triggered L1 MR MAC CEs can be included in a MAC PDU.

Observation 4: UE configured to transmit candidate beams not satisfying an LTM event condition, transmits a truncated L1 MR MAC CE when an available UL grant can accommodate all the candidate beams that satisfies an LTM event condition but cannot accommodate candidate beams that does not satisfy the LTM event condition.

Observation 5: UE configured to transmit current serving beam information, transmits a truncated L1 MR MAC CE when an available UL grant can accommodate all the candidate beams that satisfies an LTM event condition but cannot accommodate the current serving beam information.

Proposal 4: A normal L1 MR MAC CE is transmitted if available uplink grant is sufficient to accommodate a normal L1 MR MAC CE, comprising beam information of all the candidate beams satisfying the LTM event condition, plus its subheader.

Proposal 5: L1 measurement report is cancelled when beam information for all the candidate beams satisfying entry condition of an LTM event for TTT is transmitted.

Proposal 6: A normal L1 MR MAC CE that transmitted after transmitting a truncated L1 MR MAC CE for a triggered measurement event does not contain the beam information for the candidate beams included in the truncated L1 MR MAC CE.

Observation 6: There is no procedure to handle a scenario where a UE does not receive an uplink grant after transmitting a truncated L1 MR MAC CE.

Proposal 7: Introduce a timer for truncated L1 MR MAC CE, with the following behavior:

- The timer is started after transmitting a truncated L1 MR MAC CE for a triggered measurement event.

- The timer is stopped when an uplink grant is received.

- When the timer expires, transmit SR.

Proposal 8: Introduce an explicit indication in MR MAC CE to inform the number of candidate beams included in the MR MAC CE.

Proposal 9: Introduce an explicit indication in MR MAC CE to indicate whether the MR MAC CE contains the current serving beam information.

Proposal 10: Introduce an explicit indication in MR MAC CE to indicate the current serving beam.

Proposal 11: RAN2 to discuss whether some MAC CE for L1 MR is considered as lower LCH priority than MAC CE for (Enhanced BFR).

# References

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4

- Chairman Notes: TSG RAN WG2 Meeting #129

- Chairman Notes: TSG RAN WG2 Meeting #128

- Chairman Notes: TSG RAN WG2 Meeting #127bis

- Chairman Notes: TSG RAN WG2 Meeting #126

- TS 38.321: NR; Medium Access Control (MAC) protocol specification