3GPP TSG-RAN WG2 Meeting #129bis	R2-2502613

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.9.2

Source:	Huawei, HiSilicon, Turkcell

Title:	Further consideration on Store and Forward

Document for: Discussion and Decision

# 1	Introduction

In the last meeting, the following agreements were made regarding the Store & Forward (S&F) satellite operation [1]:

Agreements:

1.	RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change

2.	The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information)

3.	Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time).

4.	A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

In addition to this RAN2 received the reply LS on satellite IDs from SA2 as follows [2]:

- RAN2 understanding: The UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

[SA2 response]: SA2 has discussed the scenario and clarified this in the TS 23.401 clause 4.13.9.

Question: If a satellite is included in the satellite list signalled by the MME to a UE, does it mean one of the below options, or possibly other option(s):

- the satellite has the UE context but does not necessarily support store-and-forward operation; or

- the satellite has the UE context and must support store-and-forward operation (but may be operating in normal IoT NTN mode or in store-and-forward mode when the satellite serves the area where the UE is located)?

[SA2 Answer]: SA2 confirms that option 2 is the correct understanding.

In this contribution, we provide some further considerations on the Store and Forward satellite operation.

# 2	Discussion

## 2.1 Remaining time of current operating mode

### 2.1.1 Switching from normal mode to S&F mode

In the previous meetings, it was agreed that the remaining time for the satellite mode switch from S&F to normal would be broadcasted, while the switch from normal mode to S&F mode remains an FFS.

If the serving cell switches from normal mode to S&F mode, it indicates that the feeder link will become unavailable at certain point in time.

In the legacy mechanism, the t-service in SIB19 indicates when the current serving cell will stop providing service in the area it is currently covering. If the cell supports both normal mode and S&F mode, it may become unavailable to legacy UEs when switching to S&F mode. However, at this point, the t-service time has not yet been reached, creating uncertainty how NW and legacy UEs should behave in this situation.

Even if the NW releases the legacy UEs to RRC_IDLE, legacy UEs may not perform neighbouring cell measurement before t-Service time. One possible network implementation is to set the t-Service to the switching time from the normal mode to S&F mode. In this case, t-service serves as the indication of the remaining time of current operating mode. For the S&F UEs, a new t-service time (e.g., t-serviceSF) would be required to indicate the  stop time for the SF mode.

In that case, the UEs that do not support S&F will begin measurements and transition to normal cells by the time of the mode switch (i.e., t-service).  Meanwhile UEs that support S&F will continue to be served by the satellite until the new t-service time (e.g., t-serviceSF) is reached.

Proposal 1: The legacy t-Service is reused as the transition timing from normal mode to “S&F operation” mode while a new t-ServiceSF is introduced to indicate the stop-serving time of the SF mode.

### 2.1.2 Switching from S&F mode to normal mode

Regarding mode switching from S&F to normal, the following was noted as FFS:

1) FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode;

We believe the UE behaviour in spec should be clear as per the agreement. Specifically, UEs supporting S&F may choose not to initiate certain NAS procedures (e.g., ATTACH, TAU) immediately, but instead wait until the feeder link becomes available and then initiate the procedure. For the UEs that do not supporting S&F mode, they should wait until the cell switches to normal mode before camping on the cell.

Proposal 2: UEs supporting S&F may delay certain NAS procedures until the feeder link becomes available, while UEs not supporting S&F should wait until the feeder link is available before camping on the cell.

## 2.2	Paging in S&F satellite operation

In the legacy paging procedure, the arrival of MT data triggers the network to page UE. In the S&F scenario, when feeder link is available, the CN sends MT data to satellite, which receives and stores it. When service link is available, the satellite pages UE or the UE initiates RRC connection, at which point the network sends MT data to UE. However, if the satellite does not receive any MT data when feeder link is available, there is no need for the satellite to page UE when service link is available. In this case, the UE doesn’t need to monitor paging or initiate RRC connection.

Therefore, if UE is informed when the NW does not have any MT data buffered for an area, the UE does not need to monitor the paging or initiate RRC connection, which helps save UE power.

Proposal 3: If the NW doesn’t have any MT data buffered for an area, it informs the UE so that the UE doesn’t need to monitor paging or initiate RRC connection.

## 2.3	Access control

For both S&F architectures, the UE’s DL NAS data can be stored at the MME on board the satellite serving the UE at next pass. In this case, if the UE is unable to access the intended satellite, the stored data will be lost. Given that the current access control mechanism may lead to rejection of the UE’s RRC connection request due to the load status, it is beneficial for the eNB to prioritize the access of the UE, if it is subsequently re-attempting the access to the NW, to avoid potential data loss. The re-attempt from the UE can be indicated to the network while performing RRC Connection Establishment procedure

Proposal 4: S&F NW should prioritize the access of the UE, if it is subsequently re-attempting the access to the NW. The re-attempt from the UE can be indicated to the network while performing RRC Connection Establishment procedure.

## 2.4	Issue of satellite ID

According to the reply LS and the updated TS 23.401, the following RAN2 understanding is confirmed by SA2:“the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.”

In order to camp on the satellites that are not included in the NAS satellite ID list, the UE may need to continuously perform measurement for cell selection evaluation after being released to RRC_IDLE., This is to ensure that the UE  will not miss any passing satellites . However, in this case, the NAS wait timer becomes ineffective, leading to increased   power consumption for the UE. To alleviate this issue, when determine whether to perform measurements, the UE should also take into account the information for the subsequent satellites in SIB32, which was introduced for discontinuous coverage in Rel-17. In other words, UE if the t-ServiceStart for a satellite in SIB32 occurs earlier than the NAS wait timer, UE need to start performing measurement based on the t-ServiceStart to ensure UE can access that satellite.

Proposal 5: When determining whether to perform measurement in IDLE, the UE should consider both NAS information (i.e., Satellite list and NAS wait timer) and the information in SIB32 (i.e., t-ServiceStart and ephemeris).However, if the UE detects that the satellite indicated in SIB32 (but not contained in NAS satellite list) is operating in SF mode, it should not  access this satellite, as there would be no UE context on  this satellite. If there were, the CN would have included this satellite into the NAS satellite list.Proposal 6: If a satellite indicated in SIB32 but not  included in the NAS satellite list is operating in SF operation, the UE should not  access it.

# 3	Conclusion

This contribution discusses the store and forward operation for IoT NTN and we have the following proposals:

Remaining time of current operating mode

Proposal 1: The legacy t-Service is reused as the transition timing from normal mode to “S&F operation” mode while a new t-ServiceSF is introduced to indicate the stop-serving time of the SF mode.

Proposal 2: UEs supporting S&F may delay certain NAS procedures until the feeder link becomes available, while UEs not supporting S&F should wait until the feeder link is available before camping on the cell.

Paging in S&F satellite operation

Proposal 3: If the NW doesn’t have any MT data buffered for an area, it informs the UE so that the UE doesn’t need to monitor paging or initiate RRC connection.

Access control

Proposal 4: S&F NW should prioritize the access of the UE, if it is subsequently re-attempting the access to the NW. The re-attempt from the UE can be indicated to the network while performing RRC Connection Establishment procedure.

Issue of satellite ID

Proposal 5: When determining whether to perform measurement in IDLE, the UE should consider both NAS information (i.e., Satellite list and NAS wait timer) and the information in SIB32 (i.e., t-ServiceStart and ephemeris).Proposal 6: If a satellite indicated in SIB32 but not  included in the NAS satellite list is operating in SF operation, the UE should not  access it.

# 4	References

- R2-2501333, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 129.

- S2-2502450, Reply LS on Satellite IDs for store-and-forward operation, SA2.

- R2-2410913, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 128.