3GPP TSG-RAN WG2 #129-bis	R2-2502625

Wuhan, China, 7th – 11th April 2025

Source:	NTT DOCOMO, INC.

Title:	Random access types supported by AIoT devices

Document for:	Discussion

Agenda Item:	8.2.3

- Introduction

This contribution is to discuss random access in Ambient IoT system. As a background we agreed to support both 3-step CBRA and CFRA in RAN2#129 [1].

Agreements

- For Rel-19, only 3-step CBRA is supported for A-IoT

- We will specify both CBRA and CFRA.

- Re-use the subsequent paging message to trigger re-access.  There is no need to differentiate msg1 resource for initial access vs re-access.

- Discussion

The reader indicates Ambient IoT devices to perform either a 3-step CBRA or a CFRA in the A-IoT paging message. The CFRA indication is FFS on explicitly or implicitly, but the device reads the paging message to select the RA type to send Msg1 appropriate for the instructed RA type.

The comparison of the 3-step CBRA and CFRA procedures is shown in Figure 1.

Figure 1. 3-step CBRA and CFRA.

However, we are wondering whether all Ambient IoT devices can support both 3-step CBRA and CFRA. We particularly want to clarify this question for device type 1, which is considered to have a very simple design and is within the scope of Release 19. If some devices only support either 3-step CBRA or CFRA, the reader needs to indicate the RA type supported by the target device when sending the A-IoT paging message.

The simplest solution to this question is to assume that all devices support both 3-step CBRA and CFRA. If this is the case, the reader can freely indicate the RA type according to the situation.

Proposal 1.	All devices are assumed to support both 3-step CBRA and CFRA.

If we should allow devices not supporting both RA types, when the reader performs paging for multiple devices using a temporary device group ID, it must not include devices that only support CFRA. This imposes a significant constraint on the design of temporary device group IDs, which is likely managed by OAM, so we have a concern on this. Therefore, we should assume that all devices support at least 3-step CBRA, and CFRA can be optionally supported.

Proposal 2.	If Proposal 1 is not agreeable, devices are mandated to support 3-step CBRA and can optionally support CFRA.

Additionally, it is important to note that devices should not optionally support RA types without capability since it means that the reader may indicate random access not supported by target device, which results in frequent inventory failure.

From the reader's perspective, it is necessary to know the RA type supported by the target device before sending the paging message. As shown in Figure 1, to make this possible, it is essential to receive assistance information about the RA type within the service request from the CN (AIOTF). But this discussion needs a checking by SA2.

Proposal 3.	If Proposal 1 is not agreeable, the reader decides RA type based on assistance information from CN.

Proposal 4.	If Proposal 1 is not agreeable, RAN2 sends LS to SA2 whether the capability information on RA type per device is available.

- Summary and proposal

Proposal 1.	All devices are assumed to support both 3-step CBRA and CFRA.

Proposal 2.	If Proposal 1 is not agreeable, devices are mandated to support 3-step CBRA and can optionally support CFRA.

Proposal 3.	If Proposal 1 is not agreeable, the reader decides RA type based on assistance information from CN.

Proposal 4.	If Proposal 1 is not agreeable, RAN2 sends LS to SA2 whether the capability information on RA type per device is available.

- References

[1]	Chair notes, RAN2#129.