3GPP TSG-RAN WG2 #129bis					      		 R2-2502626

Wuhan, China – April 7 - 11, 2025		                          Revision of R2-2500935

Agenda item:	8.13.4

Source: 	Kyocera

Title: 	Service Continuity for U2N multihop relay

Document for:	Discussion

# Introduction

In this contribution, we discussed the issues related to service continuity objectives for U2N multihop relay.

# Discussion

According to the Objective from the WID [1], the following are the objectives for multihop U2N relay for service continuity.

2. Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE):

First Priority:

A.	Intra-gNB multi-hop indirect to direct path switching using existing framework

B.	Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

Second Priority in order of importance:

C.	Intra-gNB direct to multi-hop indirect path switching

D.	Intra-gNB single-hop indirect to multi-hop indirect path switching

The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this contribution, we focus on key details for Scenarios C and D since the main aspects of Scenarios A and B have already been captured, while we also consider if the legacy measurement events are sufficient for Scenarios A and B.

2.1	Measurement configuration and reporting for Scenarios C & D

During the RAN2#129 meeting, the following agreements are already reached for Scenarios A and B:

For multi-hop i2d, Remote UE evaluates both relay link (for the link between remote UE and serving first relay UE) and Uu link, where the sidelink relay measurement report shall include at least serving first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

For multi-hop i2d, first relay UE evaluates adjacent upstream relay link and Uu link, where the sidelink relay measurement report shall include at least measured relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.  FFS if intermediate relay UE reports.

For multi-hop i2i, the remote UE reports serving first relay UE and candidate single-hop relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information.

For multi-hop i2i, first evaluates adjacent upstream relay link and candidate relay link, where the sidelink relay measurement report shall include at least serving intermediate/ last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.  FFS if intermediate relay UE reports.

For Scenario C (direct to multihop indirect path switch), the flow diagram in Figure 1 could be considered, which is similar to the Rel-17 case for intra-gNB direct to indirect path switch in Figure 16.12.6.2-1 of [2], except the indirect path has multiple relay UEs (Intermediate Relay UE  Last Relay UE  gNB).

Figure 1: Procedure for direct to multihop indirect path switch (Scenario C)

1.	The Remote UE reports one or multiple candidate Relay UE(s) and Uu measurements, after it measures/discovers the candidate Relay UE(s), based on measurement configuration using the legacy events: Event Y1 (PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2) or Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold).

2.	The gNB decides to switch the Remote UE to a target indirect path consisting of the Intermediate Relay UE and the Last Relay UE within the same gNB.

3.	The gNB sends an RRCReconfiguration messages to both target Relay UEs, which includes at least the remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

4.	The gNB sends the RRCReconfiguration message to the Remote UE. The RRCReconfiguration message includes at least the target Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic, and the associated end-to-end radio bearer(s).

5.	The Remote UE establishes PC5-RRC connection with target Intermediate Relay UE, if it’s not yet PC5-RRC connected.

6.	The Remote UE sends the RRCReconfigurationComplete message to the gNB via the target Relay UEs.

7. The data path is switched from direct path to indirect path between the Remote UE and the gNB.

- Proposal 1	RAN2 should adopt Figure 1 as the baseline procedure for the direct to multihop indirect path switch (Scenario C).

In order to simplify the procedure for Scenario C, the WID stated that “The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.”, which means both target Relay UEs may be considered to be fully controllable by the gNB and no RRC establishment are needed for the target Relay UEs.  However, this also means only candidate relay UEs that are RRC Connected are useable for path switch.  Since the remote UE does not know whether any of the candidate relay UEs are already in RRC Connected, the following two options should be considered.

- Option 1: The remote UE will include all candidate relay UEs in the measurement report and it is up to the gNB to determine which of the candidate relay UE’s are in RRC Connected and applicable for the path switch.

- Option 2: The candidate relay UEs should indicate to remote UEs of its RRC state:

With Option 1, the gNB should be able to figure out which candidate relay UEs are in RRC Connected.  And, if necessary (e.g., all the candidate relay UEs are not in RRC Connected), the gNB has the option to page the candidate relay UEs and move the desired Candidate relay UEs to RRC Connected before executing the path switch, although this would may increase significant delay in completing the path switch.

With Option 2, only the candidate relay UEs in RRC Connected will be included in the measurement report. Furthermore, only the candidate relay UEs in RRC Connected would be considered for event triggering. For example, neither event Y1 nor event Y2 would be triggered based on candidate relay UEs that are not in RRC Connected. Furthermore, this reduces signalling overhead, which is helpful esp. when the Uu link has already degraded.

- Proposal 2	RAN2 should consider if the remote UE should know the RRC state of the candidate relay UEs.

Based on the discussions in RAN2#128, it was further agreed in the control plane discussion that “In one approach (“approach 2”) of U2N relays, Intermediate Relay UEs (other than the Last Relay UE) can be in any RRC state when the U2N remote UE is in RRC_CONNECTED.”; therefore, it is necessary for the gNB to decide whether the path switch will be for Approach 2 or Approach 1.  And it should also be further considered if the remote UE is allowed to perform path switch based on either Approach 1 or Approach 2.

- Proposal 3	RAN2 should consider if path switch from direct to multihop indirect path switch can be supported with Approach 2.

For Scenario D (single-hop indirect to multihop indirect path switch), the flow diagram in Figure 2 could be considered, which is similar to the Rel-17 case for intra-gNB indirect to indirect path switch in Figure 16.12.6.3-1of [2],

Figure 2: Procedure for single-hop indirect to multihop indirect path switch (Scenario D)

1.	The Remote UE reports one or multiple candidate Relay UE(s) and Uu measurements, after it measures/discovers the candidate Relay UE(s), based on measurement configuration using the legacy events: Event X2 (Serving L2 U2N Relay UE becomes worse than threshold), Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold) and Event Z1 (Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2).

2.	The gNB decides to switch the Remote UE to a target indirect path consisting of the Intermediate Relay UE and the Last Relay UE within the same gNB.

3.	The gNB sends an RRCReconfiguration messages to both target Relay UEs, which includes at least the remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

4.	The gNB sends the RRCReconfiguration message to the Remote UE. The RRCReconfiguration message includes at least the target Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic, and the associated end-to-end radio bearer(s).

5.	The Remote UE establishes PC5-RRC connection with target Intermediate Relay UE, if it’s not yet PC5-RRC connected.

6.	The Remote UE sends the RRCReconfigurationComplete message to the gNB via the target Relay UEs.

7. The data path is switched from direct path to indirect path between the Remote UE and the gNB.

- Proposal 4	RAN2 should adopt Figure 2 as the baseline procedure for the single-hop indirect to multihop indirect path switch (Scenario D).

During the discussion for Scenario B, it was decided that the target Last relay UE cannot be the intermediate relay UE from the source indirect path, using a single procedure.  For Scenario D, since the source relay UE is already the last relay UE it can also serve as the last relay UE in the target indirect path.  This assumes the target intermediate relay UE is connected to the gNB via the same last relay UE as in the source last relay UE.  However, in case the gNB’s decision to perform the path switch in Scenario D is triggered by radio link degradation on the Uu link, it should also be considered if the source relay UE is allowed to serve as the target intermediate relay UE in the target indirect path.

- Proposal 5	RAN2 should consider if there’s any restriction for the source relay UE to serve as a target intermediate relay UE in Scenario D.

2.2	Measurement events considerations for Scenarios A and B

Currently, the existing event triggers are targeted for the remote UE to perform measurement reports when one of the existing events (i.e., Events X1, X2, Y1, Y2 and Z1) is fulfilled.  With multihop relay, the RRC Connected intermediate relay UEs will also perform its own measurement reports to the gNB as a “remote UE” based on its measurement configuration.  Therefore, the measurement report from the intermediate relay UE (i.e., in case of Scenario A or B), could trigger to the gNB to decide to perform path switch.  This assumes the gNB’s latest measurement report from the remote UE is sufficiently accurate, which is not always the case.  In Rel-17 U2N relay, it was assumed the relay UE’s radio link to the gNB is sufficiently stable (in order for it to serve as a relay UE), so the degradation of the source indirect path would likely be triggered by the PC5 connection between the remote UE and the relay UE.  In our view, the following options should be considered:

- Option A: Leave it for NW implementation using legacy measurement configuration and event triggers.

- Option B: Specify new event(s) to trigger the remote UE to perform measurement reports (e.g., to synchronize the timing of measurement reports by both intermediate relay UE and the remote UE).

- Option C: Allow the use of Conditional path switch.

With Option A, if the gNB receives measurement report from one of the intermediate relay UEs and the gNB requires updated measurement report from the remote UE, it has the option to request a measurement report from the remote UE, assuming the radio condition of the source indirect path is still good enough.

With Option B, the intermediate relay UE that was triggered to send the measurement report may provide a notification to its child intermediate relay UE and/or the remote UE to send an updated measurement report to the gNB.

With Option C, the remote UE may be configured to perform path switch when the condition for path switch is met, similar to legacy CHO, with the exception that the target is a relay UE.  Since the path switch is limited to intra-gNB, additional complexity for cell preparation isn’t needed.

- Proposal 6	RAN2 should consider if the legacy measurement events are sufficient for support of Scenarios A and B.

2.3	Path switch for intermediate relay UE

Similar to discussions regarding the intermediate relay UE’s relay reselection, e.g., as discussed in [3], it should be further discussed what the remote UE’s behaviour should be when the intermediate relay UE, acting as a remote UE also needs to perform path switch to a target direct or indirect path.  It is assumed that both the intermediate relay UE and the remote UE are expected to provide their respective measurement reports to the gNB.   The result of the measurement reports may indicate that intermediate relay UE requires service continuity via a target direct/indirect path to the same gNB, while no measurement report is triggered from remote UE (i.e., no change is needed for the remote UE’s PC5 connection to the intermediate relay UE).

- Observation 1	In some cases, based on measurement reports received at the gNB, only the intermediate relay UE is triggered for path switch.

In the scenario described above, if the legacy notification principle is used, the intermediate relay UE would need to notify the PC5 connected remote UE prior to path switch, or the gNB can optionally release the remote UE from RRC connection since there are no alternative connection for the remote UE for path switch (i.e., no measurement report is triggered from the remote UE).  If such legacy rule is applied, the remote UE will need to establish or re-establish a connection to the gNB which will be highly inefficient and disruptive to the service, which is counter to the principle of service continuity.

- Observation 2	If the remote UE is required to always release the PC5 connection or RRC connection whenever the intermediate relay UE performs path switch, it will result with inefficient path switch for the remote UE.

To mitigate the inefficiency in path switch, it should be possible for the intermediate relay UE to perform direct or indirect path switch to the same cell without sending notification to the remote UE prior to path switch and the PC5 connection to the remote UE is kept.

- Proposal 7	It should be an option for intermediate relay UE to perform direct or indirect path switch to the same cell without sending notification to the remote UE prior to path switch.

After the path switch is completed and it is towards the same cell, it is likely that the path switch is transparent to the remote UE since it remains PC5 connected to the same intermediate relay UE.  It’s only the case when the intermediate relay UE’s path switch resulted in a change in target cell when the remote UE needs to be reconfigured with updated SL-RLC and SRAP.  To avoid the complexity that result in cell change, the intermediate relay UE will only need to notify the remote UE in case the path switch failed, or the path switch resulted in a cell change. The notification will allow the remote UE to perform its own RRC re-establishment in case it cannot be path switched to any candidate relay UE/cell.

- Proposal 8	The intermediate relay UE will only need to notify the PC5 connected remote UE in case the path switch fails or the path switch resulted in a cell change.

# Conclusion

In this contribution, we addressed the issues that may arise with multihop relay for service continuity.  RAN2 is kindly asked to take into account the observations and proposals below:

- Proposal 1	RAN2 should adopt Figure 1 as the baseline procedure for the direct to multihop indirect path switch (Scenario C).

- Proposal 2	RAN2 should consider if the remote UE should know the RRC state of the candidate relay UEs.

- Proposal 3	RAN2 should consider if path switch from direct to multihop indirect path switch can be supported with Approach 2.

- Proposal 4	RAN2 should adopt Figure 2 as the baseline procedure for the single-hop indirect to multihop indirect path switch (Scenario D).

- Proposal 5	RAN2 should consider if there’s any restriction for the source relay UE to serve as a target intermediate relay UE in Scenario D.  -

- Proposal 6	RAN2 should consider if the legacy measurement events are sufficient for support of Scenarios A and B.

- Observation 1	In some cases, based on measurement reports received at the gNB, only the intermediate relay UE is triggered for path switch.

- Observation 2	If the remote UE is required to always release the PC5 connection or RRC connection whenever the intermediate relay UE performs path switch, it will result with inefficient path switch for the remote UE.

- Proposal 7	It should be an option for intermediate relay UE to perform direct or indirect path switch to the same cell without sending notification to the remote UE prior to path switch.

- Proposal 8	The intermediate relay UE will only need to notify the PC5 connected remote UE in case the path switch fails or the path switch resulted in a cell change.

# References

- RP-242349, “Revised WID on NR sidelink multi-hop relay”

- TS 38.300, “NR; NR and NG-RAN Overall Description”, Stage 2

- R2-25xxxxx, “Relay (re)selection under multihop relay”, Kyocera