3GPP TSG-RAN WG2 #129bis	R2-2502650

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.3.5

Source:	Ericsson

Title:	Discussion on LCM for AI for Mobility

Document for:	Discussion

# 1	Introduction

The study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 [1]. One of the objectives included in the SID is the Life Cycle Management (LCM) of the AI/ML model:

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

This contribution discusses the LCM framework for the AI/ML for mobility use cases.

# 2	Discussion

## General LCM framework

LCM of an AI/ML model is a series of procedures which allow the correct configuration and operation of the AI/ML model. It typically consists of the following main components:

- Data collection for training, inference and performance monitoring

- AI functionality management (e.g. activation, deactivation, switching, and fallback operation).

- Inference operation and the configurations

- Functionality level performance monitoring

A functional framework showing the different components of LCM (adopted from AI for PHY study) and their relations in depicted in Figure 1

Figure 1: Functional framework for LCM

In general LCM applies to both UE side model and NW side model and the relevant functionalities and procedures need to be studied in RAN2.

In particular, for UE-side model, the main aspects to consider are related to the data collection (triggering and reporting), inference configuration, functionality management (i.e., applicability, activation and deactivation of the AI based functionality), and functionality level performance monitoring and applicability reporting.

For the NW side model, the main aspect to consider is the data collection from the UE, beneficial for the NW to perform model training, inference, and its evaluation.

For UE-side model, RAN2 consider the aspects related to data collection configuration for model training, inference configuration, functionality management (i.e., activation and deactivation), functionality level performance monitoring and applicability reporting.

For network-side model, RAN2 consider the aspects related to data collection (triggering, logging and reporting) from the UE for the sake of model training, input for inference, and its output evaluation.

In the following sections we discuss some of the above aspects from standardization impacts that need to be taken into consideration.

## UE-side model – Discussion on the signaling framework and configuration

During the study item RAN2 has investigated the performance of the AI/ML model for various mobility use cases targeting two main goals

- Reducing the measurement overhead at the UE which can be achieved using temporal, spatial and frequency domain predictions

- Enhancing mobility procedure KPI which can be achieved by temporal predictions

We have observed that for each of the above target and the prediction dimension (temporal, spatial and frequency) different configuration might be needed to enable the inference engine for the UE sided model. We believe, similar to AI for PHY such UE configurations should be set by network node serving the UE.

Similar to the AI for PHY WI, RAN2 agree that for the AI for Mobility, network configures the UE with the inference configuration, including the necessary settings to run the AIML based functionality.

When it comes to transmit the AIML related configurations (e.g., inference configuration), we think a preliminary step for discussion is to agree over which layer and using what signaling the inference configuration should be configured. Given that CU-CP component of the gNB is the main consumer of the AI for mobility use cases in RRC_Connected mode, we believe for the Rel-19 discussion RRC layer with dedicated RRC signaling should be chosen for the inference configuration.

RAN2 agree that for Rel-19 discussion dedicated RRC signalling is used for configuring the UE with AI/ML related configuration.

In RRC layer, different options may exist in configuring the UE with AIML inference configuration, we think the existing RRM measurement framework - consisting of the triplet of measConfig, measObject and reportConfig - provides a very good foundation to build the inference configuration.

RRM measurement framework, consisting of the triplet - measurement config, measurement object and report config - can potentially form the essential foundation for RRM measurement predictions configuration.

In fact, the measurement configuration triplet consists of two following main components:

- measObject: Indicating the carrier frequency wherein the UE should perform the measurements – indicating what to measure

- reportConfig: Indicating the content of the measurement report and the report type (event triggered or periodic) – indicating what and how to report

The above configuration information is linked using a measID which is configured as part of measurement configuration. The relation between these pieces of information configured as part of measurement configuration (measConfig IE) is schematically shown in the following figure.

Figure 2: Schematic view of RRM measurement configuration consisting the triplet of measConfig including a list of measIDs, measurement object (indicated by the measObjectID) and report configuration (indicated by reportConfigID).

For the AI for Mobility use case, we think the predictions at the UE side can be treated as measurements and hence the configuration can follow a similar signaling solution.

RRM measurement configuration framework (principle of the triplet similar to measurement config, measurement object and report config) is taken as baseline for configuring the UE with inference configuration.

If the above proposal is agreeable, we think it is straightforward to consider RRM measurement reporting framework (e.g., MeasurementReport) for reporting the measurements to the network.

RRM measurement report can serve as the foundation for the prediction-based reports, enabling trigger-based reporting and incorporating predictions within the report.

RRM Measurement Report is taken as baseline for reporting predictions to the network.

## Consideration on event prediction

Mobility events and reporting are designed to provide input to the network which the network can use when taking handover decisions. Events A1 and A2 are normally used as input to configure A3, A4 and A5 events and the A3 and A5 events are mainly used for the legacy L3 based handover decisions. Therefore, events A1, A2, A3 and A5 seem suitable to be considered for measurement event predictions in this study item. Event A4 and A6 are useful when it comes to CA and DC operations. Therefore, we propose to consider the above events for the normative phase.

Standard impact of the predictions of Events A1 to A6 are considered for the normative phase.

In the following we focus on the A3 event, but the same analysis can be applicable to other events.

Event A3 is defined in the RRC TS 38.331 according to the following, wherein the event is fulfilled if all the measurements in the (TTT) period fulfil the entry condition (Inequality A3-1) of the A3 event.

Mn and Mp represent the measurements of a neighbouring cell and an SpCell respectively at the present time (t0).

Therefore, prediction of the A3 event can be seen as applying the same A3 entry condition but for the predicted RSRP value in the future instead of the actual measurements (i.e., using temporal prediction), as shown below.

[t] indicates the future time instance the UE should perform the predictions and evaluate the entry conditions. Once the UE predictions and evaluation indicates fulfilment of the A3 event for a time period of Time-To-Trigger (TTT), the UE triggers a report to the network including the available measurements and predictions.

In this example temporal predictions are used. However, we believe the spatial and frequency domain predictions can be equally useful for the event prediction. For instance, the neighbour cell measurement (Mn) could be substituted with spatial or frequency domain predicted measurements at the current time instance. More precisely, utilizing temporal domain RRM measurement predictions in event evaluation can enhance the reliability of the mobility decisions by preponing the triggering of the measurement reports (earlier preparation of the handover). Additionally spatial and frequency domain predictions can reduce the measurement overhead when evaluating the mobility events at the current time instance.

According to the analysis above, it is proposed to reuse the event prediction with the purpose of mobility event predictions.

Temporal, spatial and frequency domain RRM predictions (on neighbouring cells and serving cell) should be reused for different event predictions.

During the study phase, companies have discussed and evaluated different methodologies known as direct and indirect event prediction methods. It has been observed that the performance of the AIML model in both methods is comparable. However, the indirect method provides an advantage by maximizing the reusability of the AIML models developed for the RRM measurements prediction. Specifically, AIML models trained for the predicting RRM measurements in temporal, spatial and frequency domains, can also be reused for mobility event predictions. This reduces the overhead complexity of the AIML signalling solutions in RRC specification.

For the sake of maximizing the reusability for RRM prediction models, RAN2 only consider indirect event prediction.

However, if the above solution is not agreeable, we believe the RRC solution should stand methodology-agnostic. Developing AIML models using different methodologies should not results in two separate RRC signalling solutions for the two mentioned methodologies (e.g., separate set of parameters per AIML methodology should be avoided). For event prediction we believe if 3GPP decides to support both methodologies the RRC signalling solution should remain independent of the used methodology.

If the above proposal is not agreeable, RAN2 agree that developing AIML models using different methodologies (direct/indirect) should not results in two separate RRC signalling solutions for each method (e.g., separate set of parameters per AIML methodology should be avoided)

## Network sided model – data collection framework and a basic principle

For the network side model, the primary consideration in RAN2 discussions should focus on three aspects related to data collection

- Data collection required for model training (used by the network to train AIML model)

- Data collection for model inference at the network side.

- Ground truth collection (used by the network to assess model performance)

RAN2 study the data collection for from the UE for the following cases

- Data collection from UE for model training

- Data collection from UE for model inference at the network side (model input)

- Ground truth collection from UE for evaluation of the model output

Given that we need to study the standard impact we think one primary discussion should be on the signalling solutions to enable the above data collection for different purposes at the network side. In light of the above, we think the RRM measurement framework, with some potential enhancement (with respect to triggering, logging and report retrieval procedure) can be considered as baseline for the data collection for the various purposes mentioned above.

RRM measurement framework is considered for the data collection for model training, inference and ground truth collection for assessment of network side model.

Modification of RRM measurement framework (e.g., how to trigger data collection, whether/how to log the data and how to report the data) is considered to enable data collection for model training, inference and ground truth collection for assessment of network side model.

However, when collecting data for training network-side models for mobility use cases, certain mobility-specific aspects (distinct from those relevant to AI-based PHY beam management use case) should be carefully considered. For example, the inter-frequency measurements should be performed by the UE as part of data collection procedure. However, in our understanding the data collection is a slow running process that should be conducted by long measurement campaign. The overhead of the inter-frequency measurements for the data collection purpose, can be significantly deteriorate the other higher priority functionalities at the UE such as legacy RRM measurements used for immediate mobility decisions. Another functionality that can be impaired by the long running data collection campaign with inter-frequency measurements is the uplink/downlink data transmission that could potentially be interrupted by the UE if the UE uses measurement gaps for the data collection purpose. Based on the above analysis we have the following proposals.

Data collection for network side model training is a slow process and need to be running for a long time. Hence it should not induce any impairment on the high priority features such as data transmission and RRM measurements used for immediate mobility decisions.

Inter-frequency measurements for the data collection purpose should not impair the RRM measurements and the UL/DL data transmission.

RAN2 investigate solutions on how to avoid impairment of the inter-frequency data collection on the other high priority process such as RRM measurements and UL/DL data transmission

## Network state and the associated ID for mobility use case

The notion of associated ID was discussed and agreed in AI for PHY WI, and enables the UE and network to be in-synch on the network state at the time of training and inference. The model trained with a configuration associated with the associated ID may provide the best performance at the time of inference if the network uses the same configuration represented by the same associated ID.

Similar to AI for PHY, what exactly the associated ID represents can be left to the network implementation. However, unlike the AI for PHY use cases, wherein the associated ID was limited to the state of a single cell (serving cell), a solution based on associated ID for mobility purpose shall be able to represent the state of the serving cell as well as multiple neighboring cells. Therefore, we propose as part of LCM study that RAN2 discusses how to enable representation of neighboring cells by means of Associated IDs.

Associated ID from AIML for PHY Beam Management is taken as baseline for Mobility use case.

Similar to Beam Management use case, what the associated ID represents for mobility use case is left to network implementation.

Unlike Beam Management use case, wherein the associated ID was limited to a single cell, for the mobility use case the solution shall be able to represent the state of multiple neighbouring cells

RAN2 discuss how to extend the associated ID to consider the state of the neighbouring cells in model training.

## Discussion on the use cases

In the previous meeting some companies discussed the use cases of the RRM predictions e.g., using them merely in CHO preparation. In our understanding RRM measurement predictions reported by the UE to the network can be used for various purposes and we should not limit the use cases upfront, in particular, if utilizing such the predictions in such use cases come for free.

RRM measurement predictions and event predictions can be considered for any relevant use cases e.g., mobility, dual connectivity and carrier aggregation.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	RRM measurement framework, consisting of the triplet - measurement config, measurement object and report config - can potentially form the essential foundation for RRM measurement predictions configuration.

Observation 2	RRM measurement report can serve as the foundation for the prediction-based reports, enabling trigger-based reporting and incorporating predictions within the report.

Observation 3	Data collection for network side model training is a slow process and need to be running for a long time. Hence it should not induce any impairment on the high priority features such as data transmission and RRM measurements used for immediate mobility decisions.

Observation 4	Unlike Beam Management use case, wherein the associated ID was limited to a single cell, for the mobility use case the solution shall be able to represent the state of multiple neighbouring cells

Based on the discussion in the previous sections we propose the following:

Proposal 1	For UE-side model, RAN2 consider the aspects related to data collection configuration for model training, inference configuration, functionality management (i.e., activation and deactivation), functionality level performance monitoring and applicability reporting.

Proposal 2	For network-side model, RAN2 consider the aspects related to data collection (triggering, logging and reporting) from the UE for the sake of model training, input for inference, and its output evaluation.

Proposal 3	Similar to the AI for PHY WI, RAN2 agree that for the AI for Mobility, network configures the UE with the inference configuration, including the necessary settings to run the AIML based functionality.

Proposal 4	RAN2 agree that for Rel-19 discussion dedicated RRC signalling is used for configuring the UE with AI/ML related configuration.

Proposal 5	RRM measurement configuration framework (principle of the triplet similar to measurement config, measurement object and report config) is taken as baseline for configuring the UE with inference configuration.

Proposal 6	RRM Measurement Report is taken as baseline for reporting predictions to the network.

Proposal 7	Standard impact of the predictions of Events A1 to A6 are considered for the normative phase.

Proposal 8	Temporal, spatial and frequency domain RRM predictions (on neighbouring cells and serving cell) should be reused for different event predictions.

Proposal 9	For the sake of maximizing the reusability for RRM prediction models, RAN2 only consider indirect event prediction.

Proposal 10	If the above proposal is not agreeable, RAN2 agree that developing AIML models using different methodologies (direct/indirect) should not results in two separate RRC signalling solutions for each method (e.g., separate set of parameters per AIML methodology should be avoided)

Proposal 11	RAN2 study the data collection for from the UE for the following cases

	Data collection from UE for model training

	Data collection from UE for model inference at the network side (model input)

	Ground truth collection from UE for evaluation of the model output

Proposal 12	RRM measurement framework is considered for the data collection for model training, inference and ground truth collection for assessment of network side model.

Proposal 13	Modification of RRM measurement framework (e.g., how to trigger data collection, whether/how to log the data and how to report the data) is considered to enable data collection for model training, inference and ground truth collection for assessment of network side model.

Proposal 14	Inter-frequency measurements for the data collection purpose should not impair the RRM measurements and the UL/DL data transmission.

Proposal 15	RAN2 investigate solutions on how to avoid impairment of the inter-frequency data collection on the other high priority process such as RRM measurements and UL/DL data transmission

Proposal 16	Associated ID from AIML for PHY Beam Management is taken as baseline for Mobility use case.

Proposal 17	Similar to Beam Management use case, what the associated ID represents for mobility use case is left to network implementation.

Proposal 18	RAN2 discuss how to extend the associated ID to consider the state of the neighbouring cells in model training.

Proposal 19	RRM measurement predictions and event predictions can be considered for any relevant use cases e.g., mobility, dual connectivity and carrier aggregation.

# References

RP-234055,	SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR