3GPP TSG-RAN WG2 Meeting #129bis                                    	R2-2502653

Wuhan, China, April 7th – 11th, 2025

Agenda item:	7.0.2.17

Source:	Qualcomm Incorporated

Title:	Way forward for soft satellite switch with resync

Document for:	Discussion and Decision

# Introduction

RAN2 has received LS reply from RAN1 and RAN4 in [1] and [2] with further counter questions. In this document, we provide a way forward not to extend the discussion and complicate the Rel-18 soft satellite switch with resync feature of a frozen specification.

# Discussion

The current procedure for soft satellite switch with resynchronization is defined as follows in TS 38.331.

Figure 1 time variant SMTC

In NTN, the propagation delay is inherently time variant. As shown in figure 1, the SMTC configuration needs to be updated by network periodically as in RRC_IDLE, we do not have mechanism of autonomous SMTC adjustment in RRC_CONNECTED mode. Therefore, we do not have a mechanism to save a RRC reconfiguration for the purpose of SMTC update.

In NTN, the propagation delay is inherently time variant. As shown in figure 1, the SMTC configuration needs to be updated by network periodically.

In RAN2#126, it was clarified that when the UE completes satellite switch in HARD satellite switch case, the SSB location of the cell is not changed.

RAN2#126 agreement:

RAN2 understands that in hard switch (t-serviceStart absent), cell timing/SSB location is unchanged (at gNB)

In case of soft satellite switch, i.e., when t-serviceStart is present, it should be clear that cell timing/SSB location will change depending on the value of ssb-TimeOffset. However, the MIB is not supposed to be changed as it is same serving cell.

It means if MIB is acquired in SFN x, then the SFN where MIB was acquired from the target satellite after ssb-TimeOffset should also be SFN x, i.e., SSB and SFN mapping should work in the same way in the target satellite.

Due to this, the SMTC may also need to be updated in the new serving satellite. However, as mentioned earlier, regular update of SMTC is inevitable and there is no need for adding complexity to the feature such as shifting the SSB location back and forth.

Figure 2 An example of satellite switch with resync operation

In our view, immediately after switching to the new satellite (i.e., after t-Service), there is no need for shifting the SSB location back due to following reasons.

  - Current procedure of SSB to SFN mapping should be preserved.

  - During period between [t-ServiceStart and t-Service], new UEs may find the cell and join the cell.

  - Regular SMTC update anyway would be needed and there is no guarantee that a RRC reconfiguration will be avoided.

  - The previously configured PDD reporting will not work. This has to be reconfigured with new satellite with a new RRC reconfiguration message. So even the new solution does not guarantee to avoid this.

  - When a UE performs satellite switch with resync, the UE may not immediately have updated list of neighbor satellites to perform measurements.

    - As exact time to acquire SIB19 is left to UE implementation.

  - After joining at t-Service, the new satellite’s t-Service would still be far away. There must be still enough service duration with the new satellite which gives no motivation to immediately start neighbor cell measurements for handover.

Immediately after joining the new serving satellite, handover is not expected and so is the measurements of the neighbor cells. As exact time to acquire SIB19 is left to UE, the UE may not immediately have up to date information of the neighbor satellites.

After connecting to the new satellite, (1) the frame boundary and SFN number is based on the SSB location in target satellite, (2) the neighbor satellite may have gone through the soft satellite switch procedure and stored list of neighbor satellites may not be useful for measurement, (3) the exact time to acquire SIB19 for the new updated neighbor satellite information is up to UE and the UE may not be able to immediately start neighbor satellite measurements, and (4) the UE expects RRC reconfiguration as part of periodic SMTC update.

Therefore, in our view, the existing procedure captured in the current specification for soft satellite switch with resync is sufficient and we suggest sending LS response with the clarification.

- RAN2 decides not to change the current specification texts related to satellite switch with resynchronization feature.

For measurements, clarify that the UE takes into account the fact that the exact time to acquire SIB19 for the new updated neighbor satellite information is up to UE, the neighbor satellite may have gone through the soft satellite switch procedure and stored list of neighbor satellite may not be useful for measurement until new SIB19 is acquired, measurement for handover is not expected immediately after joining the new satellite with full remaining service duration.

The UE may still perform measurement with the configured SMTC and the UE expects RRC reconfiguration as part of periodic SMTC update same as in existing procedure.

Send LS response to RAN1 and RAN4 with the above clarification.

Response to RAN1 and RAN4 questions

We have also received some questions from RAN4 in [1] and from RAN1 in [2].

As defined currently in the specification, the offset of the SSB of the target is ssb-TimeOffset from the SSB of the source at Uplink Synchronization Reference point (ULSRP). The subframe boundaries of the source and target at ULSRP should align. The mapping of SSB to SFN should not be changed for target satellite. Therefore, we propose to clarify this to RAN4. Draft is provided below.

In addition, RAN4 would like RAN2 to confirm which one of the following is the correct understanding of parameter ssb-TimeOffset, - Option 1: the relative offset between the SSB of the source satellite and the SSB of the target satellite, and the starting boundaries of SFN0 of the source satellite and the target satellites are aligned at the uplink time synchronization reference point - Option 2: the relative offset between the starting boundaries of SFN0 of the source satellite and the target satellite RAN2 Answer: RAN2 thinks Option 2 is correct understanding and would like to clarify that the starting boundaries of subframes of source and target satellites are aligned at the Uplink Synchronization Reference Point (ULSRP). But starting boundaries of SFN 0 of the target satellite and (SFN 0 + ssb-TimeOffset) of the source satellite are aligned at ULSRP.

Similarly, we can clarify to RAN1 that we do not expect any RAN1 specification change as a result of this solution. Draft is provided below.

RAN1 Question: Is RAN2 asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service, or whether RAN2 assumes that some RAN1 specification work may be needed? RAN2 Answer: RAN2 didn’t assume any RAN1 specification work is needed. RAN1 Question: RAN1 assumes that the meaning of “the SSB pattern is changed” is that only the time offset is changed (may be determined by ssb-TimeOffset). RAN2 Answer: RAN2 confirms that “the SSB pattern is changed” is that only the time offset determined by ssb-TimeOffset is changed. RAN1 Question: RAN1 assumes that the SIB19 of the source satellite includes ephemeris information of the target satellite, and that such information will not get outdated by the time the satellite switch occurs. RAN2 Answer: RAN2 confirms that SIB19 of the source satellite provides the target satellite information with epoch time and validity duration based on the timing of the cell served by the source satellite.

In response to RAN4 question, RAN2 clarifies that the starting boundaries of subframes of source and target satellites are aligned at the Uplink Synchronization Reference Point (ULSRP) and starting boundaries of SFN 0 of the target satellite and (SFN 0 + ssb-TimeOffset) of the source satellite are aligned at ULSRP.

In response to RAN1 question, clarify that RAN2 didn’t assume any RAN1 specification work and confirm the understanding in second and third questions.

# Conclusion

Following proposals are made.

Observation 1.	In NTN, the propagation delay is inherently time variant. As shown in figure 1, the SMTC configuration needs to be updated by network periodically.

Observation 2.	Immediately after joining the new serving satellite, handover is not expected and so is the measurements of the neighbor cells. As exact time to acquire SIB19 is left to UE, the UE may not immediately have up to date information of the neighbor satellites.

Proposal 1	RAN2 decides not to change the current specification texts related to satellite switch with resynchronization feature.

Proposal 2	For measurements, clarify that the UE takes into account the fact that the exact time to acquire SIB19 for the new updated neighbor satellite information is up to UE, the neighbor satellite may have gone through the soft satellite switch procedure and stored list of neighbor satellite may not be useful for measurement until new SIB19 is acquired, measurement for handover is not expected immediately after joining the new satellite with full remaining service duration.

Proposal 3	The UE may still perform measurement with the configured SMTC and the UE expects RRC reconfiguration as part of periodic SMTC update same as in existing procedure.

Proposal 4	Send LS response to RAN1 and RAN4 with the above clarification.

Proposal 5	In response to RAN4 question, RAN2 clarifies that the starting boundaries of subframes of source and target satellites are aligned at the Uplink Synchronization Reference Point (ULSRP) and starting boundaries of SFN 0 of the target satellite and (SFN 0 + ssb-TimeOffset) of the source satellite are aligned at ULSRP.

Proposal 6	In response to RAN1 question, clarify that RAN2 didn’t assume any RAN1 specification work and confirm the understanding in second and third questions.

# Reference

[1] R2-2501741, “reply LS on soft satellite switch with re-sync”, Source RAN4, To RAN2, RAN1

[2] R2-2501717, “Reply LS on soft satellite switch with re-sync”, Source RAN1, To RAN2