3GPP TSG-RAN WG2 Meeting #129bis                                    	R2-2502654

Wuhan, China, April 7th – 11th, 2025

Agenda item:	6.1.2

Source:	Qualcomm Incorporated

Title:	Corrections on Power saving features in NTN

Document for:	Discussion and Decision

# Introduction

DCP (DCI with CRC scrambled by PS-RNTI) is Rel-16 feature for power saving. Whether DCP can be supported in NTN has never been discussed. From the UE capability reporting perspective, there is no TN and NTN differentiation for the DCP feature. Therefore, this will bring the confusion whether DCP is also supported in NTN.

Similarly, CSI masking is setup for power saving such that UE is limited to send the periodic CSI report only during C-DRX ON duration. However, due to UE’s TA pre-compensation, UE may not be able to send the report if the UE would be in C-DRX sleep. In this document we provide details on how to address the issue.

# Discussion

Issue#1: Support of DCP

DCP (DCI with CRC scrambled by PS-RNTI) is introduced in Rel-16 as PDCCH-based wakeup indication in RRC_CONNECTED. There are specific mandated UE procedures defined if DCP is configured by the network. If this is configured in NTN, the UE would have to decode the DCI format 2_6 in NTN to make decision whether to start the drx-onDurationTimer. RAN5 may also need to look at enabling test case for NTN.

In general, we have introduced TN and NTN differentiation via “NTN-Parameters” UE capability for most of the lower layers (MAC and PHY) features. However, this feature DCP (i.e., drx-Adaptation-r16) was overlooked, and its TN/NTN differentiation is missing. Here is how it is missing.

The support of feature DCP is indicated via drx-Adaptation-r16 for FR1 and via drx-Adaptation-r17 for FR2. The drx-Adaptation-r16 can only be signaled via MAC-ParametersFRX-Diff-r16 and drx-Adaptation-r17 via MAC-ParametersFR2-2-r17. However, the NTN-Parameters currently cannot include MAC-ParametersFRX-Diff-r16 and MAC-ParametersFR2-2-r17.

Therefore, we think that it should be clarified whether we assume the DCP is not supported in NTN or DCP can be supported in NTN and introduce new UE capability signalling MAC-ParametersFRX-Diff-r16 separately for NTN.

Instead of fixing the particular feature (i.e., drx-Adaptation-r17) we think it is better to introduce new UE capability signaling to include MAC-ParametersFRX-Diff-r16 for NTN. The reason is below:

- NTN can include only MAC-Parameters or UE-NR-CapabilityAddFRX-Mode, i.e., without suffix.

- The MAC-ParamatersFRX-Diff cannot be included in NTN-Parameters.

- That means drx-Adaptation-r17 cannot have TN and NTN differentiation.

- In Rel-19 and beyond, new capability signaling for new features may be added under MAC-ParamatersFRX-Diff and same issue of TN NTN differentiation will arise again.

Since, it is assumed that device with support of FR2 NTN will not work in FR1, it is sufficient to add Rel-17 late noncritical extension to signal MAC-ParametersFRX-Diff-r16 for FR1. Text proposal is shown below:

-- Late non-critical extensions from Rel-17 onwards:

UE-NR-Capability-v17c0 ::=               SEQUENCE {

mac-Parameters-v17c0                     MAC-Parameters-v17c0                                         OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v17d0                                                   OPTIONAL

}

UE-NR-Capability-v17d0 ::=               SEQUENCE {

ntn-Parameters-v17d0                         NTN-Parameters-v17d0                                   OPTIONAL,

nonCriticalExtension                         SEQUENCE {}                                              OPTIONAL

}

//Next change

-- ASN1START

-- TAG-NTN-PARAMETERS-START

NTN-Parameters-r17 ::= SEQUENCE {

inactiveStateNTN-r17                ENUMERATED {supported}                                OPTIONAL,

ra-SDT-NTN-r17                      ENUMERATED {supported}                                OPTIONAL,

srb-SDT-NTN-r17                     ENUMERATED {supported}                                OPTIONAL,

measAndMobParametersNTN-r17         MeasAndMobParameters                                  OPTIONAL,

mac-ParametersNTN-r17               MAC-Parameters                                        OPTIONAL,

phy-ParametersNTN-r17               Phy-Parameters                                        OPTIONAL,

fdd-Add-UE-NR-CapabilitiesNTN-r17   UE-NR-CapabilityAddXDD-Mode                           OPTIONAL,

fr1-Add-UE-NR-CapabilitiesNTN-r17   UE-NR-CapabilityAddFRX-Mode                           OPTIONAL,

ue-BasedPerfMeas-ParametersNTN-r17  UE-BasedPerfMeas-Parameters-r16                       OPTIONAL,

son-ParametersNTN-r17               SON-Parameters-r16                                    OPTIONAL

}

NTN-Parameters-v17d0 ::= SEQUENCE {

mac-ParametersNTN-v17d0               MAC-Parameters-v1610                                        OPTIONAL,

fr1-Add-UE-NR-CapabilitiesNTN-v17d0   UE-NR-CapabilityAddFRX-Mode-v1610                           OPTIONAL

}

NTN-Parameters-v1820 ::= SEQUENCE {

fr2-Add-UE-NR-CapabilitiesNTN-r18   UE-NR-CapabilityAddFRX-Mode                           OPTIONAL

}

-- TAG-NTN-PARAMETERS-STOP

-- ASN1STOP

NTN-Parameters field descriptions

fdd-Add-UE-NR-CapabilitiesNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, fdd-Add-UE-NR-Capabilities applies to NTN.

fr1-Add-UE-NR-CapabilitiesNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, fr1-Add-UE-NR-Capabilities applies to NTN.

fr1-Add-UE-NR-CapabilitiesNTN-v17c0 NTN related capabilities which the UE supports in NTN differently than in TN. If absent, fr1-Add-UE-NR-Capabilities-v1610 applies to NTN.

fr2-Add-UE-NR-CapabilitiesNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, fr2-Add-UE-NR-Capabilities applies to NTN.

mac-ParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, mac-Parameters applies to NTN.

mac-ParametersNTN-c17c0 NTN related capabilities which the UE supports in NTN differently than in TN. If absent, mac-Parameters-v1610 applies to NTN.

measAndMobParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, measAndMobParameters applies to NTN.

phy-ParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, phy-Parameters applies to NTN.

son-ParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, son-Parameters-r16 applies to NTN.

ue-BasedPerfMeas-ParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, ue-BasedPerfMeas-Parameters-r16 applies to NTN.

Add Rel-17 late noncritical extension to signal MAC-ParametersFRX-Diff-r16 in NTN. Text proposal is provided above.

Issue#2: Periodic CSI reporting

In NTN, the logical DL symbol “n” and logical UL symbol “n” are not aligned.

When CSI masking is configured, UE’s periodic CSI reporting is limited to only when drx-onDurationTimer is running. As per current procedural text in TS 38.321, if the CURRENT symbol “n” (i.e., when UE checks this symbol now) is not within the ON duration, the UE will not send CSI report. However, when the CURRENT symbol “n” is within the ON duration, there will not be enough time for UE to be able to send the CSI report due to large TA compensation in NTN.

We think similar issue exists for periodic SRS reporting as UE is not required to wake up when NOT in DRX active time except for HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS. The network can always trigger the aperiodic CSI or SRS and resolve the issue of insufficient CSI report. However, we think some clarification on “current symbol n” would be needed for NTN. For example, it can be left to UE implementation to wake up from sleep mode to be able to send the CSI report for symbol “n” which falls during ON duration because waking up too early will drain UE’s battery. Text proposal is shown below.

Clarify that in NTN, if drx-onDurationTimer would not be running prior to symbol n (i.e., UE is in DRX sleep prior to symbol n), it is up to UE implementation whether to wakeup from C-DRX sleep prior to symbol n to report periodic CSI.

1> if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3; and 1> if the current symbol n occurs within drx-onDurationTimer duration; and 1> if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause: 2> if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and 2> if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or if all multicast DRXes would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions are configured with multicast DRX: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7]; 3> not report semi-persistent CSI configured on PUSCH; 3> not report semi-persistent CSI on PUCCH; 3> if ps-TransmitPeriodicL1-RSRP is not configured with value true: 4> not report periodic CSI that is L1-RSRP on PUCCH. 3> if ps-TransmitOtherPeriodicCSI is not configured with value true: 4> not report periodic CSI that is not L1-RSRP on PUCCH. 1> else: 2> in current symbol n, if a DRX group would not be in Active Time considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and 2> if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if all multicast DRXes corresponding to the DRX group would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7] in this DRX group; 3> not report CSI on PUCCH and semi-persistent CSI configured on PUSCH in this DRX group. 2> if CSI masking (csi-Mask) is setup by upper layers: 3> in current symbol n, if drx-onDurationTimer of a DRX group would not be running considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and 3> if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if drx-onDurationTimerPTM(s) of all multicast DRXes corresponding to the DRX group would not be running considering DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX: 4> not report CSI on PUCCH in this DRX group. NOTE 4: If a UE multiplexes a CSI configured on PUCCH with other overlapping UCI(s) according to the procedure specified in TS 38.213 [6] clause 9.2.5 and this CSI multiplexed with other UCI(s) would be reported on a PUCCH resource either outside DRX Active Time of the DRX group in which this PUCCH is configured or outside the on-duration period of the DRX group in which this PUCCH is configured if CSI masking is setup by upper layers, it is up to UE implementation whether to report this CSI multiplexed with other UCI(s). NOTE 5: In NTN, if drx-onDurationTimer would not be running prior to symbol n, it is up to UE implementation whether to wakeup from C-DRX inactive time prior to symbol n to report periodic CSI.

# Conclusion

Following proposals are made.

Proposal 1	Add Rel-17 late noncritical extension to signal MAC-ParametersFRX-Diff-r16 in NTN. Text proposal is provided above.

Proposal 2	Clarify that in NTN, if drx-onDurationTimer would not be running prior to symbol n (i.e., UE is in DRX sleep prior to symbol n), it is up to UE implementation whether to wakeup from C-DRX sleep prior to symbol n to report periodic CSI.