3GPP TSG-RAN WG2 Meeting #129bis                                    	R2-2502656

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.9.3

Source:	Qualcomm Incorporated

Title:	CB-Msg3 and Msg4 enhancements

Document for:	Discussion and Decision

- Introduction

In RAN2#129, following agreements were made.

Agreements:

1.	It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used

2.	RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC

3.	RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects)

-	Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN)

-	Frequency domain resources for (N)PUSCH occasions

-	repetition number

-	(N)PDCCH resource

-	MCS

4.	For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels.

5.	The UE shall at most have one ongoing CB EDT procedure at any time.

6.	The CB EDT Config has one minimum RSRP threshold (as agreed in RAN2#128) to use CB EDT.

7.	The CB EDT Config has two RSRP thresholds for NB-IoT for the three CE levels.

8.	CB EDT Config has three RSRP thresholds for eMTC for the four CE levels.

7.	As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config.

8.	RAN2 consider a new CBEDT-ConfigSIB-NB IE for configuring the CB EDT feature

Working assumption:

- One CB-MSG4 can target multiple UEs simultaneously (FFS on the details)

- For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today

The UE first selects the next DSA transmission window and then randomly select K replicasinside the window.

RAN2 assumes that a pointer solution is not needed in Rel-19

In this document, we provide details for DSA solution.

- Discussion

## OCC

Following is FFS from the RAN2#129 meeting.

- It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used

We think OCC is important to improve further by multiplexing two users. As RAN1 indicated, this can be used during CB-Msg3 transmission, the network would still need know whether the UE is using OCC or not. Since there can be UEs not supporting OCC, we think the cleaner solution is that separate CB-Msg3 resources are configured for CB-Msg3 transmission.

Separate resources are configured for CB-Msg3 transmission using OCC.

## Diversity Slotted Aloha (DSA)

As explained in [1], for DSA, the network should configure a number of replicas (i.e., K) allowed for transmission. There could be N EDT occasions configured for UE to choose randomly and UE selects randomly K occasions (i.e., a Msg3 transmission and K replicas) out of N occasions. Following was agreed in RAN2#128.

- The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific

- There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts)

If K = 1 (i.e., number of replicas = 0), then UE will have only one PUSCH resource to select. If K = 2, then UE will choose randomly 2 PUSCH resources in different time occasions and transmit one original msg3 transmission and one replica of the Msg3 transmission. It is to note that at a time occasion, a UE may randomly choose different frequency domain or OCC domain resources.

In RAN2#129, following working assumption is made. We think this can be simply confirmed.

- For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today

- The UE first selects the next DSA transmission window and then randomly select K replicas inside the window.

Confirm the working assumption for the CB-Msg3 transmission window.

We think the DSA configuration such as mixing the K = 1 (SA) and K > 1 (DSA) should be possible for new UEs, for example, resources for two CE levels configured one with K = 1 for CE level 0 and K = 2 for CE level 1. However, UEs not supporting DSA feature should not use the resource with K >1 for cleaner procedure, e.g. support of group RNTI. It may be better to separate the resource configuration for CB-Msg3 (i.e., DSA with K = 1) and DSA with K>1.

Consider separate resource configuration for CB-Msg3 (or DSA resource with K = 1) and DSA resource (with K > 1).

Based on the configuration of DSA resource UE knows the network’s capability. Based on the use of the DSA resource, network knows the UE’s capability to support DSA.

Based on the configuration of DSA resource UE knows the network’s capability. Based on the use of the resource, network knows the UE’s capability to support DSA or CB-Msg3 transmission.

RNTI calculation

In RAN2#128, following agreement was made.

- For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

It should be clear how the Msg4 transmission is performed. If there are multiple PDCCH monitoring windows (i.e., one window per replica), the RNTI to be monitored will be different for each window. This would require UE to monitor multiple RNTIs as windows may overlap. In addition, if the UE receives rescheduling associated with the first window (i.e., first Msg3 replica transmission), then the benefit of DSA is not achieved as the successful Msg4 response associated with the second window (i.e., second replica of the Msg3) may collide with or may be scheduled after the retransmission grant of the first Msg3 replica.

To resolve it, the network can wait until the resource occasion N (i.e., last transmission occasion of the group) to decide on retransmission. Then UE will always receive the successful response first.

This can be achieved with single PDCCH monitoring window. The other benefit of single PDCCH monitoring window is that network can take maximum benefit from the Msg4 multicast enhancements as a single RNTI associated with the window would be sufficient. Single RNTI per window will also resolve the risk of RNTI collision and UE’s complexity on monitoring multiple RNTIs.

A single RNTI specific to a PDCCH monitoring window is used. FFS on details for RNTI derivation.

A single PDCCH monitoring window is considered.

Contention resolution

In our view, existing contention resolution timer can be used for the PDCCH monitoring window. After every replica transmission the CR timer can be started considering the last repetition plus UE-eNB RTT. To take maximum benefit of Msg4 multicast enhancement, the CR timer can be defined to be started after the last replica transmission occasion plus UE-eNB RTT. However, for having a possibility to improve the delay for some UEs by responding the Msg4 response earlier, the CR timer can be started after the first replica transmission plus UE-eNB RTT. The issue with this approach is that subsequent replica transmission occasion may overlap with the CR timer and a rule needs to be defined to prioritize the replica transmission over Msg4 monitoring. The contention resolution can still be handled in the same existing way, i.e., by the use of contention resolution timer and contention resolution MAC CE in Msg4.

The contention resolution timer is used as PDCCH monitoring window. It is started after the last repetition of the last PUSCH occasion of the transmission window plus UE-eNB RTT.

For Msg4, existing contention resolution procedure can be used, i.e., contention resolution timer and contention resolution MAC CE.

## CB-Msg3 transmission attempt

There should also be a limit on how many attempts the UE is allowed for the CB-Msg3 transmission if the UE declares the contention resolution failure or contention resolution timer expires. After the limit, the UE can fall back to normal 4 step RACH-based procedure, e.g., RACH or EDT. We think this can be configured by network via system information.

If CB-Msg3 transmission fails, network configures via system information the maximum number of re-attempts allowed for CB-Msg3 transmission before falling back to 4 step RACH-based procedure.

The maximum TBS size depends on the coverage level. Therefore, we think the pending UL data size can be different for each CE level to make decision whether the CB-Msg3 can be used.

Maximum TBS size for CB-Msg3 is different for different CE level.

After CB-Msg3 transmission, if the network wants UE to move to RRC_CONNECTED mode, the same RNTI cannot converted to the C-RNTI. It is because the RNTI is derived from the CB-Msg3 resource and should not be reused in RRC_CONNECTED. Therefore, the network should provide a new C-RNTI (e.g. by RRC message) to be used when moved to RRC_CONNECTED.

After CB-Msg3 transmission, if a UE is moved to RRC_CONNECTED, a new C-RNTI is provided to the UE.

Also, it is possible that network wants UEs to take back off or fall back to the 4 RACH procedure. We this option should also be possible for the network.

Support network indication of backoff or fallback to 4 step RACH procedure.

## Msg4 enhancements

The second objective of the EDT enhancement is to reduce the congestion/overhead for transmission of Msg4 to several UEs. For example, many UEs may initiate EDT and network may exhaust the PDCCH or PDSCH resources while scheduling Msg4 (which comprises a PDCCH and at least one PDSCH) to each individual UE. There may be cases where network may not have any downlink response data and network would need to schedule Msg4 just including RRCEarlyDataComplete message (without DL NAS PDU) and contention resolution MAC CE of multiple UEs.

Instead of sending individual Msg4’s in such instances, a “multiplexed Msg4” to several UEs may be considered. An example procedure where multiplexed transmissions to several UEs using a single PDSCH are performed exists in case of Random-Access response as shown in figure 1. Network can schedule multiple RARs in a single MAC PDU.

Figure 1 TS 36.321 Figure 6.1.5-4: Example of MAC PDU consisting of a MAC header and MAC RARs

Multicast Msg4 (multi-user Msg4 multiplexing, similar to RAR or NR 2-step RACH response) or multi-user Msg4 scheduled by a single DCI (as in multi-TB Msg4 scheduling) can be studied as a solution.

Figure 2 Msg4 enhancement for EDT

In this case there could be several possible options for the enhancements, some examples being:

- Reusing a PUR-like L1 ACK procedure

  - The L1 ACK may also be used to resolve contention between UEs

  - The L1 ACK can move UE back to RRC_IDLE

- Reusing concept of RAR-like multiplexing (e.g., of multiple users RAPIDs in a single RAR)

  - A single PDU can multiplex contention resolution MAC CEs (or Msg4) of more than one UE.

- Others such as existing Multi-TB scheduling for Msg4 could also be studied for multi-user scenario.

A PUR-like L1 ACK concept, as a response to the EDT transmission, can be considered.

The Msg4 enhancements can be on the top of existing 4 steps EDT procedure as shown in figure 2, or applicable for the case of CB-msg3 / DSA.

The enhancements done for Msg4 transmission can be applicable to normal 4 step RACH or CB-Msg3 or DSA transmission.

## Aspects related to RAN1

Similar to what was done in EDT, the introduction of CB-Msg3 may include some small specification updates in RAN1 specifications. For instance, the following should be considered by RAN1:

- Design of search spaces for response.

- Derivation of CB-Msg3 RNTI

- Support of anchor and non-anchor carrier

- Aspects related to retransmission of msg3-CB (e.g. DCI design)

- TBS size per CE level

- Any other feedback on msg3-CB transmission (e.g. DMRS design, scrambling of NPUSCH, periodicity of TOs within a group etc.)

- Resource configuration/activation for OCC

Send LS to RAN1 informing potential alignment work associated with the CB-Msg3 transmission for DSA and non-DSA.

- Conclusion

Following proposals are made:

Proposal 1	Separate resources are configured for CB-Msg3 transmission using OCC.

Proposal 2	Confirm the working assumption for the CB-Msg3 transmission window.

Proposal 3	Consider separate resource configuration for CB-Msg3 (or DSA resource with K = 1) and DSA resource (with K > 1).

Proposal 4	Based on the configuration of DSA resource UE knows the network’s capability. Based on the use of the resource, network knows the UE’s capability to support DSA or CB-Msg3 transmission.

Proposal 5	A single RNTI specific to a PDCCH monitoring window is used. FFS on details for RNTI derivation.

Proposal 6	A single PDCCH monitoring window is considered.

Proposal 7	The contention resolution timer is used as PDCCH monitoring window. It is started after the last repetition of the last PUSCH occasion of the transmission window plus UE-eNB RTT.

Proposal 8	For Msg4, existing contention resolution procedure can be used, i.e., contention resolution timer and contention resolution MAC CE.

Proposal 9	If CB-Msg3 transmission fails, network configures via system information the maximum number of re-attempts allowed for CB-Msg3 transmission before falling back to 4 step RACH-based procedure.

Proposal 10	Maximum TBS size for CB-Msg3 is different for different CE level.

Proposal 11	After CB-Msg3 transmission, if a UE is moved to RRC_CONNECTED, a new C-RNTI is provided to the UE.

Proposal 12	Support network indication of backoff or fallback to 4 step RACH procedure.

Proposal 13	Multicast Msg4 (multi-user Msg4 multiplexing, similar to RAR or NR 2-step RACH response) or multi-user Msg4 scheduled by a single DCI (as in multi-TB Msg4 scheduling) can be studied as a solution.

Proposal 14	A PUR-like L1 ACK concept, as a response to the EDT transmission, can be considered.

Proposal 15	The enhancements done for Msg4 transmission can be applicable to normal 4 step RACH or CB-Msg3 or DSA transmission.

Proposal 16	Send LS to RAN1 informing potential alignment work associated with the CB-Msg3 transmission for DSA and non-DSA.

- References

[1] R2-2500534, “CB-Msg3 and Msg4 enhancements”, Athens, Greece, February 17th – 21st, 2025.