3GPP TSG-RAN WG2 Meeting #129bis	R2-2502665

Wuhan, China, Apr. 7th – 11st, 2025

Agenda item:	8.12.2

Source:	Samsung

Title:	Discussion on Asymmetric DL sTRP/UL mTRP

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision2

# Introduction

In the so-called “asymmetric DL sTRP/UL mTRP” scenario, we consider dense UL-only TRPs, where UEs receive DL from a macro DL/UL TRP (a.k.a. anchor TRP) and can transmit UL to micro UL-only TRPs, as shown in the Figure below.

In this scenario, we consider a pathloss offset between the DL pathloss RS from the DL TRP and the actual pathloss for UL-only TRP.  Due to this pathloss offset, UL power control to the UL-only TRP are determined differently from the legacy power control. The PL offset is used in UL power control in a way that UE reuses the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset. The PL offset is applied in PUCCH, PUSCH, SRS, PDCCH-order PRACH as RAN1 agreed.

The objective of this topic is specified in WID [1].

Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this paper, we discuss remaining issue related to the pathloss offset.

# Discussion

## PL offset MAC CE

After discussion in previous meeting, the PL offset update MAC CE is introduced using the following baseline format.

The MAC CE can include a flexible number of pairs of TCI state ID and PL offset. UE can know the size of the MAC CE from the L field in the MAC subhead. One remaining issue mentioned in the last meeting is whether we need to restrict the maximum number of PL offsets.

The maximum number of TCI states can be configured in the joint TCI state list is 128. If a TCI state ID bitmap is used, it requires 128bits (i.e., 16 octets). As each TCI state ID takes 7 bits, which occupies an octet (8 bits), if a MAC CE carries more than 16 TCI state IDs, it will take more than 128 bits. In this case (i.e., carrying more than 16 TCI state IDs), using TCI state ID has no advantage compared to using a bitmap in terms of saving bits. Using TCI state ID can save bits compared to bitmap only when there are less than 16 TCI state IDs in the MAC CE.

In practice, the number of updated PL offsets can be small, for example, up to 2 indicated joint/UL TCI states (for mTRP) or up to 16 activated joint/UL TCI states (for mTRP) for PUCCH, PUSCH, PRACH, and SRS transmission that follows unified TCI; up to 2 joint/UL TCI states configured in SRS-resource or indicated in MAC CE for SRS that does not follow unified TCI or for periodic SRS for beam management. Only half of these TCI states (i.e., for the UL-only TRP), which are for UL-only, need PL offset update.

NW can include all updated PL offsets in one MAC CE or if needed send multiple PL offset update MAC CE separately and more flexibly in terms of DL resource. We can leave the flexibility for NW implementation, thus, there seems no strong technical motivation to specify the maximum number of PL offsets in the MAC CE.

Proposal 1: No restriction on the maximum number of PL offsets in the MAC CE.

Regarding how to apply the configured/update PL offset, we made a working assumption in the last meeting.

- Working assumption: UE applies the latest PL offset value received in RRC or MAC CE. Can revisit if new issue is found.

The only concern raised by a company is that “the latest” would cause ambiguity. In our view, “the latest” received value is up to UE capability/implementation to process. A similar MAC CE used to update RRC configured value is SRS Pathloss Reference RS Update MAC CE, which updates the pathloss reference RS for an SRS-resource. From RAN2 perspective, we can confirm the working assumption as a RAN2 understanding.

Proposal 2: RAN2 assumes that UE applies the latest PL offset value received in RRC or MAC CE.

## PHR

Regarding the impact to PHR, currently in TS 38.321 clause 5.4.6, the key trigger of PHR is that pathloss of any PL RS changes more than a threshold.

- phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission; NOTE 1: The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between. The current pathloss reference for this purpose does not include any pathloss reference configured using pathlossReferenceRS-Pos in TS 38.331 [5].

In the last meeting, we have built a common understanding:

RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

The remaining issue is whether/how to specify this. For PHR, TS 38.321 clause 5.4.6 specifies the PHR trigger and MAC CE report procedure, and the actual reported Type 1/2/3 PH value(s) is determined by lower layer, which is specified in TS 38.213 clause 7.7. Note RAN1 does not specify how to determine the PL offset change to trigger PHR.

Some companies think we can leave it to RAN1 to specify the PL offset is applied in the PL calculation. However, RAN1 has only agreed to apply the PL offset in the power control formula on top of the calculated PL for PUSCH/PUCCH/SRS/PRACH, which is specified in TS 38.213 clause 7.1/2/3/4. It is not agreed to apply the PL offset directly in the PL calculation. Thus, RAN1 will not specify the PL offset in the PL formula.

Agreement RAN1#116 For the asymmetric DL sTRP/UL mTRP deployment scenarios, support to associate a UL TCI state with a PL offset: - When a UL TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this UL TCI state. - Reuse the legacy uplink power control formulation by replacing legacy PL with UL PL which is derived from the DL PL RS and the PL offset. - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW. - Note: it does not intend to increase the number of maintained PLs per cell. - FFS: whether to support associating joint TCI state (if supported) with a PL offset. Further study whether/how to apply a PL offset on PDCCH-order PRACH transmission too. - FFS: how to determine the Tx beam of PRACH towards UL TRP - Note: this does not imply to support 2 TA for single-DCI based system. Agreement RAN1#116bis For FR1, a joint TCI state can be associated with a PL offset. - When a joint TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this joint TCI state. - Reuse the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset. - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW.

The UL power control formula and the PL formula are copied below from TS 38.213.

PPUSCH,b,f,ci,j,qd,l=minPCMAX,f,ciPO_PUSCH,b,f,cj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,cj∙PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l [dBm] PPUCCH,b,f,ci,qu,qd,l=minPCMAX,f,ciPO_PUCCH,b,f,cqu+10log102μ∙MRB,b,f,cPUCCHi+PLb,f,cqd+∆F_PUCCHF+∆TF,b,f,ci+gb,f,ci,l [dBm] [dBm], where the pathloss in the above formula is calculated as the difference between the reference signal power and the filtered RSRP. PLb,f,c(qd) = referenceSignalPower – higher layer filtered RSRP, where referenceSignalPower is provided by higher layers and RSRP is defined in [7, TS 38.215] for the reference serving cell and the higher layer filter configuration provided by QuantityConfig is defined in [12, TS 38.331] for the reference serving cell If the UE is not configured periodic CSI-RS reception, referenceSignalPower is provided by ss-PBCH-BlockPower. If the UE is configured periodic CSI-RS reception, referenceSignalPower is provided either by ss-PBCH-BlockPower or by powerControlOffsetSS providing an offset of the CSI-RS transmission power relative to the SS/PBCH block transmission power [6, TS 38.214]. If powerControlOffsetSS is not provided to the UE, the UE assumes an offset of 0 dB.

Furthermore, according to RAN1 previous discussion (R1-2405347), the impact of PL offset on PHR trigger is left to RAN2. For the sake of progress, we propose to add a note similar to the existing NOTE 1 in TS 38.321 clause 5.4.6, to clarify that the PL change is determined based on PL offset.

Observation 1: RAN1 has only agreed to apply the PL offset in the power control formula on top of the calculated PL, but not to apply the PL offset directly in the PL calculation. PHR trigger due to PL change and PHR report procedure are specified in MAC, RAN1 specification only specifies the calculation of PH value to be reported.

Proposal 3: For PHR trigger, add a note in MAC specification: If a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset.

One discussion point mentioned by a company is whether PL offset is applied to calculate Type 3 PH for SRS transmission. By checking RAN1 discussion, we found the issue was discussed for several meetings. In the last meeting RAN1 has agreed there is no consensus to support PL offset in Type 3 PH calculation.

Technically, the view shared by majority companies in RAN1 is that currently Type 3 PHR is only used when PUSCH is not configured which is not the case for UL-only TRP. Type 3 PHR for UL-only TRP is a marginal case without strong motivation. We share the same view. Meanwhile, the calculation of PH value for PHR is specified in RAN1 specification and is out of the scope of RAN2. As RAN1 has made conclusion, which should be respected, we think RAN2 does not need to further discuss on this.

Observation 2: RAN1 has no consensus to support PL offset in Type 3 PH calculation. The calculation of PH value for PHR is specified in RAN1 specification and is out of the scope of RAN2, no need to further discuss in RAN2.

## 2TA for single-DCI mTRP

In Rel-18, 2TA operation has been introduced for mDCI mTRP. For Rel-19, the scope of the latest WID includes 2TA for sDCI mTRP to support asymmetric DL sTRP/UL mTRP. RAN1 informs in LS R1-2410758 the following agreement regarding 2TA for sDCI mTRP.

RAN1#118bis Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

In the current 2TA operation, each TCI state is applied with a TA and associated to the corresponding TAG. To extend 2TA operation to asymmetric DL sTRP/UL mTRP, Rel-18 RA procedure with 2TA and the procedure of TAT/TAG management can be directly applied since mDCI/sDCI is transparent to the TA operation in MAC. The current 2TA operation can be applied with minor changes on RRC configuration.

Based on the RAN1 agreement, the following changes are needed in 2TA configuration.

- remove in RRC field tag2 the restriction that coresetPoolIndex needs to be configured for the 2TA;

- a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario.

Proposal 4: For 2TA in asymmetric DL sTRP/UL mTRP scenario, Rel-18 2TA operation is applied with the following RRC changes:

- remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value;

- a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario.

Directly removing the restriction “tag2 is configured only if coresetPoolIndex is configured with more than one value” for 2TA configuration may cause non-backward compatibility issue. For example, if a Rel-18 UE supporting mDCI-mTRP-based 2TA is connected to a Rel-19 gNB and receives 2TA configuration with coresetPoolIndex is not configured with more than one value, the Rel-18 UE may consider this is an error configuration. To avoid such an issue, a new UE capability for Rel-19 sDCI-mTRP-based 2TA should be introduced. NW should ensure configuration according to UE capability. The corresponding new UE features has been introduced as R1-2501399 informed.

Observation 3: A new UE capability for Rel-19 sDCI-mTRP-based 2TA is needed.

Meanwhile, UE has to distinguish between Rel-18 2TA feature and Rel-19 2TA feature. One option is to introduce a new RRC parameter that explicitly enables the Rel-19 sDCI-mTRP-based 2TA. Since tag2 is configured per serving cell and sDCI/mDCI operation is per BWP, the new RRC parameter has to be defined per BWP to distinguish Rel-18 2TA and Rel-19 2TA for a certain BWP.

Another option is to use coresetPoolIndex to implicitly enable Rel-18 2TA feature or Rel-19 2TA feature, i.e., if tag2 is configured for a serving cell and coresetPoolIndex is configured with more than one value for a BWP, Rel-18 2TA is enabled for the BWP; if tag2 is configured for a serving cell and coresetPoolIndex is configured with no more than one value for a BWP, Rel-19 2TA is enabled for the BWP.

Proposal 5: Discuss how to enable Rel-19 sDCI-mTRP-based 2TA and distinguish from Rel-18 mDCI-mTRP-based 2TA.

- Option 1: introduce a new RRC parameter per BWP that explicitly enables the Rel-19 sDCI-mTRP-based 2TA.

- Option 2: use coresetPoolIndex to implicitly enable Rel-18 2TA or Rel-19 2TA, i.e., if tag2 is configured for a serving cell and coresetPoolIndex is configured with more than one value for a BWP, Rel-18 2TA is enabled for the BWP; if tag2 is configured for a serving cell and coresetPoolIndex is configured with no more than one value for a BWP, Rel-19 2TA is enabled for the BWP.

## PL offset in RACH

For PRACH transmission, RAN1 has agreed the following aspects.

- Support applying PL offset on PDCCH-order PRACH towards a UL-only TRP in FR1.

- Support to apply PL offset on PDCCH-order PRACH in FR2. The design for applying PL offset on PDCCH-order PRACH in FR1 is fully reused here. Note: there is no extra enhancement for Tx beam determination for PDCCH-order PRACH in FR2 in Rel-19.

- The PL offset associated with one of the indicated joint/UL TCI state for UL-only TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission.

- A one-bit DCI field indicates whether the PL offset of the indicated joint/UL TCI state is used for PRACH transmission.

RAN1 has also clarified in the last meeting that PDCCH-order RACH for UL-only TRP is CFRA.

Conclusion In asymmetric DL sTRP/UL mTRP deployment scenario, the PDCCH-order PRACH to UL TRP is CFRA.

Observation 4: RAN1 has also clarified that PDCCH-order RACH for UL-only TRP is CFRA.

On the other hand, it is also not clear whether PL offset is needed for PRACH in UE initiated RA (e.g., for BFR, SR, etc.) or not. In our view, PL offset is not necessary for UE initiated RACH for BFR, SR, etc. The reason is that the macro DL TRP (i.e., anchor TRP) is always able to receive the PRACH as legacy operation without applying PL offset. This is different from PDCCH-order PRACH to the UL-only TRP for which the aim is TA acquisition and UL-only TRP has to receive PRACH to determine the TA.

We propose to discuss whether to support UE initiated RACH to UL-only TRP. If this is supported, further discussion is needed regarding how to determine the PL offset for UE-initiated PRACH transmission, which may involve RAN1.

Proposal 6: RAN2 discuss whether to support UE initiated RACH to UL-only TRP.

Moreover, in the current Random Access procedure UE measures RSRP of DL pathloss RS and compares it with a RSRP threshold to determine RA resource selection (e.g., for NUL/SUL, 4-step/2-step, Msg1/3 repetition, preamble group A/B) for UL coverage consideration.

As specified in TS 38.213 clause 7, the pathloss in the above formula is calculated as the difference between the reference signal power and the filtered RSRP.

PLb,f,c(qd) = referenceSignalPower – higher layer filtered RSRP,

where referenceSignalPower is provided by higher layers and RSRP is defined in [7, TS 38.215] for the reference serving cell and the higher layer filter configuration provided by QuantityConfig is defined in [12, TS 38.331] for the reference serving cell

If the UE is not configured periodic CSI-RS reception, referenceSignalPower is provided by ss-PBCH-BlockPower. If the UE is configured periodic CSI-RS reception, referenceSignalPower is provided either by ss-PBCH-BlockPower or by powerControlOffsetSS providing an offset of the CSI-RS transmission power relative to the SS/PBCH block transmission power [6, TS 38.214]. If powerControlOffsetSS is not provided to the UE, the UE assumes an offset of 0 dB.

In legacy operation, UE measures RSRP from a DL TRP and transmits PRACH to the same TRP. However, in asymmetric DL/UL scenario, UE measures RSRP from a DL TRP and transmits PRACH to a UL-only TRP. When transmitting to UL-only TRP instead of to DL TRP, a path different from DL RS is taken. Consequently, the pathloss changes by an offset, and the pathloss change leads to a change in the measured RSRP.

PLb,f,c(qd) + PLoffset = referenceSignalPower – higher layer filtered RSRP + PLoffset

= referenceSignalPower – (higher layer filtered RSRP– PLoffset)

Equivalently, from physical channel perspective, received power (i.e., RSRP) equals the difference between the transmit power and channel pathloss. When the PL offset is changed by an offset, so is the RSRP.

higher layer filtered RSRP– PLoffset = referenceSignalPower – (PLb,f,c(qd) + PLoffset)

An example is shown in the Figure below.

For UE1, the pathloss for DL-TRP-to-UE1 and the pathloss for UL-only-TRP-to-UE1 may not be much different, so the actual measured RSRP for DL RS can be used as the virtual measured RSRP from UL-only TRP. Thus, the RSRP of DL PL RS can still work well in the comparison with the configured RSRP threshold.

For UE2 and UE3, the pathloss from DL TRP and the pathloss from UL-only TRP may be much different due to the different distances, so that the actual measured RSRP for DL RS deviates from the virtual measured RSRP from UL-only TRP. As a result, the RSRP of DL PL RS may not work well.

Here, we take Msg1 repetition as an example. UE2 is far from DL TRP and outside the boundary of RSRP threshold, and the measured RSRP for DL PL RS is smaller than the configured RSRP threshold. In this case, a larger repetition number will be selected for PRACH transmission according to the current RA procedure. However, UE2 is close to the UL-only TRP and the virtual RSRP should be larger than the configured RSRP threshold, so for PRACH towards UL-only TRP a smaller repetition number should be selected. Similarly, UE3 will select a smaller repetition number as the measured RSRP is larger than the configured RSRP threshold. However, UE3 is far away from the UL-only TRP and the virtual RSRP should be smaller than the configured RSRP threshold, so for PRACH towards UL-only TRP a larger repetition number should be selected.

Proposal 7: Confirm the RSRP of DL RS in RA procedure is deficient for PRACH towards UL-only TRP.

To solve the issues, pathloss offset can be applied to the RSRP of DL RS for RA resource selection. Note the asymmetric TRP scenario is only consider for intra-band case as specified in WID. PDCCH-order CFRA is 4-step RA with Msg1/2, and there is no RA type selection or Msg3 repetition, and Msg-2 repetition number is configured in contention-free Random Access Resources. So, there is no RACH procedure impact for PDCCH-order CFRA. If UE initiated RACH is supported for UL-only TRP, we need to discuss PL offset impacts to 4-step/2-step, Msg1/3 repetition, preamble group A/B selection.

Observation 6: For asymmetric TRP scenario, PDCCH-order CFRA procedure does not apply RSRP of DL RS.

Observation 7: For UE-initiated RACH towards UL-only TRP, it is not clear how to determine the PL offset for PRACH, and need to discuss the PL offset impacts to RSRP in 4-step/2-step, Msg1/3 repetition, preamble group A/B selection.

## Others

One potential issue raised by some is that Rel-18 TCI state activation MAC CE for sDCI mTRP does not work for asymmetric TRP scenario since only one joint/DL TCI state is needed for the anchor TRP. Based on our investigation of the issue, RAN1 has discussed and concluded the existing uTCI signaling can work with no enhancements, as below

Conclusion RAN1#117 For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

In our view, Rel-18 TCI state activation MAC CE for sDCI mTRP can work in couple of ways without enhancement, e.g., NW always activates one joint/DL TCI state for each codepoint, or always indicates one of the two activated TCI states for a codepoint, or indicates both activated TCI states for a codepoint but provides same QCL information for the two indicated joint/DL TCI states so that from UE perspective it is sTRP operation.

Observation 8: The existing Rel-18 TCI state activation MAC CE for sDCI mTRP can work without enhancement for asymmetric TRP scenario where only one joint/DL TCI state is needed for the anchor TRP.

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: No restriction on the maximum number of PL offsets in the MAC CE.

Proposal 2: RAN2 assumes that UE applies the latest PL offset value received in RRC or MAC CE.

Observation 1: RAN1 has only agreed to apply the PL offset in the power control formula on top of the calculated PL, but not to apply the PL offset directly in the PL calculation. PHR trigger due to PL change and PHR report procedure are specified in MAC, RAN1 specification only specifies the calculation of PH value to be reported.

Proposal 3: For PHR trigger, add a note in MAC specification: If a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset.

Observation 2: RAN1 has no consensus to support PL offset in Type 3 PH calculation. The calculation of PH value for PHR is specified in RAN1 specification and is out of the scope of RAN2, no need to further discuss in RAN2.

Proposal 4: For 2TA in asymmetric DL sTRP/UL mTRP scenario, Rel-18 2TA operation is applied with the following RRC changes:

- remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value;

- a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario.

Observation 3: A new UE capability for Rel-19 sDCI-mTRP-based 2TA is needed.

Proposal 5: Discuss how to enable Rel-19 sDCI-mTRP-based 2TA and distinguish from Rel-18 mDCI-mTRP-based 2TA.

- Option 1: introduce a new RRC parameter per BWP that explicitly enables the Rel-19 sDCI-mTRP-based 2TA.

- Option 2: use coresetPoolIndex to implicitly enable Rel-18 2TA or Rel-19 2TA, i.e., if tag2 is configured for a serving cell and coresetPoolIndex is configured with more than one value for a BWP, Rel-18 2TA is enabled for the BWP; if tag2 is configured for a serving cell and coresetPoolIndex is configured with no more than one value for a BWP, Rel-19 2TA is enabled for the BWP.

Observation 4: RAN1 has also clarified that PDCCH-order RACH for UL-only TRP is CFRA.

Proposal 6: RAN2 discuss whether to support UE initiated RACH to UL-only TRP.

Proposal 7: Confirm the RSRP of DL RS in RA procedure is deficient for PRACH towards UL-only TRP.

Observation 6: For asymmetric TRP scenario, PDCCH-order CFRA procedure does not apply RSRP of DL RS.

Observation 7: For UE-initiated RACH towards UL-only TRP, it is not clear how to determine the PL offset for PRACH, and need to discuss the PL offset impacts to RSRP in 4-step/2-step, Msg1/3 repetition, preamble group A/B selection.

Observation 8: The existing Rel-18 TCI state activation MAC CE for sDCI mTRP can work without enhancement for asymmetric TRP scenario where only one joint/DL TCI state is needed for the anchor TRP.

# References

- RP‑242356	Revised Work Item: NR mobility enhancements Phase 4, RAN#105.