3GPP TSG-RAN WG2 Meeting #129bis	R2-2502666

Wuhan, China, Apr. 7th – 11st, 2025

Agenda item:	8.12.3

Source:	Samsung

Title:	Discussion on UE-initiated Beam Reporting and CSI enhancement

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision

# Introduction

In this paper, we share our views on way forward for UE initiated/event-triggered (UEI) beam reporting (BR) from RAN2 perspective and discuss some RRC parameters based on offline discussion outcome.

The high-level procedure of UEI beam reporting is summarized based on RAN1 agreements as follows.

- Measurement resource: intra/inter-cell SSB, and periodic CSI-RS for BM

  - The same RS type for current beam and new beam is ensured implicitly:

    - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition (i.e., new beam is CSI-RS for BM), Scheme-1 is enabled, which means RS for current beam is the QCL RS in the indicated TCI state (Note QCL RS of the indicated TCI state is always a CSI-RS);

    - Otherwise (i.e., new beam is SSB), Scheme-2 is enabled, meaning the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

- Events

  - Event-2:

    - Current beam: QCL RS of indicated TCI state, or SSB QCLed to the indicated TCI state

    - New beam: either CSI-RS or SSB, explicitly configured in RS resource set associated to the report configuration

    - Report content: Only a single UEI beam report is carried in the second PUSCH. Report CRI/SSBRI and L1-RSRP (differential value) for N ≥ 1 beam(s) (and current beam if enabled by RRC) in the report instance, with at least one beam meeting the event.

  - Event-1:

    - Quality of the current beam is worse than a certain threshold.

    - Same type of beam comparison: If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

  - Event-7:

    - Quality (e.g., L1-RSRP) of at least one new beam becomes a threshold (RRC configured) better than the Q-th best beam among the activated TCI states

    - Same type of beam comparison:

      - Scheme-1: ‘RS for current beam’ is the QCL RS in the activated TCI state with the Q-th best quality.

      - Scheme-2: ‘RS for current beam’ is the SSB which is QCLed with the QCL RS in the activated TCI state with the Q-th best quality.

- Report trigger

  - Basic trigger: L1-RSRP of at least one new beam satisfies an event.

  - Optional trigger by UE capability: If within a configurable time window, the number of event instances for at least one same new beam is greater than or equal to M (counting is per new beam).

  - Down-select from 4 window-counting options, see Appendix.

- Report procedure

- Mode A (report in UCI using DG)

- Step 1: Transmit a one-bit report notification in UCI in PUCCH

  - dedicated PUCCH resource, not SR

- Step 2: Receive DCI indicating CSI trigger state associated with UEI beam report configuration(s) and DG for UCI,

- Step 3: Transmit report in UCI using DG in PUSCH,

  - Mode B (report in UCI using type-1 CG)

    - Step 1: Transmit a one-bit report notification in UCI in PUCCH

  - dedicated PUCCH resource, not SR

    - Step 2: Transmit report in UCI using type-1 CG PUSCH, after at least X symbols

      - One-to-one mapping between the PUCCH resource and type-1 CG PUSCH resource: Only one periodic PUCCH resource and only one type-1 CG with the same periodicity.

      - type-1 CG PUSCH for dedicated for the beam report, which can carry any other UCI but can NOT carry UL-SCH.

  - Support cross-CC report for both modes, the report notification in PUCCH and report in PUSCH should be in the same CG and can be from the same or different CC(s).

# Discussion

## 2.1 UEI beam reporting

### MAC impact

- Report trigger evaluation

In the last meeting, RAN2 has made a working assumption.

Working assumption: RAN2 assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification. The final decision is up to RAN1, we can revisit this if R1 has a different decision. No need to send a LS to R1 on this.

Meanwhile, RAN1 has reviewed the TS 38.214 draft CR which has specified the whole procedure of UEI BR including window-counting-based report triggering. Given the Editor’s CR and RAN2 working assumption it was discussed during the CR review whether RAN1 or RAN2 specifies the window-counting-based report triggering. The TS 38.214 draft CR including window-counting-based report triggering is finally endorsed [1].

On the other hand, RAN1 has sent a LS R1-2500846 informing MAC impacts based on the latest progress.

As indicated in the LS, there is no additional MAC impact, including UEI BR. RAN2 should wait for RAN1 further discussion and agreements.

Observation 1: RAN1 has endorsed TS 38.214 draft CR for the whole UEI BR procedure including window-counting-based report triggering. RAN1 has informed by LS there is no need of MAC support so far for UEI BR.

Technically, we think it is more reasonable to specify window-counting-based report triggering in TS 38.214. The reason is UEI BR is designed intentionally as a PHY layer operation with no necessary involvement of MAC layer:

- the new beam is measured in PHY;

- event evaluation (i.e., whether an event is satisfied) is determined in PHY;

- window-counting-based report triggering?

- the report notification is carried by UCI in PUCCH;

- the actual report is carried by UCI in PUSCH.

While, the UEI BR operation is designed differently from BFD/BFR:

- beam is measured in PHY;

- beam failure instance is determined in PHY;

- BFD is evaluated using timer and counter in MAC;

- BFR by MAC CE or RACH is performed in MAC.

As the report notification and the actual report for UEI BR are all handled by PHY, we don’t see any need of a detour to the MAC layer for window-counting-based report triggering in the middle of the procedure.

Observation 2: As the report notification and the actual report for UEI BR are all handled by PHY, there is no need of a detour to the MAC layer for window-counting-based report triggering in the middle of the procedure. The artificial PHY-MAC interaction in specification should be avoided.

Proposal 1:  RAN2 assumes the UEI BR procedure including window-counting-based report triggering is specified in RAN1 specification unless RAN1 informs RAN2 to specify.

While waiting for RAN1 decision, RAN2 can discuss the window-counting options listed in RAN1 agreement in case it is to be specified in MAC. There are 4 options so far, see Appendix for the illustration.

Option 1: evaluation window of length T_window is periodic, same as PUCCH occasion’s periodicity. The interval between window ending and PUCCH occasion is T_proc. In each evaluation window, count the event-met instances per beam. If at least one beam’s counting reaches M, report is trigger.

Option 3: evaluation window of length T_window is periodic, with an independent periodicity which may be different from the periodicity of PUCCH occasion. In each evaluation window, count the event-met instances per beam. If the most recent window before a PUCCH occasion (at least T_proc earlier) has at least one beam’s counting reach M, report is triggered.

Option 2: evaluation window is sliding. From each event-met instance, count backward for a duration of T_window. If the most recent window before a PUCCH occasion (at least T_proc earlier) has at least one beam’s counting reach M, report is triggered.

Option 4: Run a timer for each beam. A timer for a beam starts at an event-met instance for the beam. The time expires after T_window. When the timer is running, count the event-met instances for the beam. For example, the counting for a beam reaches M before its timer expires and before the next PUCCH occasion, immediately the timer for the same beam starts again and the counting is less than M before the PUCCH occasion, should a report be triggered for the previous counting? If the counting for a beam reaches M before the next PUCCH occasion but its timer expires after PUCCH occasion, should a report be triggered for the counting?

In our view, we have to taken into account the periodic PUCCH occasion and the processing time to determine when to evaluate report trigger. Option 1 is simple in this sense. Option 3 is similar in terms of fixed periodic window. For option 2 UE has to memorize the past event instances and counting backwards for a window ending at each event instance. For option 4 UE has to run a timer for each new beam and the complexity is large for tens of beams. Meanwhile, it is hard to decide when to trigger a report as the timer can be expired at any time with respect to the PUCCH occasion, we have to discuss all cases exhaustively.

Observation 3: For the window-counting options in RAN1 agreement, option 2 and Option 4 have large complexity in terms of running window/timer/counting. For option 4, the timer with counting met M can be expired at any time with respect to the PUCCH occasion, we have to discuss for each case whether a report is triggered or not.

Proposal 2: If it is decided RAN2 specifies the report triggering evaluation, for the window-counting operation, select from option 1 and option 3 listed in the RAN1 agreement.

- DRX

For mode-A UEI BR, once a report is triggered, UE first transmits a notification in UCI on PUCCH and wait for DCI assign dynamic grant for the actual report in UCI to be transmitted on PUSCH. It means after sending first PUCCH, UE needs to monitor PDCCH. If UE is configured with DRX, UE only monitors PDCCH during the DRX active time. Thus, the DRX active time should include the time while UE waiting for the DG for UEI BR.

Proposal 3: DRX active time includes the time while UE waiting for the PDCCH indicating a DG for UEI BR after transmitting a report notification in PUCCH.

As currently specified for DRX, aperiodic CSI is transmitted regardless of PDCCH monitoring.

TS 38.321 clause 5.7 … Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] on the Serving Cells in the DRX group when such is expected.

Similarly, for UEI BR, once a report is triggered, UE has to transmit the first PUCCH using the configured periodic PUCCH resource for UEI BR, and transmits the second PUSCH using the DG/CG, which is regardless of PDCCH monitoring.

Proposal 4: Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits UEI BR on PUSCH when it is triggered.

- UL multiplexing

Regarding UL skipping, it is specified currently that UL skipping is not applied to UCI multiplexed on the PUSCH. Since UEI BR is transmitted in UCI, the highlighted sentence below already excludes the UEI BR.

TS 38.321 clause 5.7 … The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity.

Proposal 5: The current UL skipping procedure already exclude UEI BR since it is transmitted in UCI, no additional MAC impact.

In addition, based on RAN1 agreement below, the type-1 CG PUSCH for mode-B UEI BR is dedicated for the report which cannot multiplex UL-SCH data or other CSI report. Thus, the type-1 CG allocated for the mode-B UEI BR is transparent to MAC, which should be exclude from MAC UL-SCH transmission procedure.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least Option-3 is supported: - Option-1: The same Type-1 CG PUSCH can carry UL-SCH, any other UCI, and the beam report. - Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI. - Option-3: The Type-1 CG PUSCH is a type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can carry any other UCI. FFS: whether Type-1 CG PUSCH can be transmitted if the pre-configured Type-1 CG PUSCH does NOT carry the beam report Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH does NOT carry the beam report, for the second UL channel in Mode-B, Option-2 is supported: - Option-2: Type-1 CG PUSCH can NOT be transmitted

Proposal 6: The type-1 CG allocated for the mode-B UEI BR is transparent to MAC and excluded from MAC UL-SCH transmission procedure.

### RRC impact

During the RRC running CR review, we discussed whether to add UEI BR parameters in CSI-ReportConfig or in CSI-MeasConfig.

The IE CSI-ReportConfig currently includes periodic/semi-persistent/aperiodic CSI report configuration. Some existing fields in CSI-ReportConfig can be considered as baseline for UEI beam reporting, for example, reportConfigId, carrier, resourcesForChannelMeasurement, reportConfigType, reportQuantity.

CSI-ReportConfig ::=                SEQUENCE {

reportConfigId                          CSI-ReportConfigId,

carrier                                 ServCellIndex                   OPTIONAL,   -- Need S

resourcesForChannelMeasurement          CSI-ResourceConfigId,

csi-IM-ResourcesForInterference         CSI-ResourceConfigId            OPTIONAL,   -- Need R

nzp-CSI-RS-ResourcesForInterference     CSI-ResourceConfigId            OPTIONAL,   -- Need R

reportConfigType                        CHOICE {

periodic                                SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH                   SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH                   SEQUENCE {

reportSlotConfig                        ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

reportSlotOffsetList                SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32),

p0alpha                                 P0-PUSCH-AlphaSetId

},

aperiodic                               SEQUENCE {

reportSlotOffsetList                SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32)

}

},

reportQuantity                          CHOICE {

none                                    NULL,

cri-RI-PMI-CQI                          NULL,

cri-RI-i1                               NULL,

cri-RI-i1-CQI                           SEQUENCE {

pdsch-BundleSizeForCSI                  ENUMERATED {n2, n4}                                         OPTIONAL    -- Need S

},

cri-RI-CQI                              NULL,

cri-RSRP                                NULL,

ssb-Index-RSRP                          NULL,

cri-RI-LI-PMI-CQI                       NULL

},

reportFreqConfiguration                 SEQUENCE {

cqi-FormatIndicator                     ENUMERATED { widebandCQI, subbandCQI }                          OPTIONAL,   -- Need R

pmi-FormatIndicator                     ENUMERATED { widebandPMI, subbandPMI }                          OPTIONAL,   -- Need R

csi-ReportingBand                       CHOICE {

subbands3                               BIT STRING(SIZE(3)),

subbands4                               BIT STRING(SIZE(4)),

subbands5                               BIT STRING(SIZE(5)),

subbands6                               BIT STRING(SIZE(6)),

subbands7                               BIT STRING(SIZE(7)),

subbands8                               BIT STRING(SIZE(8)),

subbands9                               BIT STRING(SIZE(9)),

subbands10                              BIT STRING(SIZE(10)),

subbands11                              BIT STRING(SIZE(11)),

subbands12                              BIT STRING(SIZE(12)),

subbands13                              BIT STRING(SIZE(13)),

subbands14                              BIT STRING(SIZE(14)),

subbands15                              BIT STRING(SIZE(15)),

subbands16                              BIT STRING(SIZE(16)),

subbands17                              BIT STRING(SIZE(17)),

subbands18                              BIT STRING(SIZE(18)),

...,

subbands19-v1530                        BIT STRING(SIZE(19))

}   OPTIONAL    -- Need S

}                                                                                                           OPTIONAL,   -- Need R

timeRestrictionForChannelMeasurements           ENUMERATED {configured, notConfigured},

timeRestrictionForInterferenceMeasurements      ENUMERATED {configured, notConfigured},

codebookConfig                                  CodebookConfig                                              OPTIONAL,   -- Need R

dummy                                           ENUMERATED {n1, n2}                                         OPTIONAL,   -- Need R

groupBasedBeamReporting                     CHOICE {

enabled                                     NULL,

disabled                                    SEQUENCE {

nrofReportedRS                          ENUMERATED {n1, n2, n3, n4}                                 OPTIONAL    -- Need S

}

},

...omitting other fields...

}

We understand that some existing parameters in CSI-ReportConfig can be reused for UEI BR, so it seems natural to be included in CSI-ReportConfig as RAN1 informed. If UEI BR parameters are included in CSI-ReportConfig, we identify some clarifications are needed.

- The existing reportConfigType and the new report type UEI BR cannot be configured simultaneously.

- In reportQuantity, either cri-RSRP or ssb-Index-RSRP is configured for UEI-BR, other values are not applicable.

- Currently, timeRestrictionForChannelMeasurements, timeRestrictionForInterferenceMeasurements, and groupBasedBeamReporting are mandatory present in CSI-ReportConfig. Seems these fields are not applicable for UEI-BR, which should be confirmed by RAN1. UE should ignore these field for UEI BM.

If UEI BR parameters are included in CSI-MeasConfig, a clean design is possible.

Proposal 7:  RAN2 wait for stable RRC parameter list to decide whether to add UEI BR parameters in CSI-ReportConfig or in CSI-MeasConfig.

## 2.2 Other RRC parameters

- CBSR

Here we discuss the CBSR (codebook subset restriction) parameters which are informed in particular by RAN1 LS R2-2404114.

[116bis] Agreement For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, refine the legacy CBSR as follows: - Only 1-bit hard restriction is supported (analogous to Rel-18 Type-II) - Moving (N1, N2) configuration out from CBSR IE and the CBSR can be optional configured - Send LS to RAN2, and subject to RAN2 consent - Group-based CBSR granularity where each bit in the CBSR is associated with a set of X1X2 SD basis vectors, where the set includes X1 adjacent SD basis vectors along the N1 direction and/or X2 adjacent SD bases along the N2 direction - FFS: Value(s) of X1 and X2 and detailed design/spec impact FFS: Whether/how to enable shared CBSR in RRC configuration for Type-I/-II codebooks with a same (N1,N2). [116bis] Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding CBSR design: - 1-bit hard restriction is supported (analogous to Rel-15 Type-I) - FFS: 3-bit scaling factor for soft restriction is supported with the scaling factor taken into account in CQI/PMI calculation - Moving (N1, N2) configuration out from CBSR IE and the CBSR can be optional configured - Send LS to RAN2, and subject to RAN2 consent - N1⋅O1⋅N2⋅O2X1⋅X2-bit CBSR where each bit in the CBSR is associated with a set of X1X2 SD basis vectors, where the set includes X1 adjacent SD basis vectors along the N1 direction and/or X2 adjacent SD bases along the N2 direction - FFS: Value(s) of X1 and X2 and detailed design/spec impact FFS: Whether/how to enable shared CBSR in RRC configuration for Type-I/II codebooks with a same (N1,N2).

Further, in RAN1 LS R1-2410758 more information on the relevant parameters are provided. For Rel-19 type-I single-panel codebook, the following parameters are required by RAN1.

As informed, CBSR bit string is optional. The length of the CBSR bit string is (N1O1N2O2)/(X1X2), where (N1, N2) and (X1, X2) take the defined values, and O1=O2=4 according to RAN1 agreement. Table1 below shows the CBSR bit string length for different cases for Rel-19 type-I single-panel codebook.

Table 1: CBSR bit string length for Rel-19 type-I single-panel codebook

For Rel-19 type-II and type-II Doppler codebook, the following parameters are required by RAN1.

As informed, CBSR bit string is optional. The length of the CBSR bit string is (N1N2)/(X1X2), where (N1, N2) and (X1, X2) take the defined values. Table 2 below shows the CBSR bit string length for different cases for Rel-19 type-II and type-II Doppler codebook.

Table 2: CBSR bit string length for Rel-19 type-II codebook

In Rel-15/16/17 (N1, N2) and CBSR configuration, the signaling of (N1, N2) and CBSR bit string are coupled since the length of CBSR bit string is a function of (N1, N2). Although CBSR is not a mandatory parameter for a configured codebook, it is always signaled with (N1, N2) because (N1, N2) is a mandatory parameter for a configured codebook. The cost of such signaling design is the overhead. Even if CBSR is not configured, a bit string of all 0s is still signaled. In Rel-18, in order to make CBSR bit string optional, a CHOICE of no CBSR is used, as shown below.

N1-N2-CBSR-r18 ::=     CHOICE {

two-one-r18            CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (8))},

two-two-r18            CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (27))},

four-one-r18           CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (16))},

three-two-r18          CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (35))},

six-one-r18            CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (24))},

four-two-r18           CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (43))},

eight-one-r18          CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (32))},

four-three-r18         CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (59))},

twelve-one-r18         CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (48))},

four-four-r18          CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (75))},

eight-two-r18          CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (75))},

sixteen-one-r18        CHOICE {no-cbsr-r18 NULL, cbsr-r18 BIT STRING (SIZE (64))}

}

For Rel-19 codebook, as informed by RAN1 LS R2-2404114, (N1, N2) configuration should be moved out from CBSR IE. If we reuse the Rel-18 CHOICE structure, the CBSR bit string has to be signaled also as a CHOICE of (X1, X2), which leads to CHOICE embedded in 3 layers. Here we provide some other options for discussion.

Option 1:

Reuse the Rel-18 CHOICE structure for the CBSR of Rel-19 type-1 and type-2 codebooks, i.e., bit string length is signaled as a CHOICE of (N1, N2), a CHOICE of (no CBSR), and a CHOICE of (X1, X2), which leads to CHOICE embedded in 3 layers. This option does not need separate parameters for N1, N2, X1, X2, but cannot reflect nonapplicable (X1, X2) pairs so extra configuration restrictions have to be capture in field description.

Option 2:

Alternatively, (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2. Note there are many (X1, X2) values not applicable to type-II and type-II Doppler codebook, but only applicable to type-I single-panel codebook. This option does not need separate parameter/description for X1, X2, and can directly reflect applicable (X1, X2) pairs in the CHOICE of (X1, X2). An example is shown below.

n1-n2-r19		ENUMERATED {eight-three, six-four, sixteen-two, eight-four, sixteen-four, eight-eight},

typeI-codebookSubsetRestriction-r19		typeI-X1-X2-CBSR-r19							 OPTIONAL    -- Need R

typeI-X1-X2-CBSR-r19 ::=     CHOICE {

one-one-r19            CHOICE {twentyfour BIT STRING (SIZE (384)), thirtytwo BIT STRING (SIZE (192)), sixtyfour BIT STRING (SIZE (1024))},

two-one-r19            CHOICE {twentyfour BIT STRING (SIZE (192)), thirtytwo BIT STRING (SIZE (256)), sixtyfour BIT STRING (SIZE (512))},

two-two-r19            CHOICE {twentyfour BIT STRING (SIZE (96)), thirtytwo BIT STRING (SIZE (128)), sixtyfour BIT STRING (SIZE (256))},

four-one-r19           CHOICE {twentyfour BIT STRING (SIZE (96)), thirtytwo BIT STRING (SIZE (128)), sixtyfour BIT STRING (SIZE (256))},

four-two-r19           CHOICE {twentyfour BIT STRING (SIZE (48)), thirtytwo BIT STRING (SIZE (64)), sixtyfour BIT STRING (SIZE (128))},

four-four-r19          CHOICE {twentyfour BIT STRING (SIZE (24)), thirtytwo BIT STRING (SIZE (32)), sixtyfour BIT STRING (SIZE (64))}

}

typeII-codebookSubsetRestriction-r19		typeII-X1-X2-CBSR-r19							 OPTIONAL    -- Need R

typeII-X1-X2-CBSR-r19 ::=     CHOICE {

one-one-r19            CHOICE {twentyfour BIT STRING (SIZE (24)), thirtytwo BIT STRING (SIZE (32)), sixtyfour BIT STRING (SIZE (64))},

two-one-r19            CHOICE {twentyfour BIT STRING (SIZE (12)), thirtytwo BIT STRING (SIZE (16)), sixtyfour BIT STRING (SIZE (32))},

two-two-r19            CHOICE {twentyfour BIT STRING (SIZE (6)), thirtytwo BIT STRING (SIZE (8)), sixtyfour BIT STRING (SIZE (16))},

four-one-r19           CHOICE {twentyfour BIT STRING (SIZE (6)), thirtytwo BIT STRING (SIZE (8)), sixtyfour BIT STRING (SIZE (16))},

four-two-r19           CHOICE {thirtytwo BIT STRING (SIZE (4)), sixtyfour BIT STRING (SIZE (8))}

}

Option 3:

(N1, N2) is signaled as a separate parameter, and CBSR is optionally signaled as a variable BIT STRING. Variable BIT STRING is widely used in UE capability signaling, which can be utilized to simplify the signaling of CBSR. This option needs extra parameters for (X1, X2) and capture nonapplicable pairs in FD. An example is shown below.

n1-n2-r19		ENUMERATED {eight-three, six-four, sixteen-two, eight-four, sixteen-four, eight-eight},

typeI-x1-x2-r19		ENUMERATED {one-one, two-one, two-two, four-one, four-two, four-four}	OPTIONAL    -- Need R

typeII-x1-x2-r19		ENUMERATED {one-one, two-one, two-two, four-one, four-two}	OPTIONAL    -- Need R

typeI-codebookSubsetRestriction-r19		BIT STRING (SIZE (24, 32, 48, 64, 96, 128, 192, 256, 384,   512, 1024))							 OPTIONAL    -- Need R

typeII-codebookSubsetRestriction-r19		BIT STRING (SIZE (4, 6, 8, 12, 16, 24, 32, 64))		OPTIONAL    -- Need R

Based on the discussion in RRC running CR review, we will select from Option 2 and Option 3. Option 3 is the cleanest signalling which separates n1-n2, x1-x2, and CBSR, but lack of the mapping of CBSR bit string length to x1-x2. We also need to check whether it is possible to signal variable BIT STRING with non-consecutive size in ASN.1.

Proposal 8: For Rel-19 codebook CBSR parameters, for option 3 discuss how to reflect the mapping of CBSR bit string length to x1-x2, check whether it is possible to signal variable BIT STRING with non-consecutive size in ASN.1.

- Option 2: (N1, N2) is configured as a separate mandatory parameter not coupled with CBSR, and CBSR is optionally configured as a CHOICE of (X1, X2) and a CHOICE of N1N2, i.e., 2-layer CHOICE structure.

- Option 3: (N1, N2) is configured as a separate mandatory parameter not coupled with CBSR, (X1, X2) is optionally configured as a separate parameter, and CBSR is optionally configured using variable BIT STRING.

- additionalOneSlotOffsetDoppler

| CSI-Type-I/II | NZP-CSI-RS-ResourceSet | additionalOneSlotOffsetDoppler | additionalOneSlotOffset Dopp | New |  | For Rel-19 Type-II based on Rel-18 Type-II Doppler: 1-slot offset (per NZP-CSI-RS-Resource Group) relative to the slot offset configured by aperiodicTriggeringOffset in NZP-CSI-RS-ResourceSet | {enabled} |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CSI-Type-I/II | NZP-CSI-RS-ResourceSet | kdopp-r19 | Kdopp | new |  | The number of configured resource groups, each resource group comprises K NZP CSI-RS resources, where K={2,3,4} | 4, 8, 12 |

In the current running CR, it is only one bit (without per resource group).

kdopp-r19                           SEQUENCE {

numberOfResourceGroups-r19           ENUMERATED {n4, n8, n12},

numberOfResourcesPerGroup-r19        ENUMERATED {n2, n3, n4}

}                                                                                                      OPTIONAL,   -- Need R

additionalOneSlotOffsetDoppler      ENUMERATED{enabled}                                                    OPTIONAL   -- Need R

However, as RAN1 informed the one-bit enabled is per NZP-CSI-RS-Resource Group, and the number of configured resource groups is {4,8,12}. Based on the information in RAN1 RRC list, we think the additionalOneSlotOffsetDoppler should be a sequence of {enabled}, where the sequence size is configured by numberOfResourceGroups-r19.

Proposal 9: additionalOneSlotOffsetDoppler is a sequence of {enabled}, where the sequence size is configured by numberOfResourceGroups-r19.

# Conclusion

Based on the discussion, we have the following proposals.

Observation 1: RAN1 has endorsed TS 38.214 draft CR for the whole UEI BR procedure including window-counting-based report triggering. RAN1 has informed by LS there is no need of MAC support so far for UEI BR.

Observation 2: As the report notification and the actual report for UEI BR are all handled by PHY, there is no need of a detour to the MAC layer for window-counting-based report triggering in the middle of the procedure. The artificial PHY-MAC interaction in specification should be avoided.

Proposal 1:  RAN2 assumes the UEI BR procedure including window-counting-based report triggering is specified in RAN1 specification unless RAN1 informs RAN2 to specify.

Observation 3: For the window-counting options in RAN1 agreement, option 2 and Option 4 have large complexity in terms of running window/timer/counting. For option 4, the timer with counting met M can be expired at any time with respect to the PUCCH occasion, we have to discuss for each case whether a report is triggered or not.

Proposal 2: If it is decided RAN2 specifies the report triggering evaluation, for the window-counting operation, select from option 1 and option 3 listed in the RAN1 agreement.

Proposal 3: DRX active time includes the time while UE waiting for the PDCCH indicating a DG for UEI BR after transmitting a report notification in PUCCH.

Proposal 4: Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits UEI BR on PUSCH when it is triggered.

Proposal 5: The current UL skipping procedure already exclude UEI BR since it is transmitted in UCI, no additional MAC impact.

Proposal 6: The type-1 CG allocated for the mode-B UEI BR is transparent to MAC and excluded from MAC UL-SCH transmission procedure.

Proposal 7:  RAN2 wait for stable RRC parameter list to decide whether to add UEI BR parameters in CSI-ReportConfig or in CSI-MeasConfig.

Proposal 8: For Rel-19 codebook CBSR parameters, for option 3 discuss how to reflect the mapping of CBSR bit string length to x1-x2, check whether it is possible to signal variable BIT STRING with non-consecutive size in ASN.1.

- Option 2: (N1, N2) is configured as a separate mandatory parameter not coupled with CBSR, and CBSR is optionally configured as a CHOICE of (X1, X2) and a CHOICE of N1N2, i.e., 2-layer CHOICE structure.

- Option 3: (N1, N2) is configured as a separate mandatory parameter not coupled with CBSR, (X1, X2) is optionally configured as a separate parameter, and CBSR is optionally configured using variable BIT STRING.

Proposal 9: additionalOneSlotOffsetDoppler is a sequence of {enabled}, where the sequence size is configured by numberOfResourceGroups-r19.

# References

- RP‑242356	Revised Work Item: NR mobility enhancements Phase 4, RAN#105.

- R1-2501669, Introduction of UE-initiated/event-driven beam management, draft CR, Nokia.

- R1-2500846, LS to RAN2 on MAC impacts for Rel-19 NR MIMO Ph5, Samsung, MediaTek.

# Appendix: UEI BR window-counting options

Agreement

Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, further study the following options for possible down-selection

- Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured.

- Option-2: The measurement window is from T_Instance – T_window to T_Instance, where T_Instance is an evaluation occasion of event instance, and T_proc is RRC configured.

  - The UEI beam report in the second PUSCH is based on the most recent measurement for new/current beam RS(s).

- Option-3: The length, slot offset and periodicity of a measurement window are configured per CSI report configuration by NW.

- Option-4: If an Event-2 instance for a new beam is obtained at the time t, UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window)

- Note: T_window is the agreed time window parameter for measurement.

- Note: Other options are not precluded.