3GPP TSG-RAN WG2 Meeting #129bis	R2-2502667

Wuhan, China, Apr. 7th – 11st, 2025

Agenda item:	8.8.2

Source:	Samsung

Title:	Discussion on Downlink Coverage Enhancement

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

For Rel-19 NR NTN, the latest WID has confirmed the objective of DL coverage enhancement (DL-CE) [1].

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

According to the objective, link-level enhancements to be specified are PDCCH repetition and PDSCH repetition, system-level enhancement to be specified is SSB periodicity extension up to 160ms.

For link-level enhancements, we expect to wait more RAN1 progress before we can identify the relevant RAN2 impacts. In this contribution, we share our views on RAN2 impacts of SSB periodicity extension.

# Discussion

## Bar or not bar

One aspect we have discussed a lot is whether/how to bar legacy UEs to access DL-CE cells. RAN1 has reached an observation regarding the backward compatibility in RAN1#119.

In the last meeting RAN1#120, it is further agreed that only 160 ms is supported in additional to the existing default periodicity of 20ms for initial access.

Agreement

For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

- The additional default value assumed by UE during initial access (apart from the existing 20ms value) is 160 ms.

Our interpretation is that legacy UEs assuming default SSB periodicity of 20ms may have difficulties in initial search if the cell used for initial frequency scan has SSB periodicity 160ms, but they may still be able to camp on such a cell. It is possible that a legacy UE happens to detect SSBs within 20ms even for a cell with SSB periodicity 160ms, or legacy UE is able to detect longer-period SSBs by implementation. Once the legacy UE camps on a DL-CE cell, no issue is identified for UE’s operation since the current specification already supports SSB periodicity up to 160ms. The only potential issue for legacy UE would be occurred during initial access, as UE may assume default SSB periodicity of 20ms. For initial access, we analyze the possible cases of legacy UE.

1. When a legacy UE assuming SSB periodicity of 20ms happens to scan, detect and decode SSBs of periodicity 160ms for a DL-CE cell, it can acquire MIB and SIB1 by the current operation. If MIB and SIB1 are acquired, UE will become aware of the SSB periodicity by reading SIB1 -> ServingCellConfigCommonSIB -> ssb-PeriodicityServingCell, so that it can camp on the DL-CE cell with no issue.

2. When a legacy UE assuming SSB periodicity of 20ms happens to scan, detect and decode SSBs of periodicity 160ms for a DL-CE cell but failed to acquire MIB and SIB1 (e.g., not support the enhanced MIB/SIB1 periodicity), as currently specified in TS 38.304 UE itself will treat the cell as barred due to being unable to acquire the MIB/SIB1.

3. When a legacy UE assuming SSB periodicity of 20ms failed to scan, detect or decode SSBs of periodicity 160ms for a DL-CE cell, e.g., failed in decoding PSS/SSS to synchronize and identify PCI, or failed in decoding PBCH to acquire MIB, UE cannot identify the cell or acquire the MIB, as currently specified in TS 38.304 UE itself will treat the cell as barred due to being unable to acquire the MIB/SIB1.

Observation 1: A legacy UE assuming default SSB periodicity of 20ms can still access DL-CE cells with SSB periodicity 160ms if it happens to scan, detect and decode SSBs for such a cell. A legacy UE may be able to access DL-CE cells with longer SSB periodicity by implementation.

Observation 2: As already specified, legacy UE will treat DL-CE cells as barred due to being unable to acquire the MIB/SIB1 (e.g., when UE fails to scan, detect or decode SSBs of longer periodicity).

Based on the above observation, in our view for Rel-19 there is no need to bar legacy UE from accessing DL-CE using an explicit bar bit. Therefore, introducing a new bar bit is not necessary.

Proposal 1: In Rel-19, do not introduce any new bar bit to bar legacy UE from accessing DL-CE cells.

Similarly, considering the desire to provide NTN service with a broader coverage, it is not strongly motivated either to prevent legacy NTN UEs from reselecting DL-CE cells with longer SSB periodicity. The current SMTC periodicity {sf5, sf10, sf20, sf40, sf80, sf160} can imply the SSB periodicity of neighbor cells per frequency, and UE can select cells properly by implementation, e.g., with or without (de)prioritization of cells operating with larger SSB periodicity.

Proposal 2: Do not proactively prevent legacy NTN UEs to (re)-select DL-CE cells with SSB periodicity 160ms.

## SMTC periodicity and offset

Question 1: Do we need multiple SMTC periodicities per frequency layer?

In the last meeting, we have agreed different SSB periodicities per frequency layer is possible.

- RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer

The current specification supports SSB periodicities {ms5, ms10, ms20, ms40, ms80, ms160}. SMTC is used to configure the window for neighbor cells’ SSB measurement. Currently, SMTC, configured per frequency layer for neighbor cell measurement in connected mode (in measConfig) and in idle/inactive mode (in SIB2/4), supports one periodicity among {sf5, sf10, sf20, sf40, sf80, sf160}, one duration among {sf1, sf2, sf3, sf4, sf5}, and up to 4 different offsets for NTN. In our view, the current one SMTC periodicity per frequency layer can already support measurement on SSBs of different periodicities.

First consider a common offset. In one scenario, it is possible that all intra-frequency cells are deployed with same SSB periodicity, and cells with different SSB periodicities are deployed on different frequencies. In this case, a single SMTC periodicity is enough for each frequency layer.

In another scenario, cells with different SSB periodicities are deployed on intra-frequency. To accommodate different SSB periodicities, it is possible that a single SMTC periodicity is configured as the largest SSB periodicity so that in each SMTC window UE can detect SSBs of all periodicities. It is also possible that a smaller SMTC periodicity is used to reduce measurement delay (e.g., 40ms in Figure 1). When UE performs measurement, UE has to identify cells by detecting and decoding SSBs on a certain frequency layer and be aware of which cells are measured or not measured in each SMTC window so that the measurement result per cell is properly consolidated, filtered, and reported. There is no difference for TN and NTN on this aspect. Thus, a single SMTC periodicity per frequency layer can also work well for UE to measure SSBs of different periodicities.

Figure 1: One SMTC for SSBs of different periodicities (offset=0).

Next consider diverse offsets in NTN. In addition to different SSB periodicities, neighbor cells’ SSBs can be seen with different offsets due to different satellite’s propagation delay or satellite beam hopping. Multiple SMTCs with different offsets are applied to accommodate diverse offsets. In this case, if a small SMTC periodicity is used (e.g., periodicity 40ms in Figure 2), UE may not be able to detect any SSBs in certain SMTC windows. By setting the SMTC periodicity as the largest SSB periodicity on the frequency layer (e.g., periodicity 80ms in Figure 2), void SMTC window can be avoided despite the drawback of longer measurement delay.

Figure 2: Two SMTCs with same periodicity and different offsets.

Observation 3: A same periodicity for SMTC(s) per frequency layer can support UE to measure SSBs of different periodicities.

Proposal 3: RAN2 confirms that a single SMTC periodicity per frequency layer can support UE to measure SSBs of different periodicities among neighbor cells.

Question 2: Do we need more than 4 SMTC offsets per frequency layer?

If neighbor cells are illuminated at different time due to satellite beam hopping, do we need more than 4 SMTC offsets for intra-frequency measurement? In our view, beam hopping occurs for the purpose of satellite energy saving. In TR 38.821, the satellite beam to PCI/SSB mapping is investigated.

Moreover, in the last meeting we build some common understanding that all NR beams of a cell (e.g., with SSB beam sweeping) are considered as active when satellite beam(s) is illuminated for the cell. This means satellite beam hopping is in a cell-level granularity.

- RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame

- RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell)

Based on satellite beam hopping in a cell-level granularity, we further reached the following observations.

- From RAN2 perspective, we support option a (clustered cells illuminated) and option b (scattered cells illuminated) for further discussion on SMTC. No LS is sent to RAN1.

- RAN2 observes that if the cells active simultaneously are in clusters, existing SMTC mechanism (a maximum of 4 SMTCs per frequency) may be sufficient (FFS if any solution identified for option b applies to option a as well).

- RAN2 observes that if the cells active simultaneously are scattered, (for the case of intra-frequency neighbouring cells), the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough.

Regardless of the number of offsets, SMTC window should cover SSB burst position in time so that UE can detect and measure SSBs at each SMTC window (e.g., Figure 3 below). Basically, NW should configure SMTC properly to ensure the cells associated to the SMTC are illuminated during the corresponding SMTC window. There is no difference for RRC connected mode or RRC idle/inactive mode on this aspect.

Observation 4: As legacy operation, regardless of the number of SMTC(s), SMTC window should cover SSB position in time so that UE can detect and measure SSBs at each SMTC window.

Figure 3: Six SMTCs with same periodicity and different offsets.

For RRC_CONNECTED UE, RRM measurement objects and the associated SMTCs are UE dedicated configuration under NW control and up to UE capability. NW should configure SMTC(s) no more than the supported number. UE can support at most 4 SMTC(s) and in the last meeting we have agreed that at any time UE only applies at most 4 SMTCs per frequency layer.

- The number of SMTC/gaps a UE needs to consider at any time will not be increased further

If more than 4 SMTCs are introduced, UE autonomous selection is needed. However, UE autonomous selection is not desired in RRC CONNECTED as it will cause more issues for NW scheduling. Then, based on UE capability NW should not configure more than 4 SMTCs in RRC_CONNECTED.

Proposal 4: The number of SMTCs (i.e., offsets) configured per frequency layer in measurement object is at most 4 as legacy for RRC_CONNECTED.

From NW perspective, UE location can be obtained or estimated. The following ways are supported currently.

- NW configures coarseLocationRequest and/or includeCommonLocationInfo in the report configuration, and UE reports location in periodical or event-triggered measurement report.

- NW configures coarseLocationRequest in UEInformationRequest message, and UE reports location if available in UEInformationResponse message.

- NW configures eventD1/D2 for event-trigger measurement report to track UE distance to a reference location.

Based on the reported/estimated UE location, NW can configure SMTC(s) only for close neighbor cells. As in Figure 3, UE 1 can be configured with SMTCs for neighbor cell 1, 2, 3, UE 2 can be configured with SMTCs for neighbor cells 4, 5, 6.

Observation 5: For RRC_CONNECTED, NW can configure UE dedicated SMTC(s) for close neighbor cells based on UE location estimated from measurement report or UE assistance information.

Proposal 4: UE autonomous SMTC selection for RRC_CONNECTED is not supported.

For RRC_IDLE/INACTIVE, SMTCs only for surrounding neighbor cells are needed, for instance, up to 6 neighbor cells in the commonly used cellular deployment. Consider an extreme case that all 6 neighbor cells are intra-frequency (i.e., frequency reuse factor is 1). To accommodate scenario of the scattered satellite beam illumination where all 6 intra-frequency neighbor cells are illuminated at different time, 6 SMTC offsets might be needed. A simple solution is to extend the number of SMTC offsets per frequency layer from 4 to 6 for RRC_IDLE/INACTIVE.

Proposal 6: Consider to extend the number of SMTCs from 4 to 6 in SIB2/4 for RRC_IDLE/INACTIVE to introduce additional offsets.

## SMTC selection

For RRC_IDLE/INACTIVE UE, as specified currently in SIB2/4, if UE supports less than 4 SMTCs, it is by UE implementation to select the SMTC(s) to be applied in RRC_IDLE/INACTIVE up to UE capability. If the number of SMTCs is further increased to more than 4, SMTC selection can also be done by UE, considering conventionally it is not specified how to measure neighbor cell once measurement is initiated.

smtc4list Measurement timing configuration list for NTN deployments, see clause 5.5.2.10. The offset of each SSB-MTC4 in smtc4list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in this list, it is up to the UE to select which SMTCs to consider.

Observation 6:  In Rel-17/18 NTN, for a UE that supports less SMTCs than configured in SIB2/4, it is up to the UE to select which SMTCs to consider.

From UE power saving perspective, UE does not need to measure neighbor cells that are not in its proximity. So, UE can select less SMTC(s) and/or neighbor cells for cell reselection measurement. However, this is not an issue specific to Rel-19 beam hopping but a general problem for NTN. In Rel-17/18, we did discuss extensively on location-based neighbor cell measurement for cell reselection. At that time, there was not consensus to support it. In our view, SMTC selection shares the same motivation, i.e., UE power saving, and can be achieved based on location similarly.

One approach is location-based SMTC selection. In our understanding, location-based SMTC can work in such a way that SMTC is configured with association to a reference location and a distance threshold. UE determines its location and calculates its distance to the reference location of a SMTC, if smaller than the distance threshold, UE apply that associated SMTC for neighbor cell measurement.

For location-based SMTC selection, UE only needs to acquire its location to determine the applied SMTC when neighbor cell measurement is initiated, but does not need to acquire its location all the time which leads more power consumption. As location-based measurement initiation is already supported, for which UE has to acquire its location, the acquired location can be directly used to determine the neighbor cells to measure, meaning there is no extra UE power consumption is needed for location-based SMTC selection in this case. During the measurement, UE is unlikely to frequently acquire UE location as UE movement is negligible with respect to NTN cell.

Observation 7: For location-based SMTC selection for cell reselection, UE can select a SMTC if the distance to an associated reference location is smaller than a threshold. UE location acquired for location-based measurement initiation can be directly used to select SMTC.

A second approach mentioned during offline discussion (R2-2501415 [2]) is SSB-based SMTC selection. In our understanding, this approach is also based on UE location, with the assumption that UE location can be somehow reflected by the SSB index. For example, if UE is located in an area covered the an SSB beam, the corresponding SSB index can be used to imply the UE location. Based on this, SMTC can be configured with association to an SSB index, UE selects an SSB index and applies the associated SMTC for neighbor cell measurement.

For SSB-based SMTC selection, one concern is that SMTC selection does not work in case of a single SSB index used per PCI. In case of multiple SSB indexes per PCI, it is also doubted whether SMTC selection can work well since SSB selection is up to UE implementation but cannot be specified, thus, it cannot be ensured that the selected SSB index correctly reflects the UE location. One may consider UE can select the SSB with the strongest measured RSRP. However, due to the large NTN cell and long propagation distance, the variation of the received beam power can be small in NTN, and after consolidation and filtering the measured RSRP may not be distinguishable between SSB beams for UE at the overlapping area of two SSB beams, where UE may use both beams to consolidate SSB detection, decoding, and measurement.

Observation 8: Assuming UE location can be somehow reflected by the selected SSB index associated to a SMTC, SSB-index based SMTC selection for cell reselection does not work in case of a single SSB index per PCI, and in case of multiple SSB indexes per PCI, it may not work well without specifying SSB selection.

Based on the above analysis, location-based SMTC selection is preferred.

Proposal 7: For location-based SMTC selection for cell reselection, provide location information for the associated SMTCs to help UE measure only close-enough neighbor cells. UE can select a SMTC if the distance to an associated reference location is smaller than a threshold.

# Conclusion

In this contribution we have the following proposals.

Observation 1: A legacy UE assuming default SSB periodicity of 20ms can still access DL-CE cells with SSB periodicity 160ms if it happens to scan, detect and decode SSBs for such a cell. A legacy UE may be able to access DL-CE cells with longer SSB periodicity by implementation.

Observation 2: As already specified, legacy UE will treat DL-CE cells as barred due to being unable to acquire the MIB/SIB1 (e.g., when UE fails to scan, detect or decode SSBs of longer periodicity).

Proposal 1: In Rel-19, do not introduce any new bar bit to bar legacy UE from accessing DL-CE cells.

Proposal 2: Do not proactively prevent legacy NTN UEs to (re)-select DL-CE cells with SSB periodicity 160ms.

Observation 3: A same periodicity for SMTC(s) per frequency layer can support UE to measure SSBs of different periodicities.

Proposal 3: RAN2 confirms that a single SMTC periodicity per frequency layer can support UE to measure SSBs of different periodicities among neighbor cells.

Observation 4: As legacy operation, regardless of the number of SMTC(s), SMTC window should cover SSB position in time so that UE can detect and measure SSBs at each SMTC window.

Proposal 4: The number of SMTCs (i.e., offsets) configured per frequency layer in measurement object is at most 4 as legacy for RRC_CONNECTED.

Observation 5: For RRC_CONNECTED, NW can configure UE dedicated SMTC(s) for close neighbor cells based on UE location estimated from measurement report or UE assistance information.

Proposal 4: UE autonomous SMTC selection for RRC_CONNECTED is not supported.

Proposal 6: Consider to extend the number of SMTCs from 4 to 6 in SIB2/4 for RRC_IDLE/INACTIVE to introduce additional offsets.

Observation 6:  In Rel-17/18 NTN, for a UE that supports less SMTCs than configured in SIB2/4, it is up to the UE to select which SMTCs to consider.

Observation 7: For location-based SMTC selection for cell reselection, UE can select a SMTC if the distance to an associated reference location is smaller than a threshold. UE location acquired for location-based measurement initiation can be directly used to select SMTC.

Observation 8: Assuming UE location can be somehow reflected by the selected SSB index associated to a SMTC, SSB-index based SMTC selection for cell reselection does not work in case of a single SSB index per PCI, and in case of multiple SSB indexes per PCI, it may not work well without specifying SSB selection.

Proposal 7: For location-based SMTC selection for cell reselection, provide location information for the associated SMTCs to help UE measure only close-enough neighbor cells. UE can select a SMTC if the distance to an associated reference location is smaller than a threshold.

# Reference

RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

R2-2501415, Report of [AT129][304][R19 NR NTN] LS to RAN1 on DL coverage enhancements (Huawei)