3GPP TSG RAN WG2 #129-bis	R2-2502685

Wuhan, Hubei, PRC, April 7th - 11th, 2025

Source:	Panasonic

Title:	Store & Forward: Mode change indication

Agenda Item:	8.9.2  Support of Store & Forward

Document for:	Discussion, decision

# 1. Introduction

Store & Forward operation of satellites is seen as a key enhancement of IoT-NTN in order to facilitate cost-effective deployment of IoT-NTN to allow for delay-tolerant, non-real-time IoT-NTN services [1].

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required.

Furthermore, the NTN sessions of the last RAN2 meeting #129 in Athens resulted in the following S&F-related results (see [4]):

- RAN2 currently assumes that a S&F mode changes does not necessarily imply an MME change. - The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information). - Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time). - A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply.

The aim of this document is to propose a proper solution for indicating the mode changes – on the basis of the highlighted agreements above and earlier decisions.

# 2. Discussion on indicating changes between S&F and RT modes – in both directions

# 2.1 Targets to be reached with the mode change signalling

  - Agreed targets:

- The change from S&F to RT/normal mode shall be indicated.
This is useful for UEs in so far as they can estimate the point of time their uploaded data will be delivered to the network. UEs in the area where RT mode will be in place can use that satellite as any other IoT-NTN satellite – but should be obeying the point of time that satellite will be changing back to S&F mode (see below).

  - Not yet agreed targets:

- Indication of the change from RT/normal to S&F mode.
As stated above, the indication of the point of time the change from RT to normal mode will happen is helpful for UEs in so far as they can make decisions depending on the remaining time the satellite will be operating in RT mode, i.e. an upload can be scheduled early enough for immediate download from the satellite to the network or the UE can prepare for making use of the upcoming S&F mode.

  - Observation 1:	There is no agreement achieved yet regarding the need for indicating the change from RT/normal mode to S&F mode.

## Proposal 1:	RAN2 to decide, if an indication of the change from RT/normal mode to S&F mode is needed/useful.

## Proposal 2:	Should P1 be agreed, i.e. a related decision is accepted as being appropriate, we suggest to indicate the point of time the next change from RT/normal mode to S&F mode will take place.

  - 2.2 Current status of the indications for stopped and started coverage – for IoT-NTN and for NR-NTN

  - The current version V18.4.0 of the RRC specifications TS 36.331 [2] and TS 38.331 [3] consists of the following definitions for t-Service and related fields.

- IoT-NTN: TS 36.331 V18.4.0 [2]:

  - t-ServiceStart (part of SIB32 for remote satellites):

    - Time information on when the incoming satellite is going to start serving the area for quasi-earth fixed cell.

  - t-ServiceStartNeigh (part of SIB33 for neighbouring satellites):

    - Indicates the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell(s) served by the satellite indicated by satelliteId, see 5.5.3.1, 5.5.8 and 36.304 [4]. This field is only present for the NTN quasi-Earth fixed neighbour cell(s).

- NR-NTN: TS 38.331 V18.4.0 [3]:

  - t-Service (field in SIB19 consisting of most NTN-related signalling):

    - Indicates the time information on when a cell provided via NTN is going to stop serving the area it is currently covering. This field applies for both service link switches in NTN quasi-Earth fixed cell and feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). The exact stop time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field. The reference point for t-Service is the uplink time synchronization reference point of the cell. This field is only present in an NTN cell.

  - t-ServiceStart (field in SIB19 consisting of most NTN-related signalling):

    - Indicates the time information on when the target satellite is going to start serving the area currently covered by the serving satellite. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1st January 1900 (midnight between Sunday, December 31, 1899, and Monday, January 1, 1900). The exact start time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field. The reference point for t-ServiceStart is the uplink time synchronization reference point of the serving satellite.

  - What we derive from that signalling and those corresponding definitions is:

- In the IoT-NTN case, the two fields t-ServiceStart (in SIB32 indicating remote satellite’s attributes) and t-ServiceStartNeigh (in SIB33 indicating neighbouring satellite’s attributes) so far apply to QEF cells only.
The two fields indicate when the incoming satellite (remote Sat4 in the example below) is going to start serving the area currently served by the serving satellite (Sat1 in example). T-ServiceStartNeigh occurs together with a satellite Id for identification of the relevant neighbouring satellite.

## LEO example constellation with four satellites and one gateway
(capture range angle illustrated with light yellow circle segment),
each satellite illuminating a small single spot on the ground – as exemplified with Sat3

- The current definitions of the two IoT fields in question are not covering the case of S&F<->RT change indication – see the beginning of this clause.

And these fields apply to QEFC’s only. Extending the existing definitions towards the intended purpose is suited for leading to unwanted ambiguities. Furthermore, there is no signalling available for indicating the opposite instance, i.e. the stop of coverage – as far as the existing meaning of the signalling is concerned.

- In the NR-NTN case – opposite to the IoT-NTN case – the definition of t-Service envisages the stop of serving the area currently served caused by a feeder link switch event for both, quasi-Earth-fixed (QEFCs) and Earth-moving cells (EMCs). In very general terms, the change between S&F and RT/normal mode can be seen as a feeder link switch case, but – in fact – the change in question is not one that once built the basis for the creation of t-Service. Envisaged was a short interruption occurring between the loss of the feeder link to one gateway and establishing the connection to another gateway.

In our case, the interruption might last for more than an hour and (re-)establishment of feeder link connection might be realized with the same gateway as before or another one.

- In the NR-NTN case, it is currently not formulated explicitly to which orbit types t-ServiceStart applies. Exactly those t-Service applies to? From a logical point of view, that would make most sense. Anyway, that is an NR-NTN-specific issue and not directly connected to the targets we’re heading for here.

Observation 2:	The current IoT-NTN fields t-ServiceStart (in SIB32 indicating remote satellite’s attributes) and t-ServiceStartNeigh (in SIB33 indicating neighbouring satellite’s attributes) are not covering the indication of changes between S&F and RT/normal modes. It would be impractical extending the definitions in order to include the aforementioned change indication.

Observation 3:	The NR-NTN field t-Service is also not covering the application we have in mind. Furthermore, to IoT-NTN t-Service would be a new – so far not existing – field.

Proposal 3:	RAN2 to neither (re-)use the existing IoT-NTN fields t-ServiceStart and/or t-ServiceStartNeigh nor introduce the NR-NTN field t-Service for IoT-NTN purposes.

  - 2.3 Proposed solution

Re-using existing parameters as those outlined with clause 2.2. above for the purposes in question – see clause 2.1 – might lead to confusion regarding orbit type and the meaning of the signalling. Therefore we’re suggesting the adoption of new fields as follows:

## Proposal 4:	For indicating the point of time the mode of operation changes from S&F to RT/normal mode and vice-versa, a new field t-changeSFRTSF is added to SIB31 in a fully backwards compatible way.

Proposal 5:	The change from S&F mode to RT/normal mode is signalled by means of timeInfoUTC as used by e.g. SIB16, i.e. with an absolute time indication. The period of time the satellite continues to operate in RT/normal mode – e.g. 20 minutes – is indicated with RTmTimeOffset. The granularity should be seconds.

3. Conclusions

In this document, we discussed how to proceed with the indications between the S&F and RT/normal modes – in both directions.

The following observations and proposals were made:

  - Observation 1:	There is no agreement achieved yet regarding the need for indicating the change from RT/normal mode to S&F mode.

Observation 2:	The current IoT-NTN fields t-ServiceStart (in SIB32 indicating remote satellite’s attributes) and t-ServiceStartNeigh (in SIB33 indicating neighbouring satellite’s attributes) are not covering the indication of changes between S&F and RT/normal modes. It would be impractical extending the definitions in order to include the aforementioned change indication.

Observation 3:	The NR-NTN field t-Service is also not covering the application we have in mind. Furthermore, to IoT-NTN t-Service would be a new – so far not existing – field.

## Proposal 1:	RAN2 to decide, if an indication of the change from RT/normal mode to S&F mode is needed/useful.

## Proposal 2:	Should P1 be agreed, i.e. a related decision is accepted as being appropriate, we suggest to indicate the point of time the next change from RT/normal mode to S&F mode will take place.

Proposal 3:	RAN2 to neither (re-)use the existing IoT-NTN fields t-ServiceStart and/or t-ServiceStartNeigh nor introduce the NR-NTN field t-Service for IoT-NTN purposes.

## Proposal 4:	For indicating the point of time the mode of operation changes from S&F to RT/normal mode and vice-versa, a new field t-changeSFRTSF is added to SIB31 in a fully backwards compatible way.

Proposal 5:	The change from S&F mode to RT/normal mode is signalled by means of timeInfoUTC as used by e.g. SIB16, i.e. with an absolute time indication. The period of time the satellite continues to operate in RT/normal mode – e.g. 20 minutes – is indicated with RTmTimeOffset. The granularity should be seconds.

- References

[1]	RP-240776 Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2]	3GPP TS 36.331 V18.4.0 (2024-12)				3rd Generation Partnership Project; Technical
                    	Specification Group Radio Access Network;
                    	Evolved Universal Terrestrial Radio Access (E-
                    	UTRA); Radio Resource Control (RRC);
                    	Protocol specification (Release 18)

[3]	3GPP TS 38.331 V18.4.0 (2024-12)		3rd Generation Partnership Project; Technical 
                                                                                  Specification Group Radio Access Network; 
                                                                                  NR; Radio Resource Control (RRC) protocol
                                                                                  specification (Release 18)

[4]	R2-129 NR-NTN-IoT-NTN (Sergio)_ EOM