3GPP TSG RAN WG2 #129bis	R2-2502691

Wuhan, China, March 7th – 11th, 2025

Agenda Item:	8.2.3

Source:	Fraunhofer HHI, Fraunhofer IIS

Title:	Discussion on random access procedures

Document for:	Discussion

# Introduction

In the last RAN#106 plenary, a new work item on solutions for Ambient-IoT (Internet of Things) in NR was approved [1]. Ambient-IoT is a new 3GPP IoT technology that relies on ultra-low complexity devices with ultra-low power consumption for very-low-end IoT applications. As a starting point, the following guidance was given to RAN2 WG:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signaling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included.

Additionally, in the last RAN2#129 [2] the following agreements were made in regards random access

Agreements for re-access: - For Rel-19, only 3-step CBRA is supported for A-IoT - We will specify both CBRA and CFRA. - Re-use the subsequent paging message to trigger re-access. There is no need to differentiate msg1 resource for initial access vs re-access. - NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback - RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

In alignment with the guidance outlined in the WID, and the agreements made in the last RAN2 meeting, our contribution further explores these functionalities and discuss potential areas for improvement.

# Discussion

## NACK based feedback for re-access

In the last meeting, a NACK-based feedback mechanism for Msg3 was discussed and agreed. In essence, this mechanism aims to regulate re-access attempts while minimizing signaling overhead. However, there is still open FFS in how to determine the success or failure of Msg3 transmission, which can be handled in different ways.

One option is an implicit indication through a timer, where a device assumes success if no NACK is received within a predefined period. Although this approach aims to minimize signaling, it may introduce uncertainty, particularly in cases where feedback is delayed or lost. If a device receives a subsequent indication with the same transaction ID, while still waiting for a potential NACK, it may prematurely initiate re-access, leading to unnecessary contention and resource inefficiency.

Observation 1. The use of an implicit mechanism based on a timer may lead to premature re-access attempts and inefficient resource usage.

To prevent this, an explicit success indication could be introduced to signal when a device has successfully completed its transmission. In this context, under the 8.2.2 agenda item, the following was agreed:

- Agreements on paging - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field - The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

From our perspective, if way-1 is adopted, the new R2D message could also be leveraged to provide an explicit success indication, ensuring that devices that have completed their transmission do not unnecessarily re-attempt access. This would help reduce redundant transmissions and improve efficiency. Also, such an indication could serve as a next-slot indicator for other devices still awaiting transmission opportunities, enhancing overall coordination.

Proposal 1. Support an explicit success indication, which can be in the form of a “QueryRep-like” message.

## Re-access procedure

Although it was agreed in RAN2#129 to re-use subsequent paging messages for re-access,  the introduction of a “QueryRep-like” message as a success/failure indication can be laveraged to enable re-access in the same round. This is particularly relevant in high-demand scenarios where multiple devices contend for resources, as waiting for the next paging cycle could significantly impact their access delay.

One of the key advantages of this approach is its ability to distribute re-access attempts more effectively, rather than concentrating them at the start of a new paging round. If re-access is solely dependent on the next scheduled paging message, devices that experience access failures early in the round may remain idle for an extended period, leading to inefficient resource usage. By contrast, triggering re-access opportunities dynamically within the same round ensures that devices can rapidly retry without impacting those that have yet to attempt access. Additionally, by allocating separate Msg1 resources for re-accessing devices, the likelihood of repeated collisions is reduced, as these resources can be managed independently from those assigned for initial access.

From a network coordination perspective, leveraging an R2D message for re-access signaling provides greater flexibility in adapting to varying traffic conditions. For instance, in low-traffic scenarios where access failures are minimal, the network may decide not to allocate additional Msg1 resources, conserving resources for subsequent rounds. Conversely, in high-traffic situations where congestion and collisions are more common, the ability to dynamically allocate additional access slots within the round can enhance reliability and ensure a higher success rate in the random access procedure.

Proposal 2. The reader can use a “QueryRep-like” indication to allocate additional access occasions within the same round for the purpose of re-access.

# Conclusion

In this contribution we discussed the A-IoT random access. Based on the discussion in the previous sections we made the following observations and proposals:

Observation 1. The use of an implicit mechanism based on a timer may lead to premature re-access attempts and inefficient resource usage.

Proposal 1. Support an explicit success indication, which can be in the form of a “QueryRep-like” message.

Proposal 2. The reader can use a “QueryRep-like” indication to allocate additional access occasions within the same round for the purpose of re-access.

# References

- 3GPP RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG-RAN #106, Madrid, Spain, Dec 9th-12th, 2024.

- 3GPP “Draft Report of 3GPP TSG RAN WG2 meeting #129”, 3GPP TSG RAN WG2 #129, Athens, Greece, Feb. 17th-21st, 2025