# 3GPP TSG-RAN WG2 Meeting #129bis                                         R2-2502697

Wuhan, China, Apr. 7th – 11st, 2025

Source: 			CMCC

Title:  		    Discussion on LCM for positioning

Agenda item:	8.1.2.3

Document for: 	Discussion

Introduction

With the past RAN2 meetings, RAN2 discussed general procedures for LCM for UE sided model for AI positioning case 1. The following agreements have been reached.

#128 1 For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF . - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS 2 As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered #129 - Introduce AI/ML positioning Case 1 as a new positioning method. 2: Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extension can be discussed while drafting the stage 3 CR. 3: FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed. FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation. 4: The content of error cause is discussed while drafting stage3 CRs. 5: As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list. 6: UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality. FFS if any additional LMF control is needed.

In the contribution, we will discuss the remaining controversial issues.

Discussion

Fallback configuration

RAN2 has agreed that if the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning and reply with LPP ProvideLocationInformation message with error cause. We understand that the details on error cause can be discussed in stage3. Regarding fallback from AIML based positing to legacy positioning, we do not think any pre-configuration about fallback is needed. The fallback should be handled via LPP reconfiguration under network control, instead of UE autonomously fallback according to pre-configuration.

Proposal 1: The fallback should be handled via LPP reconfiguration under network control, instead of UE autonomously fallback according to pre-configuration.

RAN1 LS and progress impacts

With RAN1#120 meeting, RAN1 concluded that for Rel-19 AI/ML based positioning Case 3b, if the gNB cannot report measurements according to measurement type (A) or measurement type (B) requested by LMF, the gNB responds to LMF that the gNB cannot provide measurement with the requested measurement type. The gNB does not respond to LMF with a measurement report where the measurement type is different from requested. It is up to RAN3 to decide the signalling details. It is RAN1 understanding that this is the same behavior as in legacy when the NG-RAN node cannot support the requested measurement.

RAN1 had sent an LS to RAN3 and RAN2 in [2]. For case 3b direct AI/ML positioning, NG-RAN node assisted positioning with LMF-side model, it should be low priority in RAN2 discussion as the main signaling is between gNB and LMF, which is RAN3’s scope. From RAN2’s perspective, the discussion on case 3b should be postponed until RAN3 makes progress on it.

Proposal 2: RAN2 postponed the discussion on case 3b until RAN3 makes progress on it.

With RAN1#116bis and RAN1#117, we have the following work assumptions and agreement:

Work assumptions: For training data generation of AI/ML based positioning Case 1/2a/2b/3b, the label and its related data (e.g., time stamp) can be generated by:

- PRU

- Non-PRU UE with estimated location

- LMF

Note: transfer of label and its related data is out of RAN1 scope.

For the case that the ground truth label is generated by LMF, LMF generates the ground truth label based on some measurement transmitted by UE and TRP’s location coordinate. LMF provides the corresponding ground truth label and its related data (e.g. time stamp) to UE for model training. The existing signaling ProvideAssistanceData can be reused.

Proposal 3: At least ground truth label and its related data (e.g. time stamp) should be provided by LMF via signaling ProvideAssistanceData.

As per RAN1 progress in last meeting, for both UE initiated and LMF initiated case, the DL PRS configuration and existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD are needed to be provided by LMF to UE.

For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference, - for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other; - Companies can provide inputs on further enhancements of existing assistance data, including new information - Note: this does not mean that training and inference phases are mentioned in assistance data. Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable.

Proposal 4: For data collection for UE sided POS model training, at least DL PRS configuration, existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD need to be provided by LMF to UE.

Conclusion

According to the above discussion, the following observations and proposals are given:

Fallback

Proposal 1: The fallback should be handled via LPP reconfiguration under network control, instead of UE autonomously fallback according to pre-configuration.

RAN1 LS and progress impacts

Proposal 2: RAN2 postponed the discussion on case 3b until RAN3 makes progress on it.

Proposal 3: At least ground truth label and its related data (e.g. time stamp) should be provided by LMF via signaling ProvideAssistanceData.

Proposal 4: For data collection for UE sided POS model training, at least DL PRS configuration, existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD need to be provided by LMF to UE.

Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[2] R1-2501627 LS on AI/ML Posting case 3b, RAN1.