3GPP TSG-WG2 Meeting #129bis			R2-2502717

Wuhan, China, Apr. 7th-11th, 2025

Agenda item:	8.2.3

Title:	Further consideration on A-IoT random access

Source:	CMCC

Document for:	Discussion

Introduction

The last meeting [1] discusses some critical issues on random access for Ambient IoT and leaves the following agreements and FFS:

Agreements

- For Rel-19, only 3-step CBRA is supported for A-IoT

- We will specify both CBRA and CFRA.

- Re-use the subsequent paging message to trigger re-access. There is no need to differentiate msg1 resource for initial access vs re-access.

- NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback.

- RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

This contribution discusses further details on random access for A-IoT, especially on format and content for RACH messages.

Discussion

2.1 Unified RACH

Based on former discussion, A-IoT devices are characterized by ultra-low power consumption and complexity and often relying on ambient energy harvesting. In the most common single-device deployment, CFRA is prioritized as it is most efficient in time and energy.

A significant portion of A-IoT use cases involve single-device scenarios where the need for a random ID is minimal. In CFRA, the device typically uses AS ID assigned by reader or may be requested to generate a random ID when necessary. The transmission of a random ID in such scenarios would unnecessarily consume energy without providing tangible benefits, making this an inefficient solution for A-IoT deployments.

However, existing discussion implies that the unified RACH mandate device to include a random ID in both CBRA and CFRA, regardless of the fact that device in CFRA can be assigned to a unique AS ID by reader.

Observation 1: In CFRA, the unsolicited generation and transmission of the random ID incurs more energy consumption and overhead since the reader can assign unique AS ID to device.

Proposal 1a: Device should not send random ID in the first D2R message in CFRA.

Hence, it’s proposed that device should send its device in the first D2R message in CFRA to identify itself by default so that reader can determine whether it’s the correct device.

Proposal 1b: The first D2R message in CFRA should consist of the device ID to identify itself.

2.2 Re-access

As a baseline, the random ID in A-IoT Msg1 is for contention resolution, receiving A-IoT Msg2 containing the correct random ID is considered as access success to A-IoT device.

However, failures in A-IoT Msg1 are expected to be relatively common, particularly in scenarios where multiple A-IoT devices transmit Msg1 in the same access occasion, leading to collisions. In such cases, the reader can only initiate re-access for the affected devices. Another potential reason for A-IoT Msg1 failure is insufficient device energy. If a device lacks sufficient energy to send Msg1, it will inevitably fail in the RACH procedure anyway, making it unnecessary to specify procedure for energy-related Msg1 failures.

Proposal 2: A-IoT Msg1 failure can exclusively be addressed by re-access.

For A-IoT Msg3 failures, the situation differs as contention has already been resolved. In such cases, the device may not need to retransmit A-IoT Msg1, and the reader may explicitly indicate the device to retransmit A-IoT Msg3.

While re-access will mandate device to send A-IoT Msg1 again, potentially prolonging the whole RACH process. By contrast, a direct retransmission of A-IoT Msg3 can be a more efficient solution, avoiding the need for the device to resend Msg1. But it should not be ignored that re-access could be beneficial since it offers more opportunities for device recharging.

Observation 2a: Resending A-IoT Msg2 can inherently trigger a retransmission of A-IoT Msg3.

Observation 2b: Retransmitting A-IoT Msg3 avoids the need to resend the random ID and can expedite the RACH process.

Proposal 3a: A-IoT Msg3 failure can be addressed by re-access triggered by A-IoT paging message, or retransmission triggered by A-IoT Msg2.

Proposal 3b: A-IoT device should consider this round of access as failure when:

- not receiving Msg2 with the same random ID in A-IoT Msg1,

- receiving NACK message after A-IoT Msg3.

For CFRA, which shares similarities with A-IoT Msg3 failures in terms of contention resolution, re-access may not always be necessary. If a CFRA attempt fails, the reader has the flexibility to initiate a new CFRA procedure or indicate a data retransmission instead.

Proposal 4: Reader should not mandate device to re-access if CFRA attempt failures, i.e., reader can send new A-IoT paging message to initiate a new round of CFRA.

2.3 Multiplexing, segmentation and D2R grant

Multiplexing and segmentation has impact on the format and content design for A-IoT MAC PDU and A-IoT message. Hence, they need to be specified before the discussion on the details of message content and format.

Since A-IoT Msg2 multiplexing has already been agreed, and in FDMA, A-IoT Msg3 may be transmitted according to the radio resource to send A-IoT Msg1. Similarly, A-IoT Msg4, or NACK message should also support multiplexing for overhead reduction.

Proposal 5a: A-IoT Msg4 (NACK message) for multiple A-IoT devices can be multiplexed into one A-IoT MAC PDU, with one message header to indicate message type.

For A-IoT messages except A-IoT Msg2 and Msg4, the intention of the multiplexing for such messages are not clear and RAN2 haven’t evaluated this issue yet. For other R2D message, e.g., A-IoT paging and read/write command, multiplexing them may prolong the minimum time when device needs to monitor PRDCH. The benefits of multiplexing same type of message needs clarification.

Moreover, multiplexing different type of A-IoT messages can lead to a complicated format of MAC PDU and increase device implementation complexity.

Proposal 5b: As a baseline, at least different types of A-IoT message should not be multiplexed into one A-IoT MAC PDU.

For segmentation, if a device lacks sufficient energy to transmit the full Msg1, it should treat this as an initial access failure and wait for re-access, rather than attempting to segment Msg1. This avoids additional complexity and ensures a straightforward failure-handling mechanism.

Proposal 6a: A-IoT Msg1 should not be segmented.

However, considering that A-IoT Msg3 is kind of D2R data, segmentation of A-IoT Msg3 should also be supported. Likely, the segmentation of A-IoT Msg3 should not occur autonomously by device, i.e., segmentation must be executed only upon explicit reader indication.

If a device lacks sufficient energy to transmit the complete A-IoT Msg3, it may withhold transmission to conserve energy. This behaviour implicitly informs the reader of the device’s low-energy condition, prompting the reader to indicate that A-IoT Msg3 should be segmented. Moreover, permitting segmentation of A-IoT Msg3 is advantageous since devices with limited energy storage may be incapable of transmitting the entire message in one attempt.

Observation 3a: Devices with limited energy storage or low recharging power may be unable to transmit the complete A-IoT Msg3 in one D2R message, potentially resulting in a definitive access failure and re-access cannot help.

Observation 3b: Segmented D2R data transmission is already agreed.

Given that a device can reliably transmit A-IoT Msg1, it should also be capable of transmitting at least a 16-bit segmented version of A-IoT Msg3. Consequently, segmentation offers a viable solution for devices constrained by low charging power or limited energy storage.

Additionally, if the reader fails to decode A-IoT Msg3, it is beneficial for the reader to request retransmission by sending an A-IoT Msg2 with the same AS ID and a new grant for A-IoT Msg3, as re-access would otherwise necessitate reinitiating the contention resolution and AS ID assignment procedure.

Proposal 6b: A-IoT Msg3 can be segmented based on reader indication in A-IoT Msg2.

Considering the absence of PDCCH and only PDRCH is available, RAN1 intends to embed scheduling information within the PDRCH; consequently, it is advisable for A‑IoT Msg2 to carry scheduling-related details to support the subsequent (segmented) A‑IoT Msg3.

Agreement The payload size (i.e. TBS-like) for PDRCH transmission with variable size is explicitly indicated in the corresponding R2D control information.

Proposal 7: RAN2 can wait for RAN1’s progress on the detail of the D2R grant.

2.4 Message contents and formats

Message formats and content are essential in optimizing the RACH process, particularly in low-power, resource-constrained situation. A key consideration is the content and format of the messages, which need to balance clarity and efficiency without unnecessarily burdening A-IoT devices or the network.

Legacy LTE/NR mandates 8-bit alignment to simplify parsing but introduces padding. A-IoT MAC PDU’s small payloads magnify this inefficiency. For example, A-IoT Msg2 may contain a 16-bit random ID, if a message header is necessary, with 8-bit or byte alignment, the minimal length of A-IoT Msg2 will be 24-bit, as shown in the following.

Fig.1: An example of overhead comparison for byte- alignment and non-byte-alignment

Hence, byte alignment introduces additional padding and overhead, which can be inefficient for A-IoT.

Observation 4: A-IoT MAC PDUs are generally small in size (e.g., <1000 bits). If byte alignment is mandatory, a significant portion of the A-IoT MAC PDU could include padding bits, reducing transmission efficiency.

Proposal 8a: As the reader allocates radio resource to the A-IoT device, whether the transmission block is byte alignment are fully controlled by the reader.

Proposal 8b: Byte alignment is not mandatory for either A-IoT R2D or D2R message.

Unlike legacy MAC PDU, which primarily consists of MAC SDUs and MAC CEs, A-IoT MAC PDUs serve a more dedicated purpose, e.g., RACH signalling, read/write commands, and upper-layer data. Introducing a lightweight message header within the A-IoT MAC PDU can help indicate the message type while keeping the payload structure flexible. This design allows for efficient encapsulation of A-IoT messages while maintaining compatibility with different transmission needs.

As it’s not clear whether MAC SDU and MAC CE will be reused, payload is used for simplicity. RAN2 may revisit this issue if necessary. The L field of A-IoT message is absent as it can be inferred by scheduling related information.

Proposal 9a: A-IoT MAC PDU should consist of at most one A-IoT message header which indicates the message type and one or more optional A-IoT MAC subPDUs.

Proposal 9b: Each A-IoT MAC subPDU consists of at least the following:

- An L field to indicate its length and the payload.

- The payload only.

The following is an example of A-IoT MAC PDU.

Fig.2: An example of A-IoT MAC PDU

For now, the reader controls the RACH procedure and data transmission, meaning it has prior knowledge of what type of D2R data will be transmitted by the device and its size. To minimize complexity and improve energy efficiency, the device can omit the A-IoT message header in D2R messages.

Proposal 9c: As the reader may have expectation on what D2R message will be sent, the message header in D2R message can be omitted.

A noteworthy fact in A-IoT is that the occurrence probabilities of different R2D/D2R messages are highly uneven. For instance, A-IoT Msg1 and Msg2 are transmitted significantly more frequently than commands such as read/write command, paging, or NACK. Applying a fixed-length header to all messages would lead to inefficient resource utilization and unnecessary energy consumption.

To optimize efficiency, an entropy coding-like approach can be applied, where frequently occurring messages are assigned shorter headers, while less frequent messages use longer headers. This method, similar to Huffman coding, reduces overall transmission overhead and conserves device energy.

Observation 5a: Certain A-IoT messages, e.g., Msg1 and Msg2, occur more frequently than others, e.g., A-IoT paging or read/write/NACK messages.

Observation 5b: Using fixed-length message headers for messages with varying occurrence frequencies leads to inefficient resource and energy usage. Variable-length encoding, such as Huffman coding, is commonly used to address this issue.

Proposal 9d: The A-IoT message header should be variable-length, with shorter headers assigned to more frequently transmitted messages.

Additionally, timers should be generally avoided in A-IoT devices due to their complexity and power constraints. The time relationship in the RACH procedure is discussed at the RAN1 at this moment, making explicit timers unnecessary.

Proposal 10: To simplify device implementation, no explicit timer should be defined for the RACH procedure, device can reply on timing relationship defined by RAN1.

2.3.1 Msg1

A-IoT Msg1 is expected to be the most frequently transmitted message by the device. To maximize efficiency, it is beneficial to omit the message header in A-IoT Msg1, ensuring that it consists solely of a compact 16-bit random ID.

Proposal 11: A-IoT Msg1 should have a fixed length of 16 bits, containing only the 16-bit random ID, with omitted message header for efficiency.

The following is an example of A-IoT Msg1:

Fig.3: An example of A-IoT Msg1

2.3.2 Msg2 or ACK message

Considering the 1‑bit ACK message may lead to ambiguity because it provides minimal differentiation, it is recommended that A‑IoT Msg2 at least includes a concise message header and the 16‑bit random ID received, as both are critical to avoid misinterpretation among nearby devices.

With respect to the AS ID discussion, the AS ID can be allocated within A-IoT Msg2, suggesting that an optional AS ID should be included. In addition, since A-IoT Msg2 may incorporate multiple random IDs for multiplexing purposes, a new A-IoT Msg2 format for multiplexing is necessary.

For A-IoT Msg2 multiplexing in FDMA, the use of a bitmap indication associated with the Msg1 time-frequency domain is not preferred, as it could introduce additional ambiguity; in TDMA, employing multiplexing for A-IoT Msg2 may extend the duration during which the device must monitor R2D messages, and is therefore not recommended.

Based on these considerations, the components and formats for the various A-IoT Msg2 types can be systematically determined.

Proposal 12a: If A-IoT Msg3 is not segmented, A-IoT Msg2 should consist of a short message header, a random ID, an A-IoT Msg3 grant, and an optional AS ID.

The following is an example of A-IoT Msg2 without segment indication:

Fig.4: An example of A-IoT Msg2 for single and multiple unsegmented A-IoT Msg3.

If the reader decides to segment A-IoT Msg3, the format of A-IoT Msg2 will be correspondingly affected. For the first A-IoT Msg2 in the segmented A-IoT Msg3 scenario, the message should include the random ID and an optional AS ID if the random ID is not unique, followed by the subsequent A-IoT Msg2 containing segmentation indication.

Since this initial A-IoT Msg2 assigns the AS ID and signals to the device that the complete A-IoT Msg3 has not yet been transmitted, any subsequent A-IoT Msg2 should include the AS ID, the segmented A-IoT Msg3 grant, and the segmentation offset.

Proposal 12b: If A-IoT Msg3 is segmented, there should be two types of A-IoT Msg2:

- The first A-IoT Msg2 for AS ID, consisting of random ID, D2R grant for the first segment for A-IoT Msg3, and an optional AS ID, A-IoT device can be implicitly indicated to segment A-IoT Msg3 based on the D2R grant,

- The subsequent A-IoT Msg2, consisting of AS ID, grant for the next segment, and offset.

The following is an example of A-IoT Msg2 that indicates a segmented A-IoT Msg3:

Fig.5-1: An example of first and following A-IoT Msg2 for segmented A-IoT Msg3.

A-IoT Msg3 segmentation has no impact on A-IoT Msg2 with multiplexing format design, as there will be an L field and device is aware of whether to segment A-IoT Msg3 based on the D2R grant.

The following is an example of multiplexed A-IoT Msg2 that indicates segmented and unsegmented A-IoT Msg3:

Fig.5-2: An example of multiplexed A-IoT Msg2 that indicates segmented and unsegmented A-IoT Msg3.

The reader may efficiently reuse A-IoT Msg2 as the ACK message for read/write commands, eliminating the need for a separate ACK command and thus avoiding additional overhead.

In scenarios where Msg2 is reused as the ACK message for segmented data transmission, the existing A-IoT Msg3 grant can be retained for data segmentation transmission, and a segmentation indicator. e.g., an offset, should also be included; otherwise for unsegmented data transmission, they all can be omitted.

Proposal 13a: A-IoT Msg2 with AS ID can be reused as the ACK message for unsegmented A-IoT data transmission.

Proposal 13b: A-IoT Msg2 with AS ID, grant for segmentation and offset can be reused as ACK message for segmented A-IoT data transmission.

The following is an example of reusing A-IoT Msg2 as ACK message:

Fig.6: An example of reusing A-IoT Msg2 as ACK message

2.3.3 Msg3

In line with the design of A-IoT Msg1, A-IoT Msg3 is intended to carry only the device ID, as the reader managing the RACH procedure is already aware of the transmission timing and necessity of Msg3. Therefore, an explicit message header for A-IoT Msg3 is deemed redundant.

Proposal 14a: If the A-IoT Msg3 is not segmented, it should consist of the device ID only.

The following is an example of A-IoT Msg3 without segmentation:

Fig.7: An example of A-IoT Msg3 without segmentation

Device should send a segmented A-IoT Msg3 if reader indicates in A-IoT Msg2. As reader doesn’t have the device ID yet, the segmented A-IoT Msg3 should contain the AS ID assigned by reader to identify itself.

Proposal 14b: If the A-IoT Msg3 is segmented, it should consist of the AS ID and the segmented device ID.

The following is an example of A-IoT Msg3 with segmentation:

Fig.8: An example of A-IoT Msg3 with segmentation

2.3.4 Msg4 or NACK message

The A-IoT Msg4, or NACK message, should consist of a concise message header and the AS ID to address the device. If device receives A-IoT with the correct AS ID, device should consider the current access attempt as failed and wait for re-access.

Proposal 15a: A-IoT Msg4 should consist of a message header and the AS ID, which is the end mark of the access attempt.

The following is an example of A-IoT Msg4:

Fig.9: An example of A-IoT Msg4.

For A-IoT Msg3, as device considers its random access as success after receiving A-IoT Msg2, it would still consider random access is successful even if A-IoT Msg3 fails in transmission or retransmission. If reader doesn’t send NACK message, device would assume that random access is successful and skip re-access procedure.

Observation 6: A-IoT device regards random access attempt as success after A-IoT Msg2 even with A-IoT Msg3 failure.

Therefore, an explicit NACK message is necessary in case reader needs device to re-access to address A-IoT Msg3 failure.

Proposal 15b: Reader is expected to send an explicit NACK message if reader intends to address A-IoT Msg3 failure by re-access.

Furthermore, A-IoT Msg4 may be reused as a NACK command for data transmission, which signals both failure and termination of the data transmission. There is no obvious benefit to include grant in A-IoT Msg4 for retransmission or data transmission failure handing as A-IoT Msg2 or ACK message is already capable of this purpose, i.e., it can be handled by (re)sending A-IoT Msg2.

Proposal 15c: A-IoT Msg4 can be reused as the NACK message in A-IoT data transmission, as the failure indication and end mark of the data transmission.

Conclusion

This contribution offers the following observations and proposals:

Unified RACH:

Observation 1: In CFRA, the unsolicited generation and transmission of the random ID incurs more energy consumption and overhead since the reader can assign unique AS ID to device.

Proposal 1a: Device should not send random ID in the first D2R message in CFRA.

Proposal 1b: The first D2R message in CFRA should consist of the device ID to identify itself.

Re-access:

Proposal 2: A-IoT Msg1 failure can exclusively be addressed by re-access.

Observation 2a: Resending A-IoT Msg2 can inherently trigger a retransmission of A-IoT Msg3.

Observation 2b: Retransmitting A-IoT Msg3 avoids the need to resend the random ID and can expedite the RACH process.

Proposal 3a: A-IoT Msg3 failure can be addressed by re-access triggered by A-IoT paging message, or retransmission triggered by A-IoT Msg2.

Proposal 3b: A-IoT device should consider this round of access as failure when:

- not receiving Msg2 with the same random ID in A-IoT Msg1,

- receiving NACK message after A-IoT Msg3.

Proposal 4: Reader should not mandate device to re-access if CFRA attempt failures, i.e., reader can send new A-IoT paging message to initiate a new round of CFRA.

Multiplexing:

Proposal 5a: A-IoT Msg4 (NACK message) for multiple A-IoT devices can be multiplexed into one A-IoT MAC PDU, with one message header to indicate message type.

Proposal 5b: As a baseline, at least different types of A-IoT message should not be multiplexed into one A-IoT MAC PDU.

Segmentation for D2R RACH message:

Proposal 6a: A-IoT Msg1 should not be segmented.

Observation 3a: Devices with limited energy storage or low recharging power may be unable to transmit the complete A-IoT Msg3 in one D2R message, potentially resulting in a definitive access failure and re-access cannot help.

Observation 3b: Segmented D2R data transmission is already agreed.

Proposal 6b: A-IoT Msg3 can be segmented based on reader indication in A-IoT Msg2.

D2R grant

Proposal 7: RAN2 can wait for RAN1’s progress on the detail of the D2R grant.

Byte alignment for A-IoT message:

Observation 4: A-IoT MAC PDUs are generally small in size (e.g., <1000 bits). If byte alignment is mandatory, a significant portion of the A-IoT MAC PDU could include padding bits, reducing transmission efficiency.

Proposal 8a: As the reader allocates radio resource to the A-IoT device, whether the transmission block is byte alignment are fully controlled by the reader.

Proposal 8b: Byte alignment is not mandatory for either A-IoT R2D or D2R message.

A-IoT MAC PDU format:

Proposal 9a: A-IoT MAC PDU should consist of at most one A-IoT message header which indicates the message type and one or more optional A-IoT MAC subPDUs.

Proposal 9b: Each A-IoT MAC subPDU consists of at least the following:

- An L field to indicate its length and the payload.

- The payload only.

A-IoT message header:

Proposal 9c: As the reader may have expectation on what D2R message will be sent, the message header in D2R message can be omitted.

Observation 5a: Certain A-IoT messages, e.g., Msg1 and Msg2, occur more frequently than others, e.g., A-IoT paging or read/write/NACK messages.

Observation 5b: Using fixed-length message headers for messages with varying occurrence frequencies leads to inefficient resource and energy usage. Variable-length encoding, such as Huffman coding, is commonly used to address this issue.

Proposal 9d: The A-IoT message header should be variable-length, with shorter headers assigned to more frequently transmitted messages.

Timing:

Proposal 10: To simplify device implementation, no explicit timer should be defined for the RACH procedure, device can reply on timing relationship defined by RAN1.

A-IoT Msg1:

Proposal 11: A-IoT Msg1 should have a fixed length of 16 bits, containing only the 16-bit random ID, with omitted message header for efficiency.

A-IoT Msg2:

Proposal 12a: If A-IoT Msg3 is not segmented, A-IoT Msg2 should consist of a short message header, a random ID, an A-IoT Msg3 grant, and an optional AS ID.

Proposal 12b: If A-IoT Msg3 is segmented, there should be two types of A-IoT Msg2:

- The first A-IoT Msg2 for AS ID, consisting of random ID, D2R grant for the first segment for A-IoT Msg3, and an optional AS ID, A-IoT device can be implicitly indicated to segment A-IoT Msg3 based on the D2R grant,

- The subsequent A-IoT Msg2, consisting of AS ID, grant for the next segment, and offset.

Proposal 13a: A-IoT Msg2 with AS ID can be reused as the ACK message for unsegmented A-IoT data transmission.

Proposal 13b: A-IoT Msg2 with AS ID, grant for segmentation and offset can be reused as ACK message for segmented A-IoT data transmission.

A-IoT Msg3:

Proposal 14a: If the A-IoT Msg3 is not segmented, it should consist of the device ID only.

Proposal 14b: If the A-IoT Msg3 is segmented, it should consist of the AS ID and the segmented device ID.

A-IoT Msg4:

Proposal 15a: A-IoT Msg4 should consist of a message header and the AS ID, which is the end mark of the access attempt.

Observation 6: A-IoT device regards random access attempt as success after A-IoT Msg2 even with A-IoT Msg3 failure.

Proposal 15b: Reader is expected to send an explicit NACK message if reader intends to address A-IoT Msg3 failure by re-access.

Proposal 15c: A-IoT Msg4 can be reused as the NACK message in A-IoT data transmission, as the failure indication and end mark of the data transmission.

References

[1]	R2_129_ChairNotes_25-02-21_final