# 3GPP TSG-RAN WG2 Meeting #129bis	                                                        R2-2502724

# Wuhan, China, Apr. 7th – 11th, 2025

# Agenda item:	8.5.2

# Source:	CMCC

# Title:	Discussion on on-demand SSB for SCell

# Document for:	Discussion, Decision

- Introduction

In RAN2#129 meeting, RAN2 reached the following agreements on on-demand SSB for SCell [1]:

Agreements on OD-SSB SCell

- RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

In this paper, we further discuss open issues related to OD-SSB transmission configuration/indication and measurements.

- Discussion

  - OD-SSB MAC CE design

In last meeting, RAN2 discussed on OD-SSB MAC CE design, but there’s no consensus on this issue:

R2-2500256 Discussion on on-demand SSB for Scell Apple discussion Rel-19 Netw_Energy_NR_enh-Core Proposal 1: The OD-SSB MAC-CE has a fixed size and includes a bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE). Proposal 2: The OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells. Proposal 3: Considering forward compatibility, configuration indices of candidate SSB periodicities in RRC (rather than actual values of SSB periodicities) are included in the OD-SSB MAC-CE. [Xiaomi]: Understand RAN1 already agreed both implicit deactivation and explicit deactivation. If we signal both of them with the single bit-map, it may bring the additional complexities. [Ericsson]: In addition to periodicity, RAN1 also agreed additional ones (something like reference point). It may not be easy to handle them with a fixed MAC CE. [Samsung]: Understand it is almost impossible to have a fixed MAC CE since some SCell activates OD-SSB and some other SCell deactivates OD-SSB, and different information is applied to activation and deactivation. Revisit the issues that raised in P1-P3 next meeting.

The concern on MAC CE design is mainly due to RAN1’s agreements to support implicit and explicit activation/deactivation of OD-SSB and potential parameters like periodicity included in the new MAC CE. Following are the related RAN1 agreements:

RAN1#118bis meeting Agreement[2] Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB FFS: Any other relevant parameters Agreements For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios FFS: Details related to each of the above options

RAN2#119 meeting agreements [3]: For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

Observation 1: Based on RAN1’s agreements, periodicity of the on-demand SSB can be indicated in OD-SSB transmission indication MAC CE, and on-demand SSB deactivation can be indicated by explicit indication  or the configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB.

The intention of RAN1 is to provide more configuration flexibility, but from RAN2 perspective, including more parameters in OD-SSB transmission indication MAC CE may enlarge the size of the MAC CE. Since only periodicity is agreed to be indicated in MAC CE by RAN1, we think RAN2 can just take periodicity into consideration for the new MAC CE design, and send LS to RAN1 for further confirm, as the work item time is limited. And considering the complexity of MAC CE design, it is preferred to use configuration index included in the MAC CE to simplify the new MAC CE.

Proposal 1: Periodicity of OD-SSB indication is consider in the OD-SSB MAC CE design, except for activation/deactivation indication. And RAN2 sends LS to RAN1 for further confirm.

Proposal 2: If multiple-configuration of OD-SSB transmission is supported, i.e., multiple periodicity values of OD-SSB, the index of applicated configuration can be indicated in OD-SSB transmission indication MAC CE.

What’s more, according to TS 38.321 [x], the legacy SCell Activation MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells. Therefore, the new OD-SSB MAC-CE should also support the two formats.

Proposal 3: Two formats of OD-SSB MAC CE are supported: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

  - OD-SSB configuration

Besides, RAN1 also discussed OD-SSB related configuration and reached consensus as following:

RAN1#117 meeting Agreement[4] - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused

RAN1#118bis meeting Agreement[2] For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1. - Sub-carrier spacing of the on-demand SSB - FFS if this can be absent - Physical Cell ID of the on-demand SSB - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index - Downlink transmit power of on-demand SSB - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters

RAN1#120bis Agreement [5] - For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC, - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e., - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) - By defining reference point as SFN where AO-SSB burst exists Agreement - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2. - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) - FFS: Whether this parameter can be absent for Case #2 - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) - FFS: Whether this parameter can be absent for Case #2 - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity) - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing) - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB. - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId) - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB. - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex) - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower) - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB. - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx) - FFS: Which of the above parameters are a list

Based on RAN1 progress, the following parameters can be configured in RRC signalling:

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - Sub-carrier spacing of the on-demand SSB

  - Time location of on-demand SSB burst

  - Physical Cell ID of the on-demand SSB

  - Downlink transmit power of on-demand SSB

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

Proposal 4: The following parameters can be configured in RRC signalling:

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - Sub-carrier spacing of the on-demand SSB

  - Time location of on-demand SSB burst

  - Physical Cell ID of the on-demand SSB

  - Downlink transmit power of on-demand SSB

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

As mentioned by RAN1, a high layer parameter should be introduced for OD-SSB configuration, and RAN2 should discuss the relationship of the high layer parameter for OD-SSB configuration and SCell configuration IE.

Proposal 5: Introduce a high layer parameter for OD-SSB configuration, and RAN2 discuss the relationship of the new IE and SCell configuration IE.

  - Measurement considering OD-SSB

In last RAN2 meeting, RAN2 reached the consensus that how to measurement OD-SSB and AO-SSB depends on RAN4 decision:

Agreements on OD-SSB 1. RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

And following are the related RAN4 agreements for OD-SSB measurement in Case#1 and Case#2 [6].

Case#1 Agreement: - The time window is defined with Tuncertain plus actual measurement duration. - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows. - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time. - Treport is the first OD-SSB based L3 measurement reporting - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow. - A detected cell: OD-SSB based measurement period - A newly detectable cell: OD-SSB based cell identification period No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify). Agreement: Case#2 Agreement: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), - Option 1: The same approach in Case 1 can be reused. - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell). - Option 2: - If the serving cell is unknown, the same approach in Case 1 can be reused. - If the serving cell is known, UE follows legacy requirements based on measCycleSCell. - The known condition reuses the principle of legacy known condition. - This scenario applies at least when UE sends the meas. reporting. - FFS the case if the L3 meas. report cannot be triggered. - The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

Based RAN4 agreements, no matter in Case#1 or Case#2, the same measurement mechanism is used, that is UE will measure OD-SSB based on SSB pattern within the fast measurement window, and measured based on measCycleSCell as legacy if there’s OD-SSB or AO-SSB.

Observation 2: No matter in Case#1 or Case#2, the same measurement mechanism is used, that is UE will measure OD-SSB based on SSB pattern within the fast measurement window, and measured based on measCycleSCell as legacy if there’s OD-SSB or AO-SSB.

It should be noticed that in Case#2, there are several sub-cases considering whether OD-SSB and AO-SSB are overlapped on time/frequency domain:

RAN1#118bis meeting Agreement [2] For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

What’s more, RAN1 also agreed that in case OD-SSB and AO-SSB are on different frequency location, there’s no need to measure them both:

RAN1#120 meeting Agreements [5] Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

In case AO-SSB and OD-SSB are on the same frequency location,  and there’s overlapping in time domain,  AO-SSB can be measured in the fast measurement window defined by RAN4. And it should be noticed that measurement of SCell SSB can be configured as servingCellMo, and in legacy, only one servingCellMo is configured for a specific SCell [7].

ServingCellConfig information element

-- ASN1START

-- TAG-SERVINGCELLCONFIG-START

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated                                                OPTIONAL,   -- Cond TDD

initialDownlinkBWP                  BWP-DownlinkDedicated                                                    OPTIONAL,   -- Need M

downlinkBWP-ToReleaseList           SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                               OPTIONAL,   -- Need N

downlinkBWP-ToAddModList            SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink                         OPTIONAL,   -- Need N

firstActiveDownlinkBWP-Id           BWP-Id                                                                   OPTIONAL,   -- Cond SyncAndCellAdd

bwp-InactivityTimer                 ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30,

ms40,ms50, ms60, ms80,ms100, ms200,ms300, ms500,

ms750, ms1280, ms1920, ms2560, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1 }    OPTIONAL,   --Need R

defaultDownlinkBWP-Id               BWP-Id                                                                  OPTIONAL,   -- Need S

uplinkConfig                        UplinkConfig                                                            OPTIONAL,   -- Need M

supplementaryUplink                 UplinkConfig                                                            OPTIONAL,   -- Need M

pdcch-ServingCellConfig             SetupRelease { PDCCH-ServingCellConfig }                                OPTIONAL,   -- Need M

pdsch-ServingCellConfig             SetupRelease { PDSCH-ServingCellConfig }                                OPTIONAL,   -- Need M

csi-MeasConfig                      SetupRelease { CSI-MeasConfig }                                         OPTIONAL,   -- Need M

sCellDeactivationTimer              ENUMERATED {ms20, ms40, ms80, ms160, ms200, ms240,

ms320, ms400, ms480, ms520, ms640, ms720,

ms840, ms1280, spare2,spare1}       OPTIONAL,   -- Cond ServingCellWithoutPUCCH

crossCarrierSchedulingConfig        CrossCarrierSchedulingConfig                                            OPTIONAL,   -- Need M

tag-Id                              TAG-Id,

dummy1                              ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

pathlossReferenceLinking            ENUMERATED {spCell, sCell}                                              OPTIONAL,   -- Cond SCellOnly

servingCellMO                       MeasObjectId                                                            OPTIONAL,   -- Cond MeasObject

...,

[[

lte-CRS-ToMatchAround               SetupRelease { RateMatchPatternLTE-CRS }                                OPTIONAL,   -- Need M

rateMatchPatternToAddModList        SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern       OPTIONAL,   -- Need N

rateMatchPatternToReleaseList       SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId     OPTIONAL,   -- Need N

downlinkChannelBW-PerSCS-List       SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier                     OPTIONAL    -- Need S

]],

[[

supplementaryUplinkRelease-r16      ENUMERATED {true}                                                       OPTIONAL,   -- Need N

tdd-UL-DL-ConfigurationDedicated-IAB-MT-r16    TDD-UL-DL-ConfigDedicated-IAB-MT-r16                         OPTIONAL,   -- Cond TDD_IAB

dormantBWP-Config-r16               SetupRelease { DormantBWP-Config-r16 }                                  OPTIONAL,   -- Need M

ca-SlotOffset-r16                   CHOICE {

refSCS15kHz                         INTEGER (-2..2),

refSCS30KHz                         INTEGER (-5..5),

refSCS60KHz                         INTEGER (-10..10),

refSCS120KHz                        INTEGER (-20..20)

}

……

For Case#1, there’s no AO-SSB before OD-SSB, therefore the servingCellMo should be configured based on OD-SSB, but it is not needed to be measured until OD-SSB is activated.

For Case#2, based on RAN1 agreements, OD-SSB is a not a CD-SSB, while for AO-SSB, it can be a CD-SSB or NCD-SSB. If AO-SSB is an NCD-SSB, the frequency of AO-SSB and OD-SSB  can be the same or different, but if AO-SSB is a CD-SSB, the frequency of AO-SSB and OD-SSB is definitely different.

If AO-SSB and OD-SSB are on different frequency location, as mentioned previous, UE is not required to measure both AO-SSB and OD-SSB, in other words, the servingCellMo may be changed from AO-SSB to OD-SSB once OD-SSB is activated, one way to achieve this, is to perform a UE RRC reconfiguration to change the servingCellMo from AO-SSB to OD-SSB, and the other way to configure both two SSB in the servingCellMo, and similar to Casse, OD-SSB related measurement is only performed when OD-SSB is activated. When it comes to the case that AO-SSB and OD-SSB are on the same frequency location, no matter there’s  overlapping in timed domain or not, since the frequency is the same, the servingCellMo can be configured as AO-SSB or OD-SSB, while UE performs measurement based on RAN4’s design as described in Observation 2.

Proposal 4: For the measurement of OD-SSB:

- For Case#1, servingCellMo is configured based on OD-SSB, and it is not measured until OD-SSB is activated;

- For  Case#2

- if AO-SSB and OD-SSB are on the same frequency, the network can configure servingCellMo as AO-SSB or OD-SSB;

- if AO-SSB and OD-SSB are on different frequencies, RAN2 discuss the following two ways to configure servingCellMo:

        - Solution 1: RRCReconfiguration procedure is initiated before OD-SSB activation, to change servingCellMo to OD-SSB;

        - Solution 2 (preferred): servingCellMo IE is enhanced to included more than one measurement IDs, i.e., one ID is associated with AO-SSB, and the other is associated with OD-SSB, and OD-SSB related measurement ID is not valid until OD-SSB is activated.

- Conclusion

In this contribution, we discuss configuration/indication of OD-SSB transmission and measurement related issues. Following are the proposals and observations made in this contribution:

Observations:

Observation 1: Based on RAN1’s agreements, periodicity of the on-demand SSB can be indicated in OD-SSB transmission indication MAC CE, and on-demand SSB deactivation can be indicated by explicit indication  or the configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB.

Observation 2: No matter in Case#1 or Case#2, the same measurement mechanism is used, that is UE will measure OD-SSB based on SSB pattern within the fast measurement window, and measured based on measCycleSCell as legacy if there’s OD-SSB or AO-SSB.

Proposals:

Proposal 1: Periodicity of OD-SSB indication is consider in the OD-SSB MAC CE design, except for activation/deactivation indication. And RAN2 sends LS to RAN1 for further confirm.

Proposal 2: If multiple-configuration of OD-SSB transmission is supported, i.e., multiple periodicity values of OD-SSB, the index of applicated configuration can be indicated in OD-SSB transmission indication MAC CE.

Proposal 3: Two formats of OD-SSB MAC CE are supported: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

Proposal 4: The following parameters can be configured in RRC signalling:

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - Sub-carrier spacing of the on-demand SSB

  - Time location of on-demand SSB burst

  - Physical Cell ID of the on-demand SSB

  - Downlink transmit power of on-demand SSB

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

Proposal 5: Introduce a high layer parameter for OD-SSB configuration, and RAN2 discuss the relationship of the new IE and SCell configuration IE.

Proposal 4: For the measurement of OD-SSB:

- For Case#1, servingCellMo is configured based on OD-SSB, and it is not measured until OD-SSB is activated;

- For  Case#2

- if AO-SSB and OD-SSB are on the same frequency, the network can configure servingCellMo as AO-SSB or OD-SSB;

- if AO-SSB and OD-SSB are on different frequencies, RAN2 discuss the following two ways to configure servingCellMo:

        - Solution 1: RRCReconfiguration procedure is initiated before OD-SSB activation, to change servingcellmo to OD-SSB;

        - Solution 2 (preferred): servingCellMo IE is enhanced to included more than one measurement IDs, i.e., one ID is associated with AO-SSB, and the other is associated with OD-SSB, and OD-SSB related measurement ID is not valid until OD-SSB is activated.

- References

- Report of 3GPP TSG RAN WG2 meeting #129;

- Report of 3GPP TSG RAN WG1 meeting #118bis;

- Report of 3GPP TSG RAN WG1 meeting #119;

- Report of 3GPP TSG RAN WG1 meeting #117;

- Report of 3GPP TSG RAN WG1 meeting #120;

- R4-2502598 WF on RRM requirements for Netw_Energy_NR_enh_Part1;

- TS 38.331.