# 3GPP TSG-RAN WG2 Meeting #129bis	                                                        R2-2502727

# Wuhan, China, Apr. 7th – 11th, 2025

# Agenda item:	8.6.2

# Source:	CMCC

# Title:	Discussion on inter-CU LTM

# Document for:	Discussion, Decision

- Introduction

In RAN2#129 meeting, RAN2 reached the following agreements on inter-CU LTM [1]:

Agreements on RRC issues:

- For inter-CU SCG LTM, introducing the SK counter list in LTM-ConfigSCG-r19, each list associated with one ltm-NoSecurityChangeID.

- Reuse the selectedSK-Counter-r18 to indicate the selected SK counter upon the execution of inter-CU SCG LTM.

- The reference configuration related to inter-SN LTM is included within the new ltm-ConfigSCG field.

- UE does not combine reference configurations from ltm-Config and ltm-ConfigSCG.

- FFS whether only one indication is needed to indicate LTM configuration as complete configuration.

- The existing field mrdc-ReleaseAndAdd is always included when SCG configuration is included during an MCG LTM execution. Can be revisited if there is any issue.

- The release of the LTM configuration (MCG or SCG) follows the Rel-18 principles.

- A maximum number of 8 candidate configurations (LTM plus CLTM candidate configurations) can be configured.

- The LTM-Candidate IE is re-used to model an CLTM candidate cell configuration.

- Re-use existing L1 events for CLTM.

- For field ltm3-Offset-r19, the granularity is 0.5 dB.

- Add extension mark “…” to LTM-ConfigSCG-r19, adopt the principle that fields only applicable to inter-CU SCG change should be defined in the root of LTM-ConfigSCG-r19, not LTM-Config-r18.

Agreements on inter-CU LTM:

- Using the term “LTM” to represent “intra-CU LTM” and “inter-CU LTM” in MAC specification, and introduce separate term for “conditional intra-CU LTM” in the specification(s).

- Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES.

- Same as Rel-18 intra-CU LTM, CFRA resource configuration could be included in LTM cell switch command MAC CE for Rel-19 inter-CU LTM.

- For inter-CU MCG LTM, when Rel-19 ID of candidate cell is different from serving cell, the UE performs PDCP re-establishment for SRB1/SRB2.

- For inter-CU MCG LTM, when the Rel-19 ID of candidate cell is the same with serving cell, the UE performs PDCP SDU discard for SRB1/SRB2.

- How to support shared RACH resources for inter-CU LTM is left to RAN3.

In this paper, we further discuss the open issues of inter-CU LTM.

- Discussion

  - Issues related to SA3’s LS

SA3 replies RAN2’s LS on security handling for inter-CU LTM in non-DC case in [2]:

1. Overall Description: SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases. Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover. 2. Actions: To RAN2 and RAN3 ACTION: SA3 kindly asks RAN2 and RAN3 to take the above information into account. To RAN3 ACTION: SA3 kindly asks RAN3 to decide and update the SA3 on the key distribution mechanism.

Based on SA3’s reply, the NCC can be transmitted to the UE in plaintext via MAC CE message, one issue should be discussed in RAN2 is whether the NCC  valued is included in LTM cell switch command MAC CE or a new MAC CE. In legacy handover, NCC value is indicated to the UE with handover command in RRCReconfiguration, therefore, the similar way can be used that NCC value is included in LTM cell switch command. What’s more, in this way, it can avoid extra Uu signalling.

Proposal 1: NCC value is include in LTM cell switch command, but not in a new MAC CE only for NCC value delivery.

Based on [3], NCC value transmitted from CN to RAN is an integer, from 0 to 7, therefore,  at least 3 bits are needed for NCC value indication:

#### 9.3.1.88 Security Context This IE provides security related parameters to the NG-RAN node which are used to derive security keys for user plane traffic and RRC signalling messages and for security parameter generation for subsequent mobility, see TS 33.501 [13]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Next Hop Chaining Count; M; INTEGER (0..7); Next Hop Chaining Counter (NCC) defined in TS 33.501 [13].. Next-Hop NH; M; Security Key 9.3.1.87; The NH together with the NCC is used to derive the security configuration as defined in TS 33.501 [13]..

Following Figure 1is a potential way to enhance LTM cell switch command for further discussion:

Figure 1 LTM Cell Switch Command with NCC value

Proposal 2: Taking Figure 1 as baseline for further discussion on the MAC CE to indicate NCC value.

The other related issue is whether to support fast recovery for inter-CU LTM. In the previous meeting discussion, RAN2 reached the following agreements:

Agreements on inter-CU LTM in RAN2#127bis

- The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

If fast recovery is supported for inter-CU LTM candidate cells, the UE may selects an inter-CU LTM candidate cell after LTM failure. However, since NCC value is indicated in MAC CE as agreed in SA3, it’s impossible for the UE to acquire NCC value. Therefore, fast recovery cannot be supported for inter-CU LTM. If the UE selects an inter-CU LTM candidate cell after LTM failure, the UE should follow the legacy RRC re-establishment procedure.

Proposal 3: Fast recovery is not supported for inter-CU LTM candidate cells. If the UE selects an inter-CU LTM candidate cell after LTM failure, the UE should follow the legacy RRC re-establishment procedure.

  - Coexistence of inter-CU LTM and other features

In last RAN2 meeting, RAN2 discussed the coexistence of inter-CU LTM with other features:

2. Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES.

In the following section, we discussed the coexistence of LTM with MIMO 2 TA/NES.

    - Coexistence with MIMO 2TA

in Rel-18, coexistence of intra-CU LT with MIMO 2TA is supported for PDCCH-order based TA measurement, but not for UE-based TA measurement.

RAN2#125 meeting agreements [4]: R2-2401924 Summary of [AT125][509][feMob] MAC offline Huawei, HiSilicon LTM with MIMO two TA For LTM with MIMO two TA, - - Use post-email discussion to discuss the TP with outcome of endurable TP for next meeting, aiming to reuse the MIMO design as much as possible; - - To use option 2, not signal additional info but use the mapping from TCI state to TAG ID to understand the applicable TAG, in the TP. RAN2#125bis meeting agreements [5]: R2-2402236 LTM and MIMO 2TA MediaTek Inc. discussion NR_Mob_enh2-Core P1: Add the field tag-Id-ptr in CandidateTCI-state and CandidateTCI-UL-State. RAN2#126 meeting agreements [6]: R2-2404300 LTM with 2TA co-existance MediaTek inc. discussion Rel-18 NR_Mob_enh2-Core - HW think all info so already there in the outer config. - ZTE think indeed if all TCI states and all SSBs are included then this is possible. - Apple think we can skip this CATT agrees UE-based TA measurement should not be configured if candidate cell is configured with Rel-18 MIMO 2TA

For PDCCH-order based TA measurement, the UE can identify the TA group of the accessed TRP according to the indicated TCI state in LTM cell switch command. For inter-CU LTM, currently only PDCCH order based TA measurement is agreed, and R18 LTM cell switch command is used as baseline, therefore, we think at least MIMO 2TA can be configured with LTM using PDCCH-order based TA measurement.

Proposal 4: MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

What’s more, whether UE-based TA measurement is supported for Rel-19 inter-CU LTM, should be discussed in RAN2. In Rel-18 intra-CU LTM, except for L2 handing, the UE’s execution of UE-based TA measurement is also based on the comparison of related IDs of the serving cell and candidate cell. In the current spec, it’s UE implementation on how to measure TA when this way is configured by the network, and there’s no restrictions on the scenarios  in which UE can be configured with UE-based TA measurement. In other words, whether to configure UE-based TA measurement to the UE is a kind of network implementation, maybe it is decided based on the prior information in the serving cell.

Observation 1: In Rel-18 intra-CU LTM, whether to configure UE-based TA measurement is up to network implementation.

When it comes to Rel-19 inter-CU LTM, we think the same principle can be applied, the serving cell can configure UE-based TA measurement to the UE. If UE cannot access to the target cell successfully, it can fallback to legacy RACH procedure.

Proposal 5: UE-based  TA  measurement can be supported in Rel-19 inter-CU LTM.

Proposal 6: If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA

    - Coexistence with NES

Many NES techniques are specified in Rel-18 and Rel-19.Some NES techniques have impact on RRC_IDLE/INACTIVE UE, while some have impact on RRC_CONNECTED UE. Considering that LTM is used for RRC_CONNECTED UE, therefore, in some sense, inter-CU LTM can coexists with NES techniques having impact on RRC_IDLE UEs, and we list the and following NES techniques for RRC_CONNECTED UEs for further discuss.

- Cell DTX/DRX

- Enhanced CHO for NES

- OD-SSB for SCell

- SSB adaptation (for SCell)

The coexistence of LTM with Cell DTX was discussed in Rel-18, and nothing is optimized for the coexistence case:

Agreements in RAN2#126 meeting [6]

Understanding is that an LTM target is not a serving cell and hence it will not apply CELL DTX/DRX until the switch is completed. Nothing is optimized or specified for RACHless

When the UE is configured with Cell DTX/DRX, the UE waits for to receive PDCCHs or SPS occasions, and transmit SR or CG. In this duration, the gNB transmission/reception of PDCCH, SPS, SR, CG, periodic and semi-persistent CSI report are not impacted for the purpose of network energy saving during the active duration of cell DTX/DRX, and there’s no impact on UE’s L1/L3 measurement. Therefore, LTM can be configured with Cell DTX/DRX simultaneously.

As to the enhanced CHO for NES, once configured, the UE may use NES-specific CHO event for executing CHO to a candidate cell when it is indicated by the network via DCI. Intrinsically it is a kind of CHO, and RAN2 agreed that CHO can be configured with LTM:

Agreements on LTM and L3 HO in RAN2#127 [7]:

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

Therefore, the enhanced CHO for NES can be configured with inter-CU LTM for a specific UE.

For the SSB related enhancements, including OD-SSB and SSB adaption, they are only applied to SCells, which may have effect on the measurement of SCells. However, LTM is used for cell switch, which is mainly related to PCell measurement. Therefore, it seems the coexistence of LTM and OD-SSB and SSB adaptation can be supported.

Based on the discussion above, we think the coexistence of NES and inter-CU LTM can be supported.

Proposal 7: The coexistence of NES and inter-CU LTM can be supported.

  - Inter-CU SCG LTM open issues

Inter-CU SCG LTM initiation

A new IE is introduced in the Running CR [8] to indicate whether inter-CU SCG LTM is allowed by MN for further discussion.

[[

configureInterSN-LTM-r19         ENUMERATED{allowed}                                              OPTIONAL

]]

Editor’s Note: FFS whether the indication whether the SN is allowed to configure an inter-CU LTM configuration is provided in the Xn signalling.

In our opinion, there’s no need to introduce extra IE. For inter-CU SCG LTM, RAN2 only agreed the SN initiation case, and the procedure design is based on SN change procedure specified in TS 37.340, in which SN initiates the procedure by send SN change required message to MN, and MN may confirm or refuse SN’s request and indicate proper cause value if MN refuses. Therefore, we think SN change request procedure can be reused, if SN is not allowed to configure an inter-CU LTM configuration, the MN may refuse SN’s request.

Proposal 8: There’s no need to introduce extra IE to indicate whether the SN is allowed to configure an inter-CU LTM configuration is provided in the Xn signalling, while the current SN change refuse procedure can be reused to not allowed inter-CU SCG LTM configuration.

Inter-CU SCG LTM execution

In Rel-18, only intra-CU LTM is supported, and UE include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate in RRCReconfigurationComplete message and sends the message to the node providing LTM configuration, i.e., MN or SN only. When it comes to inter-CU SCG LTM, one open issue is if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message. Considering that in inter-CU SCG LTM, it’s MN to provide inter-CU LTM configuration including the LTM-CandidateId, therefore, we think appliedLTM-CandidateId is only needed to be included in MCG RRCReconfigurationComplete message. And the MN informs the target SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message as captured in TS 37.340 Running CR.

Observation 2: The content of appliedLTM-CandidateId is LTM-CandidateId, which is configured by MN in inter-CU SCG LTM.

Proposal 9: appliedLTM-CandidateId is only included in MCG RRCReconfigurationComplete message.

Inter-CU SCG LTM configuration modification

In current CPAC procedure, the source SN may send the SN Modification Required message to trigger an update of CPC execution condition and/or corresponding SCG measurement configuration for CPC. And MN reconfigures the UE after SN modification. Similar procedure should be supported for inter-CU SCG LTM, since the configuration is provided to UE is generated by MN, and the medication can be used for LTM configuration updating and candidate PsCell change.

Proposal 10: SN modification required by Candidate SN should be supported for LTM configuration updating and candidate PsCell change.

Except for the changes mentioned above, LTM candidate cell release is another kind of LTM configuration modification. For this case, it should be decided by MN, and whether SN can request LTM candidate cell release can be decided by RAN3.

Proposal 11: It’s MN to decide whether release LTM candidate cell(s), but whether SN can request LTM candidate cell release can be decided by RAN3.

Coexistence with L3 SN handling

It was agreed in RAN2#127bis meeting that L3 mobility can be configured to the UE with inter-CU LTM configured. Following are the related agreements with regard to the coexistence of  L3 mobility and inter-CU MCG LTM [11]:

9. L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM): - When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network. - The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations. - For the execution order between CHO and LTM, Rel-18 principle is applied.

Therefore, when it comes to SCG LTM, the coexistence of inter-CU SCG LTM and L3 SCG handling like SN change/PsCell change/(S-)CPAC should also be discussed. And we think the similar principle can be applied, including:

- Network triggered SN change/PsCell change and (S-)CPAC can be configured to UE, when (SN initiated) inter-CU SCG LTM is configured.

  - When performing network triggered SN change/PsCell change or (S-)CPAC, the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

  - The RRCReconfiguration message to execute an SN change/PsCell change/(S-)CPAC procedure may reconfigure inter-CU LTM configurations.

  - For the execution order between (S-)CPAC and inter-CU SCG LTM, similar principle for CHO and inter-CU MCG LTM is reused.

Proposal 12: Network triggered SN change/PsCell change and (S-)CPAC can be configured to UE, when (SN initiated) inter-CU SCG LTM is configured.

-	When performing network triggered SN change/PsCell change or (S-)CPAC, the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

-	The RRCReconfiguration message to execute an SN change/PsCell change/(S-)CPAC procedure may reconfigure inter-CU LTM configurations.

-	For the execution order between (S-)CPAC and inter-CU SCG LTM, similar principle for CHO and inter-CU MCG LTM is reused.

- Conclusion

In this contribution, we discuss inter-CU LTM related issue. Following are the proposals and observations made in this contribution:

Observations:

Observation 1: In Rel-18 intra-CU LTM, whether to configure UE-based TA measurement is up to network implementation.

Observation 2: The content of appliedLTM-CandidateId is LTM-CandidateId, which is configured by MN in inter-CU SCG LTM.

Proposals:

Proposal 1: NCC value is include in LTM cell switch command, but not in a new MAC CE only for NCC value delivery.

Proposal 2: Taking Figure 1 as baseline for further discussion on the MAC CE to indicate NCC value.

Proposal 3: Fast recovery is not supported for inter-CU LTM candidate cells. If the UE selects an inter-CU LTM candidate cell after LTM failure, the UE should follow the legacy RRC re-establishment procedure.

Proposal 4: MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

Proposal 5: UE-based  TA  measurement can be supported in Rel-19 inter-CU LTM.

Proposal 6: If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA

Proposal 7: The coexistence of NES and inter-CU LTM can be supported.

Proposal 8: There’s no need to introduce extra IE to indicate whether the SN is allowed to configure an inter-CU LTM configuration is provided in the Xn signalling, while the current SN change refuse procedure can be reused to not allowed inter-CU SCG LTM configuration.

Proposal 9: appliedLTM-CandidateId is only included in MCG RRCReconfigurationComplete message.

Proposal 10: SN modification required by Candidate SN should be supported for LTM configuration updating and candidate PsCell change.

Proposal 11: It’s MN to decide whether release LTM candidate cell(s), but whether SN can request LTM candidate cell release can be decided by RAN3.

Proposal 12: Network triggered SN change/PsCell change and (S-)CPAC can be configured to UE, when (SN initiated) inter-CU SCG LTM is configured.

-	When performing network triggered SN change/PsCell change or (S-)CPAC, the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

-	The RRCReconfiguration message to execute an SN change/PsCell change/(S-)CPAC procedure may reconfigure inter-CU LTM configurations.

-	For the execution order between (S-)CPAC and inter-CU SCG LTM, similar principle for CHO and inter-CU MCG LTM is reused.

- References

- Report of 3GPP TSG RAN WG2 meeting #129;

- S3-251124, Reply LS on security handling for inter-CU LTM in non-DC cases, SA3 to RAN2, RAN3, contact: CATT;

- TS 38.413, Technical Specification Group Radio Access Network; NG-RAN; NG Application Protocol (NGAP) (Release 18);

- Report of 3GPP TSG RAN WG2 meeting #125;

- Report of 3GPP TSG RAN WG2 meeting #125bis;

- Report of 3GPP TSG RAN WG2 meeting #126;

- Report of 3GPP TSG RAN WG2 meeting #127;

- Running RRC CR for inter-CU and conditional LTM;

- Report of 3GPP TSG RAN WG2 meeting #127bis.