3GPP TSG-WG2 Meeting #129bis                                                                     R2-2502740

Wuhan, China, 7th – 11th April 2025

Agenda item:	7.0.2.17

Title:	Discussion on LS from RAN1,RAN4 on soft satellite switch with re-sync

Source:	CMCC

Document for:	Discussion, Decision

- Introduction

RAN2 sent a LS to RAN1,RAN4 on soft satellite switch with re-sync to ask for whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

RAN1 and RAN4 replied the RAN2’s LS and provided some questions for RAN2 respectively.

In this contribution, we discuss the RAN2 understandings in respond to the question mentioned in the reply LS.

- Discussion

For soft satellite switching with re-synchronization, to ensure non-colliding SSB with same PCI at the UE side, the target satellite transmits SSB during [t-ServiceStart, t-Service] which is not overlapping with SSB from the source satellite by  applying SSB pattern shift (i.e., subject to ssb-TimeOffset) .

RAN2#126 Three options were proposed: - Shifted cell timing: the SSB transmission is on the same SFN/subframes for source and target satellite, but there is a timing difference (i.e. SFTD) between source and target satellite. - Shifted SSB pattern: the SSB transmission is on the different SFN/subframes for source and target satellite. - Temporarily shifted SSB pattern: the target satellite uses different SSB transmission pattern with source satellite during [t-Start, t-Service], and then change back to the source satellite SSB pattern after t-Service. RAN2#127 Agreement The field description of ssb-TimeOffset is updated as “Indicates the time offset of the SSB from target satellite at its uplink time synchronization reference point with respect to the SSB from source satellite at its uplink time synchronization reference point. It is given in number of subframes”. RAN2#128 Agreement Send a LS to RAN1/RAN4 on the feasibility of option 3. RAN2 will work on solution 2b or solution 3 depending on the LS response.

RAN2 has discussed one possible solution to introduce a temporarily shifted SSB pattern during [t-ServiceStart, t-Service](option 3). At t-Service, the new serving satellite SSB pattern (e.g., SSB offset) is restored to the same as the previous source satellite.RAN2 wonders whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

- 2.1 Replies to RAN1

RAN1’s reply LS asks for further clarification on the above question and provides the question for RAN2’s LS as follows: - Is RAN2 asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service, or whether RAN2 assumes that some RAN1 specification work may be needed? - RAN1 assumes that the meaning of “the SSB pattern is changed” is that only the time offset is changed (may be determined by ssb-TimeOffset). - RAN1 assumes that the SIB19 of the source satellite includes ephemeris information of the target satellite, and that such information will not get outdated by the time the satellite switch occurs.

Regarding the first question: One condition for soft satellite switching without PCI change is “non-colliding SSB with same PCI at UE side”. To manipulate this SSB timing difference, the SSB transmission pattern of source satellite and target satellite should be different, which is illustrated by the parameter “ssb-TimeOffset-r18”. During the RAN2 discussion, RAN2 assumes that there is no impact on RAN1 specification and no additional RAN1 specification work may be needed.

Proposal 1: RAN2 assumes that there is no impact on RAN1 specification and no additional RAN1 specification work may be needed.

Regarding the first assumption: Based on the above agreement of RAN2#127, the meaning of “the SSB pattern is changed” is that only the time offset is changed (may be determined by ssb-TimeOffset).

Proposal 2: RAN2 confirms that the meaning of “the SSB pattern is changed” is that only the time offset is changed (may be determined by ssb-TimeOffset).

Regarding the second assumption: RAN2 echos RAN1’s assumption: the ephemeris information of the target satellite has already provided in the satSwitchWithReSync-r18 within the SIB19 of source satellite.

Proposal 3: RAN2 assumes that the SIB19 of the source satellite includes ephemeris information of the target satellite, and that such information will not get outdated by the time the satellite switch occurs.

- 2.2 Replies to RAN4

RAN2#128 Agreement Send a LS to RAN1/RAN4 on the feasibility of option 3. RAN2 will work on solution 2b or solution 3 depending on the LS response. RAN4’s reply LS Under following assumptions, RAN4 reached the conclusion that the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. - Starting boundaries of SFN0 of the source satellite and the target satellite are aligned at the uplink time synchronization reference point; - The parameter ssb-TimeOffset is the relative offset between the SSB of the source satellite and the SSB of the target satellite during [t-ServiceStart, t-Service].

Proposal 4: Based on RAN4’s reply LS, RAN2 confirms the Option 3 is feasible.

RAN4’s reply LS provides the questions for RAN2’s LS as follows: - In addition, RAN4 would like RAN2 to confirm which one of the following is the correct understanding of parameter ssb-TimeOffset, - - Option 1: the relative offset between the SSB of the source satellite and the SSB of the target satellite, and the starting boundaries of SFN0 of the source satellite and the target satellites are aligned at the uplink time synchronization reference point - - Option 2: the relative offset between the starting boundaries of SFN0 of the source satellite and the target satellite - Regarding the assumptions above, if option 1 is the correct understanding, RAN4 kindly ask RAN1/2: 1) Whether it is feasible to configure ssb-TimeOffset with values other than multiple of 5. 2) How the half-frame indication and SFN indication bits are configured during [t-ServiceStart, t-Service].

Regarding the first question: Based on the above agreement of RAN2#127, the field description of ssb-TimeOffset is given in number of subframes. In addition, Option 2 corresponds to ‘Shifted cell timing’ solution (Option 1 in RAN2) which has been ruled out in RAN2. Therefore, the option 1 fulfills requirements.

Proposal 5: RAN2 confirms that Option 1 in RAN4’s reply LS is the correct understanding of parameter ssb-TimeOffset.

Regarding the second question: RAN2 envisaged solutions where SSB patterns are shifted (permanently or temporarily, i.e. option 2b and 3 previously discussed) referred to a SSB shift with a half-frame granularity. In the solutions, it is prefer to configure ssb-TimeOffset with values multiple of 5.

Proposal 6: RAN2 assumes that it is prefer to configure ssb-TimeOffset with values multiple of 5.

Regarding the third question: The SSB transmission pattern of target satellite can be determined by the parameter “ssb-TimeOffset-r18” which indicates the time offset of the SSB from target satellite at its uplink time synchronization reference point with respect to the SSB from source satellite at its uplink time synchronization reference point. In addtional, the half-frame indication and SFN indication bits can be acquired from SSB as legacy.

Proposal 7: RAN2 assumes that there is no need to require source satellite to configure the half-frame indication and SFN indication bits of target satellite during [t-ServiceStart, t-Service].

Proposal 8: Please consider the Draft LS in the annex.

- Conclusion

- Proposal 1: RAN2 assumes that there is no impact on RAN1 specification and no additional RAN1 specification work may be needed.

- Proposal 2: RAN2 confirms that the meaning of “the SSB pattern is changed” is that only the time offset is changed (may be determined by ssb-TimeOffset).

- Proposal 3: RAN2 assumes that the SIB19 of the source satellite includes ephemeris information of the target satellite, and that such information will not get outdated by the time the satellite switch occurs.

- Proposal 4: Based on RAN4’s reply LS, RAN2 confirms the Option 3 is feasible.

- Proposal 5: RAN2 confirms that Option 1 in RAN4’s reply LS  is the correct understanding of parameter ssb-TimeOffset.

- Proposal 6: RAN2 assumes that it is prefer to configure ssb-TimeOffset with values multiple of 5.

- Proposal 7: RAN2 assumes that there is no need to require source satellite to configure the half-frame indication and SFN indication bits of target satellite during [t-ServiceStart, t-Service].

- Proposal 8: Please consider the Draft LS in the annex.

- References

- Chairman's Notes RAN2#128

- Chairman's Notes RAN2#129

- R2-2410971, Report of [AT128][301][R18 NR NTN] SMTC issues, Huawei, HiSilicon

- Annex

3GPP TSG-RAN WG2 Meeting #129bis

Wuhan, China, April 7th – 11th, 2025

Title:	[draft LS] soft satellite switch with re-sync

Response to:	R1-2501606, R4-2502701

Release:	Rel-18

Work Item:	NR_NTN_enh-Core

Source:	RAN2

To:	RAN1, RAN4

Cc: 	-

# Contact Person:

# Name:	Li Chai

E-mail Address:	 chaili@chinamobile.com

Attachments:	None

# 1. Overall Description:

RAN2 would like to thank for the reply LS R1-2501606 and R4-2502701.

RAN1’s reply LS provides the question for RAN2’s LS as follows: - Is RAN2 asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service, or whether RAN2 assumes that some RAN1 specification work may be needed? - RAN1 assumes that the meaning of “the SSB pattern is changed” is that only the time offset is changed (may be determined by ssb-TimeOffset). - RAN1 assumes that the SIB19 of the source satellite includes ephemeris information of the target satellite, and that such information will not get outdated by the time the satellite switch occurs.

RAN2 discussed the above questions and has the following agreement.

- RAN2 is asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. RAN2 assumes that there is no impact on RAN1 specification and no additional RAN1 specification work may be needed. - The following two assumptions are correct: - The meaning of “the SSB pattern is changed” is that only the time offset is changed (may be determined by ssb-TimeOffset). - The SIB19 of the source satellite includes ephemeris information of the target satellite, and that such information will not get outdated by the time the satellite switch occurs.

RAN4’s reply LS provides the question for RAN2’s LS as follows: - In addition, RAN4 would like RAN2 to confirm which one of the following is the correct understanding of parameter ssb-TimeOffset, - - Option 1: the relative offset between the SSB of the source satellite and the SSB of the target satellite, and the starting boundaries of SFN0 of the source satellite and the target satellites are aligned at the uplink time synchronization reference point - - Option 2: the relative offset between the starting boundaries of SFN0 of the source satellite and the target satellite - Regarding the assumptions above, if option 1 is the correct understanding, RAN4 kindly ask RAN1/2: 1) Whether it is feasible to configure ssb-TimeOffset with values other than multiple of 5. 2) How the half-frame indication and SFN indication bits are configured during [t-ServiceStart, t-Service].

RAN2 discussed the above questions and has the following agreement.

- RAN2 confirms the Option 1 in RAN4’s reply LS is the correct understanding of parameter ssb-TimeOffset. - RAN2 assumes that it is prefer to configure ssb-TimeOffset with values multiple of 5. - RAN2 assumes that there is no need to require source satellite to configure the half-frame indication and SFN indication bits of target satellite during [t-ServiceStart, t-Service].

Furthermore, RAN2 confirms the Option 3 (Temporarily shifted SSB pattern: the target satellite uses different SSB transmission pattern with source satellite during [t-Start, t-Service], and then change back to the source satellite SSB pattern after t-Service.)  is feasible.

# 2. Actions:

# To RAN1:

RAN2 respectfully asks RAN1 to take the above clarification into consideration and provide feedback on whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

# To RAN4:

RAN2 respectfully asks RAN4 to take the above clarification into consideration and raise further questions, if any.

# 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #130		19th May – 23rd May 2025			Malta, MT

TSG RAN WG2 Meeting #131		25th August – 29th August 2025		TBC, India