3GPP TSG-RAN WG2 Meeting #129bis	R2-2502762

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.7.6

Source:	OPPO

Title:	Discussion on XR Rate Control

Document for:	Discussion, Decision

Introduction

This paper discusses the following objective in Rel-19 XR WID [1]:

- Specify uplink congestion signalling [RAN2, RAN3]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

Discussion

RAN2 previously discussed the granularity of the rate control indication for XR in the congestion case. The ongoing debate about “per QoS flow or DRB” in RAN2 has continued through several meetings, and RAN2#128 eventually concludes to implement QoS flow level indication.

- RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

The follow-up question is whether the QoS flow level information is contained in a MAC CE (i.e., Option 1) or RRC signalling (i.e., Option 2). Some companies prefer RRC signalling since they are concerned that having the QoS flow level awareness at the MAC layer could negatively impact the CU/DU split architecture.

However, according to TS 38.473, the DU can now obtain information about the QoS flow(s) and the associated DRB through interaction with the CU. As a result, the MAC layer can derive/generate a MAC CE that includes QoS flow identification to control the data rate with minimal effort.

Furthermore, RAN3 shows that both Option 1 and Option 2 affect the F1 interface [2]. Thus, RAN2 can continue discussing or analysing how to indicate the QoS flow level information.

RAN3 thanks RAN2 for the LS on XR UL bit rate control. RAN3 discussed it and agreed both options have an impact on the F1 interface. RAN3 will further discuss whether gNB-DU can support per QoS flow data bit rate control when TU is available.

We believe that using RRC to indicate which QoS flows should be throttled is inefficient and slow during times of congestion. The QoS flow requiring throttling can differ between congestion scenarios. For example, in cases of slight congestion, the network might only need to adjust one QoS flow linked to a single DRB, whereas in the case of severe congestion, it may need to adjust multiple QoS flows for the same DRB. As a result, the network has to often update the relevant QoS flow(s) via RRC before sending a MAC CE, which leads to unexpected latency and increased signalling overhead.

Introduce a new MAC CE that contains QFI information to indicate uplink rate adaption.

One XR application may be associated with multiple QoS flows, each having distinct QoS requirements (e.g., video, audio, etc). In the event of congestion, it may become necessary for the gNB to simultaneously adapt the bit rates of these multiple QoS flows. To minimize the signalling overhead associated with transmitting multiple MAC CEs (assuming one MAC CE per QoS flow), it is recommended to consolidate information for multiple QoS flows in a single MAC CE.

The new MAC CE designed for XR rate control can include bit rate indications for multiple QoS flows.

Conclusion

We have the following proposals:

Proposal 1	Introduce a new MAC CE that contains QFI information to indicate uplink rate adaption.

Proposal 2	The new MAC CE designed for XR rate control can include bit rate indications for multiple QoS flows.

Reference

[1] RP-250107 Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur)

[2] R2-2501508 Reply LS on XR UL Rate Control (R3-250805; contact: Meta)