3GPP TSG-RAN WG2 Meeting #129bis	R2-2502763

Wuhan, China,7th  – 11th  April,  2025

Agenda item:	8.1.2.2

Source:	Nokia

Title:	LCM for UE-side Beam Management

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we discuss issues related to functionality-based LCM for UE-side AI/ML beam management. First, we discuss features concerning the configuration of inference-related parameters. Then, we discuss the remaining issues concerning applicability such as reporting structure, reconfiguration due to poor performance and handling the case when an active applicable configuration for periodic beam prediction becomes inapplicable. Next, we discuss fallback configurations. This document concludes with our Stage 2 text proposals concerning Applicability Reporting Clause detailed in Section 7.

# 2	Configuration for Inference-Related Parameters

The NW can provide the UE with one or more sets of inference-related parameters to be evaluated for applicability. These sets of inference-related parameters are not sufficient on their own to configure the UE for inference rather require that the gNB further configures the UE after determining applicability.

Because the inference-related parameters proposed by RAN1 can be found in CSI-ReportConfig, we propose that the inference-related parameters are provided in CSI-ReportConfigs. Because the UE’s behaviour is different depending on whether it is provided a full inference configuration or a set of inference-related parameters, a flag should be included to indicate that a CSI-ReportConfig corresponds to inference-related parameters instead of a full inference configuration. With the flag differentiating the two, the inference-related parameters can also be placed in csi-ReportConfigToAddModList in CSI-MeasConfig.

Observation 1: The inference-related parameters suggested by RAN1, including any configurations that will be added for beam prediction, will be included in CSI-ReportConfig.

Proposal 1: CSI-ReportConfig, included in CSI-MeasConfig -> csi-ReportConfigToAddModList, is used to transmit one or multiple sets inference-related parameters for beam prediction.

To differentiate between a full configuration and a set of inference-related parameters, a field would be necessary, which at least could be one of the following, including options to make it obvious or non-obvious that this is about ML.

- infererenceRelatedParameters or relatedParameters		ENUMERATED {true} OPTIONAL

- applicabilityConfig										ENUMERATED {full, inference-set} OPTIONAL

Proposal 2: Add a flag to CSI-ReportConfig to indicate that a configuration contains one or multiple sets of inference-related parameters instead of a full configuration. This flag could also be used  as an indication that the configuration is subject to the applicability determination procedure.

Alternatively, inference-related parameters could be provided in CSI-MeasConfig, in a new SEQUENCE which stores configurations of inference-related parameters. To aid in the indexing of the CSI-ReportConfigIds, we could discuss whether to share CSI-ReportConfigIds between the lists such that they do not overlap. That way the applicability report can simply contain CSI-ReportConfigIds without any further specification of whether the CSI-ReportConfigId corresponds to a full configuration or to inference-related parameters. An example is shown below in Figure 2.1-1.

CSI-MeasConfig ::=           SEQUENCE {

<removed>

csi-ReportConfigToAddModList  SEQUENCE SIZE(1..maxNrOfCSI-ReportConfigurations) OF

CSI-ReportConfig OPTIONAL,

csi-ReportConfigToReleaseList SEQUENCE SIZE(1..maxNrOfCSI-ReportConfigurations) OF

CSI-ReportConfigId OPTIONAL

<removed>
<for inference-related parameters>

[[

csi-ReportConfigRelatedToAddModList-r19  SEQUENCE SIZE(1..maxNrOfCSI-ReportConfigurations) OF

CSI-ReportConfig OPTIONAL,

csi-ReportConfigRelatedToReleaseList-r19 SEQUENCE SIZE(1..maxNrOfCSI-ReportConfigurations) OF

CSI-ReportConfigId OPTIONAL

]]

Figure 2.1-1 Alternative Configuration of Inference-Related Parameters

Proposal 3: As an alternative to providing inference-related parameters in CSI-MeasConfig -> csi-ReportConfigToAddModList, a new list of CSI-ReportConfigs containing inference-related parameters could be defined.

# 3	Applicability

## 3.1	Structure of the Applicability Report

The RRC applicability report will be used to report the applicability of any AI/ML-enabled configuration, or any configuration subject to the applicability determination procedure, for all RRC-based use cases, e.g., CSI feedback enhancement, Beam Prediction, and Mobility, in the future. Every configuration which we have discussed thus far in the use cases has an ID, and that ID, e.g., CSI-ReportConfigId, can be used to identify AI/ML-enabled configurations and to report their applicability, and updates thereto.

Below, we provide two versions of the ApplicabilityReport-r19, which we find to be extensible to future use cases. Lastly we provide ASN.1, which closely matches the running CR for RRC for AI/ML Phy: ApplicabilityReportList-r19; UEAssistanceInformation-v1900-IEs; and RRCReconfigurationComplete-v1900-IEs.

In Alt 1., shown in Figure 3.1-1, the applicabilityId is a choice of IDs already existing in the specification for which applicability could be determined and reported. In this alternative, new applicabilityId types would be added through nonCriticalExtensions in further releases.

ApplicabilityReport-r19 ::= SEQUENCE { applicabilityId CHOICE { csi-ReportConfigId CSI-ReportConfigId, <similar additions for other use cases in Rel-19> } OPTIONAL, applicability ENUMERATED {applicable, inapplicable}, nonCriticalExtensions SEQUENCE {} }

Figure 3.1-1: Alt 1. ApplicabilityReport-r19

In Alt 2., show in Figure 3.1-2, the applicabilityIdType is an ENUMERATED value which identifies the type of ID, e.g., csi-ReportConfigId which is pointed to by applicabilityId, which is an OCTET STRING, or an INTEGER with a size to be determined, which can hold any ID type. The benefit of this option is that we can include spare values in the ENUMERATED and then use nonCriticalExtensions for additional features in the future instead of additional ID types.

ApplicabilityReport-r19 ::= SEQUENCE { applicabilityIdType ENUMERATED { csi-ReportConfigId, use-case-X-Rel-19, use-case-Y-Rel-19, spare3, spare4, spare5, spare6, spare 7 }, applicabilityId OCTET STRING, applicability ENUMERATED {applicable, inapplicable}, nonCriticalExtensions SEQUENCE {} }

Figure 3.1-2: Alt 2. ApplicabilityReport-r19

The supporting elements for signalling the ApplicabilityReport are shown in Figure 3.1-3 below, and are captured similarly in the running CR. The same ApplicabilityReportList container would be used in UEAssistanceInformation and in RRCReconfigurationComplete, where applicable.

ApplicabilityReportList-r19 ::= SEQUENCE (SIZE(1..maxNrOfApplicabilityReports)) OF ApplicabilityReport-r19 UEAssistanceInformation-v1900-IEs ::= SEQUENCE { applicabilityReportList ApplicabilityReportList-r19 OPTIONAL -- Need R nonCriticalExtensions SEQUENCE {} } RRCReconfigurationComplete-v1900-IEs ::= SEQUENCE { applicabilityReportList ApplicabilityReportList-r19 OPTIONAL -- Need R nonCriticalExtensions SEQUENCE {} }

Figure 3.1-3: ApplicabilityReportList-r19, UEAssistanceInformation-v1900-IEs, and RRCReconfigurationComplete-v1900-IEs Adapted from the running CR.

Proposal 4: To allow future proof extensibility, adopt the structure of alternative 1 or alternative 2 of the ApplicabilityReport-r19 and its inclusion in UEAssistanceInformation-v1900-IEs and RRCReconfigurationComplete-v1900-IEs.

As is written in Section 2, inference-related parameters could be stored in CSI-ReportConfig, identified by a CSI-ReportConfigId, which would be non-conflicting in the case that full configurations an inference-related configurations are stored in separate lists. Therefore, whether a CSI-ReportConfigId identifies a full configuration or inference-related parameters, it could be reported with the same ID type in the ApplicabilityReport.

## 3.2 Reporting with Multiple Serving Cells

To support applicability reporting on component carriers in a carrier aggregation configuration, a UE may be provided inference configurations for more than one cell. As such, the gNB needs to know how to identify to which cell the applicability report applies. Since the primary cell, or PCell would be responsible for receiving applicability report on behalf of the SCells. we think applicabilityCellId could be updated to indicate the SCellIndex whose range is between 1 and 31. Correspondingly, the first index is implicitly reserved for the PCell. An ASN.1 example is shown below.

applicabilityCellId-r19                      INTEGER (0..maxNrOfSCells),

The editor’s note below from the running CR for 38.331 indicates this is an open item.

Editor's Note: FFS if applicability reporting is supported for multiple serving cells and applicabilityCellId is needed to unambiguously identify CSI report configurations for prediction.

Therefore, we make the following proposal.

Proposal 5: The applicabilityCellId-r19 corresponds to the PCell or to the SCellIndex corresponding to the configuration upon which applicability is being reported. The range of applicabilityCellId-r19 is 0 to maxNrOfSCells, where 0 indicates the PCell.

## 3.3	Reporting Applicability of Modified Configurations

The behaviour for initial configuration has been provided, but it isn’t clear what the UE should do when a full inference configuration or a set of inference-related parameters for beam prediction is modified through the ToAddMod structure, which would be used for full inference configurations, and possibly for inference-related parameters. The simplest approach would be to treat a modification to a configuration the same way as the addition. That is, anytime a full inference configuration or set of inference-related parameters is modified, the applicability determination procedure should be triggered.

Proposal 6: The modification of a full inference configuration or set of inference-related parameters triggers an applicability determination and reporting procedure.

## 3.4	Applicability and Poor Performance

RAN2 agreed that the UE is the entity responsible for determination of the applicability of an AI/ML-enabled configuration, enabled by a UE-side model. The applicability depends in part on the evaluation of conditions involved in the consistency between training and inference. Some conditions are based on a comparison between model meta data and the configuration provided to the UE and others are evaluated based on aspects related to the UE, e.g., its velocity or the inputs from various sensors. What has not been considered thus far, however, is the consideration of historical performance of a configuration enabled by AI/ML models.

A UE could frequent a cell, each time reporting that a configuration is applicable. However, due to a change in conditions, such as the construction of a new building or the presence of a large vehicle in front of the cell, or the lack of consideration for different weather conditions, a configuration reported as applicable consistently performs poorly.

Observation 2: A poorly performing inference configuration could consistently perform poorly because the applicability determination procedure is unable to account for unforeseen factors which could impact model performance.

To prevent the UE from repeatedly reporting a poorly performing functionality/configuration as applicable, we propose that the NW should be able to provide feedback to the UE while reconfiguring the UE due to a poorly performing AI/ML-enabled configuration. The UE could use the feedback to augment its applicability determination criteria.

Observation 3: An indication of poor performance when reconfiguring a UE configured with an inference configuration could be used by the UE to help determine applicability of a configuration.

Proposal 7: The NW can provide a cause with reconfiguration, e.g., modifying or removing an inference configuration, including that the configuration performed poorly.

## 3.5	Handling of Active Periodical Inference Configurations Which Become Inapplicable

In RAN2#129, it was agreed “if option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5)” [1].  Moreover, according to the RAN2#128 agreement “when a functionality becomes non-applicable the UE doesn’t autonomously deactivate, the NW is expected to deactivate an activated functionality when it receives a report from UE that it is inapplicable” [2]. As such, if a full inference configuration for beam prediction is provided with reportConfigType=’periodic’, the UE can report when the applicability of the configuration becomes inapplicable using UAI. Because the UE cannot deactivate the inference reporting autonomously, a potential ambiguity is introduced because the quality and usefulness of the inference reports are unknown. See Figure 3.5-1 for details.

Figure 3.5-1. Ambiguity of UE behaviour after an active periodic inference configuration becomes inapplicable

Observation 4: An active beam prediction configuration may become inapplicable after it is activated. During this time, the outputs of the model enabling the configuration could be unreliable or ambiguous.

One approach to resolve the above issue is to define a new UE reporting behaviour. Instead of reporting Set A beam predictions, the UE would report Set B beam measurements after the UE reports the inapplicability of a beam prediction reporting configuration. This avoids wasting configured air interface resources but also supports performance monitoring at the NW (e.g., in case of Type-1 Option 1).

Proposal 8 When an active beam prediction configuration becomes inapplicable, the UE shall suspend the transmission of Set A beam predictions and report Set B measurements corresponding to the newly inapplicable, activated beam prediction inference configuration until the UE is reconfigured by the NW.

Proposal 9: RAN2 to endorse Proposal 8 within Applicability Reporting Clause for Stage 3 running CR (changes are highlighted in orange in Section 7).

# 4	Fallback

Fallback to non-AIML means de-configuring one AI/ML beam prediction configuration and activating a non-AI/ML beam prediction configuration. The purpose of fallback is to mitigate performance losses by reverting to a non-AI/ML method for beam management or beam measurement reporting. However, the NW may fall into an unnecessary ping-pong between non-AI/ML and AI/ML-enabled beam prediction configurations, which could lead to signalling overhead to activate or deactivate an AI/ML configuration on one hand and imposing unnecessary activation delays on the other hand.

Because the performance of the AI/ML beam prediction configuration n was satisfactory prior to the decision to fall back, it could be reasonably assumed that the AI/ML configuration will perform well once again.

Observation 5: Because the performance of a beam prediction configuration could have been satisfactory prior to the decision to fall back, it could be reasonably assumed that the AI/ML configuration will perform well once again.

Therefore, monitoring a beam prediction configuration during fallback could be useful to help the NW determine when to exit fallback and resume the use of the beam prediction configuration to make beam prediction decisions. During this type of fallback, the beam prediction reporting configuration would still be active, but it could be configured with a lower reporting frequency.

Proposal 10: Discuss whether during fallback if the UE can be configured to report inference outputs, e.g., at a lower periodicity, of a configuration which performed poorly after initially performing well to aid the NW in exiting fallback.

# 5	Stage 2 Specification of Applicability Reporting

The latest Stage 2 draft on the applicability reporting [3] lacks the presentation of the applicability reporting procedure. This was commented during the email discussion of by several companies [3]. We think that it is significant to  present  all the steps involved in the  procedure in the Stage 2 specification phase, since  Stage 3 specifications mainly capture the protocol level procedures without clarifying the relation between  different protocol level procedures. On the other hand, we believe that capturing protocol level agreements (e.g., which information elements are carrying which information) is not relevant from Stage 2 perspective, as it will be specified during Stage 3 specification phase. Therefore, we propose to adopt the revision of the Applicability Reporting clause included in the Annex of this paper, which includes the complete procedure and removes the unnecessary Stage 3 details.

Proposal 11: RAN2 to adopt the text proposal of Annex in Section 7 on Applicability Reporting Clause  as a Stage 2 specification baseline for the next version of the Stage 2 running CR.

# 6	Conclusion

This document has made the following observations:

Observation 1: The inference-related parameters suggested by RAN1, including any configurations that will be added for beam prediction, will be included in CSI-ReportConfig.

Observation 2: A poorly performing inference configuration could consistently perform poorly because the applicability determination procedure is unable to account for unforeseen factors which could impact model performance.

Observation 3: An indication of poor performance when reconfiguring a UE configured with an inference configuration could be used by the UE to help determine applicability of a configuration.

Observation 4: An active beam prediction configuration may become inapplicable after it is activated. During this time, the outputs of the model enabling the configuration could be unreliable or ambiguous.

Observation 5: Because the performance of a beam prediction configuration could have been satisfactory prior to the decision to fall back, it could be reasonably assumed that the AI/ML configuration will perform well once again.

And the following proposals:

Proposal 1: CSI-ReportConfig, included in CSI-MeasConfig -> csi-ReportConfigToAddModList, is used to transmit one or multiple sets inference-related parameters for beam prediction.

Proposal 2: Add a flag to CSI-ReportConfig to indicate that a configuration contains one or multiple sets of inference-related parameters instead of a full configuration. This flag could also be used  as an indication that the configuration is subject to the applicability determination procedure.

Proposal 3: As an alternative to providing inference-related parameters in CSI-MeasConfig -> csi-ReportConfigToAddModList, a new list of CSI-ReportConfigs containing inference-related parameters could be defined.

Proposal 4: To allow future proof extensibility, adopt the structure of alternative 1 or alternative 2 of the ApplicabilityReport-r19 and its inclusion in UEAssistanceInformation-v1900-IEs and RRCReconfigurationComplete-v1900-IEs.

Proposal 5: The applicabilityCellId-r19 corresponds to the PCell or to the SCellIndex corresponding to the configuration upon which applicability is being reported. The range of applicabilityCellId-r19 is 0 to maxNrOfSCells, where 0 indicates the PCell.

Proposal 6: The modification of a full inference configuration or set of inference-related parameters triggers an applicability determination and reporting procedure.

Proposal 7: The NW can provide a cause with reconfiguration, e.g., modifying or removing an inference configuration, including that the configuration performed poorly.

Proposal 8 When an active beam prediction configuration becomes inapplicable, the UE shall suspend the transmission of Set A beam predictions and report Set B measurements corresponding to the newly inapplicable, activated beam prediction inference configuration until the UE is reconfigured by the NW.

Proposal 9: RAN2 to endorse Proposal 8 within Applicability Reporting Clause for Stage 3 running CR (changes are highlighted in orange in Section 7).

Proposal 10: Discuss whether during fallback if the UE can be configured to report inference outputs, e.g., at a lower periodicity, of a configuration which performed poorly after initially performing well to aid the NW in exiting fallback.

Proposal 11: RAN2 to adopt the text proposal of Annex in Section 7 on Applicability Reporting Clause  as a Stage 2 specification baseline for the next version of the Stage 2 running CR.

# 7	Text Proposal for 38.300 Applicability Reporting Procedure

Note: The changes in the section below are on top of the latest available version of the running CR [3].

#### X.Y.2.3	Applicability Reporting

Editor's note: The content in this sub-clause will be further refined/update based on further progress.

UE indicates its AI/ML capabilities to the Network via UE capability information. The UE may not always have a suitable model to enable a specific configuration related to a AI/ML capability, and thus explicitly signals the applicability of configurations, subject to applicability determination. These configurations are related to AI/ML-enabled feature. The applicability reporting procedure for a full inference configuration is depicted in Figure X.Y.2.3-1

Editor's note: The definition of NW side additional conditions, UE-side additional conditions, the ‘model’ needs to be added.

Figure X.Y.2.3-1: Procedure for applicability reporting and full inference configuration

1.	The network provides inference configuration to UE in RRCReconfiguration message.

2.	UE provides its applicability report to the network in the RRCReconfigurationComplete message. The UE determines the applicability of the inference configuration based on network-side additional conditions (if provided), UE-side additional conditions, and availability of the model within the UE.

3.	The network activates one of the applicable inference configurations. This can happen via RRC Reconfiguration procedure or lower layer signalling.

4.	Any time after Step 2, once the applicability of the configuration changes, the UE reports the change in the applicability to the network via UEAssistanceInformation message. This message may trigger (e.g., when an active configuration becomes inapplicable) a new inference configuration (Step 1) or an activation of another inference configuration (Step 3).

Editor's note: FFS if we report explicit cause of applicability change.

When provided with periodic CSI configuration consistent with reported UE capabilities, upon reporting the applicable configurations, the UE autonomously activates the applicable AI/ML configurations. When an activated applicable AI/ML configuration with periodic CSI configuration becomes inapplicable, the UE reports only the measured beams configured in the inapplicable configuration until the network deactivates or release this activated configuration. When provided with semi-persistent CSI and/or aperiodic CSI configuration, upon reporting the applicable AI/ML configurations, applicable AI/ML configuration activation follows the legacy CSI framework, i.e., semi-persistent reporting can be activated by MAC CE/DCI and aperiodic CSI reporting can be activated by DCI.

When an activated AI/ML configuration becomes inapplicable, the UE does not autonomously deactivate it,but indicates to the network the change in the applicability (see Step 4 of Figure X.Y.2.3-1). Upon reception of UE indication of the configuration becoming inapplicable, the network should deactivate or release this activated full inference configuration.

Editor's note: FFS signaling details for option B (e.g., whether it is signaling in CSI-Report Config or otherconfig) and activation.

During handover, the UE may receive the NW-side additional conditions and the inference configuration related to the target gNB . Then the UE provides its applicability report to the target gNB after the handover completion.

Editor's note: FFS how the applicable functionality is decided if NW-side additional condition is not provided

NOTE 1:	UAI can be sent from the source gNB to the target gNB using existing signaling.

For NW-side model, the CSI report framework is used to acquire input data for inference.

NOTE 2:	For NW-side model, required network side additional condition is left up to the network implementation.

The UE can be first provided one or multiple sets of inference-related parameters instead of the full inference configuration to decrease the signalling overhead during the applicability determination procedure. These parameters are a subset of those included in a full inference configuration. The outcome of this procedure is that the gNB receives enough information to configure the UE with full inference configuration(s) as described above, which aligns with the one or multiple sets of inference-related parameters. A call flow is provided in Figure X.Y.2.3-2 below.

Figure X.Y.2.3-2: Procedure for applicability reporting and sets of inference-related parameters

The steps of the call flow of Figure X.Y.2.3-2 mirror those of Figure X.Y.2.3-1, but without the activation of inference. This procedure on its own is insufficient to configure a UE for inference, and a full inference configuration would need to follow.

# 8	References

[1] R1-2410898, “RAN2 Chair’s Notes”, 3GPP TSG RAN WG2 #129, Athens, Greece, February 17 – 21, 2025.

[2] R1-2407554, “RAN2 Chair’s Notes”, 3GPP TSG RAN WG2 #128, Orlando, US, November 18 – 22, 2024.

[3] [POST129][024][AI PHY] Stage 2 running CR (vivo) R2-25xxxxx_Introduction of AI for Air interface feature in 38300