3GPP TSG RAN WG2 #129bis			R2-2502781

Wuhan, China, April 7th – 11st, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on on-demand SIB1

Agenda Item:	8.5.3

Document for: 	Discussion and Decision

- Introduction

At the RAN#105 meeting, the new work item of “Enhancement of network energy savings for NR” was agreed [1]. In this contribution, we provide our views on on-demand SIB1 for idle/inactive mode UEs.

Agreements on OD-SIB1@RAN2#127

Scenarios:

- No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

- RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

OD-SIB1 SI conclusion:

- RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

Agreements on OD-SIB1@RAN2#127bis

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

Agreements on OD-SIB1@RAN2#128

1.	NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

2.	New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritiz-ing/deprioritizing a NES frequency.

3.	Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

4.	Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

5.	The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

6.	The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

7.	If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

8.	A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no cor-responding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

9.	UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE ac-quired WUS configuration. FFS on SI validity area rather than cell level.

10.	Working assumption: UL-WUS configuration in RRC release is not supported.

Agreements on OD-SIB1@RAN2#129

1.	There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

2.	Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

3.	The UE follows the legacy validity principle of stored SIB.

4.	For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

5.	SIB-x can be cell specific or area specific, as legacy.

6.	Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

- Discussion

2.1	Behavior on NES cell

Before a UE requests SIB1 to a cell, the UE needs to check if SIB1 is currently being broadcasted or provided on demand, as agreed in the previous meeting [2]. This agreement should be clarified at least for the following two issues: (issue 1) whether legacy UE can use a NES cell where SIB1 is broadcasted (Note: a NES cell means a cell for which UL WUS configuration is available in this context) and (issue 2) how a UE identifies if the cell transmits SIB1 in broadcast mode or on-demand mode.

On issue (1), We would like to note that, regardless of whether legacy UEs can camp on the NES cell with broadcasted SIB1, it should be up to gNB implementation to switch NES cell (including the case where SIB1 is on-demand or broadcasted) to legacy/normal cell that legacy UEs can camp on. In our understanding, such operation can be implemented in a spec transparent manner, and spec never prevent such operation.

Proposal 1:

- Clarify whether or not legacy UE can use (camp on) a cell with SIB1 currently being broadcasted for which SIB1 request (UL WUS) configuration is available.

  - Note: Regardless of the above discussion, it should be up to gNB implementation to switch NES cell (including the case where SIB1 is on-demand or broadcasted) to legacy/normal cell that legacy UEs can camp on.

On top of broadcast of SIB1 on NES cell, we understand that a cell for which SIB1 request configuration is available, can periodically broadcast any other SI. From operators’ perspective, NW should have flexibility to broadcast other SI as well according to situation (e.g., considering the amount of request to SI)

Proposal 2:

- A cell for which SIB1 request (UL WUS) configuration is available can periodically broadcast any other SI message(s) in addition to SIB1.

On issue (2), the related RAN1 agreement was made as follows.

Agreement@RAN1#120 If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2, down select from the following: - Alt. 1: Only if K_SSB=30 for FR1 and K_SSB=14 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission; Otherwise, UE doesn’t monitor Type 0 PDCCH - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified. - Alt. 2: UE monitors Type 0 PDCCH for SIB1 transmission - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified. - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission - Alt. 4: UE doesn’t monitor Type 0 PDCCH for SIB1 transmission. - Alt. 5: UE monitors Type 0 PDCCH for SIB1 based on some repurposed parameters in MIB or PBCH Note: If the UE detects a SSB where K_SSB<=23 for FR1 or K_SSB<=12 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission as legacy. Note: From Ericsson point view, Alt 3 and Alt 4 may be inconsistent with existing RAN2 agreements Note: The above cases are for SSB on the sync raster.

We would like to clarify the original intention of the RAN2 agreement that UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell. In our understanding, the wording of “SIB1 is currently being broadcasted” means SIB1 is transmitted as the legacy behavior, i.e., periodically transmitted following configuration of CORSET0 and search space0. In this case, the related parameters, kssb and PDCCH-configSIB1 in MIB/PBCH should be set properly, which means kssb is not set to more than 24 in FR1 and 13 in FR2. In other words, if kssb with being more than 24 in FR1 is detected in a cell, a UE can request OD-SIB1 in the cell by using provided UL WUS configuration. In this sense, the RAN2 agreement does not require any additional mechanism to check the transmission status of OD-SIB1 and the already agreed mechanism that kssb is set as “no SIB1 indication” when the cell is operating OD-SIB1 to be transmitted according to UE’s request.

The other interpretation can be that it just means SIB1 is now transmitted whichever the SIB1 is transmitted in the on-demand manner or the legacy manner. The interpretation of the RAN2 agreement would impact on th e direction discussed in RAN1, so we would like to clarify the original intention of RAN2 agreement.

Proposal 3:

- Clarify the meaning of the RAN2 agreement of “SIB1 currently being broadcasted”, (1) or both of (1) and (2)

  - (1) SIB1 is transmitted as legacy and MIB/PBCH including the parameters kssb and PDCCH-SIB1 is set as legacy of CD-SSB

  - (2) SIB1 is transmitted as OD-SIB1, i.e., SIB1 is transmitted in response to UL WUS transmission and within time window

In the legacy specification, there might be some cases where a UE needs to check the contents of SIB1. So far, RAN2 has mainly discussed the case of cell reselection where OD-SIB1 in the NES cell is requested so that a UE decides whether the NES cell can be camped on. Besides, after and during camping on the NES cell, in order to obtain and keep the valid version of other SI, a UE needs to check broadcast status of other SI configured in SIB1. We think the overall legacy procedure for obtaining SI should be kept, and during the procedure, it should be clarified that a UE needs to check SIB1 transmission status first and then obtaining necessary information included in SIB1. How to check the SIB1 transmission status should be same as the above discussion.

Proposal 4:

- Before requesting on-demand SI, a UE should check if SIB1 is currently being broadcasted or provided on demand

  - if SIB1 is broadcasted, the UE should check the broadcasting status in SIB1

  - if SIB1 is provided on demand, the UE should request on-demand SIB1 transmission first.

Proposal 5:

- It should be up to NW implementation whether other SI is on-demand or broadcast transmission that is provided by on-demand SIB1.

- Conclusion

In this contribution, we provided the following observations and proposals of on-demand SIB1 operation for idle/inactive mode UEs network energy saving.

Proposal 1:

- Clarify whether or not legacy UE can use (camp on) a cell with SIB1 currently being broadcasted for which SIB1 request (UL WUS) configuration is available.

  - Note: Regardless of the above discussion, it should be up to gNB implementation to switch NES cell (including the case where SIB1 is on-demand or broadcasted) to legacy/normal cell that legacy UEs can camp on.

Proposal 2:

- A cell for which SIB1 request (UL WUS) configuration is available can periodically broadcast any other SI message(s) in addition to SIB1.

Proposal 3:

- Clarify the meaning of the RAN2 agreement of “SIB1 currently being broadcasted”, (1) or both of (1) and (2)

  - (1) SIB1 is transmitted as legacy and MIB/PBCH including the parameters kssb and PDCCH-SIB1 is set as legacy of CD-SSB

  - (2) SIB1 is transmitted as OD-SIB1, i.e., SIB1 is transmitted in response to UL WUS transmission and within time window

Proposal 4:

- Before requesting on-demand SI, a UE should check SIB1 transmission status

  - if SIB1 is broadcasted, the UE should check the broadcasting status of SI in SIB1

  - if SIB1 is not transmitted, the UE should request on-demand SIB1 transmission first.

Proposal 5:

- It should be up to NW implementation whether other SI is on-demand or broadcast transmission that is provided by on-demand SIB1.

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

- 3GPP, RAN2#127bis meeting, Chairman’s notes

- 3GPP, RAN2#128 meeting, Chairman’s notes

- 3GPP, RAN1#120 meeting, Chairman’s notes

- 3GPP, RAN2#129 meeting, Chairman’s notes