3GPP TSG-RAN RAN2 #129bis					       R2-2502792

Wuhan, China, 7 - 11 April, 2025

Title: 	Discussion on SONMDT for NTN

Source: 	Huawei, HiSilicon

Agenda item:	8.10.4

Document for:	Discussion/Decision

1   Introduction

According to the RAN3 #126 meeting, the LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT [1] was sent to RAN2, among which the SON/MDT-for-NTN related issues are as follows:

- Regarding SON/MDT for NTN: On MRO for NTN, RAN3 agreed that UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not. On MDT for NTN, RAN3 assumes that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP. Further, RAN3 would like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, to help RAN3 decide whether to specify geographical area and/or mapped cell ID over NGAP.

In the last #129 meeting, RAN2 reached agreements on the first LS to reuse the existing IEs for CHO [2]:

Existing IEs for CHO are used to report the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”.

In this paper, we further discuss the second issue of MDT for NTN based on RAN3’s LS.

2   Discussion

As for the Area Scope for the logged MDT for NTN, considering that 1) the legacy area scope is defined by cell ID, and 2) the area scope in NTN is dynamic and large, the fine-grained area scope definition is needed. Additionally, for UEs in idle/inactive state, the NW cannot get the accurate location or speed of UEs. Hence, merely reusing the current definition with some NW-side enhancement may be insufficient, and the geographical area defined for MBS NTN may be considered to meet such requirements. Thus, to support the logged MDT for NTN according to RAN3’s LS, we think the geographical area (coverageAreaInfoList-r18 defined in SIB 25) is needed.

–	SIB25

SIB25 contains TN coverage information to assist neighbour cell measurements for the UEs in an NTN cell.

SIB25 information element

-- ASN1START

-- TAG-SIB25-START

SIB25-r18 ::=                  SEQUENCE {

coverageAreaInfoList-r18       CoverageAreaInfoList-r18         OPTIONAL,  -- Need R

lateNonCriticalExtension       OCTET STRING                     OPTIONAL,

...

}

CoverageAreaInfoList-r18 ::=   SEQUENCE (SIZE (1..maxTN-AreaInfo-r18)) OF CoverageAreaInfo-r18

CoverageAreaInfo-r18 ::=       SEQUENCE {

tn-AreaId-r18                  TN-AreaId-r18,

tn-ReferenceLocation-r18       ReferenceLocation-r17,

tn-DistanceRadius-r18          INTEGER(0..65535)

}

-- TAG-SIB25-STOP

-- ASN1STOP

SIB25 field descriptions

coverageAreaInfoList Contains a list of TN coverage area's information to assist skipping TN measurements for NTN UEs in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20].

tn-DistanceRadius Distance from the TN coverage area reference location. It is used for skipping TN measurements in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 1m.

Hence, we proposed to reuse the geographical area defined for MBS NTN for Area Scope of logged MDT over Uu for NTN.

At RAN2#129 meeting, some companies were concerned about UE impacts due to geographical area checking in MDT for NTN, e.g. this would drain the UE battery.

Firstly, we think this geographical area checking has been introduced in MBS for NTN, and here are relevant agreements made at RAN2#127 [3]:

For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

It can be seen that this feature is only applicable for NTN UEs and there should be uniform UE behaviours regarding geographical area checking, so the UE impacts should be minimized.

Secondly, we think that the UE is not always required to get position information for this feature, so we can discuss UE behaviours to minimize negative impacts, e.g. the UE performs geographical area checking only if it has available position information.

In summary, we have the following observations:

Observation 1: The UE behaviour of checking the geographical area in MDT for NTN can follow RAN2 agreements in MBS for NTN.

Observation 2: The negative impacts can be minimized by some means, e.g. the UE performs geographical area checking only if it has available position information.

Then we have the following proposals:

Proposal 1: It is proposed RAN2 to agree on reusing the geographical area defined for MBS NTN for Area Scope of logged MDT over Uu for NTN.

Proposal 2: It is proposed RAN2 to discuss whether to introduce some means to minimize negative impacts at UE side, e.g. the UE performs geographical area checking only if it has available position information.

3   Conclusion

In this paper, we provide the following observations and proposals:

Observation 1: The UE behaviour of checking the geographical area in MDT for NTN can follow RAN2 agreements in MBS for NTN.

Observation 2: The negative impacts can be minimized by some means, e.g. the UE performs geographical area checking only if it has available position information.

Proposal 1: It is proposed RAN2 to agree on reusing the geographical area defined for MBS NTN for Area Scope of logged MDT over Uu for NTN.

Proposal 2: It is proposed RAN2 to discuss whether to introduce some means to minimize negative impacts at UE side, e.g. the UE performs geographical area checking only if it has available position information.

4   Reference

[1] R3-247908, LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT.

[2] R2_129_Maint_SONMDT_Mattias_20250221_Final.

[3] R2-127 NR-NTN-IoT-NTN (Sergio)_ EOM