3GPP TSG-RAN WG2 Meeting #129bis					R2-2502793

Wuhan, China, 7 - 11 April, 2025

Title: 	Open issue list for running 37.320 CR for R19 AI for PHY

Source: 	Huawei, HiSilicon, Ericsson

Agenda item:	8.1.1

Document for:	Discussion/Decision

- Introduction

When drafting running 37.320 CR, the CR rapporteur observed some open issues. After checking with Ericsson (WI rapporteur for RAN2), it is suggested to discuss these issues and perhaps also discuss them with other working groups, before progressing on the running 37.320 CR in RAN2.

So this contribution is to provide technical analysis on open issues.

- Discussion

For RAN2, the relevant agreements are copied in section 5. Generally, the running 37.320 CR is to capture impacts due to OAM-centric data collection, while the measurement framework will be captured in the running 38.300 CR.

At RAN2#126 meeting, immediate MDT is used for OAM-centric data collection, and the relevant use case is beam management. In current TS 37.320, some sections are about immediate MDT, which have been listed below:

Existing sections for immediate MDT

5.1.2 Immediate MDT procedures

>5.1.2.1 Measurement configuration

>5.1.2.2 Measurement reporting

>5.1.2.3 MDT context handling during handover and UE context retrieval

5.1.3 MDT initiation

5.1.4 UE capabilities

5.4 NR solutions

> 5.4.0 General

> 5.4.1 RRC_CONNECTED

In general, we think the existing immediate MDT definitions can be re-used for NW-side data collection, and the following aspects can be discussed.

(1) signalling based MDT and management based MDT

There are two cases that RAN should initiate a MDT measurements collection task. One is that the MDT task is initiated without targeting a specific UE by the cell traffic trace, i.e. management based trace function from OAM. The other is that the MDT task is initiated towards a specific UE by the signalling trace activation messages from CN nodes, i.e. the Initial Context Setup message, the Trace Start message or the Handover request message in E-UTRAN or NR, the CN Invoke Trace message in UTRAN. The detailed procedures to transfer the MDT configurations to RAN are specified in TS 32.422.

For NW-side data collection, we think it is sufficient to only adopt management based MDT, due to the following reasons:

(a) from network point of view, the data should be collected from many UEs instead of just one UE. In this case, management based MDT is more reasonable and has less impacts on a specific UE

(b) signalling based MDT involves RAN3, so it may bring some workload to RAN3 if signalling based MDT is also supported for NW-side data collection

Proposal 1: It is proposed to only adopt management based immediate MDT for NW-side data collection.

(2) MR-DC

In current TS 37.320, immediate MDT for MR-DC is supported, and here is the relevant text.

- 5.4.1.3 Immediate MDT for MR-DC Immediate MDT is supported for (NG)EN-DC, NE-DC and NR-DC scenario. Signalling based immediate MDT is triggered for both MN and SN as default, if the MN only indication is provided by OAM, the MDT configuration applies to MN only. In signalling based immediate MDT, MME provides MDT configuration for both MN and SN towards MN including multi RAT SN configuration, specifically E-UTRA and NR MDT configuration. MN then forwards the NR MDT configuration towards SN (EN-DC scenario, SN is always NR). In management-based immediate MDT, OAM provides the MDT configuration to both MN and SN independently. For both MN and SN, Management based MDT should not overwrite signalling based MDT. For immediate MDT configuration, MN and SN can independently configure and receive measurement from the UE. For MN terminated SCG bearer and SN terminated MCG bearer, the terminated node, e.g., MN in case of MN terminated SCG bearer, configures the configuration to UE. For configuring UL PDCP packet average delay (as specified in clause 4.3.1.1 in TS 38.314 [18]) in case of split bearer: only the terminated node of the split bearer can configure the measurement to UE, and the UE reports the measurement result to corresponding node where the configuration was received from.

For NW-side data collection, RAN2 has not discussed MR-DC scenarios. In our opinion, there may be some options:

Option A: immediate MDT for NW-side data collection can support all MR-DC combinations, i.e. (NG)EN-DC, NE-DC and NR-DC scenario

Option B: for the NW-side data collection in NR-DC, only the MN can configure and receive reports from the UE in NR-DC scenario

Option C: for the NW-side data collection in NR-DC, the MN and SN can be configured independently by the OAM, and the MN and SN can independently configure and receive the reports from the UE (as in existing TS 37.320). For this option, we assume that there be no need for co-ordination between MN and SN, and data collection on MN and SN are independent. In addition, it is expected that the following highlighted text are applicable for Option C

- 5.4.1.3 Immediate MDT for MR-DC Immediate MDT is supported for (NG)EN-DC, NE-DC and NR-DC scenario. Signalling based immediate MDT is triggered for both MN and SN as default, if the MN only indication is provided by OAM, the MDT configuration applies to MN only. In signalling based immediate MDT, MME provides MDT configuration for both MN and SN towards MN including multi RAT SN configuration, specifically E-UTRA and NR MDT configuration. MN then forwards the NR MDT configuration towards SN (EN-DC scenario, SN is always NR). In management-based immediate MDT, OAM provides the MDT configuration to both MN and SN independently. For both MN and SN, Management based MDT should not overwrite signalling based MDT. For immediate MDT configuration, MN and SN can independently configure and receive measurement from the UE. For MN terminated SCG bearer and SN terminated MCG bearer, the terminated node, e.g., MN in case of MN terminated SCG bearer, configures the configuration to UE. For configuring UL PDCP packet average delay (as specified in clause 4.3.1.1 in TS 38.314 [18]) in case of split bearer: only the terminated node of the split bearer can configure the measurement to UE, and the UE reports the measurement result to corresponding node where the configuration was received from.

Option A may have the most specification impacts, and Option B puts unnecessary restrictions to network side. Option C is flexible and the specification impacts are moderate, so it is preferred.

Proposal 2: It is proposed to support immediate MDT for NW-side data collection for NR-DC, i.e. for the NW-side data collection in NR-DC scenario, the MN and SN can be configured independently by the OAM, and the MN and SN can independently configure and receive the reports from the UE (as in existing TS 37.320).

For P1 and P2, we think RAN2 can first discuss them, and if agreeable, RAN2 can consult SA5 and RAN3 of our agreements and also check with them if there are any concerns with those agreements.

Proposal 3: It is proposed to send a LS to SA5 and RAN3 to check with them if there are any concerns with RAN2 agreements.

- Conclusion

In this paper, the 37.320 CR rapporteur provides open issue list for the running 37.320 CR, and there are the following proposals:

Proposal 1: It is proposed to only adopt management based immediate MDT for NW-side data collection.

Proposal 2: It is proposed to support immediate MDT for NW-side data collection for NR-DC, i.e. for the NW-side data collection in NR-DC scenario, the MN and SN can be configured independently by the OAM, and the MN and SN can independently configure and receive the reports from the UE (as in existing TS 37.320).

Proposal 3: It is proposed to send a LS to SA5 and RAN3 to check with them if there are any concerns with RAN2 agreements.

For proposal 3, a draft LS has been prepared in [2].

- References

Draft_RAN2_129_Meeting_Report_v1

R2-2502794, Draft LS on OAM-centric solution for NW-side data collection, Huawei

- Relevant RAN2 agreements on OAM-centric solution

RAN2#126

Beam management

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signalling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting

RAN2#125bis

Beam management

Agreements:

- RAN2 to consider an RRC configuration to configure radio measurements and the related reporting to enable data collection for NW-side training

- For AI/ML based beam management, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW side AI/ML model inference.  FFS if further enhancements are needed

- There is no specification impact associated to gNB-side model inference, depending on further RAN1 input.

- FFS whether rhere is specification impact associated to gNB-side model monitoring.

Agreements

1	For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches

2	We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

3	RAN2 supports enhancements to MDT for data collection framework for training.  FSS Whether to enhance logged or immediate MDT