3GPP TSG-RAN WG2 Meeting #129bis	R2-2502799

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.6.3

Source:	Samsung

Title:	Remaining Issues of L1 Event Triggered Measurement Report

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

Based on agreements RAN2 made, event-triggered L1 measurement enhancements for LTM has implemented in the MAC/RRC running CRs [1][2]. In addition, RAN2 had an offline discussion for RRC issues [3] and made proposals to implement the RAN2 agreements into the RRC running CR [2].

In addition, RAN1 sent LS (R1-2501500) [5] on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells to RAN2. This LS include following RAN1 agreement that the semi-persistent CSI-RS resource is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting.

Agreement - Confirm the following working assumption made in RAN1#118bis Working Assumption(RAN1#118bis) In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2

In this contribution, we will further address the above issues on the event triggered L1 measurement reporting and support of semi-persistent CSI-RS activation/deactivation for LTM.

Discussion

Semi-persistent CSI-RS resource activation/deactivation MAC CE

In Rel-19, CSI-RS resources are common measurement for all kind of LTM features, so we think RAN2 first need to make a high-level agreement that this CSI-RS resources including semi-persistent CSI-RS resources are reported by UCI, event-triggered L1 measurement report MAC CE and it is used for evaluation of conditional LTM. We think it is not big deal to support SP CSI-RS resource could be associated with any LTM MR signalling i.e. no impact on the current running CRs, only some association between CSI resource configuration and report configuration need to be checked.

Proposal 1: CSI-RS resources for LTM including semi-persistent CSI-RS resources are reported by UCI, event-triggered L1 measurement report MAC CE, and this resources are used for evaluation of the conditional LTM.

According to the RAN1 LS [5], RAN2 needs to discuss how to support the measurement procedure for semi-persistent CSI-RS resource which could be activated/deactivated by source cell. For this purpose, we think following two RAN2 aspects need to be studied:

- Issue 1: How to handle the activated semi-persistent CSI-RS resources after LTM cell switch (i.e. support the Subsequent LTM)

- Issue 2: LTM SP CSI-RS resource activation/deactivation MAC CE design

For the first issue, we think all Rel-19 LTM features are supported after LTM cell switch (i.e. subsequent LTM) without additional RRC reconfiguration. CSI-RS resource measurement for LTM is supported in Rel-19 and we think it should be supported after LTM cell switch, this measurement of semi-persistent CSI-RS resource are also kind of this CSI-RS resource measurement for LTM. Then, we think it is general for UE to measure the activated semi-persistent CSI-RS resources after LTM cell switch unless NW explicitly signal the deactivation of semi-persistent CSI-RS resources.

However, the legacy semi-persistent CSI-RS resources are only measured in the source cell when it is activated i.e. if semi-persistent CSI-RS resources are activated and the HO is triggered by gNB then UE should stop to measure the activated semi-persistent CSI-RS resources.

In order to support the CSI resource measurement for subsequent LTM, the CSI-RS resource configuration for LTM should be common for all candidate cells, then UE could continuously monitor/measure the CSI-RS resources after LTM cell switch.

For example, following scenario could be considered for this issue:

- Step 1: Semi-persistent CSI-RS resources in candidate cell #1/#2/#3 are activated by the source cell and UE measure/report the activated semi-persistent CSI-RS resources results.

- Step 2: Based on MR, network triggers the LTM cell switch to candidate cell #1.

- Step 3: After cell switch, UE could measure these semi-persistent CSI-RS resources because there was no explicit MAC CE for deactivation i.e. UE consider these resources as activated.

We also think it is also possible that these semi-persistent CSI-RS resources are considered as deactivated when the LTM cell switch is executed, and it could be simple in terms of UE procedure perspective. We summarize two options with some observation for each option.

- Option 1: UE stop to measure the activated semi-persistent CSI-RS resources after cell switch

  - Assume the activated SP CSI-RS resources are not valid after cell switch

  - NW could provide the semi-persistent CSI-RS activation MAC CE for activation after cell switch, if needed

  - Need some rule in both RAN2/3 specifications that the activated semi-persistent CSI-RS resources are considered as deactivated upon LTM cell switch

    - The success of LTM cell switch indication is provided to all candidate cells in order that candidate cell could provide the new state of semi-persistent CSI-RS resources again after cell switch. Some new indication could be added in F1/Xn message.

- Option 2: UE continues to measure the activated semi-persistent CSI-RS resources after cell switch

  - Each candidate cell has provided the state of semi-persistent CSI-RS resources which are associated with the candidate cell, source cell could know the state of these resources

    - In order to support subsequent LTM for this semi-persistent CSI measurement, this state information of semi-persistent CSI-RS resources for each candidate cell could be shared to all candidate cells, then target cell could just associate the activated semi-persistent CSI resource with the report configuration.

    - It means the state of semi-persistent CSI resources should be shared all candidate cells not only for the source cell.

    - However, it makes the signaling complicate to support.

- Option 3: UE stop to measure the activated semi-persistent CSI-RS resources of the candidate cells (i.e. other than target cell) after cell switch. UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch

    - If companies want to handle the activated SP resources for target cell and other candidate cells different, this kind of approach could be considered as well.

Proposal 2: RAN2 decide How to handle the activated semi-persistent CSI-RS resources after LTM cell switch (i.e. support the Subsequent LTM) between following options.

- Option 1: UE stop to measure the activated semi-persistent CSI-RS resources after cell switch

- Option 2: UE continues to measure the activated semi-persistent CSI-RS resources after cell switch

- Option 3: UE stop to measure the activated semi-persistent CSI-RS resources of the candidate cells (i.e. other than target cell) after cell switch. UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch

Regarding the RRC configuration of the semi-persistent CSI-RS resources, the current CSI-RS resource configuration is only configured in the candidate configuration (i.e. LTM-Candidate) and how to associate between the measurement CSI-RS resources and candidate cells in LTM-CSI-ResourceConfig is put as FFS. From our understanding, this CSI-RS resource configuration including semi-persistent CSI resources should be common and the CSI-RS resource configuration should include all resources in candidate cells. Therefore, it should be visible in LTM-CSI-ResourceConfig, the detail signaling should be further determined based on RAN1/2 discussion.

In short, actual resource configuration is per candidate given by ltm-NZP-CSI-RS-ResourceSetToAddModList-r19 and ltm-NZP-CSI-RS-ResourceToAddModList-r19 in LTM-Candidate-r18. In addition we need follow singlaing structure for CSI-RS resource configurations for Rel-19 LTM.

LTM-CSI-RS-ResourceSet-r19  NZP-CSI-RS-ResourceId in LTM-CSI-ResourceConfig-r18 refers to ltm-NZP-CSI-RS-ResourceToAddModList-r19.

Proposal 3: CSI-RS resource configuration for LTM including semi-persistent CSI resources should be common, i.e. semi-persistent CSI resources for all candidate cells should be configured in LTM-CSI-ResourceConfig.

For semi-persistent CSI resources activation/deactivation MAC CE design, the legacy MAC CE for SP CSI-RS/CSI-IM Resource Set Activation/Deactivation could be a baseline but it should be revised considering the LTM feature.

Figure 1 SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE

Since the LTM CSI resource configurations already have the association information of candidate cell index and the resource information, there are no need to provide the serving cell and bwp information in this MAC CE. It is clear to define the A/D field to indicate the activation or deactivation of the SP CSI-RS resources.

Then, how to indicate the activated/deactivated SP CSI-RS resources should be decided, and this is very related with the P3, so we provide following summarized options:

- Option1: Use existing ltm-TCI-Info-r18 in LTM-Candidate for CSI-RS reference configuration

  - Since the ltm-TCI-Info-r18 is used to refer the TCI state of the candidate cell, it is not for the actual resource conifugration.

  - This CSI-RS resources which are configured in ltm-TCI-Info-r18 is only used for TCI state field in the SP CSI-RS resource AD MAC CE.

- Option2: Include a separate CSI-RS reference configuration in LTM-Candidate

  - As already implemented in the RRC running CR, each candidate cell provide the actual CSI-RS configuration in LTM-Candidate and this resource could be directly indicated by SP CSI-RS resource AD MAC CE.

  - If we follow this structure, UE needs to decode all candidate cell configuration and store it, and further this configuration should be used in the source cell. Because UE will be indicated the SP AD MAC CE in the source cell to measure other cell’s SP resources.

  - Candidate cell ID and ltm-NZP-CSI-RS-ResourceConfigId is used for indicating the SP CSI-RS resources

- Option3: Include a separate CSI-RS configuration directly in LTM-CSI-ResourceConfig-r18

  - As we proposed in P3, this structure is good for aligning Rel-18 LTM structure.

  - UE have to measure all resources in all candidate cells without decoding all candidate cell configurations.

  - ltm-CSI-ResourceConfigId (or new LTM SP CSI-RS resource set ID) in LTM-CSI-ResourceConfig-r18 is used for indicating the SP CSI-RS resources.

Plus, CSI-IM resource set is not needed for LTM because L1-SINR as a quantity metric is not supported for Rel-19 LTM, and the TCI state information which is associated with the each indicated SP CSI-RS resources need to be indicated in MAC CE. We think the TCI-StateId indicating which CandidateTCI-State in LTM candidate configuration could be a reference for this field. However, it means UE needs to decode all candidate cell configuration and store it to apply this configuration.

In addition, either “Model A: Resource set is indicated” or “Model B: multiuple resources are indicated” could be considered. The detail MAC CE format could be decided after the RAN1 parameter is fixed.

Proposal 4: SP CSI-RS Activation/Deactivation MAC CE for LTM include following information (see the Figure 2).

- A/D field: indicate either activation or deactivation of the SP CSI resources in this MAC CE

- LTM SP CSI-RS resource set ID field:

  - ltm-CSI-ResourceConfigId (or LTM SP CSI-RS resource set ID) in LTM-CSI-ResourceConfig-r18 is used for indicating the SP CSI-RS resources.

  - Detail reference RS ID would be decided after fixing the RRC structure, Option 1/2/3 could be considered.

    - Option1: Use existing ltm-TCI-Info-r18 in LTM-Candidate for CSI-RS reference configuration

    - Option2: Include a separate CSI-RS reference configuration in LTM-Candidate

    - Option3: Include a separate CSI-RS configuration directly in LTM-CSI-ResourceConfig-r18

- TCI state field: it is associated with the indicated SP CSI-RS resources.

Figure 2. Example of SP CSI-RS Activation/Deactivation MAC CE for LTM (Model A: Resource set and B: 
Resources)

Event-triggered L1 measurement report

In RAN2#129 meeting, RAN2 made many agreements on L1 event-triggered measurement report and most of agreements are applied in the MAC running CR [1]. In this section, we want to discuss remaining issues on L1 Event Triggered Measurement Report MAC CE.

First, RAN2 needs further discussion to make detail signalling for L1 event-triggered measurement report MAC CE:

- Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT

- Explicit indication in MR MAC CE to inform the leaving event condition is met

Since, the explicit RSRP value is used for the first indicated beam but it could not the triggered beam based on the RAN2 decision, and the differential RSRP is used for following beams. Then, we think the easiest signalling would be the one bit indicator for both triggered beam and leaving condition.

- Per beam explicit indication inform which beam(s) meets the event condition with TTT (T field)

- Per beam explicit indication to inform the leaving event condition is met (L field)

Figure 3. Example of event triggered L1 measurement report MAC CE

There might be some redundant signalling for the beam met the leaving condition for above approach but it is straightforward to easy understanding when UE receives the MAC CE.

Proposal 5: Event triggered L1 measurement report MAC CE includes:

- Per beam explicit indication inform which beam(s) meets the event condition with TTT (T field)

- Per beam explicit indication to inform the leaving event condition is met (L field)

RAN2 made following agreements for truncated event triggered L1 measurement report MAC CE in RAN2#129 meeting.

- Truncated MR MAC CE includes the triggered beam information:

- One report config ID (event config ID)

- At least one triggered beam id (SSBRI or CRI) + L1 RSRP

- Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur.

- For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used.

Then the detail procedure for UE after UE send the truncated event triggered L1 measurement report MAC CE should be determined.

From our understanding, there are two candidate UE behaviour:

- UE behaviour 1: UE cancel to send the (truncated) MR MAC CE similar as BFR MAC CE

  - NW doesn’t need to provide the additional uplink grant for remaining information of Truncated LTM MR MAC CE.

  - If UE send the MAC PDU including any (Truncated) MR MAC CE to the NW, UE considers the LTM MR procedure is completed and cancel the LTM MR procedure for the reported ID.

  - How to handle this complete report ID:

    - Option 1: UE will not trigger this event again, if needed NW re-configure this event again.

    - Option 2: UE re-evaluate this event for LTM MR, and if it is triggered, UE try to send the new LTM MR MAC CE (not for the remaining information of Truncated LTM MR MAC CE)

    - Option 3: If prohibit timer will be introduced, the same report ID could be re-evaluated from UE side and reported if the valid MR is triggered.

- UE behaviour 2: UE pending to send the (truncated) MR MAC CE similar as BSR MAC CE

  - It is beneficial to provide all information of triggered/configured LTM MR MAC CE for the configured/measured event.

  - After sending the truncated MR MAC CE, UE just wait the UL grant. If NW provides the UL grant then UE send the (truncated) MR MAC CE.

  - Which MAC CE format is used after sending the first LTM MAC CE?

    - Option 1: Only provide the remaining part/information of LTM MR MAC CE i.e. Truncated LTM MR MAC CE

    - Option 2: Provide the full set of information of LTM MR MAC CE i.e. LTM MR MAC CE

From our understanding the truncated event triggered L1 measurement report MAC CE provides at least one set of full information of triggered beam, so it might be enough for the network to use this measurement report to decide the cell switch. We think this truncated MR MAC CE scenario is not the normal case, so the specification impact to handle this corner case is minimized. Based on our observation for each case, canceling to send the (truncated) MR MAC CE after UE send the (truncated) MR MAC CE is simple i.e. No further UE behavior and the network doesn’t need to provide the additional uplink grant for remaining information of Truncated LTM MR MAC CE.

Proposal 6: UE considers the LTM MR procedure is completed and cancel the LTM MR procedure after UE send the truncated event triggered L1 measurement report MAC CE for the reported ID.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: CSI-RS resources for LTM including semi-persistent CSI-RS resources are reported by UCI, event-triggered L1 measurement report MAC CE, and this resources are used for evaluation of the conditional LTM.

Proposal 2: RAN2 decide How to handle the activated SP CSI-RS resources after LTM cell switch (i.e. support the Subsequent LTM) between following options.

- Option 1: UE stop to measure the activated semi-persistent CSI-RS resources after cell switch

- Option 2: UE continues to measure the activated semi-persistent CSI-RS resources after cell switch

- Option 3: UE stop to measure the activated semi-persistent CSI-RS resources of the candidate cells (i.e. other than target cell) after cell switch. UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch

Proposal 3: CSI-RS resource configuration for LTM including semi-persistent CSI resources should be common, i.e. semi-persistent CSI resources for all candidate cells should be configured in LTM-CSI-ResourceConfig.

Proposal 4: SP CSI-RS Activation/Deactivation MAC CE for LTM include following information (see the Figure 2).

- A/D field: indicate either activation or deactivation of the SP CSI resources in this MAC CE

- LTM SP CSI-RS resource set ID field:

  - ltm-CSI-ResourceConfigId (or LTM SP CSI-RS resource set ID) in LTM-CSI-ResourceConfig-r18 is used for indicating the SP CSI-RS resources.

  - Detail reference RS ID would be decided after fixing the RRC structure, Option 1/2/3 could be considered.

    - Option1: Use existing ltm-TCI-Info-r18 in LTM-Candidate for CSI-RS reference configuration

    - Option2: Include a separate CSI-RS reference configuration in LTM-Candidate

    - Option3: Include a separate CSI-RS configuration directly in LTM-CSI-ResourceConfig-r18

- TCI state field: it is associated with the indicated SP CSI-RS resources.

Proposal 5: Event triggered L1 measurement report MAC CE includes:

- Per beam explicit indication inform which beam(s) meets the event condition with TTT (T field)

- Per beam explicit indication to inform the leaving event condition is met (L field)

Proposal 6: UE considers the LTM MR procedure is completed and cancel the LTM MR procedure after UE send the truncated event triggered L1 measurement report MAC CE for the reported ID.

References

[1]	R2-250xxxx		Running MAC CR for enhanced mobility Ph4	vivo	38.321	18.4.0		NR_Mob_Ph4-Core	Rel-19.

[2]	R2-250xxxx		Introduction of event-triggered measurement reporting for RRC spec	Huawei	38.331	18.4.0		NR_Mob_Ph4-Core	Rel-19.

[3]	R2-250xxxx		Summary of [POST128][108][MOB] RRC running CR (Huawei)	Huawei.

[4]	R2-2406021		Enhancements to measurement report [meas_report_enh]	TEI18		38.331	4803	Rel-18.

[5]	R1- 2501500	LS on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells	Rel-19	NR_Mob_Ph4-Core, 	Source: RAN1	To: RAN2	CC: RAN3.