3GPP TSG-RAN WG2 Meeting #129bis	R2-2502801

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.11.3

Source:	Samsung

Title:	Other Aspects of SBFD

WID/SID:	NR_duplex_evo-Core

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss other aspects/objective which are not directly related to RA operation, e.g. DC support and CLI and CSI-RS measurement in SBFD.

Discussion

CLI measurement and reporting overall

Just for clarification, below issues are also handled in RRC offline discussion below, and we provide the view in this offline. This contribution is used to explain our comment more detail.

- [Post129][217][SBFD] List of open issues of RRC impact (Huawei)

Intended outcome: List of open issues of RRC impact to submit to the next meeting, taking into account related proposals in this meeting

Deadline:  Long

It is necessary to measure and report CLI in symbol level time scale since SBFD configuration can be per symbol level. L3 time average blur the exact CLI level in smaller time scale so the L1 based CLI measurement operation for SBFD is needed. From our understanding, RAN1 considers this principle and we provide the overall procedure to support the L1 based UE-to-UE CLI measurement and reporting.

- Step 1: Exchange of resource information to measure CLI between gNBs

Victim gNB receives the SRS resource configuration that aggressor gNB will assign to the aggressor UE via Xn message which can be used for SBFD configuration delivery. Once victim gNB receives the SRS resource configuration for the aggressor gNB, it should adjust or translate the received resource from aggressor cell to the victim cell resource:

- Only overlapped resources can be used for CLI resource (inter-freq case);

- Each SRS or CLI resources should be indicated in one of serving cells for victim UE, and using DL BWP of that cell, which includes those SRS or CLI resource configuration.

- Above adjusted / translated resource information should be signalled to the victim UE by victim gNB

- Step 2: CLI measurement configuration at (victim) gNB

- Serving cell configures resources for CLI measurement to a specific UE (possible victim UE)

  - Measuring quantity: SRS-RSRP, CLI-RSSI same as legacy L3 based CLI resources.

    - SRS-RSRP: Victim UE will measure aggressor UE’s SRS transmission

    - CLI-RSSI: Victim UE will measure aggressor UE’s UL transmission plus any interference power on its DL reception resource.

      - This resource is determined by victim gNB and there is no explicit exchange of TX resource from aggressor gNB to the victim gNB.

    - Etc.

  - Each measurement resource would be either periodic resource, semi-persistent resource or aperiodic resource.

- Based on resource type above, measurement report is at least aperiodic report on PUSCH; FFS for other reporting (e.g. semi-persistent, periodic reporting)

In RAN1#118bis meeting, RAN1 made following agreements for CLI handling which has RAN2 impact on CLI measurement and reporting in SBFD. RAN1 agreed more detail operation but we only captures the CLI measurement resource and report configuration aspects.

Agreement Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement. - The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic - The number of periodic/semi-persistent CLI measurement resource sets is restricted to 1 in a CSI-ResourceConfig as in current specifications. - Note: No new usage needs to be defined for SRS resource sets Agreement For aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured is configured by higher layer parameter. - FFS: Whether to reuse the legacy SRS resource set - FFS: Whether available slot offset list is also reused For aperiodic CLI-RSSP reporting using aperiodic CLI-RSSI measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured is configured by higher layer parameter. Agreement For aperiodic CLI reporting on PUSCH - Reuse CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig - Reuse the existing DCI field 'CSI request' to trigger an aperiodic CLI report - Introduce an indication in CSI-AssociatedReportConfigInfo that selects a set from the list of CLI-RSSI measurement resource sets or SRS-RSRP measurement resource sets configured in CSI-ResourceConfig - Note: As in current specifications, the parameter for indication (e.g., resourceSet) is always present, but only used in the case that the CSI-ResourceConfig is configured with multiple sets of aperiodic CLI measurement resources. Agreement In CSI-ReportConfig, reuse resourcesForChannelMeasurement providing the ID of a CSI-ResourceConfig that contains configuration of measurement resources for L1-SRS-RSRP and/or L1-CLI-RSSI.

The legacy CSI framework is consist of Report setting, Resource setting and Trigger state:

- Resource setting (CSI-ResourceConfig): what signal to use to compute CSI

- Report setting (CSI-ReportConfig): what CSI to report and when to report it

- Trigger state (CSI-AperiodicTriggerStateList): associates CSI report and CSI resource settings

Below sections provides the detail signalling aspect to support CLI measurement resource and report configuration using CSI framework. We agree that the detail RAN2 discussion will be based on further RAN1 impact but we think it is good to know before RAN2 get the request from RAN1 for stage3 work easier.

CLI resource configuration

Based on above RAN1 agreements, it is confirmed that two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI could be configured in CSI-ResourceConfig and the detail configuration follows the Rel-16 CLI measurement resource configuration. We provide the ASN.1 signalling based on RAN1 agreements as an example and this could be the baseline on further extension i.e. detail parameter values could be revised based on RAN1 parameters which will be provided later. Please see the TP in Annex 5.1.

In addition, we provide the issue list for CLI resource configuration in terms of ASN.1 signalling:

- If the CLI measurement resource set provides both SRS-RSRP and CLI-RSSI (if yes, this resource set use the SEQ structure, otherwise use the CHOICE structure)

- How to handle the legacy mandatory field (csi-RS-ResourceSetList) in CSI-ResourceConfig IE?

  - Additional restriction to ignore the legacy mandatory field if the new CLI measurement resource set could be provided in the field description.

- What is the additional parameters for CLI measurement resource set lists for SRS-RSRP and CLI-RSSI.

Proposal 1: RAN2 introduce the CLI measurement resource configuration considering following issues, TP in this contribution could be used for baseline.

- If the CLI measurement resource set provides both SRS-RSRP and CLI-RSSI (if yes, this resource set use the SEQ structure, otherwise use the CHOICE structure)

- How to handle the legacy mandatory field (csi-RS-ResourceSetList) in CSI-ResourceConfig IE?

  - Additional restriction to ignore the legacy mandatory field if the new CLI measurement resource set could be provided in the field description.

- What is the additional parameters for CLI measurement resource set lists for SRS-RSRP and CLI-RSSI.

We assume that this CLI measurement resource configuration is the new configuration including the SRS-RSRP and CLI-RSSI resource/resource set for CLI, and it should support periodic, semi-persistent, aperiodic CLI RS transmission. For periodic, we think UE just follows the RRC configuration if the CLI resources are configured to periodic transmission. For aperiodic, UE needs to know which resource is activated and UE has to measure based on the signaling, e.g. DCI, we assume that RAN1 will define how DCI indicate the aperiodic CLI resources either by new DCI or new field in legacy DCI. For semi-persistent CLI resource, we think the new MAC CE signaling to activate/deactivate the CLI resources which UE have to measure is required and following information could be added.

- A/D field: indicates the activate or deactivate of the indicated CLI resources

- Source cell/BWP ID which the CLI resources configuration is provided

- CLI measurement resource type (either SRS-RSRP or CSI-RSSI)

- CLI measurement resource set ID

- CLI measurement resource(s) ID

Proposal 2: RAN2 introduce the new MAC CE for semi-persistent CLI resource activation/deactivation which include at least following information.

- A/D field: indicates the activate or deactivate of the indicated CLI resources

- Source cell/BWP ID which the CLI resources configuration is provided

- CLI measurement resource type (either SRS-RSRP or CSI-RSSI)

- CLI measurement resource set ID

- CLI measurement resource(s) ID

CLI report configuration

If we follow the legacy CSI report framework by L1 signaling, following approach could be reused for the L1 based UE-to-UE CLI measurement and reporting as well. If network determines the RS type, then it will further determine the report type available for the RS type. This measurement resource and report configuration consist of a single measurement instance, i.e., ID, which can be referred by the DL MAC CE/DCI for its corresponding operation.

Table 1. L1 based CLI measurement and report configuration

| Resource Configuration | Report Configuration |  |  |
| --- | --- | --- | --- |
| RS Configuration | Periodic report | Semi-Persistent report | Aperiodic report |
| Periodic RS | No dynamic triggering/activation | Reporting on PUCCH : Triggering by MAC CE Reporting on PUSCH : Triggering by DCI | Triggered by DCI, additionally by MAC CE |
| Semi Persistent RS | Not Supported | Reporting on PUCCH : Triggering by MAC CE Reporting on PUSCH : Triggering by DCI | Triggered by DCI, additionally by MAC CE |
| Aperiodic RS | Not Supported | Not Supported | Triggered by DCI, additionally by MAC CE |

However, only aperiodic CLI measurement report was agreed now based on the RAN1 and RAN2 agreements, and other reporting types (e.g. periodic, semi-persistent) are FFS. From our understanding, those types of CLI reporting are deprioritized in this release and they can be introduced if time allows (or find critical reason to support them).

For aperiodic CLI reporting on PUSCH, RAN1 agreed that CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig is resued. From our understanding, it also means that the legacy CSI-ReportConfig IE and CSI-AperiodicTriggerStateList IEs are used for CLI measurement report configuration.

For detail configuration for CLI measurement report which could be configured under CSI-ReportConfig IE could be provided by RAN1 but we think the legacy configuration could be reused as much as possible i.e. additional parameters are only provided. For example, the two new report entities for SRS-RSRP and CLI-RSSI could be added in reportQuantity, and some restrictions of usage of reportConfigType could be added in the field description, such that “for SBFD CLI measurement report configuration, only aperiodic is set”. In addition, if the resourcesForChannelMeasurement is used (as RAN1 agreement) for referring the CLI resource configuration or the new field to refer the CLI resource configuration is needed, and how to handle the legacy mandatory field resourcesForChannelMeasurement. For example, if the resourcesForChannelMeasurement is used for referring the CLI resource configuration, the new description that this field is used for CLI resource configuration is needed. However, if RAN1/2 decides the new field to refer the CLI resource configuration is needed, the legacy mandatory field is ignored when the new field is present.

CSI-ReportConfig ::=                SEQUENCE {

reportConfigId                          CSI-ReportConfigId,

carrier                                 ServCellIndex                   OPTIONAL,   -- Need S

resourcesForChannelMeasurement          CSI-ResourceConfigId,

csi-IM-ResourcesForInterference         CSI-ResourceConfigId            OPTIONAL,   -- Need R

nzp-CSI-RS-ResourcesForInterference     CSI-ResourceConfigId            OPTIONAL,   -- Need R

reportConfigType                        CHOICE {

periodic                                SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH                   SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH                   SEQUENCE {

reportSlotConfig                        ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

reportSlotOffsetList                SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32),

p0alpha                                 P0-PUSCH-AlphaSetId

},

aperiodic                               SEQUENCE {

reportSlotOffsetList                SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32)

}

},

reportQuantity                          CHOICE {

none                                    NULL,

cri-RI-PMI-CQI                          NULL,

cri-RI-i1                               NULL,

cri-RI-i1-CQI                           SEQUENCE {

pdsch-BundleSizeForCSI                  ENUMERATED {n2, n4}                                         OPTIONAL    -- Need S

},

cri-RI-CQI                              NULL,

cri-RSRP                                NULL,

ssb-Index-RSRP                          NULL,

cri-RI-LI-PMI-CQI                       NULL

},

Proposal 3: RAN2 assume the legacy CSI-ReportConfig is reused for CLI measurement report configuration, detail parameters are FFS based on RAN1 agreements and following could be considered to be added.

- If the new report entities for SRS-RSRP and CLI-RSSI are needed in reportQuantity.

- What is the slot offset value between the slot containing the DCI and the aperiodic CLI measurement report.

- Usage of reportConfigType could be added in the field description, such that “for SBFD CLI measurement report configuration, only aperiodic is set”.

- If the resourcesForChannelMeasurement is used for referring the CLI resource configuration or the new field to refer the CLI resource configuration is needed.

  - how to handle the legacy mandatory field resourcesForChannelMeasurement.

For CSI-AperiodicTriggerStateList, the Figure 1 provides the simplified procedure for legacy aperiodic CSI report. Based on the legacy procedure, the new resource indicator for CLI measurement resource instead of resourceForChannel is needed if the legacy CSI-AperiodicTriggerStateList IE is reused for CLI aperiodic reporting configuration. In that case, the legacy mandatory resourceForChannel field should be ignored if the new resourceForChannel-r19 (for SBFD CLI resource) is present.

Figure 1. CSI aperiodic triggering state

CSI-AperiodicTriggerStateList ::=   SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState

CSI-AperiodicTriggerState ::=       SEQUENCE {

associatedReportConfigInfoList      SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo,

...,

[[

ap-CSI-MultiplexingMode-r17         ENUMERATED {enabled}                                          OPTIONAL  -- Need R

]],

[[

ltm-AssociatedReportConfigInfo-r18  LTM-CSI-ReportConfigId-r18                                    OPTIONAL  -- Need R

]]

}

CSI-AssociatedReportConfigInfo ::=  SEQUENCE {

reportConfigId                      CSI-ReportConfigId,

resourcesForChannel                 CHOICE {

nzp-CSI-RS                          SEQUENCE {

resourceSet                         INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info                            SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId

OPTIONAL  -- Cond Aperiodic

},

csi-SSB-ResourceSet                 INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfig)

},

csi-IM-ResourcesForInterference     INTEGER(1..maxNrofCSI-IM-ResourceSetsPerConfig)               OPTIONAL, -- Cond CSI-IM-ForInterference

nzp-CSI-RS-ResourcesForInterference INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)          OPTIONAL, -- Cond NZP-CSI-RS-ForInterference

...,

[[

resourcesForChannel2-r17        CHOICE {

nzp-CSI-RS2-r17                 SEQUENCE {

resourceSet2-r17                INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info2-r17                   SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId

OPTIONAL   -- Cond Aperiodic

},

csi-SSB-ResourceSet2-r17        INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfigExt)

}                                                                                             OPTIONAL,  -- Need R

csi-SSB-ResourceSetExt          INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfigExt)          OPTIONAL   -- Need R

]],

[[

resourcesForChannelTDCP-r18     SEQUENCE {

resourceSet2TDCP-r18        INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

resourceSet3TDCP-r18        INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)              OPTIONAL  -- Need R

}                                                                                                     OPTIONAL,  -- Cond TDCP

applyIndicatedTCI-State-r18     CHOICE {

perSet-r18                      ENUMERATED {first, second},

perResource-r18                 SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF ENUMERATED {first, second}

}                                                                                                OPTIONAL, -- Cond ApplyIndicatedTCI

applyIndicatedTCI-State2-r18    CHOICE {

perSet-r18                     ENUMERATED {first, second},

perResource-r18                SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF  ENUMERATED {first, second}

}                                                                                                     OPTIONAL, -- Cond SecondCSICMR

csi-ReportSubConfigTriggerList-r18  CSI-ReportSubConfigTriggerList-r18                                OPTIONAL   -- Need R

]]

}

Proposal 4: RAN2 assume the legacy CSI-AperiodicTriggerStateList is reused for the association of CLI measurement resource and report configuration, detail parameters are FFS based on RAN1 agreements and following could be considered to be added.

- Introduce the new resourceForChannel-r19 for SBFD CLI resource association.

- Add the text to ignore the legacy mandatory field of resourceForChannel.

Since the new RRC configuration is added in the legacy CSI-AperiodicTriggerStateList, the legacy Aperiodic CSI Trigger State Subselection MAC CE is reused to indicates the selection status of the Aperiodic Trigger States configured for CLI measurement report, i.e. no new Aperiodic CSI Trigger State Subselection MAC CE is needed for CLI purpose.

Proposal 5: RAN2 confirm that the legacy Aperiodic CSI Trigger State Subselection MAC CE is also used to indicate the selection status of the Aperiodic Trigger States configured for aperiodic CLI measurement report.

In addition, the SRS configuration at the aggressor gNB is dynamic because this SRS configuration is dedicated configuration per UE. Plus, the SRS activation at the aggressor gNB would be dynamic because this SRS resource from other cells are not configured by periodic manner. In this case, the exchange of the information on the activation/deactivation of SRS at aggressor gNB is not easy to be exchanged with the victim gNB. For handling this, the information exchanged between gNBs can further information for this SRS configurations but this is not a scope of this release based on the WID as below.

  - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs

Based on this information, RP meeting confirmed that the enhancement of SRS configuration coordination is not supported in Rel-19, it means that SRS CLI measurement on SP/AP resources would be only possible in intra-CU case (i.e., no Xn signaling needed) in this release.

Proposal 6: RAN2 confirm that exchange of the information on the activation/deactivation of SRS between gNBs is not supported in Rel-19.

CSI-RS measurement impact

From RAN4 RRM requiremements perstive, follow SSB based and/or CSI-RS based measurement requirements are specified as summarized in below table:

Table 1: SSB and/or CSI-RS related measurement in TS 38.133

| Serving cell RRM measurement |  |  | Neighbouring cell measurement |
| --- | --- | --- | --- |
| L3 RRM measurement | L1-BM (L1-RSRP, L1-SINR) with in CSI framework | L1 RLM L1 BFD/BFR | L3 RRM measurement |

For the RRM measurement impact, RAN1 made the below agreement for CSI reporting.

Agreement

For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options.

- Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only.

  - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only.

    - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation.

- Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD

Based on the above agreements, UE is configured with the separate CSI report on SBFD and non-SBFD, it means UE monitors the CSI-RS resources in SBFD and non-SBFD, separately and the results will be reported to the gNB. The reason why this independent CSI measurement/report is needed between SBFD and non-SBFD symbols is that the UE may experience different propagation characteristics between SBFD symbols and non SBFD symbols.

Then, the next question is how the RAN2 impacts if this separate CSI measurement and report for SBFD is introduced. We think following two features need to be considered:

- Issue 1: Separate BFD/BFR procedure for SBFD symbols

  - Main objective of BFD/BFR is checking the radio link monitoring and we think even the separate link quality is monitored by SBFD symbols, there will be legacy CSI monitoring for BFD/BFR

  - SBFD resources could be used for BFD/BFR configuration perspective, but we think the new procedure to handle this SBFD resources separately is not needed.

  - In [2], following proposals are provided, and it could be just NW configuration impact (i.e. no specification change is needed)

    - Keep the legacy BFD and BFR procedure in the MAC layer, i.e., no need for the MAC entity to trigger BFD and BFR for non SBFD symbols and SBFD symbols separately

    - The network configures the valid symbol type for CSI-RS based RLM/BFD/CBD measurement i.e. SBFD symbols could be used for this RLM/BFD/CBD measurement

- Issue 2: Separate RRM measurement

  - Based on the above RAN1 agreements, L1 CSI-RS resources for SBFD symbols could be measured separately, then it is also possible the separate CSI-RS measurement is used for L3 measurement (i.e. RRM manner).

  - It is good for gNB to know the separate measurement report for SBFD in order to decide the mobility (i.e. HO), the how to support could be considered.

    - E.g. MeasObject and/or ReportConfig configures the separate CSI-RS resources in SBFD symbol which is different with the measurement in non-SBFD symbols.

  - In short, the measurement of L1 CSI-RS resources for SBFD symbols separately is already supported, then only the new thing is reporting the measured results for L3 RRM manner. We think this feature has no big spec. impact and RAN2 could decide it.

Proposal 7: RAN2 does not support the separate BFD/BFR procedure for SBFD symbols but below proposal in [2] could be used for enhancement of BFD/BFR for SBFD without specification impact.

- Keep the legacy BFD and BFR procedure in the MAC layer, i.e., no need for the MAC entity to trigger BFD and BFR for non SBFD symbols and SBFD symbols separately

- The network configures the valid symbol type for CSI-RS based RLM/BFD/CBD measurement i.e. SBFD symbols could be used for this RLM/BFD/CBD measurement.

Proposal 8: RAN2 consider to support the separate Layer3 measurement report for CSI-RS resources in SBFD symbol.

SP CLI Measurement Resource Set Activation/Deactivation MAC CE

Based on RAN2 agreement, the current MAC running CR for SBFD [3] just capture the 1.1	SP CLI Measurement Resource Set Activation/Deactivation MAC CE but details are not added. In this section, we provide the proposals to solve the below two FFS.

Editor’s Note: FFS the initial states of the configured SP CLI measurement resource sets upon (re-)configuration by upper layers and after reconfiguration with sync.

Editor’s Note: FFS MAC CE format and field descriptions.

Since the semi-persistent CLI resource set activation/deactivation feature is similar with the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE. Only difference would be the CLI resource set is used instead of CSI-RS/CSI-IM resource set.

For the first issue, the initial state of the SP CLI measurement resource sets could be deactivated upon (re-)configuration by upper layers and after reconfiguration with sync. It is same with the UE operation for SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE, and we think it is reasonable. If we want more flexible on this feature we can provide the RRC parameter to indicate the initial state of the SP CLI measurement resource sets but we think it is too much.

Proposal 9: The configured SP CLI measurement resource sets are initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync.

For the MAC CE format and field descriptions, the details could be fixed if the RRC parameters are implemented but some high-level design and fields for this MAC CE are considered.

- A/D field: it is needed to indicate the activate or deactivate for the indicated SP CLI resource set ID in this MAC CE

- SP CLI resource set ID: We assume this SP CLI resource set ID could be signalled one of following options

  - Option 1: csi-ResourceConfigId, if the SP CLI resource set is configured in CSI-ResourceConfig

  - Option 2: SRS-RSRP resource set ID (srs-RSRP-MeasurementResourceSetId) or CLI-RSSI resource set ID (cli-RSSI-MeasurementResourceSetId), if SP CLI resource set is configured in CSI-MeasConfig.

  - The size of this field could be further determined based on the RAN1 decision. Here we use the 7 bits as an example.

- TCI State ID: TCI state what the CLI resource is associated with the actual CLI resources in CLI resource set.

  - For example, if the CLI resource set #1 in cli-MeasurementResourceSetList includes three SRS-RSRP resources then three TCI states are need to be signalled in this MAC CE to indicate the associated TCI state for SRS-RSRP resources.

  - Based on RAN1 decision, this TCI state ID field could be removed.

Figure 2. Example of SP CLI Measurement Resource Set Activation/Deactivation MAC CE

Proposal 10: RAN2 consider the above MAC CE format for SP CLI Measurement Resource Set Activation/Deactivation as a baseline.

Conclusion

The P1 to P5 is Stage3 (ASN.1) related proposals so it could be used for the reference of the Stage3 work.

Proposal 1: RAN2 introduce the CLI measurement resource configuration considering following issues, TP in this contribution could be used for baseline.

- If the CLI measurement resource set provides both SRS-RSRP and CLI-RSSI (if yes, this resource set use the SEQ structure, otherwise use the CHOICE structure)

- How to handle the legacy mandatory field (csi-RS-ResourceSetList) in CSI-ResourceConfig IE?

  - Additional restriction to ignore the legacy mandatory field if the new CLI measurement resource set could be provided in the field description.

- What is the additional parameters for CLI measurement resource set lists for SRS-RSRP and CLI-RSSI.

Proposal 2: RAN2 introduce the new MAC CE for semi-persistent CLI resource activation/deactivation which include at least following information.

- A/D field: indicates the activate or deactivate of the indicated CLI resources

- Source cell/BWP ID which the CLI resources configuration is provided

- CLI measurement resource type (either SRS-RSRP or CSI-RSSI)

- CLI measurement resource set ID

- CLI measurement resource(s) ID

Proposal 3: RAN2 assume the legacy CSI-ReportConfig is reused for CLI measurement report configuration, detail parameters are FFS based on RAN1 agreements and following could be considered to be added.

- If the new report entities for SRS-RSRP and CLI-RSSI are needed in reportQuantity.

- What is the slot offset value between the slot containing the DCI and the aperiodic CLI measurement report

- Usage of reportConfigType could be added in the field description, such that “for SBFD CLI measurement report configuration, only aperiodic is set”.

- If the resourcesForChannelMeasurement is used for referring the CLI resource configuration or the new field to refer the CLI resource configuration is needed.

  - how to handle the legacy mandatory field resourcesForChannelMeasurement.

Proposal 4: RAN2 assume the legacy CSI-AperiodicTriggerStateList is reused for the association of CLI measurement resource and report configuration, detail parameters are FFS based on RAN1 agreements and following could be considered to be added.

- Introduce the new resourceForChannel-r19 for SBFD CLI resource association.

- Add the text to ignore the legacy mandatory field of resourceForChannel.

Proposal 5: RAN2 confirm that the legacy Aperiodic CSI Trigger State Subselection MAC CE is also used to indicate the selection status of the Aperiodic Trigger States configured for aperiodic CLI measurement report.

Proposal 6: RAN2 confirm that exchange of the information for the activation/deactivation of SRS resources between gNBs is not supported in Rel-19.

Proposal 7: RAN2 does not support the separate BFD/BFR procedure for SBFD symbols but below proposal in [2] could be used for enhancement of BFD/BFR for SBFD without specification impact.

- Keep the legacy BFD and BFR procedure in the MAC layer, i.e., no need for the MAC entity to trigger BFD and BFR for non SBFD symbols and SBFD symbols separately

- The network configures the valid symbol type for CSI-RS based RLM/BFD/CBD measurement i.e. SBFD symbols could be used for this RLM/BFD/CBD measurement.

Proposal 8: RAN2 consider to support the separate Layer3 measurement report for CSI-RS resources in SBFD symbol.

Proposal 9: The configured SP CLI measurement resource sets are initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync.

Proposal 10: RAN2 consider the above MAC CE format for SP CLI Measurement Resource Set Activation/Deactivation as a baseline.

References

- RP‑241614	Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD); RAN#104

- R2-2500885	CSI-RS measurements for RLM/BFD/CBD in SBFD		Ericsson

- R2-250xxxx	MAC running CR for Evolution of NR duplex operation: Sub-band full duplex (SBFD)		Rel-19	NR_duplex_evo-Core		Samsung.

- R1-2501646		Collection of Rel-19 higher layers parameters list		Post RAN1#120.

Annex - TP

CLI resource configuration

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId  OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

bwp-Id                      BWP-Id,

resourceType                ENUMERATED { aperiodic, semiPersistent, periodic },

...,

[[

csi-SSB-ResourceSetListExt-r17      CSI-SSB-ResourceSetId                                                  OPTIONAL  -- Need R

]],

[[

cli-SBFD-ResourceSetList-r19       CHOICE {

cli-SRS-ResourceSetList-r19   SEQUENCE (SIZE (1..FFS)) OF CLI-SRS-ResourceSetId-r19

cli-CLI-ResourceSetList-r19   SEQUENCE (SIZE (1..FFS)) OF CLI-RSSI-ResourceSetId-r19

}                                                                                                         OPTIONAL  -- Need R

]]

}

CLI-SRS-ResourceSet-r19 ::=   SEQUENCE {

cli-srs-ResourceSetId                       CLI-SRS-ResourceSetId,

cli-srs-ResourceIdList (SIZE (1.. maxNrofCLI-SRS-Resources-r16)) OF SRS-ResourceConfigCLI-r19             OPTIONAL,  -- Need M

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

FFS

...

},

semi-persistent                         SEQUENCE {

FFS

...

},

periodic                                SEQUENCE {

FFS

...

}

},

FFS

}

CLI-SRS-ResourceSetId ::=                   INTEGER (0..FFS)

CLI-RSSI-ResourceSet-r19::=   SEQUENCE {

cli-rssi-ResourceSetId                       CLI-RSSI-ResourceSetId,

cli-rssi-ResourceIdList (SIZE (1.. maxNrofCLI-SRS-Resources-r16)) OF RSSI-ResourceConfigCLI-r19             OPTIONAL,  -- Need M

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

FFS

...

},

semi-persistent                         SEQUENCE {

FFS

...

},

periodic                                SEQUENCE {

FFS

...

}

},

FFS

}

CLI-RSSI-ResourceSetId::=                   INTEGER (0..FFS)

SRS-ResourceConfigCLI-r19 ::=       SEQUENCE {

srs-Resource-r19                    SRS-Resource,

srs-SCS-r19                         SubcarrierSpacing,

refServCellIndex-r19                ServCellIndex                                                  OPTIONAL,   -- Need S

refBWP-r19                          BWP-Id,

...

}

RSSI-ResourceConfigCLI-r19 ::=      SEQUENCE {

rssi-ResourceId-r19                 RSSI-ResourceId-r19,

rssi-SCS-r19                        SubcarrierSpacing,

startPRB-r19                        INTEGER (0..2169),

nrofPRBs-r19                        INTEGER (4..maxNrofPhysicalResourceBlocksPlus1),

startPosition-r19                   INTEGER (0..13),

nrofSymbols-r19                     INTEGER (1..14),

rssi-PeriodicityAndOffset-r19       RSSI-PeriodicityAndOffset-r19,

refServCellIndex-r19                ServCellIndex                                                  OPTIONAL,   -- Need S

...

}

RSSI-ResourceId-r19 ::=             INTEGER (0.. maxNrofCLI-RSSI-Resources-1-r16)

RSSI-PeriodicityAndOffset-r19 ::=   CHOICE {

sl10                                INTEGER(0..9),

sl20                                INTEGER(0..19),

sl40                                INTEGER(0..39),

sl80                                INTEGER(0..79),

sl160                               INTEGER(0..159),

sl320                               INTEGER(0..319),

s1640                               INTEGER(0..639),

...

}