3GPP TSG-RAN WG2 Meeting #129bis		       R2-2502817

Wuhan, China: 7th April – 11th April 2025

Agenda Item:	8.1.3

Source:	ASUSTeK

Title:	Discussion on NW-sided data collection

Document for:	Discussion and Decision

- Introduction

The WI NR_AIML_air agreed to work on the signalling mechanism of data collection for NW-sided models. We would like to address some of the issues related to the data collection for NW-sided models in this document.

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models

- Discussion

Agreements for beam management (RAN2 #126) 1 For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement. 2 Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model 3 Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting

(RAN1 #120) Agreement For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: whether/how to support 'aperiodic' CSI RS Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

In the previous meetings, RAN2 agreed to enhance the immediate MDT framework to support periodical reporting and support logging optional feature. RAN1 also agreed that for UE side data collection, the CSI-ReportConfig is reused. However, how to configure for periodic report and/or logging for NW side data collection is unclear.

For configure periodic reporting for NW side data collection, there are 2 options with their pros and cons respectively:

- Option 1: reuse the CSI framework

- Option 2: enhance the RRM measurement framework

For option 1:

- Pros: minimal spec impact, aligns with UE side data collection, aligns with the input for inference

- Cons: the signalling for the report of logging may not be aligned

For option 2:

- Pros: can design dedicated signalling for general data collection purposes

- Cons: large spec impact (e.g., to align with the input for inference), not aligned with other RAN1 AI/ML features (e.g., monitoring, UE side data collection)

Therefore, we propose:

Proposal 1: For configure periodic reporting for NW side data collection, reuse the CSI framework or CSI-ReportConfig to minimize spec impact.

Also, it is not clear whether periodic reporting for NW side data collection is transmitted via PUCCH, PUSCH or RRC. Therefore, we propose:

Proposal 2: RAN2 clarify whether periodic reporting for NW side data collection is transmitted via PUCCH, PUSCH or RRC.

- Conclusion

Proposal 1: For configure periodic reporting for NW side data collection, reuse the CSI framework or CSI-ReportConfig to minimize spec impact.

Proposal 2: RAN2 clarify whether periodic reporting for NW side data collection is transmitted via PUCCH, PUSCH or RRC.

References

[1] RP-240774, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”

[2] Chair Notes for RAN2 #126

[2] Minutes report for RAN1 #120

Annex