3GPP TSG RAN WG2 #129-bis				R2-2502819

Wuhan, China, Apr. 7th – 11th, 2025	Revision of R2-2500414

Agenda Item:	8.2.2

Source:		ASUSTeK

Title:				Discussion on Ambient IoT paging message design

Document for: Discussion and Decision

- Introduction

In the WID agreed in RAN#106 meeting [1], the following objectives on Ambient IoT RAN2 scope were set:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

Additionally, in RAN2#129 meeting [2], the following agreements on paging message were made:

- Agreements - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

In this contribution, we further discuss the detailed design of A-IoT paging message.

- Discussion

2.1  Contents of paging message

2.1.1  ID

As specified in WID, it is possible that a paging message comprises “no ID”, which implies that the paging message is a broadcast message for all devices. And, there are two alternatives to implement “no ID”:

Option 1:	The ID field in the paging message is optional. If the paging message is to be broadcasted to all devices, the ID field is absent. An additional field to indicate whether the ID field is present or absent may be required.

Option 2:	The ID field in the paging message is mandatory. If the paging message is to be broadcasted to all devices, the ID field is set to a specific value, e.g. 0.

To reduce device decoding complexity, a fixed format for the paging message is slightly preferred, i.e., option 2.

Proposal 1:  The ID in a paging message which is set to a specific value implies “a paging message contains no identifier.”

To achieve the purpose for forward compatibility, one way to design a paging message format such that multiple identifiers can be contained is to include a field in a paging message to indicate a number of IDs. Each ID may have its corresponding transaction ID or all IDs share the same transaction ID. And the field is always set to 1 in release 19.

Proposal 2:  To support multiple IDs in a paging message for forward compatibility, a paging message includes a field to indicate the number of IDs.

2.1.3  Device type

Moreover, since the device type includes device 1, device 2a and device 2b, the reader may page devices separately based on their device type due to different device capabilities. Since A-IoT devices have very low capability, it is important for the network to schedule these devices to perform data transmission properly, e.g., providing proper frequency resources and/or repetition number.

Although only device 1 is supported in this release, device 2a and device 2b may be included in next release. It is proposed to consider including device type information in a paging message, since the paging format should allow the devices from different releases to co-exist in the fields.

Proposal 3: To support different device types for forward compatibility, a paging message includes a field to indicate device type information.

2.1.4  D2R resource information

In RAN2 #126 meeting, it was agreed that AIoT paging message indicates information from which the device can determine resources to be used for response. In TR 38.769 [3] as below, how to provide D2R resource for Msg1 still needs further study by RAN1. Design of this part could be postponed till RAN1 agree to use higher-layer signalling to carry D2R resource.

#### 6.1.2.8 Scheduling of D2R For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via the corresponding PRDCH: - Time domain resources - Frequency domain resources - MCS-like information - Chip duration - ID associated with device(s) - Repetitions - Midamble (if supported) related information For each information, it is for further study whether higher-layer signalling and/or L1 R2D control signalling is used - see Clause 6.1.1.10.

Observation 1:  The design of D2R resource information included in a paging message could wait for RAN1’s progress.

2.2  Message to determine Msg1 resource

In RAN2#129 meeting [2], it was agreed that there would be a R2D transmission after initial paging message to determine Msg1 resource. Two options were captured:

- Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or

- Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field

We believe that both options work to allow the devices to determine Msg1 resources. Here, we would like to confirm the feasibility of Way-2. Since it has been agreed that the paging message can include a number of Msg1 resources, the subsequent R2D transmission can use the same resource field to indicate which resources are available in the following occasions corresponded to the R2D transmission. On the other hand, introducing a new R2D message could increase the decoding complexity for the device. Therefore, way-2 is preferred.

Proposal 4:  The reader reuses the same paging message to determine Msg1 resource after initial paging message.

2.3  Avoiding duplicated response to paging

Consider the occurrence of transmission failure, the reader could send subsequent paging for the same service. For a device that has responded to the paging message successfully, it should not trigger the random access procedure again in response to receiving the subsequent paging for the same service. And the transaction ID based on CN correlation ID would be included in a paging message to avoid the redundant paging response.

For example, as shown in Figure 1, the first paging message for a first service request would have transaction ID #0, the second paging message for a second service request would have transaction ID #1, and so forth. When the maximum value of the transaction ID range is reached, the transaction ID restarts from #0. During the multiple service requests, a device would not always monitor the paging messages due to limited energy. As a result, the device may receive transaction ID #0 and not receive transaction ID #1, 2 and 3. When the transaction ID comes back to 0, the device may misinterpret the wrap-up new transaction ID as an old one. Then the device would not respond to the paging.

- Figure 1: An example of wrap-up issue for transaction ID

To solve the issue, a timer could be introduced to avoid the device from being prohibit to respond to the service forever. The device could start the timer when responding the paging message successfully. For example, when the timer is running, the device may consider that subsequent paging for the same transaction ID is for re-access purpose and the device should not respond. After the timer expiry, the device responds to a paging with the same transaction ID.

Proposal 5:  Consider that the device would not always receive all the transaction IDs, a timer is introduced to avoid the wrap-up issue.

- Conclusion

Proposal 1:  The ID in a paging message which is set to a specific value implies “a paging message contains no identifier.”

Proposal 2:  To support multiple IDs in a paging message for forward compatibility, a paging message includes a field to indicate the number of IDs.

Proposal 3:  To support different device types for forward compatibility, a paging message includes a field to indicate device type information.

Observation 1:  The design of D2R resource information included in a paging message could wait for RAN1’s progress.

Proposal 4:  The reader reuses the same paging message to determine Msg1 resource after initial paging message.

Proposal 5:  Consider that the device would not always receive all the transaction IDs, a timer is introduced to avoid the wrap-up issue.

- Reference

[1] RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”

[2] Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece

[3] TR 38.769, “Study on solutions for ambient IoT (Internet of Things)”