3GPP TSG-RAN WG2 Meeting #129bis		       R2-2502822

Wuhan, China: 7th April – 11th April 2025

Agenda Item:	8.3.5

Source:	ASUSTeK

Title:	Discussion on LCM for AIML Mobility

Document for:	Discussion and Decision

- Introduction

The objectives of SI FS_NR_AIML_Mob agreed to work on the specification impact of AI/ML mobility, where enhancements should be based on the Rel19 AI/ML-air interface WID general framework. In this document, we would like to discuss the potential impacts and whether/how the Rel19 AI/ML-air interface WID general framework can be reused.

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

- Discussion

In legacy, the RRM measurement framework is based on MeasObject(s), ReportConfig(s), and MeasId(s) that associate one MeasObject with one ReportConfig. For inference, it makes sense to reuse the RRM measurement framework for configuring inference configurations for AI/ML mobility (e.g., RRM prediction and measurement event prediction).

Proposal 1: The inference configuration for AI/ML mobility can be based on RRM measurement configuration (e.g., MeasObject(s), ReportConfig(s), and/or MeasId(s)).

Based on the study in AI/ML-air interface, the reporting of applicability, can be based on the inference configuration. For beam management, the inference configuration utilizes CSI framework, where one CSI-ReportConfig can conclude the inference configuration, which makes reporting of applicability as simple as reporting the applicability of one configuration. However, the structure of RRM measurement framework is more complex. For example, MeasObject(s), ReportConfig(s), and/or MeasId(s) may all have legacy and inference variants, where different variants may associate with each other, increasing the number and complexity of configurations to report the applicability for. The applicability reporting framework should take this into consideration.

Proposal 2: The applicability reporting framework should consider the increasing number and complexity of configurations for new features.

Based on the study in AI/ML-air interface, associated ID is used to ensure consistency between training and inference. Based on the ongoing study in AI/ML mobility, to perform inference for RRM prediction, the UE may need to acknowledge the frequency, cell, prediction window related parameters, and/or observation window related parameters. These parameters could be included in the inference configuration (at least for RRM prediction).

Proposal 3: The inference configuration for RRM prediction may include associated ID, frequency information, cell information, prediction window related parameters, and/or observation window related parameters.

Based on the ongoing study in AI/ML mobility, predicted measurements and/or event can be used to trigger a measurement report. One benefit for the prediction is that the NW can acknowledge potential handover in advance and prepare for handover execution. To prepare for handover execution, the predicted measurement results may play an important role.

Proposal 4: The current measurement reporting could be enhanced to consider predicted measurements.

- Conclusion

Proposal 1: The inference configuration for AI/ML mobility can be based on RRM measurement configuration (e.g., MeasObject(s), ReportConfig(s), and/or MeasId(s)).

Proposal 2: The applicability reporting framework should consider the increasing number and complexity of configurations for new features.

Proposal 3: The inference configuration for RRM prediction may include associated ID, frequency information, cell information, prediction window related parameters, and/or observation window related parameters.

Proposal 4: The current measurement reporting could be enhanced to consider predicted measurements.

References

[1] RP-242393, “Revised SID on AIML for mobility in NR”

Annex