3GPP TSG-RAN WG2 Meeting #129bis	R2-2502823

Wuhan, China, Apr. 7th – 11th, 2025                        Revision of	 R2-2500417

Agenda Item:	8.6.3

Source: 	ASUSTeK

Title:  	Discussion on MR MAC CE for LTM

Document for:	Discussion and Decision

- Introduction

In the previous meeting RAN2#129, L1 event triggered report for LTM was discussed and has made several agreements including details of Measurement report (MR) MAC CE:

Agreements on MAC issues:

- CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell.

- If the CLTM TAT associated with one candidate cell is running, UE considers the TA for the candidate cell is valid; if CLTM TAT associated with one candidate cell expires, UE considers the TA for the candidate cell is invalid.

- If UE has valid TA value at the time CLTM execution, UE performs RACH-less CLTM, otherwise UE performs RACH base CLTM.

- The CLTM TAT is per candidate configuration.

- FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

- Absolute TA value is included in the MAC CE for CLTM early TA acquisition.

- The candidate configuration ID is included in the MAC CE for CLTM early TA acquisition. And one configuration ID for one TA in one MAC CE.

- RAN2 assumes the CSI-RS could be associated with the indicated TCI state in LTM cell switch MAC CE and in the candidate cell TCI state Activation/Deactivation MAC CE, pending RAN1 confirmation.

- The Measurement Report MAC CE is transmitted with a higher LCP than 'data from any Logical Channel, except data from UL-CCCH'.

- The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR.

- The max number of N (included in the MR MAC CE) is the total number of all candidate beams without counting current serving beam.

- There is no restriction on which serving cell(s) L1 Measurement Report MAC CE can be transmitted on, i.e. not limited to SpCell.

- Truncated event-triggered L1 measurement report MAC CE can be used when the available grant is not sufficient to accommodate the normal measurement report MAC CE plus subheader.

Agreements on L1 event-triggered MR:

- Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused".

- Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide).

- Do not create a new resource configuration just for the event triggered measurements.

- Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused".

- Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report.

- Reuse the existing RSRP values and RSRP offset values and their interpretations in TS 38.133 for event-triggered measurement report. Can revisit if RAN4 identifies any issue.

- TimeToTrigger and reportInterval of L3 measurement report can be reused. FFS on additional value(s) of reportInterval.

- Cell specific offset is not supported for LTM L1 event evaluation.

- The trigger quantity for LTM L1 event triggered measurement is the same as the report quantity.

- Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

- Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

- R18 differential RSRP encoding is used as baseline.

- LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

- Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s))

- Truncated MR MAC CE includes the triggered beam information:

- One report config ID (event config ID)

- At least one triggered beam id (SSBRI or CRI) + L1 RSRP

- Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur.

- For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used.

- S-measureConfig is applicable to in the L1 event-triggered MR. No additional change is expected on RRC and MAC.

In this contribution, we discuss further details for L1 measurement report for LTM.

- Discussion

Contents of LTM MR MAC CE

Currently, it is agreed that SSBRI/CRI will be used to indicate candidate beams reported in LTM MR MAC CE, and a maximum N of total number of all beams included into LTM MR MAC CE. In RAN2#129, it was further agreed that a MAC CE only includes one trigger event. In LTM measurement reporting, since there can be more than one candidate configuration fulfilling LTM measurement report triggering event(s) at a time (e.g., multiple SSRRIs or CRIs across candidate cells fulfilling the event(s)), it is proposed to confirm that a single LTM MR MAC CE can include measurement information of multiple candidate cells (e.g., event ID, SSBRI/CRI, or RSRP value).

Proposal 1:	A LTM MR MAC CE can include measurement information of multiple candidate cells fulfilling a LTM measurement report triggering event.

prioritization between multiple LTM MR MAC CEs

According to last meeting agreement, the Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR. As one LTM MR MAC CE can only report information of a single event, it is possible that when multiple events are fulfilled for different report configurations (LTM-CSI-ReportConfig), the UE needs to generate and includes multiple MR MAC CEs that are associated with different triggered events in one MAC PDU.

Observation:	When there are multiple triggered L1 measurement reports, the UE needs to generate and include multiple MR MAC CEs in a MAC PDU.

When multiple MAC CEs are to be included in a MAC PDU, the order between separate MR MAC CEs need to be decided during assembly and multiplexing. One simple way is to include the MR MAC CE in an ascending order of ltm-CSI-ReportConfigId (e.g., include MAC CE associated with smaller ltm-CSI-ReportConfigId first). Another way is to include MAC CEs with smaller report configuration type first (e.g., the UE includes MR MAC CE with LTM2 before include MAC CE with LTM 5).

Another direction is to have priority for each triggered event. The priority can be a fixed order according to timing when the pending events was triggered (e.g., measurement report that was triggered first in time can be included first), or the priority can be configured by network (e.g., priority of each event or measurement configuration can be configured, For example, report with triggering event LTM3 can be prioritized over LTM2).

In our view, it’d be simpler to prioritize including MR MAC CE with smaller report ID. While it may be beneficial to prioritize including MR MAC CE associated with more mobility-related events (e.g., events that considers candidate quality, LTM 3 or LTM5), it can be up to NW to assign the more important measurement configurations with smaller report IDs so that the UE can include the corresponding MAC CEs first.

Proposal 2:	When multiple MR MAC CEs are to be multiplexed in a MAC PDU, the UE prioritizes including MR MAC CEs containing smaller report ID (ltm-CSI-ReportConfigId).

Information prioritization of truncated LTM MR MAC CE

In RAN2#128, it was agreed to support truncated LTM MR MAC CE. A main reason is that available UL grants may not always be able to include all information generated by the UE for all triggered measurement. To support truncated LTM MR MAC CE, when UL grant cannot accommodate all measurement report information (e.g., cannot accommodate all N beams or cannot accommodate information of all candidate cells), the UE will need to decide which information to be included first. In our view, since the LTM MR MAC CE indicates information for mobility execution (e.g., for the NW to decide whether to perform Cell switch or early UL synchronization), it would be beneficial for the UE to prioritize including information of a candidate cell with higher quality beam so that the network can select the best candidate cell to perform LTM Cell switch/early sync for the UE. Also, information of the best beam could assist the NW to select the beam for RACH-less LTM.

In addition, the previous meeting, it was agreed that to select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s)), the same principle can be applied when selecting the beam to report in truncated MR MAC CE as well.

Proposal 3:	When a UL grant cannot accommodate a LTM MR MAC CE plus its subheader, the UE decides which information to be included in a truncated LTM MR MAC CE by:

- prioritizing including information of candidate cells with higher quality beam, and

- prioritizing including information of beams with higher quality.

- Conclusion

We have the following observation and proposals for MR MAC CE for LTM:

Proposal 1:	A LTM MR MAC CE can include measurement information of multiple candidate cells fulfilling a LTM measurement report triggering event.

Observation:	When there are multiple triggered L1 measurement report, the UE needs to generate and include multiple MR MAC CEs in a MAC PDU.

Proposal 2:	When multiple MR MAC CEs are to be multiplexed in a MAC PDU, the UE prioritizes including MR MAC CEs containing smaller report ID (ltm-CSI-ReportConfigId).

Proposal 3:	When a UL grant cannot accommodate a LTM MR MAC CE plus its subheader, the UE decides which information to be included in a truncated LTM MR MAC CE by:

- prioritizing including information of candidate cells with higher quality beam, and

- prioritizing including information of beams with higher quality.

- Reference