3GPP TSG-RAN WG2 Meeting #129bis	R2-2502824

Wuhan, China, 7th -11th April, 2025

Agenda Item:	8.6.4

Source: 	ASUSTeK

Title:  	Discussion on early TAT handling for CLTM

Document for:	Discussion and Decision

- Introduction

In the previous meeting RAN2#129, L1 event triggered report for LTM was discussed and has made several discussion and agreements including details of timer and contents of Measurement report (MR) MAC CE:

Agreements on MAC issues:

- CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell.

- If the CLTM TAT associated with one candidate cell is running, UE considers the TA for the candidate cell is valid; if CLTM TAT associated with one candidate cell expires, UE considers the TA for the candidate cell is invalid.

- If UE has valid TA value at the time CLTM execution, UE performs RACH-less CLTM, otherwise UE performs RACH base CLTM.

- The CLTM TAT is per candidate configuration.

- FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

- Absolute TA value is included in the MAC CE for CLTM early TA acquisition.

- The candidate configuration ID is included in the MAC CE for CLTM early TA acquisition. And one configuration ID for one TA in one MAC CE.

- RAN2 assumes the CSI-RS could be associated with the indicated TCI state in LTM cell switch MAC CE and in the candidate cell TCI state Activation/Deactivation MAC CE, pending RAN1 confirmation.

- The Measurement Report MAC CE is transmitted with a higher LCP than 'data from any Logical Channel, except data from UL-CCCH'.

- The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR.

- The max number of N (included in the MR MAC CE) is the total number of all candidate beams without counting current serving beam.

- There is no restriction on which serving cell(s) L1 Measurement Report MAC CE can be transmitted on, i.e. not limited to SpCell.

- Truncated event-triggered L1 measurement report MAC CE can be used when the available grant is not sufficient to accommodate the normal measurement report MAC CE plus subheader.

Agreements on L1 event-triggered MR:

- Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused".

- Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide).

- Do not create a new resource configuration just for the event triggered measurements.

- Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused".

- Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report.

- Reuse the existing RSRP values and RSRP offset values and their interpretations in TS 38.133 for event-triggered measurement report. Can revisit if RAN4 identifies any issue.

- TimeToTrigger and reportInterval of L3 measurement report can be reused. FFS on additional value(s) of reportInterval.

- Cell specific offset is not supported for LTM L1 event evaluation.

- The trigger quantity for LTM L1 event triggered measurement is the same as the report quantity.

- Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

- Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

- R18 differential RSRP encoding is used as baseline.

- LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

- Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s))

- Truncated MR MAC CE includes the triggered beam information:

- One report config ID (event config ID)

- At least one triggered beam id (SSBRI or CRI) + L1 RSRP

- Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur.

- For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used.

- S-measureConfig is applicable to in the L1 event-triggered MR. No additional change is expected on RRC and MAC.

In this contribution, we discuss further details for handling early TAT for candidate cells.

- Discussion

Handling of CLTM TAT for target Cell

In the previous meeting, it was agreed that CLTM TAT of a candidate cell will be (re)started when receiving TA value for the candidate cell sent by the MAC CE, but how to handle stopping condition of the new timer for candidate cell TA was not concluded and were postponed:

 CLTM TAT for target cell is not stopped upon CLTM cell switch execution to the target cell. Rapporteur suggests to discuss how/whether to start the timeAlignmentTimer associated with PTAG upon CLTM cell switch execution to the target cell during CB session: - Option 1 (revert the offline agreement): start the timeAlignmentTimer associated with PTAG as in Rel-18; - Option 2 (keep the offline agreement): discuss the options below in the MAC running CR o Option 2.1: clarify the running TAT to act as the timeAlignmentTimer associated with PTAG by a note in the MAC running CR; o Option 2.2: define that UE can perform UL tx when the TAT for the target cell is running. Besides, we need to add a note that, if SCell and SpCell are in the different TAG, SCell may not work when this TAT for target cell is running. o Option 2.3: the UE stops the CLTM timer and starts the PTAG using the remaining time of the CLTM timer. - Will be revisited next meeting.

In our view, it is only logical for the UE to extend validity period of the TA for a TAG or a Cell when receiving or updating a new TA of the corresponding TAG. In legacy LTM, the TA is received in LTM Cell switch command MAC CE so the UE start or restart the timeAlignmentTimer associated with the PTAG when receiving the MAC CE with a valid TA. In conditional LTM, on the other hand, the TA is not refreshed upon LTM execution, so the validity should not be prolonged when the UE determines to execute the CLTM Cell switch.

Observation:	The UE should not extend the validity period of a TA of the target cell upon CLTM execution on the target cell.

Therefore, to leverage the early TA provided by the network for RACH-less CLTM while keeping the time period of the TA validity unchanged, there are two options to handle the new CLTM TAT and legacy timeAlignmentTimer:

- The UE starts timeAlignmentTimer of PTAG upon execution of the CLTM using the remaining time of the CLTM TAT.

Figure 1

As illustrated in Figure 1, the UE receives the TA for the candidate cell at t0 and start the CLTM TAT. The UE starts timeAlignmentTimer of PTAG of the target candidate cell at t1 (upon execution of CLTM on the candidate cell) with length set to remaining time of the CLTM TAT (n- m, where m is the time difference between t1 and t0). In this option, it is open whether the CLTM TAT should be stopped or not considering the UE considers UL transmission availability based on legacy timeAlignmentTimer.

- The CLTM TAT of the candidate cell acts as timeAlignmentTimer of PTAG of the candidate cell upon execution of the CLTM and the UE can perform UL transmission on the candidate cell when CLTM TAT is running.

In this option, the UE does not stop the CLTM TAT upon CLTM execution and using the CLTM TAT to decide whether UL TA is sync or not for target Cell. The advantage of this option is that no special handling one the starting value of timeAlignmentTimer is needed when executing CLTM. On the other hand, additional issues will need to address if CLTM TAT is used as a temporary timeAlignmentTimer (e.g., when to stop considering CLTM TAT as timeAlignmentTimer, when to use/start the original timeAlignmentTimer, etc).

In our view, both options can work. We propose option 1 as it would be more similar to timeAlignmentTimer handling on legacy LTM execution and would impose less spec impact than having to determine UL alignment using additional timer.

Proposal:	The UE starts timeAlignmentTimer of PTAG of target candidate cell upon execution of the CLTM on the target candidate cell using the remaining time of the CLTM TAT of the target candidate cell.

- Conclusion

We have the following observation and proposal for C-LTM timer handling:

Observation:	The UE should not extend the validity period of a TA of the target cell upon CLTM execution on the target cell.

Proposal:	The UE starts timeAlignmentTimer of PTAG of target candidate cell upon execution of the CLTM on the target candidate cell using the remaining time of the CLTM TAT of the target candidate cell.

- Reference