3GPP TSG-RAN WG2 Meeting #129bis	R2-2502827

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.12.3

Source: 	ASUSTeK

Title:  	Discussion on PL offset value for Asymmetric DL sTRP/UL mTRP

Document for:	Discussion and Decision

- Introduction

In Rel-19 MIMO, UE-initiated beam reporting is introduced:

The detailed objectives are as follows: RAN1: …Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

In the previous meeting, a working assumption was made regarding PL offset application

- Working assumption UE applies the latest PL offset value received in RRC or MAC CE. Can revisit if new issue is found.

In this contribution, we discuss possible issues regarding applying PL offset for UL TRP.

- Discussion

Issues when applying latest PL offset value for asymmetric DL/UL TRP

In MIMO enhancement for power control on asymmetric DL/UL TRP, it was agreed that different PL offset values can be configured for different TCI states for a TRP, and RAN2 agreed to use MAC CE for updating PL offsets for TCI states:

Agreements on asymmetric DL sTRP and UL mTRP

New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID.

Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB.

In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

In the previous meeting, it was raised and discussed in R2-2501177 [1] that whether the UE should use the PL offset value (previously) configured by RRC or updated by MAC CE for a BWP after BWP switching to the BWP, or for a TCI state after receiving delta signalling without modifying the TCI state. A working assumption was made so that the UE uses the latest received value (either RRC or MAC CE) for PL offset determination. In our view, if this working assumption is applied, an issue may occur in another scenario when beam failure recovery occurs on DL TRP. When beam failure is detected in DL/UL TRP, it is likely that the currently applied beam for the DL/UL TRP would be changed due to deteriorated quality or blockage of the beam path to the DL/UL TRP. While the beam for UL-only TRP may remain unchanged, as the pathloss environment has been changed on the DL TRP side, it may not be suitable to keep using the MAC CE-updated PL offset for the UL TRP. While the MAC CE-indicated PL offset may be more fine-tuned, it may not be accurate after beam change and could lead to insufficient transmission power or causing interference to other UEs.

Observation:	The updated PL offset value from MAC CEs may be inaccurate after beam failure recovery procedures due to changes of pathloss environment.

An example is shown in Figure 1. At t1, the UE receives RRC configuration for DL/UL TRP and UL-only TRP with TCI state configuration including TCI state id, pathloss reference signal id, and pathloss offset (for TCI states associated with UL TRP). At t2, the UE receives PL offset MAC CE updating PL offset value for TCI state 2. The UE then uses the latest value, PL offset 2, for calculating transmission power on UL TRP. At t3, beam failure is detected and triggers a beam failure recovery procedure. At t4, after completing beam failure recovery and before receiving (new) TCI state (de)activation, the UE applies a new candidate TCI state, TCI state 3, on the DL/UL TRP. The issue then occurs if the UE still uses the updated PL offset value (e.g., PL offset2) on the UL TRP which is inaccurate since pathloss estimate based on PL RS 1 is not accurate due to beam failure for the path according to PL RS 1.

Figure 1

Therefore, in order to avoid applying inaccurate PL offset after beam failure recovery procedure, we propose the UE reverts to RRC-configured PL offset values for TCI states of the UL TRP when a beam failure recovery procedure is successfully completed and has not received a new PL offset update MAC CE.

Proposal:	The UE reverts to RRC-configured path loss offset values for TCI states of the UL TRP of a Serving Cell when a beam failure recovery procedure is successfully completed and has not received PL offset update MAC CE.

- Conclusion

We have the following observation and proposal for asymmetric DL/UL TRP:

Observation:	The updated PL offset value from MAC CEs may be inaccurate after beam failure recovery procedures due to changes of pathloss environment.

Proposal:	The UE reverts to RRC-configured path loss offset values for TCI states of the UL TRP of a Serving Cell when a beam failure recovery procedure is successfully completed and has not received PL offset update MAC CE.

- Reference

[1] R2-2501177	Discussion on Asymmetric DL sTRP/UL mTRP	Huawei, HiSilicon	discussion	Rel-19