3GPP TSG-RAN WG2 Meeting #129bis	R2-2502828

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.12.3

Source: 	ASUSTeK

Title:  	Discussion on MAC impact regarding UEI reporting

Document for:	Discussion and Decision

- Introduction

In Rel-19 MIMO, UE-initiated beam reporting is introduced:

The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In this contribution, we discuss possible MAC impact regarding BWP operation for UEI beam reporting.

- Discussion

MAC impact on BWP operation regarding UEI beam reporting

Regarding UEI beam reporting, RAN1 has agreed on two modes, Mode-A (dynamic scheduling of beam reporting resource) and Mode-B (pre-configured resource) with a first UL channel (UCI on PUCCH) and a second UL channel (via at least CG-PUSCH). In Mode B, the first UL channel transmission on PUCCH is used to notify network of UEI beam reporting via a corresponding CG-PUSCH transmission. In RAN1#118bis meeting, it was agreed on the one-to-one resource mapping between the first and the second UL channel:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, resource mapping/configuration between first and second UL channel in Mode-B, at least Option-1 is supported - Option-1 (one-to-one): Only one periodic PUCCH resource for the first channel and only one pre-configured resource for second UL channel can be associated with the CSI report configuration for UE-initiated/event-driven beam reporting. - Down-select one of the following in RAN1#118bis - Option-1A: Same periodicity between first PUCCH resource and pre-configured resource for second UL channel. - Option-1B: No restriction in terms of periodicity.

Observation 1:	For UEI beam reporting Mode-B, there is a one-to-one mapping between first channel (PUCCH resource) and second UL channel (CG-PUSCH resource) in a CSI report configuration.

For MAC layer, additional adaptations on BWP operation may be needed in order to ensure UEI report transmission on the second UL channel. An example is shown in Figure 1. A UE is configured with UEI beam report configuration including PUCCH resource and CG-PUSCH resource for 1st and 2nd UL channel transmission for a mode-B UEI reporting on BWP 1. The UE triggers a UEI reporting corresponding to the configuration and performs PUCCH transmission for UEI beam report (via PUCCH resource) at timing t1 to indicate a transmission on timing t3, where the PUCCH resource is paired with a CG-PUSCH resource at t3 for UEI beam report transmission (i.e., the 2nd UL transmission). At t2, bwp-InactivityTimer expires before the UE can perform the UEI beam report transmission and the UE switches its active BWP to a BWP 2 (e.g., default BWP). This would cause the UE to be unable to transmit the UEI beam report as there are no corresponding resources for the beam report transmission on the BWP 2.

Figure 1

Observation 2:	For UEI beam reporting mode-B, the UE will be unable to perform UEI beam report transmission if the UE performs UL BWP switching after PUCCH transmission as there will be no corresponding pre-configured resource for transmitting the UEI beam report on the switched BWP.

In the current specification, BWP operation leverages control on bwp-InactivityTimer to prevent unwanted BWP switching:

2> if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active BWP; or 2> if a PDCCH addressed to G-RNTI or G-CS-RNTI configured for multicast indicating downlink assignment is received on the active BWP; or 2> if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received for the active BWP; or 2> if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or 2> if a MAC PDU is received in a configured downlink assignment for unicast or MBS multicast: 3> if there is no ongoing Random Access procedure associated with this Serving Cell; or 3> if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a and 5.1.5): 4> start or restart the bwp-InactivityTimer associated with the active DL BWP.

In order to avoid unnecessary BWP changes that leads to failure on UEI beam reporting transmission, we propose the MAC entity to start or restart the bwp-InactivityTimer associated with the Cell where the CG-PUSCH resource of a CSI report configuration is configured if the MAC entity performs PUCCH transmission for the UEI beam reporting of the associated CSI report configuration.

Proposal:	For UE-initiated/event-driven beam reporting mode-B, the MAC entity shall start or restart bwp-InactivityTimer associated with the Cell where the CG-PUSCH resource of a CSI report configuration is configured if the MAC entity performs the PUCCH transmission of the CSI report configuration.

- Conclusion

We have the following observations and proposal for UEI beam reporting:

Observation 1:	For UEI beam reporting Mode-B, there is a one-to-one mapping between first channel (PUCCH resource) and second UL channel (CG-PUSCH resource) in a CSI report configuration.

Observation 2:	For UEI beam reporting Mode-B, the UE will be unable to perform UEI beam report transmission if the UE performs UL BWP switching after PUCCH transmission as there will be no corresponding pre-configured resource for transmitting the UEI beam report on the switched BWP.

Proposal:	For UE-initiated/event-driven beam reporting Mode-B, the MAC entity shall start or restart bwp-InactivityTimer associated with the Cell where the CG-PUSCH resource of a CSI report configuration is configured if the MAC entity performs the PUCCH transmission of the CSI report configuration.

- Reference