3GPP TSG RAN WG2 #129bis		                            R2-2502831

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.6.3

Source:	     KDDI Corporation

Title:	Discussion on L1 event triggered measurement reporting for LTM

WID:       	NR_Mob_Ph4-Core

Tdoc Type:	Discussion

Document for: 	Discussion and decision

- Introduction

In RAN #106, the Rel-19 WID on NR mobility enhancements Phase 4 was updated [1]:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

In RAN2#129, the following agreement was made:[2]

Agreements on L1 event-triggered MR:

- Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused".

- Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide).

- Do not create a new resource configuration just for the event triggered measurements.

- Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused".

- Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report.

- Reuse the existing RSRP values and RSRP offset values and their interpretations in TS 38.133 for event-triggered measurement report. Can revisit if RAN4 identifies any issue.

- TimeToTrigger and reportInterval of L3 measurement report can be reused. FFS on additional value(s) of reportInterval.

- Cell specific offset is not supported for LTM L1 event evaluation.

- The trigger quantity for LTM L1 event triggered measurement is the same as the report quantity.

- Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

- Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

- R18 differential RSRP encoding is used as baseline.

- LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

- Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s))

- Truncated MR MAC CE includes the triggered beam information:

- One report config ID (event config ID)

- At least one triggered beam id (SSBRI or CRI) + L1 RSRP

- Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur.

- For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used.

- S-measureConfig is applicable to in the L1 event-triggered MR. No additional change is expected on RRC and MAC.

In this contribution, we will discuss open issues form RAN2#129 and share our views on L1 event triggered MR.

- Discussion

2.1 Per Beam, explicit indication in MR MAC CE to inform the leaving event condition is met

In RAN2#129, explicit indication in MR MAC CE to inform the leaving event condition is met is agreed and whether per beam is indicated was FFS. Support explicit indications for whether a beam meets the leaving event condition is essential for network operations. This clarity allows the network to accurately determine which specific beams fulfil the event conditions. Knowing this information is critical for effective management of resources and for making informed decisions regarding cell switching and overall network performance. By providing a clear indication on a per-beam basis, the network can respond more efficiently to changing conditions, ensuring better service quality and user experience. This directly addresses the need for precise communication of beam performance, which is vital for optimizing network operations and ensuring that mobile users experience seamless connectivity.

Proposal 1: Explicit indication per beam in MR MAC CE to inform the leaving event condition is met

2.2 Candidate cell IDs in report configuration

Discussion was held on the maximum number of beam level measurement results that can be included in the MAC CE to send LS to RAN1. Including candidate cell IDs was proposed as a way to associate N beam level measurements with candidate cells. Including candidate cell IDs in the report configuration allows for accurate association of N beam-level measurements with their respective candidate cells. In legacy systems, there are typically 4 cells, each with 4 beam candidates. By using candidate cell IDs, the measurement results for each beam can be clearly mapped to the corresponding candidate cell. This method facilitates easier interpretation of measurement results and helps the network understand which beams are associated with which candidate cells.

However, it should be noted that including candidate cell IDs means that the UE will need to undergo a two-step filtering process, which may increase complexity. Therefore, if a method were chosen that allows the NW to set a list of LTM resource set IDs in the report configuration it could potentially reduce the complexity for the UE and promote more efficient measurements. Nevertheless, given the current proposal, utilizing candidate cell IDs is considered the most appropriate approach.

Proposal 2: Include candidate cell IDs in the reporting configuration.

- Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: Explicit indication per beam in MR MAC CE to inform the leaving event condition is met

Proposal 2: Include candidate cell IDs in the reporting configuration.

- References

[1] RP-250339 Revised Work Item: NR mobility enhancements Phase 4 (Apple Inc, China Telecom)

[2] RAN2 #129 Report from session on LTE V2X and NR SL (Vice Chairman: Samsung)