3GPP TSG-RAN WG2 Meeting #129bis	R2-2502841

Wuhan, China, 7 - 11 Apr, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on adaptation of common signal/channel transmissions

Agenda Item:	8.5.4

Document for:	Discussion and decision

Introduction

In this contribution, we provide our considerations on adaptation of common signal/channel transmissions.

Discussion

## 2.1 Adaptation of paging

In RAN2#129, the following has been agreed for paging adaptation:

Agreements on paging adaptation

- For N, values smaller than T/32 are not supported.

- The maximum possible value for Ns is 8.

- Introduce a separate PEI configuration.

- Paging adaptations are configured semi-statically and updated via system information update notification.

- A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

There are two remaining issues related two paging/PEI:

Issue 1: PO/PEI configuration

The following FFS is captured during [POST129][102][NES] Running RRC CR(Ericsson):

Editor’s note: FFS field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions. FFS: firstPDCCH-MonitoringOccasionOfPO for paging adaptations. FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

Issue 2: UE behaviour of PEI monitoring

FFS is captured during [POST129][104][NES] (Apple):

Editor’s note: FFS the UE behavior if pagingAdaptationPEI-Config is configured.

As commented during the email discussion, there could be two interpretations of R19 NES UE behaivors:

Option 1) R19 NES UEs only monitor R19 PEI

Option 2) R19 NES UEs monitor legacy PEI for legacy POs, monitor R19 PEI for R19 additional POs

Since R19 extended the Ns value to 8, there would be overlapping POs for legacy UEs and R19 NES UEs and additional POs for R19 NES UEs only. Note that the agreement in RAN2#127bis “Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration” also indicates that co-existence is feasible. In fact, we think the NW should always make R19 POs partially overlap with legacy POs, otherwise there would be no NES gain at all. As illustrated in the below picture, assuming T = 32 frames, configured legacy N = T/16 (PF appears in every 16 frames) and configured R19 N = T/32 (PF appears every 32 frames), configured legacy Ns = 4 and configured R19 Ns = 8. In this case, the R19 NES POs include the 8 POs in the first PF (the only PF), and legacy POs include 4 POs in each PF.

Taking the above assumption as an example, in the following discussion, we provide our views on how the PO/PEI configuration is provided and the intended UE behaviour.

On firstPDCCH-MonitoringOccasionOfPO, in existing spec the value range of depends on SCS and N (number of PFs in paging cycle T:

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

In R17, the parameter is extended considering the SCS of 480kHz:

firstPDCCH-MonitoringOccasionOfPO-v1710  CHOICE {

sCS480KHZoneEighthT        SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

}                                                                                  OPTIONAL  -- Need R

To ensure backward compatibility, legacy firstPDCCH-MonitoringOccasionOfPO still needs to be broadcast for legacy POs, but additional values need to be configured for additional POs in the PF, i.e. firstPDCCH-MonitoringOccasionOfPO-r19. There are two kinds of configurations:

Option 1) R19 NES UEs use firstPMO-r19 for all POs in the PF

Option 2) R19 NES UEs use legacy PMO for POs that overlap with legacy POs, use firstPMO-r19 for additional POs not overlapping with legacy POs

With Option 1, apparently the signalling overhead is increased (the signalling overhead for firstPMO-r19 is doubled). Besides, the value range needs to be extended: considering largest SCS of 480kHz and smallest N of T/32 (PF appears in every 32 frames), the maximum number of PDCCH monitoring occasions for this PF is 32 frames = 10240 slots = 143360 symbols in case of 480kHz SCS. Therefore, the existing maximum value of “(0.. 71679)” is not sufficient, “(0..143359)”.

firstPDCCH-MonitoringOccasionOfPO-v1710  CHOICE {

sCS480KHZoneEighthT        SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

}                                                                                  OPTIONAL  -- Need R

With Option 2, even though the value range is still as large as “(0..143359)”, the number of POs to indicate firstPMOs are reduced.

Observation 1: If R19 NES UEs use firstPMO-r19 for all POs in the PF, the signalling overhead of firstPMO-19 is twice as much as if R19 NES UEs use legacy PMO for POs that overlap with legacy POs and use firstPMO-r19 for additional POs not overlapping with legacy POs.

Proposal 1: (related to 38.331 FFS) A new firstPDCCH-MonitoringOccasionOfPO-r19 is added, value range is extended to “(0..143359)” to accommodate SCS=480kHz and N=T/32. RAN2 to down-select from the following options:

- Option 1) R19 NES UEs use firstPMO-r19 for all POs in the PF

- Option 2) R19 NES UEs use legacy PMO for POs that overlap with legacy POs, use firstPMO-r19 for additional POs not overlapping with legacy POs

In existing spec, PEI configuration comprises the following:

We think both pei-Config and pei-ConfigBWP are needed for R19 PEI configuration. Take firstPDCCH-MonitoringOccasionOfPEI-O as an example, this parameter is used to determine the PEI monitoring occasions after the UE determines the reference frame (based on pei-FrameOffset-r17). If firstPDCCH-MonitoringOccasionOfPEI-O is not configured, it is unclear how the UE locates the PEI, and reusing the same configuration from legacy PEI configuration largely restricts the PEI transmission flexibility. Similar to P2, the value range also needs to be extended, and the similar option should be adopted.

Proposal 2: (related to 38.331 FFS) A new firstPDCCH-MonitoringOccasionOfPEI-O-r19 is added, value range is extended to “(0..143359)” to accommodate SCS=480kHz and N=T/32. RAN2 to down-select from the following options:

- Option 1) R19 NES UEs use firstPMO-PEI-r19 for all POs in the PF

- Option 2) R19 NES UEs use legacy PMO-PEI for POs that overlap with legacy POs, use firstPMO-PEI -r19 for additional POs not overlapping with legacy POs

The above proposal is more or less related to the open issue during 38.304 review:

Option 1) R19 NES UEs only monitor R19 PEI

Option 2) R19 NES UEs monitor legacy PEI for legacy POs, monitor R19 PEI for R19 additional POs

If Option 2 is adopted in proposal P1 and P2, then Option 2 here should also be adopted. Option 2 not only brings less signalling overhead, but allows the PEI for R19 POs to span two PFs, as illustrated in the picture below:

Observation 2: If R19 NES UEs only monitor R19 PEI, the signalling overhead for PEI configuration is larger, and the R19 PEI cannot span two PFs when each PF has 9 POs.

Proposal 3: (related to 38.304 FFS) RAN2 to down-select from the following options:

- Option 1) R19 NES UEs only monitor R19 PEI

- Option 2) R19 NES UEs monitor legacy PEI for legacy POs, monitor R19 PEI for R19 additional POs

## 2.2 Adaptation of RACH

In the last meeting, the following agreements have been made for RACH adaptation:

Agreements on RACH adaptation

- Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.

2a.	RAN2 starts CBRA for RACH adaptation.

2b. 	RAN2 starts 4-step RACH adaptation.

3. 	From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request.

- From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.

5a. 	From R2 perspective, RACH adaptation is not modelled as RA feature(s).

5b.	From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

- Will follow legacy mechanism regarding how to select RACH resource.

Since RAN2 has agreed not to incorporate the additional RACH resources into RACH partitioning framework, and the additional RACH resources will be used for CBRA, it is straightforward to add the additional RACH resources in BWP-ConfigCommon in the form of RACH-ConfigCommon.

Proposal 4: New RACH-ConfigCommon is added in BWP-ConfigCommon for R19 NES UEs.

On the other hand, RAN2 agreed that RACH partitioning with all the features and feature combinations are supported. It is still unclear how the RACH resource selection is performed when PRACH adaptation is used jointly with RACH partition features, for instance, how to enable RACH adaptation for a Redcap UE.

If RACH adaptation is enabled for Redcap UEs:

If the Redcap UE selects RA resources from Branch 1, it cannot benefit from RACH adaptation;

If the Redcap UE selects RA resources from Branch 2, it cannot apply early identification.

To solve this, either a Redcap dedicated RACH-ConfigCommon is added to BWP-UplinkCommon (i.e. add a RACH-ConfigCommonRedcapNES-r19 on top of RACH-ConfigCommonNES-r19) or a NES dedicated RACH-ConfigCommon is added to AdditionalRACH-Config-r17 (i.e. add a RACH-ConfigCommonNES-r19 on top of RACH-ConfigCommon):

The similar mechanism needs to be applied for other features (e.g. SDT, Slicing) if these features can also be applied jointly with RACH adaptation.

Proposal 5: RAN2 to down-select from the following for (e)Redcap UEs:

- Option 1: A Redcap-specific R19 RACH-ConfigCommon is added to BWP-UplinkCommon;

- Option 2: An NES-specific RACH-ConfigCommon is added to AdditionalRACH-Config-r17;

- Option 3: RACH adaptation is not supported for (e)Redcap UEs.

The similar approach applies to other RA feature (e.g. SDT, Slicing).

According to RAN1 agreement, the additional RA resources can be adapted by DCI:

Agreement For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE), - At least DCI based adaptation is supported. No introduction of new DCI format.

In legacy RACH partitioning framework, the RACH resource set selection is performed after carrier selection, BWP selection and feature applicability determination. After RACH resource set selection (in which UE will choose from legacy RA resource set and R19 NES resource set), the UE performs RA type selection, initialize RA variables, and starts msg1 transmission.

If the RAR reception is not successful or contention resolution is not successful, the UE will select preambles to retransmit, until preambleTransMax is reached. However, in the context of RACH adaptation, it is possible that after UE selects the additional RA resources, the RA resource is deactivated before successful completion of RA procedure. In this case it is not preferred to let the UE keep retransmitting preambles consider the maximum number could be as many as 200:

preambleTransMax                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200},

Proposal 6: RAN2 to discuss whether the RA procedure can be terminated earlier if the selected RA resource is deactivated.

## 2.3 Adaptation of SSB

In RAN2#128, the following agreements were made for SSB adaptation:

Agreements on SSB adaptation

- RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell.

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

RAN2 agreed to use an additional SMTC on SCell for SSB adaptation for serving cell measurements. In case of intra-frequency measurements, the SMTC configured in servingCellMO will be used for both serving cell and intra-frequency neighbour cells, there is no motivation to deviate from this principle and exclude intra-frequency neighbour cells. Besides, it is still FFS when this additional SMTC comes into use. In our understanding, the additional SMTC is preconfigured to the UE, and will replace the legacy SMTC once SSB adaptation is indicated. RAN1 has already agreed to use DCI for SSB adaptation, there is no need to introduce MAC CE in RAN2 for the same purpose:

Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

Proposal 7: The additional SMTC configured on SSB for SSB adaptation is used for serving cell and intra-frequency neighbour cell measurements when SSB adaptation is indicated by DCI, otherwise legacy SMTC is used for serving cell and intra-frequency neighbour cell measurements.

Proposal 8: RAN2 will not introduce new MAC CE for SSB adaptation.

Another open issue is whether to have a unified L3 RRM measurement design for both OD-SSB and SSB adaptation. We think this is not possible given the current RAN2 and RAN4 progress. RAN2 has already agreed SSB adaptation will be based on SMTC, while RAN4 has agreed that OD-SSB will be based on OD-SSB pattern rather than MO configuration:

Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF.

Proposal 9: RAN2 does not aim at a unified L3 RRM measurement design for both OD-SSB and SSB adaptation.

Conclusion

In this contribution, we discussed the adaptation of common signal/channel transmissions, and made the following proposals:

Observation 1: If R19 NES UEs use firstPMO-r19 for all POs in the PF, the signalling overhead of firstPMO-19 is twice as much as if R19 NES UEs use legacy PMO for POs that overlap with legacy POs and use firstPMO-r19 for additional POs not overlapping with legacy POs.

Proposal 1: (related to 38.331 FFS) A new firstPDCCH-MonitoringOccasionOfPO-r19 is added, value range is extended to “(0..143359)” to accommodate SCS=480kHz and N=T/32. RAN2 to down-select from the following options:

- Option 1) R19 NES UEs use firstPMO-r19 for all POs in the PF

- Option 2) R19 NES UEs use legacy PMO for POs that overlap with legacy POs, use firstPMO-r19 for additional POs not overlapping with legacy POs

Proposal 2: (related to 38.331 FFS) A new firstPDCCH-MonitoringOccasionOfPEI-O-r19 is added, value range is extended to “(0..143359)” to accommodate SCS=480kHz and N=T/32. RAN2 to down-select from the following options:

- Option 1) R19 NES UEs use firstPMO-PEI-r19 for all POs in the PF

- Option 2) R19 NES UEs use legacy PMO-PEI for POs that overlap with legacy POs, use firstPMO-PEI -r19 for additional POs not overlapping with legacy POs

Observation 2: If R19 NES UEs only monitor R19 PEI, the signalling overhead for PEI configuration is larger, and the R19 PEI cannot span two PFs when each PF has 9 POs.

Proposal 3: (related to 38.304 FFS) RAN2 to down-select from the following options:

- Option 1) R19 NES UEs only monitor R19 PEI

- Option 2) R19 NES UEs monitor legacy PEI for legacy POs, monitor R19 PEI for R19 additional POs

Proposal 4: New RACH-ConfigCommon is added in BWP-ConfigCommon for R19 NES UEs.

Proposal 5: RAN2 to down-select from the following for (e)Redcap UEs:

- Option 1: A Redcap-specific R19 RACH-ConfigCommon is added to BWP-UplinkCommon;

- Option 2: An NES-specific RACH-ConfigCommon is added to AdditionalRACH-Config-r17;

- Option 3: RACH adaptation is not supported for (e)Redcap UEs.

The similar approach applies to other RA feature (e.g. SDT, Slicing).

Proposal 6: RAN2 to discuss whether the RA procedure can be terminated earlier if the selected RA resource is deactivated.

Proposal 7: The additional SMTC configured on SSB for SSB adaptation is used for serving cell and intra-frequency neighbour cell measurements when SSB adaptation is indicated by DCI, otherwise legacy SMTC is used for serving cell and intra-frequency neighbour cell measurements.

Proposal 8: RAN2 will not introduce new MAC CE for SSB adaptation.

Proposal 9: RAN2 does not aim at a unified L3 RRM measurement design for both OD-SSB and SSB adaptation.

References

- R2-2501331, Report from session on V2X/SL, R19 NES and MOB, Vice Chairman (Samsung)