3GPP TSG-RAN2 Meeting #129bis	R2- 2502850

# Wuhan, China, Apr 7th – 11st, 2025

Agenda item:		8.11.2 (NR_duplex_evo)

Source:	LG Electronics Inc.

Title: 	Discussion on Random Access procedure in SBFD

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 SBFD[1], it is agreed to support the Random Access procedure using the SBFD symbols.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - … - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - …

This document further discusses RAN2 aspects on RA procedure for SBFD.

2.	Discussion

Details on NW indication to select between SBFD RO and legacy RO

In RAN2#129 meeting [2], basic principle on RO type selection between the legacy RO and additional RO is agreed as follows:

For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. FFS whether RO type selection is performed before or after the RA type selection. FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

In detail, for CBRA case, it is agreed that the SBFD-aware UE selects the indicated RO for initial transmission, if the network indication is provided. Here, it was remained as an FFS on how to provide the network indication.

For the details on the signaling for network indication, followings can be considered:

- Option 1) RRC signaling, e.g., additional field in RACH-ConfigCommon IE to indicate RO type;

- Option 2) MAC CE using broadcast RNTI; or

- Option 3) PDCCH indication (e.g., DCI scrambled with P-RNTI, similar to time domain PRACH adaptation discussed in NES WI)

In our understanding, the main intention of the network indication is to dynamically adjust the number of UEs using additional RO and legacy RO. In this sense, following aspects should be considered:

- The network indication should be mainly transmitted via broadcast signaling. If it is transmitted via UE-specific signaling, it would cause a lot of overhead to adjust the number of UEs using the additional RO, whenever the network intends to change the load between the additional RO and legacy RO.

- The network indication should be able to change dynamically compared to the current SI change procedure. Otherwise, the benefit of the network indication is not clear, compared to updating SIB1 based on SI modification period.

- Meanwhile, it is not required for all SBFD-aware UEs to receive the network indication, e.g., it is possible to miss the network indication. If the network indication is not detected for UE, the SBFD-aware UE selects the RO type based on the RSRP threshold, which does not cause any significant problem to perform Random Access using additional RO. Therefore, no critical handling is needed for the missing case of the network indication from UE perspective.

With this understanding in mind, for Option 1, it could be easily implemented if the network indication is included in existing RACH configuration (e.g., in RACH-ConfigCommon IE). However, given that the RACH configuration is transmitted via SIB1 or RRCReconfiguration message, it may not be reasonable to dynamically adjust the load between the additional RO and legacy RO:

- If it is included in SIB1, the network indication cannot be changed dynamically, since the SIB1 content should be updated based on SI modification period. Therefore, the intention of introducing the network indication is unclear.

- If it is included in RRCReconfiguration message, it is transmitted via UE-specific signaling. Then, if the network intends to change the load between the additional RO and legacy RO, huge overload is expected in order to indicate the new network indication for each UE.

For Option 3, given that DCI-based adaptation for additional PRACH resources is defined in NES WI, similar method can be used to indicate whether to use additional RO or legacy RO (i.e., network indication by DCI with P-RNTI) as well, considering SBFD-aware UEs in idle/inactive mode and connected mode. However, it would cause additional RAN1 work, so it is doubtable whether it can be easily implemented until the competition of Rel-19, given that RAN1 has only two meetings left for Rel-19 SBFD.

In our view, it is slightly preferred to define in MAC CE which is transmitted using the broadcast RNTI (i.e., Option 2), which can be handled in RAN2 without any impact on other WG. In addition, the network indication may be dynamically changed based on the network decision. As mentioned above, it is not essential to ensure that all the SBFD-aware UEs should receive the network indication to select the RO type. In other words, for a SBFD-aware UE,

- If the SBFD-aware UE monitors the PDCCH at the time of the transmission of the indication transmission and receives the network indication, the SBFD-aware UE selects the RO type based on the network indication;

- Otherwise (including the missing case of the network indication), the SBFD-aware UE selects the RO type based on the RSRP.

Then, the load between the additional RO and legacy RO would be properly adjusted anyway.

Nevertheless, we are open to discuss on each option to define the detailed signaling on the network indication.

Proposal 1. For the detailed signaling on the network indication, select one of the followings:

- Option 1) RRC signaling, e.g., additional field in RACH-ConfigCommon IE to indicate RO type;

- Option 2) MAC CE using broadcast RNTI; or

- Option 3) PDCCH indication (e.g., DCI scrambled with P-RNTI, similar to time domain PRACH adaptation discussed in NES WI)

Meanwhile, if the network indication is received from the UE perspective, it should be clearly determined that the validity time of the network indication. Otherwise, once the network indication is detected at the UE side, it would never be transitioned to RO type selection based on the RSRP threshold, unless the UE receives the new network indication with release information. In this case, if the next network indication including information on release the previous indication is missed in the UE side, the SBFD-aware UE would select the RO type based on invalid network indication without considering channel quality, which may cause additional CLI impact. Therefore, it should be clearly determined on the validity duration of the network indication and how to cancel the previous network indication, in order to select the RO type based on RSRP.

In detail, following options can be considered:

- Option A) the received network indication is maintained until the SBFD-aware UE successfully receives the next network indication to release the previous indication

- Option B) the network indication further includes the validity time. After the validity time, the SBFD-aware UE selects the RO type based on the RSRP

- Option C) the network indication is valid within the predefined time window. After the time window, the previously received network indication is no longer valid.

Proposal 2. On the validity duration of the network indication, discussion on following options:

- Option A) the received network indication is maintained until the SBFD-aware UE successfully receives the next network indication to release the previous indication

- Option B) the network indication further includes the validity time. After the validity time, the SBFD-aware UE selects the RO type based on the RSRP

- Option C) the network indication is valid within the predefined time window. After the time window, the previously received network indication is no longer valid.

Condition for RA resource selection for initial attempt

When the network indication for RO type selection is provided for a SBFD-aware UE, additional RO should be prioritized over legacy RO as long as it is beneficial for RA latency. However, if the SBFD-aware UE performs RA procedure using SBFD RO in bad channel condition, CLI impact would occur as followings:

- When the UE in bad channel condition performs RA procedure using additional RO, the transmission of RA preamble may not be successful due to CLI impact caused by DL subband or DL slot on the network side.

- When the UE performs RA procedure using additional RO even though the UE is in cell edge, the UE would perform the power ramping procedure for re-attempt of the RA procedure, causing additional CLI impact to other UEs (e.g., to UEs in neighboring cell).

Therefore, in order to minimize the aforementioned CLI impact of using additional RO, it is suggested to use additional RO only if the channel condition is good enough to perform the RA procedure using additional RO as a baseline. In other words, if the SBFD RACH configuration is configured for SBFD-aware UE,

- additional RO is used when the channel condition is better than the threshold (i.e., when the RSRP is larger than the RSRP threshold)

- Otherwise, legacy RO is used for the current RA procedure.

Proposal 3. For the initial attempt of the RA procedure, if the network indication is not provided and RSRP threshold is configured, SBFD-aware UE prioritizes additional RACH occasion if the channel condition is better than the RSRP threshold as a baseline. Otherwise, SBFD-aware UE performs RA procedure using legacy RACH occasion.

Further, in RAN2#128 and RAN2#129 meeting, some companies suggested to use additional RO in bad channel condition, since the RA procedure using additional RO can be beneficial for coverage enhancement, e.g., when the long preamble format can be used for coverage enhancement using additional RO for RACH configuration Option 2. In our understanding, if the network configures the long preamble format for RACH configuration Option 2, it could be beneficial to use the additional RO if the UE is in bad channel condition, in order to enhance coverage using the long preamble format. In this sense, we are open to discuss on additional network indication within the RACH configuration to configure whether the additional RO is used in bad channel condition.

If the additional indication to use the additional RO bad channel condition is agreed and configured, SBFD-aware UE may select the RO type based on RSRP as follows:

- If the network configures that additional RO is used in bad channel condition,

  - Legacy RO is used when the channel condition is better than the threshold (i.e., when the RSRP is larger than the RSRP threshold)

  - Otherwise, additional RO is used for the initial transmission of the current RA procedure.

- Otherwise,

  - additional RO is used when the channel condition is better than the threshold (i.e., when the RSRP is larger than the RSRP threshold)

  - Otherwise, legacy RO is used for the initial transmission of the current RA procedure.

Proposal 4. Discuss on further network indication to indicate the additional RO is used when the RSRP of downlink pathloss reference is less than the threshold.

For the case of RSRP threshold is not configured

During the post e-mail discussion, it was discussed whether the network indication and the RSRP threshold may not be present and the corresponding UE behavior for this case. Specifically, following two Options can be considered:

- Option a) The network mandatorily configures the RSRP threshold, if the network indication is not provided and the additional RO is configured.

- Option b) The network may not configure the RSRP threshold, and the SBFD-aware UE always selects the additional RO in this case.

In our view, both are feasible, but Option b is slightly preferred for network flexibility to indicate prioritize additional RO using the semi-static signaling, i.e., within the RACH configuration.

Proposal 5. When the RSRP threshold is not configured, the SBFD-aware UE always selects the additional RO in this case unless the network indication is provided.

RA resource selection procedure for SBFD RA procedure

According to current MAC procedure [3], the Random Access resource selection is as follows:

- Step 1) Selection of UL carrier between NUL/SUL: If SUL is configured in addition to NUL, the UL carrier selection is done using downlink RSRP.

- Step 2) Selection of a set of Random Access resources (i.e., RACH partition) associated with feature

  - Determine feature applicability (e.g., Msg1 repetition + repetition number)

  - Determine availability for each set of RA resources

  - Set of RA resource selection

- Step 3) Selection of RA type between 4-step RA and 2-step RA within the selected RACH partition

- Step 4) Selection of SSB

- …

In RAN2#129 meeting [2], it was discussed on which step RO type should be selected, and there are two different views on this:

FFS whether RO type selection is performed before or after the RA type selection.

In details there are two ways to determine in which step RO type can be selected:

- Alt 1: Before the selection of a set of Random Access resources (i.e., between Step 1 and Step 2)

- Alt 2: After the selection of a set of Random Access resources and the RA type selection (i.e., between Step 3 and Step 4)

Meanwhile, in RAN1#119 meeting [4], it is agreed to configure the separated RSRP threshold to determine Msg1 repetition number for SBFD RO, for RACH configuration Option 1 and RACH configuration Option 2.

Agreement For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - msg1-RepetitionNum is configured in RACH-ConfigCommon in current specification, i.e., for RACH configuration Option 2, msg1-RepetitionNum can already be separately configured for legacy-ROs and additional-ROs. Agreement For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - Same msg1-RepetitionNum is used for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

In other words, whether to perform Msg1 repetition for this Random Access procedure and corresponding Msg1 Repetition Number should be determined based on different condition for SBFD RO and legacy RO.

Given that whether to perform the Msg1 repetition for this Random Access procedure and the corresponding Msg1 repetition number is determined during the RACH partition selection (i.e., Step 2), if the RO type is selected after the RACH partition selection, further UE complexity is expected for the RACH partitioning framework, since the UE needs to select two sets of Random Access resources (i.e., two RACH partition) during the RACH partition selection, i.e., one RACH partition for each RO type.

For example, assume that RACH partition is configured as follows:

- In legacy RO,

  - RACH partition A1: common RACH partition (i.e., RACH partition not associated with any feature)

  - RACH partition A2: Msg1 repetition with repetition number 2 + NSAG1

  - RACH partition A3: Msg1 repetition with repetition number 4 + NSAG1

- In additional RO,

  - RACH partition B1: common RACH partition (i.e., RACH partition not associated with any feature)

  - RACH partition B2: Msg1 repetition with repetition number 2

In addition, assume that the RSRP thresholds to select RO type and to select Msg1 repetition for each RO type as follows:

- RSRP threshold to select additional RO (additional RO is selected in good channel quality): 25

- RSRP threshold to perform Msg1 repetition number 4 in legacy RO: 10

- RSRP threshold to perform Msg1 repetition number 2 in legacy RO: 20

- RSRP threshold to perform Msg1 repetition number 2 in addition RO: 30

Then, for Alt 2, if an SBFD-aware UE initiates a Random Access procedure for NSAG1 (i.e. for slicing) and the RSRP of the downlink pathloss reference is 27, the RACH partition selection would be performed as follows:

- Step 1) Selection of UL carrier between NUL/SUL: If SUL is configured in addition to NUL, the UL carrier selection is done using downlink RSRP.

- Step 2) Selection of a set of Random Access resources (i.e., RACH partition) associated with feature

- For legacy RO, RACH partition A1(i.e., common RACH partition) is selected

- For additional RO, RACH partition B2 (i.e., Msg1 repetition with repetition number 2) is selected

- Step 3) Selection of RA type between 4-step RA and 2-step RA within the selected RACH partition

- ? Further discussion is needed how to select the RA type with two selected RACH partitions

  - Step 3A) Select the RO type, between the additional RO and legacy RO, i.e., additional RO is selected

  - Step 3B) Determine the one RACH partition within the selected RO type, i.e., RACH partition B2

Therefore, with the separated threshold to determine Msg1 repetition number for additional RO and legacy RO, if RO type should be selected after the RACH partition selection, huge impact on the existing RACH partitioning framework is expected. In our understanding, during the Rel-17 RACH partitioning discussion, it is agreed as a basic principle to select only one partition, and the same principle is kept until Rel-18. That’s why the RA type selection between the 2-step RA and the 4-step RA is performed within the selected RACH partition as follows [3]:

1> else if the BWP selected for Random Access procedure is configured with both 2-step and 4-step RA type Random Access Resources within the selected set of Random Access resources (as specified in clause 5.1.1b) and the RSRP of the downlink pathloss reference is above msgA-RSRP-Threshold; or 1> if the BWP selected for Random Access procedure is only configured with 2-step RA type Random Access resources within the selected set of Random Access resources according to clause 5.1.1b; or 1> if the Random Access procedure was initiated for reconfiguration with sync not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3 and if the contention-free Random Access Resources for 2-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure: 2> set the RA_TYPE to 2-stepRA. 1> else: 2> set the RA_TYPE to 4-stepRA.

Given that the existing RACH partitioning framework is already complicated, further complexity on the Random Access procedure with RACH partitioning framework should be avoided as much as possible. Therefore, in order to apply the appropriate threshold to determine Msg1 repetition number within the existing RACH partitioning frame work, RO type should be selected before RACH partition selection (i.e. Alt 1).

With Alt 1, the Random Access procedure for the above example would be performed as follows:

- Step 1) Selection of UL carrier between NUL/SUL: If SUL is configured in addition to NUL, the UL carrier selection is done using downlink RSRP.

- Step 1A) Select the RO type, between the additional RO and legacy RO, i.e., additional RO is selected

- Step 2) Selection of a set of Random Access resources (i.e., RACH partition) associated with feature, within the selected RO type. Determination of Msg1 repetition number if performed based on the separated RSRP threshold for additional RO.

  - RACH partition B2 (i.e., Msg1 repetition with repetition number 2) is selected

- Step 3) Selection of RA type between 4-step RA and 2-step RA within the selected RACH partition and RO type, i.e., 4-step RA is selected (given that additional RO is supported only for 4-step RA)

Proposal 6. The SBFD-aware UE selects between additional RO and legacy RO before selecting a set of Random Access resources.

Meanwhile, according to the current signaling structure, the RACH partition associated with feature can be differently configured in SBFD RO and legacy RO. Specifically,

- For RACH configuration Option 1, it is possible that SBFD RO may not be configured in some RACH configurations, e.g., in RACH-ConfigCommon included in AdditionalRACH-Config IE.

- For RACH configuration Option 2, due to separated RACH configuration, the preamble partitioning can be different from the legacy RACH configuration.

In other words, it is possible that the RACH partition associated with a feature is configured in legacy RO, while RACH partition associated with the feature is not configured in SBFD RO.

However, based on proposal 6, the selection between the additional RO and legacy RO is performed prior to the selection of RACH partition, in order to apply the appropriate RSRP threshold to determine Msg1 repetition number. In other words, the selection of RACH partition is performed within the selected RO type.

In our understanding, RA procedure using the SBFD symbol is to perform the Random Access procedure with better quality, e.g., for latency reduction, collision handling, and coverage enhancement. Therefore, if the follow-up procedure is different for specific feature, it may not be reasonable to always select additional RO based on the condition (e.g., RSRP), prior to select the feature/feature combination.

For example, the network may configure RACH partition for RA-SDT as follows, due to RA resource limitation in additional RO:

- In legacy RO,

  - RACH partition 1: common RACH partition (i.e., RACH partition not associated with any feature)

  - RACH partition 2: RA-SDT (i.e., RA resource to perform RA-SDT)

- In additional RO,

  - RACH partition 3: common RACH partition

Then, if the UE selects the RO type between the additional RO and legacy RO prior to the RACH partition selection, it is possible that SBFD-aware UE selects common RACH partition in additional RO (i.e., RACH partition 3), even though the SBFD-aware UE may selects the RACH partition for RA-SDT in legacy RO (i.e., RACH partition 2) as well. Then, the SBFD-aware UE would unnecessarily perform legacy RRC resume procedure (i.e., transition to RRC_CONNECTED state) even though the SBFD-aware UE is in good channel condition to perform RA-SDT procedure, causing additional delay to transmit SDT data.

Therefore, it should be discussed whether the RA procedure associated with a specific feature using legacy RO can be prioritized over the legacy RA procedure using the additional RO. For example, the network indication to select between the additional RO and legacy RO can be provided in the finer granularity, e.g. associated with feature/feature combination.

Proposal 7. Discuss whether the set of Random Access resource associated with a specific feature within legacy RO can be prioritized over the set of Random Access resource not associated with any feature within additional RO, even though the SBFD-aware UE meets the RSRP condition to select additional RO.

Fallback procedure for re-attempt

For re-attempt in one random access procedure, it is agreed to support the fallback procedure from SBFD RO to legacy RO, after the certain number of RA attempt using SBFD RO:

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

On the other hand, it was remained as FFS on whether the SBFD-aware UE is allowed to switch from legacy RO to SBFD RO [2]:

FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

However, the fallback procedure from legacy RO to additional RO does not ensure successful transmission of RA preamble, even in the case of long preamble format is used in additional RO, due to additional CLI impact anyway. In addition, given that the RO type is selected based on the channel condition in the initial attempt, supporting fallbacks for all cases are not essential.

Rather, if the fallback from legacy RO to additional RO is supported, it would cause additional discussion in RAN1 on power ramping for the fallback cases [5]:

Agreement Consider the following alternatives about power ramping when a SBFD-aware UE switches from additional-ROs to legacy-ROs and from legacy-ROs to additional-ROs (if supported) in PRACH transmission re-attempt in one RACH procedure for RACH configuration Option 1 and Option 2. - Alt-1: Increase the power ramping counter. - FFS whether to introduce a power offset to compensate the power ramping difference for RACH configuration Option 2. - Alt-2: Reset the power ramping. - Note: different alternatives can be applied for different RACH configuration options.

Specifically, if the fallback from the legacy RO to additional RO is supported, separated discussion for power ramping procedure on the fallback case for each direction is needed. In addition, it is possible that the different principle can be applied the power ramping procedure for each direction, e.g.,

- If the SBFD-aware UE switches from additional ROs to legacy ROs, the SBFD-aware UE does not need to reset the power ramping counter, since it does not cause additional CLI impact (i.e., similar to 2-step RA)

- If the SBFD-aware UE switches from legacy ROs to additional ROs and from legacy ROs, further discussion may be needed to reset the power ramping counter, considering that additional CLI impact may be occurred in additional RO anyway.

Then, the Random Access procedure would be very complicated, e.g., defining bi-directional fallback and applying different principle to decide power ramping value for the fallback case in each direction.

In addition, if the fallback from the legacy RO to additional RO is supported, further discussion is needed on whether the sequential fallback considering the fallback from 2-step RA to 4-step RA (i.e., 2-step RA using legacy RO  4-step RA using legacy RO  4-step RA using additional RO) is also supported as well.

Given that there only several meetings are left to complete SBFD WI (2 meetings for RAN1 and 3 meetings for RAN2), essential issues to support the Random Access procedure using SBFD symbol should be prioritized. If any further enhancements are adopted, it is doubtable whether the SBFD feature can be properly implemented without any critical issues, i.e., a lot of corrections are expected.

In this sense, given that current Random Access is already complicated, any additional complexity should be avoided by adding the bi-directional fallback procedure between the additional RO and legacy RO.

Proposal 8. Do not support switching from legacy RACH occasions to SBFD RACH occasions for SBFD-aware UE.

Signaling RACH configuration Option 2

According to the RAN1 parameter list [6], for RACH configuration Option 2, it is agreed to define additional RACH configuration under the BWP-UplinkCommon IE (purple-colored in Figure 1).

Figure 1.

On the other hand, in RACH partitioning framework defined in RAN2, there are additional RACH configuration for feature/feature combination (i.e., additionalRACH-Config IE). Given that feature combination using the additional RO can be supported (e.g., Msg1 repetition), it could be valuable to discuss whether the additional RACH configuration for SBFD (i.e., sbfd-RACHDualConfig IE) can also be configured within the separated RACH configuration for feature/feature combination (i.e., additionalRACH-Config IE), as blue-colored in Figure 1.

Proposal 9. Discuss whether the additional RACH configuration for SBFD (i.e., sbfd-RACHDualConfig IE) can also be configured within the separated RACH configuration for feature/feature combination (i.e., additionalRACH-Config IE).

Whether to support Msg1-based SI request

In post e-mail discussion, it seems that there are different views on whether Msg1-based SI request using the additional RO should be supported:

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

In detail, some companies [7][8] proposed to simply indicate the RO type within the Msg1-based SI request configuration (e.g., in SI-RequestResources), similar to other CFRA cases. However, it should be noted that the Random Access resource for Msg1-based SI request is common for all UEs in the cell, so it is not the CFRA nor CBRA procedure from MAC perspective. In the current MAC specification [3], Msg1-based SI request case is described as follows, which is different from other CFRA cases:

1> else if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]); and 1> if the Random Access Resources for SI request have been explicitly provided by RRC: (…omitted)

Therefore, it could be risky to dictate to use a specific RO type for all SBFD-aware UE, regardless of its channel condition. In other words, it a SBFD-aware UE is in cell-edge area, performing Msg1-based SI request using the additional RO would not be helpful for successful Random Access procedure, due to unexpected CLI impact in each UE. Note that it is different from other CFRA case, since the network may indicate the proper RO type for the specific UE for other CFRA cases, based on the channel condition of that UE.

On the other hand, if the SBFD-aware UE selects between additional RO and legacy RO based on RSRP (similar to CBRA cases), further discussion and large spec impact is expected in RRC/MAC specification in order to check the condition to select the RO type in various Msg1-based SI request cases, which causing additional workload for SBFD WI. In current specification, Msg1-based SI request is already defined for various cases, including NUL/SUL, Redcap, Positioning SI, and Msg1-based SI request with Msg1 repetition. In addition, there are combinational cases for Msg1-based SI request, for example, Msg1-based SI request with Msg1 repetition can be performed for RedCap UE on Positioning SI (i.e., Msg1 repetition + RedCap + Positioning SI). In this sense, there are already 12 separated cases to determine and initiate Msg1-based SI request in current RRC specification.

Therefore, if the Msg1-based SI request is supported using SBFD symbol, at least following aspects should be discussed:

- whether the Msg1-based SI request configuration (e.g., si-RequestConfigSBFD) should be separately defined in SIB;

- whether SBFD operation can be supported with other features (e.g., for (e)RedCap UE, SUL, Msg1 repetition, etc), in order to decide whether the separated procedure should be specified for all of the aforementioned 12 cases; and

- which condition should be checked first for Msg1-based SI request, between SBFD and other features. For example, if Msg1-based SI request using SBFD symbol and Msg1-based SI request with Msg1 repetition is supported as well,

  - Case 1: Msg1 repetition condition is checked first. In other words, if Msg1-based SI request with Msg1 repetition is configured and the corresponding condition is met, UE performs Msg1-based SI request with Msg1 repetition even though Msg1-based SI request using SBFD symbol is configured.

  - Case 2: SBFD condition is checked first. In other words, if Msg1-based SI request using SBFD symbol is configured and the corresponding condition is met, UE performs Msg1-based SI request using SBFD symbol even though Msg1-based SI request with Msg1 repetition is configured.

In our understanding, for Msg1-based SI request, the benefits of supporting RA procedure using SBFD symbol may not be essential. Specifically, it is not super urgent to request the SI message, so latency reduction for RA procedure on Msg1-based SI request is not needed. In addition, there is no collision issue on Msg1-based SI request, since any UE can perform Msg1-based SI request, and the network does not need to identify the UE performing the Msg1-based SI request. Last but not least, for coverage issue, Msg1-based SI request with Msg1 repetition is already supported in order to ensure the coverage of RA procedure for Msg1-based SI request, based on long discussion in Rel-18 CE. Thus, additional feature for coverage enhancement is not needed for Msg1-based SI request.

Therefore, given that Msg1-based SI request using SBFD symbol causes lots of further discussion with limited benefits, it should not be further discussed to define additional procedure for Msg1-based SI request using additional RO, given that only three meetings left for SBFD WI.

Proposal 10. Msg1-based SI request using the additional RO is not supported.

3.	Conclusion

In this contribution, it is discussed about RAN2 aspects on Random Access operation for SBFD. This document includes following proposals.

Proposal 1. For the detailed signaling on the network indication, select one of the followings:

- Option 1) RRC signaling, e.g., additional field in RACH-ConfigCommon IE to indicate RO type;

- Option 2) MAC CE using broadcast RNTI; or

- Option 3) PDCCH indication (e.g., DCI scrambled with P-RNTI, similar to time domain PRACH adaptation discussed in NES WI)

Proposal 2. On the validity duration of the network indication, discussion on following options:

- Option A) the received network indication is maintained until the SBFD-aware UE successfully receives the next network indication to release the previous indication

- Option B) the network indication further includes the validity time. After the validity time, the SBFD-aware UE selects the RO type based on the RSRP

- Option C) the network indication is valid within the predefined time window. After the time window, the previously received network indication is no longer valid.

Proposal 3. For the initial attempt of the RA procedure, if the network indication is not provided and RSRP threshold is configured, SBFD-aware UE prioritizes additional RACH occasion if the channel condition is better than the RSRP threshold as a baseline. Otherwise, SBFD-aware UE performs RA procedure using legacy RACH occasion.

Proposal 4. Discuss on further network indication to indicate the additional RO is used when the RSRP of downlink pathloss reference is less than the threshold.

Proposal 5. When the RSRP threshold is not configured, the SBFD-aware UE always selects the additional RO in this case unless the network indication is provided.

Proposal 6. The SBFD-aware UE selects between additional RO and legacy RO before selecting a set of Random Access resources.

Proposal 7. Discuss whether the set of Random Access resource associated with a specific feature within legacy RO can be prioritized over the set of Random Access resource not associated with any feature within additional RO, even though the SBFD-aware UE meets the RSRP condition to select additional RO.

Proposal 8. Do not support switching from legacy RACH occasions to SBFD RACH occasions for SBFD-aware UE.

Proposal 9. Discuss whether the additional RACH configuration for SBFD (i.e., sbfd-RACHDualConfig IE) can also be configured within the separated RACH configuration for feature/feature combination (i.e., additionalRACH-Config IE).

Proposal 10. Msg1-based SI request using the additional RO is not supported.

4.	Reference

[1] RP‑241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2] Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece, 17-21 February, 2025

[3] TS 38.321 v18.5.0

[4] Report of 3GPP TSG RAN WG1 #119

[5] Report of 3GPP TSG RAN WG1 #120

[6] R2-2501724 LS on Rel-19 higher layers parameters list Post RAN1#120

[7] R2-2500274 Discussion on Random Access in SBFD Symbols, CATT

[8] R2-2500884 SBFD RA aspects, Ericsson