3GPP TSG-RAN WG2#129-bis	     					      R2-2502854

Wuhan, China, April 7 – 11, 2025

Agenda item:	8.1.2.2

Source: 	Kyocera

Title: 	Remaining Issues on LCM for UE-sided model in Beam Management use case

Document for:	Discussion

Introduction

In this contribution, the remaining issues regarding LCM of the UE-sided model for the Beam Management use case, configuration for Option B, cause reporting for applicability change, monitoring configuration for periodic, aperiodic, and semi-persistent CSI reporting, and UAI prohibit timer are discussed.

Discussion

Configuration for Option B (i.e., inference related parameters for applicability report only)

This section examines whether Option B should be included in CSI-ReportConfig or OtherConfig.

First, the RAN1 Reply LS [1] includes the following description of the Applicable Functionality:

Reply LS from RAN1 (R1-2410898) […] - The concept/terminology “functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters […] - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction

At RAN2#129, the following agreements were made [2]

RAN2#129 Agreements 1. Inference configuration/parameters can be signaled in Step 3 and/or Inference configuration can be signaled in Step 5 (i.e., Option A and Option B from RAN1). 2. The full inference configuration is sent in CSI-ReportConfig. 3. Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. 4. FFS signaling details for Option B (e.g., whether it is signaled in CSI-ReportConfig or OtherConfig).

At RAN2#129, the following agreements were made [2]. The 2nd agreement states that the full inference configuration is sent in CSI-ReportConfig, which is in-line with the intention of Option A, i.e., ‘A)’ in Reply LS from RAN1. On the other hand, regarding Option B, i.e., ‘B)’ in Reply LS from RAN1, as in the 4th agreement, it’s left as FFS whether the configuration for applicability report is signalled in CSI-ReportConfig or OtherConfig.

The nature of the configuration for Option A and Option B can be characterized as follows:

- Option A provides configuration for the inference report by reusing the CSI framework; therefore, it corresponds to Layer 1 operation.

- Option B provides configuration for the applicability report, which can be considered as Layer 3 operation.

Given this structural distinction between Layer 1 and Layer 3 configurations described above, it is considered reasonable and natural for Option B configuration to be carried in OtherConfig.

CSI-ReportConfig is defined for Layer 1 configuration, so it’s not preferable to include Layer 3 configuration like Option B since it violates the principle of separation between protocol layers.

Each layer has clearly defined responsibilities. Mixing Layer 1 and Layer 3 configuration within the same structure may obscure those responsibilities and reduce architectural clarity and consistency.

From an implementation standpoint, including Layer 3 configuration in Layer 1 IE would increase parsing and processing complexity for both the UE and the gNB.

Furthermore, keeping Layer 3 configuration within OtherConfig supports future extensibility, easier specification maintenance and better backward compatibility.

Based on the above, including Option B configuration in OtherConfig is justified not only by the functional roles of each layer, but also by the practical benefits in implementation and long-term specification management.

RAN2 should agree that the applicability report-specific configuration, i.e., Option B, is signaled in OtherConfig.

Cause Reporting for Applicability Change

In RAN2#129, it was agreed to explicitly report applicability/inapplicability of AI/ML functionalities in the initial report, and subsequently only report changes in applicability. However, whether to explicitly report the cause of these changes remains as FFS [2]. This section discusses the optional reporting of explicit causes and highlights the need for a discussion on whether each specific cause for change is worth reporting.

RAN2#129 Agreement Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause

One of the main technical concerns in reporting explicit causes is the balance between potential benefits and signalling overhead. Multiple NW vendors have indicated support for cause reporting, since the additional information received when the UE applicability status changes may lead to performance improvements. Specifically, providing causes such as significant variations in prediction accuracy or UE-internal condition changes (e.g., due to UE mobility) may enable the NW to more effectively optimize or adapt inference configurations.

However, it's essential to evaluate whether each specific cause is truly worth reporting. For example, cause reporting associated with RRC signaling will be used primarily for immediate decisions by the network. Therefore, cause reporting should be considered only if it offers clear value for real-time network decisions.

It may be beneficial to adopt an optional approach for cause reporting. By keeping it optional, networks that prefer reduced complexity and signalling overhead can simply choose not to activate or utilize the feature.

However, given that the detailed conditions and specific parameters for cause reporting are currently unclear in both RAN1 and RAN2. These may depend on additional factors on either the NW-side or the UE-side, and should therefore remain FFS for further consideration.

In light of the ongoing discussions in RAN1 and RAN2 and the need for flexibility, the approach to report explicit causes should be considered as an optional feature.

RAN2 should support optional reporting of explicit causes for applicability/inapplicability alongside the applicability indication. The detailed conditions and exact parameters regarding explicit causes shall remain FFS pending further discussions in RAN1 and RAN2.

Monitoring Configuration for Periodic, Aperiodic, and Semi-persistent CSI Reporting

At the previous RAN2#129 meeting, the following agreement was reached [2]:

RAN2#129 Agreement […] 2. The full inference configuration is sent in CSI-ReportConfig.

This section introduces the concept of monitoring configuration.

Here, "monitoring" refers to evaluating the accuracy of AI/ML models by comparing their predicted results (e.g., CRI as predicted beam ID, predicted RSRP) with the actual measurements.

For the UE to perform monitoring, it is necessary to simultaneously execute both AI/ML-based inference and actual measurements. AI/ML-based inference can be performed without transmitting actual beams, as it operates on existing input data. In contrast, monitoring requires the network to transmit the beams at specific timings, so the UE can perform real measurements and compare them to the inference output. Therefore, the network needs to configure the monitoring timing through signalling to the UE.

Since AI/ML inference for beam management follows the existing CSI reporting framework, three reporting types—Periodic, Aperiodic, and Semi-persistent—can be considered. Several configuration options can be considered for monitoring timings aligned with the CSI reporting types, such as periodic configurations or event-triggered configurations.

However, it is important to note that the specific configuration of monitoring timings and the requirements for performance monitoring reports should be first discussed and agreed upon by RAN1.These performance monitoring requirements should first be defined by RAN1. Based on this, RAN2 can further discuss how monitoring for Periodic, Aperiodic, and Semi-persistent CSI reporting can be configured.

RAN2 should wait for RAN1's conclusion regarding the requirements for performance monitoring reports.

UAI Prohibit Timer

In RAN2#127-bis, RAN2 reached the following agreement on the mechanism for reporting functionality applicability [3].

Agreements (RAN2#127-bis) UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signaled. FFS on the applicability reporting content.

To date, UAI-related functionalities (e.g., DelayBudgetReport, OverheatingAssistance, etc.) have been configured with a Prohibit Timer, which has been shown to effectively suppress unnecessary reporting over short periods and reduce network load. This proposal considers the possibility of applying a similar Prohibit Timer to Applicable Functionality Reporting.

The advantages and disadvantages of introducing a Prohibit Timer are described as follows:

Advantages and Disadvantages of Applying a Prohibit Timer for UAI Reporting

- Advantages:

By suppressing repeated UAI reporting over short durations, the Prohibit Timer can reduce network load and enable efficient signal transmission.

Reusing a mechanism already adopted in other UAI-related features can standardize behavior and reduce operational complexity.

- Disadvantages:

In scenarios where rapid response to environmental or conditional changes is required, the introduction of the Prohibit Timer may cause notification delays, potentially affecting the applicability of models or the adaptability of the system.

If state transitions are infrequent, the effectiveness of introducing the Timer may be limited, posing a risk of reduced flexibility.

In cases requiring fine-grained control, multiple Prohibit Timers with different scopes may be needed, raising concerns about increased operational complexity.

It may be difficult for the network to determine an appropriate duration for the Prohibit timer. This process is not always straightforward, especially when some of the factors influencing the decision are dependent on the UE.

Based on the above pros and cons, the main disadvantage of introducing the Prohibit Timer appears to be its inability to handle frequent changes between applicable and not-applicable states. However, the legacy UAI Prohibit Timer allows the Timer value to be set within a range of, for example, DelayBudgetReportingProhibitTimer: 0, 0.4, 0.8, 1.6, 3, 6, 12, 30 seconds, enabling appropriate values to be selected even in scenarios where responsiveness is required. Based on this, it can be concluded that there are no significant issues associated with the introduction of the Prohibit Timer. Therefore, Prohibit Timer should be supported for Applicable Functionality Reporting.

Next, we consider the granularity of the Prohibit Timers.

Granularity and Responsiveness of the Prohibit Timer

The feasibility of introducing a Prohibit Timer in Applicable Functionality Reporting heavily depends on the level of granularity at which the Timer is applied and the associated requirements for responsiveness.

- Coarse Granularity (e.g., at the level of entire use cases or sub-use cases):
In this case, the Timer is set for larger functional units, resulting in lower requirements for responsiveness. The introduction of a Prohibit Timer is expected to be effective.

- Fine Granularity (e.g., at the level of individual applicable functions, such as specific report configurations, resource configurations, or resource sets):
In this case, frequent state changes may occur, and ensuring timely responses may become essential. In such scenarios, it would be necessary to configure the Timer with shorter durations or finer time steps.

Discussion on the Granularity of Applicable Functionality in RAN1

As discussed in Section 2.1.1 of this document, the granularity of Applicable Functionality can be categorized as follows:

- Based on AssociatedID only

- Based on AssociatedID along with related information

- Based on related information only

In this sense, small changes in Applicable Functionality may require reporting of applicable functionality at a fine level of granularity.

However, if multiple Timers are configured, for example, with separate timers assigned per AssociatedID, this may improve responsiveness, but it is likely to increase the complexity of the specifications and add overhead.

On the other hand, defining Timers at the use case level may result in too coarse a granularity, making control difficult. Therefore, defining Timers at the sub-use case level is a possible approach. Therefore, it is recommended that this be addressed in Stage-3.

RAN2 should introduce a Prohibit Timer to manage reporting of changes in applicable functionality, similar to the mechanism used in legacy UAI configurations.

The number of Timers and their configuration details should be determined in Stage-3, with the expectation that flexible Timer values may be needed in scenarios requiring high responsiveness.

Conclusion

This contribution discusses the issues in terms of LCM for UE-sided model in Beam Management use case. RAN2 is kindly asked to take into account the proposals below:

Proposal 1	RAN2 should agree that the applicability report-specific configuration, i.e., Option B, is signaled in OtherConfig.

Proposal 2	RAN2 should support optional reporting of explicit causes for applicability/inapplicability alongside the applicability indication. The detailed conditions and exact parameters regarding explicit causes shall remain FFS pending further discussions in RAN1 and RAN2.

Proposal 3	RAN2 should wait for RAN1's conclusion regarding the requirements for performance monitoring reports.

Proposal 4	RAN2 should introduce a Prohibit Timer to manage reporting of changes in applicable functionality, similar to the mechanism used in legacy UAI configurations.

Proposal 5	The number of Timers and their configuration details should be determined in Stage-3, with the expectation that flexible Timer values may be needed in scenarios requiring high responsiveness.

References

- “Reply LS on applicable functionality reporting for beam management UE-sided model” (Samsung) (R1-2410898)

- “RAN2 Chair’s Notes”, (InterDigital), RAN2#129

- “RAN2 Chair’s Notes”, (InterDigital), RAN2#127-bis